ETSI 3GPP TSG CN1/SMG3 WPA 	Tdoc N1-99489

Sophia Antipolis, France

May 31, June 01-04, 1999

| CHANGE REQUEST No : |  |  |  |  |  | A685 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Alcatel | Date: | 990526 |
| --- | --- | --- | --- |

| Subject: | TMSI requested by the MSC trough the Gs interface |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| Marked with an X) |  |  |  | UMTS |  |

| Reason for change: | In GSM 04.08, it is possible for the MSC/VLR to request the TMSI from the MS in the Identity Request message (sections 9.2.10, 10.5.3.4). In addition, in GSM 03.60, section 6.3.5 (MS information procedure), it is written that: - "the VLR may perform the MS Information procedure via the SGSN" and that - "If the information requested is MS identity information (e.g., IMEI) that is not known by the SGSN but is known by the MS, then the SGSN shall interrogate the MS in a similar manner to that described in subclause "Identity Check Procedures"". In this case, the SGSN sends an Identity Request message to the MS. Therefore, this CR adds the possibility for the SGSN to request the TMSI to the MS. This can be used by the MSC/VLR to request and get the TMSI through the Gs interface |
| --- | --- |

| Clauses affected: | 10.5.5.9 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications | x | ® List of CRs: | CR Axxx on GSM 09.18 |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | This CR is linked to the CR Axxx on GSM 09.18 which introduces the possibility for the MSC/VLR to request the TMSI through the Gs interface. The section 10.5.1.4 used in the CR is the one of the GSM 04.08 v6.3.0 modified by the CR A476 rev2 (Tdoc N1-99356, TSG N1/SMG3 WPA Sophia 26-29 April 1999) |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 9.4	GPRS Mobility Management Messages

### 9.4.12	Identity request

This message is sent by the network to the MS to request submission of the MS identity according to the specified identity type. See table 9.4.12/GSM 04.08.

Message type:	IDENTITY REQUEST

Significance:		dual

Direction:			network to MS

Table 9.4.12/GSM 04.08: IDENTITY REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 10.2 | M | V | 1/2 |
|  | Skip indicator | Skip indicator 10.3.1 | M | V | 1/2 |
|  | Identity request message identity | Message type 10.4 | M | V | 1 |
|  | Identity type | Identity type 2 10.5.5.9 | M | V | 1/2 |
|  | Force to standby | Force to standby 10.5.5.7 | M | V | 1/2 |

### 9.4.13	Identity response

This message is sent by the MS to the network in response to an identity request message providing the requested identity. See table 9.4.13/GSM 04.08.

Message type:	IDENTITY RESPONSE

Significance:		dual

Direction:			MS to network

Table 9.4.13/GSM 04.08: IDENTITY RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 10.2 | M | V | 1/2 |
|  | Skip indicator | Skip indicator 10.3.1 | M | V | 1/2 |
|  | Identity response message identity | Message type 10.4 | M | V | 1 |
|  | Mobile identity | Mobile identity 10.5.1.4 | M | LV | 6 - 11 |

#### 10.5.1.4	Mobile Identity

The purpose of the Mobile Identity information element is to provide either the international mobile subscriber identity, IMSI, the temporary mobile subscriber identity, TMSI/P-TMSI, the international mobile equipment identity, IMEI or the international mobile equipment identity together with the software version number, IMEISV.

The IMSI shall not exceed 15 digits, the TMSI/P-TMSI is 4 octets long, and the IMEI is composed of 15 digits, the IMEISV is 16 digits (see GSM 03.03).

For the GPRS attach procedure and when paging for GPRS services the mobile identity type shall be selected with the following priority:

1- P-TMSI: The P-TMSI shall be used if it is available.

2- IMSI: The IMSI shall be used in cases where no P-TMSI is available.

For all other transactions except emergency call establishment, emergency call re-establishment, mobile terminated call establishment, the identification procedure, the GMM identification procedure , the GMM authentication and ciphering procedure and the ciphering mode setting procedure, the mobile station and the network shall select the mobile identity type with the following priority:

1- TMSI: The TMSI shall be used if it is available.

2- IMSI: The IMSI shall be used in cases where no TMSI is available.

For mobile terminated call establishment the mobile station shall select the same mobile identity type as received from the network in the PAGING REQUEST message.

For emergency call establishment and re-establishment the mobile station shall select the mobile identity type with the following priority:

1- TMSI: The TMSI shall be used if it is available.

2- IMSI: The IMSI shall be used in cases where no TMSI is available.

3- IMEI: The IMEI shall be used in cases where no SIM is available or the SIM is considered as not valid by the mobile station or no IMSI or TMSI is available.

In the identification procedure and in the GMM identification procedure the mobile station shall select the mobile identity type which was requested by the network.

In the ciphering mode setting procedure and in the GMM authentication and ciphering procedure the mobile shall select the IMEISV.

The Mobile Identity information element is coded as shown in figure 10.5.4/GSM 04.08 and table 10.5.4/GSM 04.08.

The Mobile Identity is a type 4 information element with a minimum length of 3 octet and 11 octets length maximal. Further restriction on the length may be applied, e.g. number plans.

8     7     6     5     4     3     2     1

+-----------------------------------------------+

ª     ª       Mobile Identity IEI               ª octet 1

+-----------------------------------------------ª

ª                                               ª

ª     Length of mobile identity contents        ª octet 2

+-----------------------------------------------ª

ª                       ªodd/ ª                 ª

ª   Identity digit 1    ªeven ª Type of identityª octet 3

ª                       ªindicª                 ª

+-----------------------+-----------------------ª

ª                       ª                       ª

ª   Identity digit p+1  ª   Identity digit p    ª octet 4*

+-----------------------------------------------+

Figure 10.5.4/GSM 04.08 Mobile Identity information element

Table 10.5.4/GSM 04.08: Mobile Identity information element

+---------------------------------------------------------------+

ª Type of identity (octet 3)                                    ª

ª Bits                                                          ª

ª 3 2 1                                                         ª

ª 0 0 1    IMSI                                                 ª

ª 0 1 0    IMEI                                                 ª

ª 0 1 1    IMEISV                                               ª

ª 1 0 0    TMSI/P-TMSI                                          ª

ª 0 0 0    No Identity  note 1)                                 ª

ª                                                               ª

ª All other values are reserved.                                ª

ª                                                               ª

ª Odd/even indication (octet 3)                                 ª

ª Bit                                                           ª

ª 4                                                             ª

ª 0        even number of identity digits and also when         ª

ª          the TMSI/P-TMSI is used                              ª

ª 1        odd number of identity digits                        ª

ª                                                               ª

ª Identity digits (octet 3 etc)                                 ª

ª For the IMSI, IMEI and IMEISV this field is coded using       ª

ª BCD  coding. If the number of identity digits is  even        ª

ª then  bits  5 to 8 of the last octet shall  be  filled        ª

ª with an end mark coded as "1111".                             ª

ª                                                               ª

ª If the mobile identity is the TMSI/P-TMSI then bits 5 to 8 ofª

ª octet 3 are coded as "1111" and bit 8 of octet 4 is the       ª

ª most  significant bit and bit 1 of the last octet  the        ª

ª least significant bit. The coding of the TMSI/P-TMSI is  left ª

ª open for each administration.                                 ª

+---------------------------------------------------------------+

NOTE 1:	This can be used in the case when a fill paging message without any valid identity has to be sent on the paging subchannel.

#### 10.5.5.9	Identity type 2

The purpose of the identity type 2 information element is to specify which identity is requested.

The identity type 2 is a type 1 information element.

The identity type 2 information element is coded as shown in figure 10.5.125/GSM 04.08 and table 10.5.142/GSM 04.08.

|  | 8 7 6 5 4 3 2 1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Identity type 2 IEI | 0 spare | Type of identity | octet 1 |

Figure 10.5.125/GSM 04.08: Identity type 2 information element

Table 10.5.142/GSM 04.08: Identity type 2 information element

Type of identity (octet 1) Bits 3 2 1 0 0 1 IMSI 0 1 0 IMEI 0 1 1 IMEISV 1 0 0 TMSI All other values are interpreted as IMSI by this version of the protocol.