- Agenda Item:	8.2

- WI / Topic:	SIP call control protocol for the IM subsystem

- Source:	Nokia

- Title:		Routing of session initiation with called public user ID in E.164 format

- Effected Specifications / Releases:  24.228

- Document for:	Approval

- Date:	2001-02-22

___________________________________________________________________________

# 1. Introduction

This contribution is a follow-up contribution to N1-010232, where the following principles have been agreed:

# It has been agreed that routing within the IM Subsystem should exclusively be based on SIP-URLs.

# Accordingly, for IMS-originating calls using an E.164 number as the called party public user identifier, there is a need for address translation.

# This address translation is to be based on ENUM-DNS based database structure, or any other suitable translation database, and to be performed in the originating S-CSCF.

2. Discussion

In TSG-SA WG2 it has been agreed that routing within the IM Subsystem should exclusively be based on SIP-URLs. Accordingly, for IMS-originating calls using an E.164 number as the called party public user identifier, there is a need for address translation. This address translation is to be based on ENUM [ENUM], and to be performed in the originating S-CSCF.

The figures below illustrate some possible examples for this address translation:

- The destination E.164 number belongs to an IMS subscriber

Figure 1 - Destination E.164 belongs to an IMS subscriber

- Destination E.164 is not identified as belonging to an IMS subscriber

As a general conclusion it can be stated that the ENUM-based address translation effects the Request-URI of the INVITE request.

# 3. Proposal

It is proposed to add the following text to the appropriate steps in the MO# and S-S# call flows. These appropriate steps are the ones when the originating S-CSCF sends out the INVITE request:

- Step 4 of MO#1b

- Step 6 of MO#1a

- Step 6 of MO#2

- Step 4 of S-S#1b

- Step 4 of S-S#1a

- Step 4 of S-S#2

- Step 4 of S-S#3

An the proposed text is:

"In case the Request-URI of the incoming INVITE request to S-CSCF contains a TEL-URL, it has to be translated to a globally routable SIP-URL before applying it as Request-URI of the outgoing INVITE request. For this address translation the S-CSCF may use the services of an ENUM-DNS based database structure, or any other suitable translation database."