| 3GPP TSG-N2A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc 3GPP |  |  |  |  |  |  |  |  |  | N2A000303 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis, France, 21 - 25 Jan 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (revision of N2000302) |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 29.078 |  |  |  |  |  | CR |  |  | 048r3 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.2.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | CN#7 |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | CN WG2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  | 29/02/2000 |  |  |  |  |  |
| Subject: |  |  | Transfer of destination address to gsmSCF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | CAMEL Phase 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | The current specification of CAMEL (Phase 2) does not allow the transfer of the destination address for a call leg to the gsmSCF. This is necessary for the correct operation of prepayment services. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 5.1; (see ‘Other Comments’ below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | This CR is based on Tdoc N2-99847, presented in Visby, July 1999. This CR should have been incorporated into the draft version of 29.078 for CAMEL Phase 3. This revision has had section 11 removed due to its contents being agreed and passed on to Keijo (Nokia) to incorporate in his combined section 11 CR. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

****First/Last Modified Section ****

## 5.1	Data types

-- The Definition of Common Data Types follows

CAP-datatypes {ccitt(0) identified-organization(4) ???(0) ???Domain(1) ???-network(1) modules(3) CAP-datatypes (50) version3(0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

tc-Messages, classes FROM CAP-object-identifiers	
		{ccitt(0) identified-organization(4) ???(0) ???Domain(1) ???-network(1) module(0) CAP-object-identifiers(17) version3(0)}

InvokeIdType
FROM TCAPMessages tc-Messages

EXTENSION,

PARAMETERS-BOUND,

SupportedExtensions {}

FROM CAP-classes classes

.

.

.

EventSpecificInformationBCSM {PARAMETERS-BOUND : bound} ::= CHOICE {

routeSelectFailureSpecificInfo	[2] SEQUENCE {

failureCause			[0] Cause {bound}			OPTIONAL,

...

},

oCalledPartyBusySpecificInfo	[3] SEQUENCE {

busyCause				[0] Cause {bound}			OPTIONAL,

...

},

oNoAnswerSpecificInfo			[4] SEQUENCE {

-- no specific info defined --

...

},

oAnswerSpecificInfo				[5] SEQUENCE {

destinationAddress			CalledPartyNumber,

or-Call					[0]	NULL					OPTIONAL,

forwardedCall			[1] NULL					OPTIONAL,

...

},

oDisconnectSpecificInfo			[7] SEQUENCE {

releaseCause			[0] Cause {bound}			OPTIONAL,

...

},

tBusySpecificInfo 				[8] SEQUENCE {

busyCause				[0] Cause {bound}			OPTIONAL,

callForwarded			[50] NULL					OPTIONAL,

...

},

tNoAnswerSpecificInfo 			[9] SEQUENCE {

callForwarded			[50] NULL					OPTIONAL,

...

},

tAnswerSpecificInfo				[10] SEQUENCE {

destinationAddress			CalledPartyNumber,

or-Call					[0]	NULL					OPTIONAL,

forwardedCall			[1]	NULL					OPTIONAL,

...

},

tDisconnectSpecificInfo	[12] SEQUENCE {

releaseCause			[0] Cause {bound}			OPTIONAL,

...

},

}

.

.