3GPP TSG-GERAN WG4 Meeting #6 	TDoc G4-010641

Sophia Antipolis, France, 8 - 10 October 2001

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51.010-1 |  |  |  | CR | 428 |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.5.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | x |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Number of octets in data transfer of 43.1.1.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Setcom |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | GPRS |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 2001-08-28 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-4 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | n=1500 octets corresponds to approx. 68 RLC data blocks with CS-1. In this test case we need a larger number of RLC data blocks because until step 10 the MS shall have sent already BSN=0 … 30 in the previous steps, and in step 11-13 the MS shall sent RLC data blocks with BSN=31…94 (until the Stall Indicator bit is set, SI=1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Change N=1500 to N=3000 in step 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Test case cannot be executed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 43.1.1.4.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | This CR applies to R97 and later releases |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 43.1.1.4	Acknowledged mode / Uplink TBF / Negatively acknowledged RLC data blocks

43.1.1.4.1	Conformance requirements

1.	The transmitter shall transmit the oldest RLC data block whose corresponding element in V(B) indexed relative to V(A) has the value NACKED.

2.	If [ V(S) < V(A) + k ] modulo 128 and no RLC data blocks have a corresponding element in V(B) with the value NACKED, the RLC data block with BSN = V(S) shall be transmitted.

3.	As each RLC data block is transmitted the corresponding element in V(B) shall be set to the value PENDING_ACK.

References

GSM 04.60, section 9.1.3.

43.1.1.4.2	Test purpose

1.	To verify that the mobile station retransmits Nacked data blocks before transmission of new information.

2.	To verify that the mobile station retransmits Nacked blocks in order of age, older first.

3.	To verify that the mobile station only retransmit Nacked blocks once per negative acknowledgment.

43.1.1.4.3	Method of test

Initial Conditions

System Simulator:

1 cell, default setting, , PBCCH not present, BS_CV_MAX = 0

Mobile Station:

The MS is GPRS updated with a P-TMSI allocated, SPLIT PG CYCLE negotiated and the test PDP Context2 activated.

Related PICS/PIXIT Statement(s)

The way to initiate an acknowledged uplink transfer.

Test Procedure

The MS is made to transmit RLC data blocks. The SS randomly negatively acknowledges RLC data blocks. The MS retransmits the negatively acknowledged RLC data blocks, oldest first before transmitting new RLC data blocks.

The SS does not send further acknowledgements. The MS does not retransmit the previously negatively acknowledged RLC data blocks.

Maximum Duration of Test

5 min.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 |  | { Uplink dynamic allocation two phase access} | n = 3000 octets USF_GRANULARITY = 1 block CHANNEL_CODING_COMMAND: cs1 |
| 2 | SS -> MS | PACKET DOWNLINK DUMMY CONTROL BLOCK | Sent on the PACCH of the PDCH assigned, USF assigned to the MS. |
| 3 | MS -> SS | UPLINK RLC DATA BLOCK | Received on the assigned PDTCH |
| 4 |  |  | Repeat steps 2 and 3 until received data block BSN = 30 |
| 5 | SS -> MS | PACKET UPLINK ACK/NACK | SS acknowledges RLC data blocks with BSN 10 to 30, RBB set to 1 and negatively acknowledges the rest with RBB set to 0 USF not assigned to the MS |
| 6 |  |  | Wait for 6 blocks with no USF |
| 7 | SS -> MS | PACKET DOWNLINK DUMMY CONTROL BLOCK | Sent on the PACCH of the PDCH assigned, USF assigned to the MS. |
| A8 (optional step) | MS -> SS | UPLINK RLC DATA BLOCK | MS may retransmit block BSN = 31 if it has already been scheduled while Packet Uplink Ack/Nack is being processed. In this case repeat step 7 |
| 8 | MS -> SS | UPLINK RLC DATA BLOCK | Received on the assigned PDTCH BSN = 0..9 |
| 9 | SS |  | Repeat steps 7 & 8 ten (11 in case mobile sends BSN = 31 in step A8) times SS verifies that the Nacked data blocks are received before new data block once per negative acknowledgment. After these data blocks new data block BSN=31 (32 if MS sends BSN = 31 earlier in step A8) should be received |
| 10 | SS -> MS | PACKET UPLINK ACK/NACK | SS acknowledges retransmitted (BSN 0..9) RLC data blocks, RBB set to 1 |
| 11 | SS -> MS | PACKET DOWNLINK DUMMY CONTROL BLOCK | Sent on the PACCH of the PDCH, containing USF assigned to the MS. |
| 12 | MS -> SS | UPLINK RLC DATA BLOCK | Received on the assigned PDTCH |
| 13 | SS |  | Repeat steps 11 and 12 until the stall indication bit is set in the data block received in step 12. SS verifies that acknowledged blocks are not retransmitted |
| 14 | SS -> MS | PACKET UPLINK ACK/NACK | SS negatively acknowledges RLC data blocks with BSN 55 to 60, RBB set to 0 and acknowledges the rest with RBB set to 1 USF not assigned to the MS |
| 15 |  |  | Wait for 6 blocks with no USF. |
| 16 | SS -> MS | PACKET DOWNLINK DUMMY CONTROL BLOCK | Sent on the PACCH of the PDCH assigned, USF assigned to the MS. |
| 17 | MS -> SS | UPLINK RLC DATA BLOCK | Received on the assigned PDTCH BSN = 55..60 MS may retransmit block BSN = 32 (or 33) if it has already been scheduled while Packet Uplink Ack/Nack is being processed |
| 18 | SS |  | Repeat steps 16 & 17 until all Nacked blocks are received SS verifies that the Nacked data blocks are received before new data blocks once per negative acknowledgment. |
| 19 |  | {Completion of uplink RLC data block transfer} |  |