Compliance Templates

Introduction :

The compliance templates from Report ITU-R M.2133 (2008) Section 4.2.4, corresponding to the proposed draft Revision 6 for « LTE Release 10 & beyond (LTE-Advanced) » are provided. Given that the full context of the total terrestrial radio interface is considered to be the original submission and any previously approved updates as well as the proposed update, the tables have been updated on the basis of the 3GPP up to Release 18 Specifications as per TSG Plenary #106 (Dec 2024)

The proposed revision of M.2012 for « LTE Release 10 & beyond (LTE-Advanced) » continues to meet or improve upon the characteristics presented in Report ITU-R M.2198 Section 5.2.5.

Baseline Compliance Templates :

FDD RIT component of SRIT “LTE Release 10 & beyond (LTE-Advanced)”

#### 4.2.4.1 	Compliance template for services

|  | Service related minimum capabilities within the RIT/SRIT | Evaluator’s comments |
| --- | --- | --- |
| 4.2.4.1.1 | Support of a wide range of services Does the proposal support a wide range of services?: If bullets 4.2.4.1.1.1 - 4.2.4.1.1.3 are marked as "yes" then 4.2.4.1.1 is a "yes". YES / NO | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.1.1.1 | Ability to support basic conversational service class Is the proposal able to support basic conversational service class?: YES / NO | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.1.1.2 | Support of rich conversational service class Is the proposal able to support rich conversational service class?: YES / NO | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.1.1.3 | Support of conversational low delay service class Is the proposal able to support conversational low-delay service class?: YES / NO | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |

#### 4.2.4.2 	Compliance template for spectrum1

|  | Spectrum capability requirements |
| --- | --- |
| 4.2.4.2.1 | Spectrum bands Is the proposal able to utilize at least one band identified for IMT?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. Furthermore, based on document 3GPP TS 36.101 Section 5.5 and TS 36.104 Section 5.5 , E-UTRA is designed to operate in the operating bands defined in Table 5.5-1. Below is table based on table 5.5-1 from TS 36.101. Note that the table contains both FDD and TDD bands. Based on table 5.5-1 E-UTRA operating bands E-UTRA Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. 1; 1920 MHz; –; 1980 MHz; 2110 MHz; –; 2170 MHz; FDD. 2; 1850 MHz; –; 1910 MHz; 1930 MHz; –; 1990 MHz; FDD. 3; 1710 MHz; –; 1785 MHz; 1805 MHz; –; 1880 MHz; FDD. 4; 1710 MHz; –; 1755 MHz; 2110 MHz; –; 2155 MHz; FDD. 5; 824 MHz; –; 849 MHz; 869 MHz; –; 894MHz; FDD. 61; 830 MHz; –; 840 MHz; 875 MHz; –; 885 MHz; FDD. 7; 2500 MHz; –; 2570 MHz; 2620 MHz; –; 2690 MHz; FDD. 8; 880 MHz; –; 915 MHz; 925 MHz; –; 960 MHz; FDD. 9; 1749.9 MHz; –; 1784.9 MHz; 1844.9 MHz; –; 1879.9 MHz; FDD. 10; 1710 MHz; –; 1770 MHz; 2110 MHz; –; 2170 MHz; FDD. 11; 1427.9 MHz; –; 1447.9 MHz; 1475.9 MHz; –; 1495.9 MHz; FDD. 12; 699 MHz; –; 716 MHz; 729 MHz; –; 746 MHz; FDD. 13; 777 MHz; –; 787 MHz; 746 MHz; –; 756 MHz; FDD. 14; 788 MHz; –; 798 MHz; 758 MHz; –; 768 MHz; FDD. 15; Reserved; Reserved; FDD. 16; Reserved; Reserved; FDD. 17; 704 MHz; –; 716 MHz; 734 MHz; –; 746 MHz; FDD. 18; 815 MHz; –; 830 MHz; 860 MHz; –; 875 MHz; FDD. 19; 830 MHz; –; 845 MHz; 875 MHz; –; 890 MHz; FDD. 20; 832 MHz; –; 862 MHz; 791 MHz; –; 821 MHz; FDD. 21; 1447.9 MHz; –; 1462.9 MHz; 1495.9 MHz; –; 1510.9 MHz; FDD. 22; 3410 MHz; –; 3490 MHz; 3510 MHz; –; 3590 MHz; FDD. 231; 2000 MHz; –; 2020 MHz; 2180 MHz; –; 2200 MHz; FDD. 2417; 1626.5 MHz; –; 1660.5 MHz; 1525 MHz; –; 1559 MHz; FDD. 25; 1850 MHz; –; 1915 MHz; 1930 MHz; –; 1995 MHz; FDD. 26; 814 MHz; –; 849 MHz; 859 MHz; –; 894 MHz; FDD. 27; 807 MHz; –; 824 MHz; 852 MHz; –; 869 MHz; FDD. 28; 703 MHz; –; 748 MHz; 758 MHz; –; 803 MHz; FDD. 29; N/A; 717 MHz; –; 728 MHz; FDD2. 3015; 2305 MHz; –; 2315 MHz; 2350 MHz; –; 2360 MHz; FDD. 31; 452.5 MHz; –; 457.5 MHz; 462.5 MHz; –; 467.5 MHz; FDD. 32; N/A; 1452 MHz; –; 1496 MHz; FDD2. 33; 1900 MHz; –; 1920 MHz; 1900 MHz; –; 1920 MHz; TDD. 34; 2010 MHz; –; 2025 MHz; 2010 MHz; –; 2025 MHz; TDD. 35; 1850 MHz; –; 1910 MHz; 1850 MHz; –; 1910 MHz; TDD. 36; 1930 MHz; –; 1990 MHz; 1930 MHz; –; 1990 MHz; TDD. 37; 1910 MHz; –; 1930 MHz; 1910 MHz; –; 1930 MHz; TDD. 38; 2570 MHz; –; 2620 MHz; 2570 MHz; –; 2620 MHz; TDD. 39; 1880 MHz; –; 1920 MHz; 1880 MHz; –; 1920 MHz; TDD. 40; 2300 MHz; –; 2400 MHz; 2300 MHz; –; 2400 MHz; TDD. 41; 2496 MHz; 2690 MHz; 2496 MHz; 2690 MHz; TDD. 42; 3400 MHz; –; 3600 MHz; 3400 MHz; –; 3600 MHz; TDD. 43; 3600 MHz; –; 3800 MHz; 3600 MHz; –; 3800 MHz; TDD. 44; 703 MHz; –; 803 MHz; 703 MHz; –; 803 MHz; TDD. 45; 1447 MHz; –; 1467 MHz; 1447 MHz; –; 1467 MHz; TDD. 48; 3550 MHz; –; 3700 MHz; 3550 MHz; –; 3700 MHz; TDD. 49; 3550 MHz; –; 3700 MHz; 3550 MHz; –; 3700 MHz; TDD16. 50; 1432 MHz; -; 1517 MHz; 1432 MHz; -; 1517 MHz; TDD13. 51; 1427 MHz; -; 1432 MHz; 1427 MHz; -; 1432 MHz; TDD13. 52; 3300 MHz; -; 3400 MHz; 3300 MHz; -; 3400 MHz; TDD. 53; 2483.5 MHz; -; 2495 MHz; 2483.5 MHz; -; 2495 MHz; TDD. 54; 1670 MHz; -; 1675 MHz; 1670 MHz; -; 1675 MHz; TDD. …. 64; Reserved. 65; 1920 MHz; –; 2010 MHz; 2110 MHz; –; 2200 MHz; FDD. 66; 1710 MHz; –; 1780 MHz; 2110 MHz; –; 2200 MHz; FDD4. 67; N/A; 738 MHz; –; 758 MHz; FDD2. 68; 698 MHz; –; 728 MHz; 753 MHz; –; 783 MHz; FDD. 69; N/A; 2570 MHz; –; 2620 MHz; FDD2. 70; 1695 MHz; –; 1710 MHz; 1995 MHz; –; 2020 MHz; FDD10. 71; 663 MHz; –; 698 MHz; 617 MHz; –; 652 MHz; FDD. 72; 451 MHz; –; 456 MHz; 461 MHz; –; 466 MHz; FDD. 73; 450 MHz; –; 455 MHz; 460 MHz; –; 465 MHz; FDD. 74; 1427 MHz; –; 1470 MHz; 1475 MHz; –; 1518 MHz; FDD. 75; N/A; 1432 MHz; –; 1517 MHz; FDD2. 76; N/A; 1427 MHz; –; 1432 MHz; FDD2. 85; 698 MHz; –; 716 MHz; 728 MHz; –; 746 MHz; FDD. 87; 410 MHz; –; 415 MHz; 420 MHz; –; 425 MHz; FDD. 88; 412 MHz; –; 417 MHz; 422 MHz; –; 427 MHz; FDD. 10318; 787 MHz; –; 788 MHz; 757 MHz; –; 758 MHz; FDD. 106; 896 MHz; –; 901 MHz; 935 MHz; –; 940 MHz; FDD. NOTE 1: Band 6, 23 is not applicable NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: A UE that complies with the E-UTRA Band 65 minimum requirements in this specification shall also comply with the E-UTRA Band 1 minimum requirements. NOTE 4: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: A UE that supports E-UTRA Band 66 shall receive in the entire DL operating band NOTE 6: A UE that supports E-UTRA Band 66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_66B, CA_66C and CA_66A-66A. NOTE 7: A UE that complies with the E-UTRA Band 66 minimum requirements in this specification shall also comply with the E-UTRA Band 4 minimum requirements. NOTE 9: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. NOTE 10: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 12: A UE that complies with the E-UTRA Band 74 minimum requirements in this specification shall also comply with the E-UTRA Band 11 and Band 21 minimum requirements. NOTE 13: UE that complies with the E-UTRA Band 50 minimum requirements in this specification shall also comply with the E-UTRA Band 51 minimum requirements. NOTE 14: A UE that complies with the E-UTRA Band 75 minimum requirements in this specification shall also comply with the E-UTRA Band 76 minimum requirements. NOTE 15: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 16: This band is restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 17: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. NOTE 18: This band is restricted to NB-IoT operation only. |

#### 4.2.4.3 	Compliance template for technical performance1

| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  |  | Required value | Value(2), (3) | Require-ment met? | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test environment |  | Downlink or uplink |  |  |  |  |
| 4.2.4.3.1 Cell spectral efficiency (bit/s/Hz/cell) (4.1) | Indoor | Downlink |  | 3 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 2.25 |  | Yes  No |  |
|  | Microcellular | Downlink |  | 2.6 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 1.8 |  | Yes  No |  |
|  | Base coverage urban | Downlink |  | 2.2 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 1.4 |  | Yes  No |  |
|  | High speed | Downlink |  | 1.1 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 0.7 |  | Yes  No |  |
| 4.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) | Not applicable | Downlink |  | 15 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 6.75 |  | Yes  No |  |
| 4.2.4.3.3 Bandwidth (4.3) | Not applicable | Up to and including (MHz) |  | 40 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Scalability |  | Support of at least three band-width values(4) |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |

| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Require-ment met? | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.4 Cell edge user spectral efficiency (bit/s/Hz) (4.4) | Indoor | Downlink | 0.1 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink | 0.07 |  | Yes  No |  |
|  | Microcellular | Downlink | 0.075 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink | 0.05 |  | Yes  No |  |
|  | Base coverage urban | Downlink | 0.06 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink | 0.03 |  | Yes  No |  |
|  | High speed | Downlink | 0.04 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink | 0.015 |  | Yes  No |  |
| 4.2.4.3.5 Control plane latency (ms) (4.5.1) | Not applicable | Not applicable | Less than 100 ms |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.6 User plane latency (ms) (4.5.2) | Not applicable | Not applicable | Less than 10 ms |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.7 Mobility classes (4.6) | Indoor | Uplink | Stationary, pedestrian |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  | Microcellular | Uplink | Stationary, pedestrian, vehicular up to 30 km/h |  | Yes  No |  |
|  | Base coverage urban | Uplink | Stationary, pedestrian, vehicular |  | Yes  No |  |
|  | High speed | Uplink | High speed vehicular, vehicular |  | Yes  No |  |
| 4.2.4.3.8 Mobility Traffic channel link data rates (bit/s/Hz) (4.6) | Indoor | Uplink | 1.0 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  | Microcellular | Uplink | 0.75 |  | Yes  No |  |
|  | Base coverage urban | Uplink | 0.55 |  | Yes  No |  |
|  | High speed | Uplink | 0.25 |  | Yes  No |  |

| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Require- ment met? | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.9 Intra-frequency hand-over interruption time (ms) (4.7) | Not applicable | Not applicable | 27.5 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.10 Inter-frequency handover interruption time within a spectrum band (ms) (4.7) | Not applicable | Not applicable | 40 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.11 Inter-frequency handover interruption time between spectrum bands (ms) (4.7) | Not applicable | Not applicable | 60 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.12 Inter-system handover (4.7) | Not applicable | Not applicable | Not applicable | Not applicable | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.13 Number of supported VoIP users (active users/ sector/MHz) (4.8) | Indoor | As defined in Report ITU-R M.2134 | 50 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  | Microcellular | As defined in Report ITU-R M.2134 | 40 |  | Yes  No |  |
|  | Base coverage urban | As defined in Report ITU-R M.2134 | 40 |  | Yes  No |  |
|  | High speed | As defined in Report ITU-R M.2134 | 30 |  | Yes  No |  |
| (1) As defined in Report ITU-R M.2134. (2) According to the evaluation methodology specified in Report ITU-R M.2135. (3) Mandatory when “no” is checked, optional when “yes” is checked. (4) Refer to Report ITU-R M.2135, § 7.4.1. |  |  |  |  |  |  |

TDD RIT component of SRIT “LTE Release 10 & beyond (LTE-Advanced)”

#### 4.2.4.1 	Compliance template for services1

|  | Service related minimum capabilities within the RIT/SRIT | Evaluator’s comments |
| --- | --- | --- |
| 4.2.4.1.1 | Support of a wide range of services Does the proposal support a wide range of services?: If bullets 4.2.4.1.1.1 - 4.2.4.1.1.3 are marked as "yes" then 4.2.4.1.1 is a "yes". YES / NO | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.1.1.1 | Ability to support basic conversational service class Is the proposal able to support basic conversational service class?: YES / NO | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.1.1.2 | Support of rich conversational service class Is the proposal able to support rich conversational service class?: YES / NO | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.1.1.3 | Support of conversational low delay service class Is the proposal able to support conversational low-delay service class?: YES / NO | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |

#### 4.2.4.2 	Compliance template for spectrum1

|  | Spectrum capability requirements |
| --- | --- |
| 4.2.4.2.1 | Spectrum bands Is the proposal able to utilize at least one band identified for IMT?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. Furthermore, based on document 3GPP TS 36.101 Section 5.5 and TS 36.104 Section 5.5, E-UTRA is designed to operate in the operating bands defined in Table 5.5-1. Below is table based on table 5.5-1 from TS 36.101. Note that the table contains both FDD and TDD bands. Based on table 5.5-1 E-UTRA operating bands E-UTRA Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. 1; 1920 MHz; –; 1980 MHz; 2110 MHz; –; 2170 MHz; FDD. 2; 1850 MHz; –; 1910 MHz; 1930 MHz; –; 1990 MHz; FDD. 3; 1710 MHz; –; 1785 MHz; 1805 MHz; –; 1880 MHz; FDD. 4; 1710 MHz; –; 1755 MHz; 2110 MHz; –; 2155 MHz; FDD. 5; 824 MHz; –; 849 MHz; 869 MHz; –; 894MHz; FDD. 61; 830 MHz; –; 840 MHz; 875 MHz; –; 885 MHz; FDD. 7; 2500 MHz; –; 2570 MHz; 2620 MHz; –; 2690 MHz; FDD. 8; 880 MHz; –; 915 MHz; 925 MHz; –; 960 MHz; FDD. 9; 1749.9 MHz; –; 1784.9 MHz; 1844.9 MHz; –; 1879.9 MHz; FDD. 10; 1710 MHz; –; 1770 MHz; 2110 MHz; –; 2170 MHz; FDD. 11; 1427.9 MHz; –; 1447.9 MHz; 1475.9 MHz; –; 1495.9 MHz; FDD. 12; 699 MHz; –; 716 MHz; 729 MHz; –; 746 MHz; FDD. 13; 777 MHz; –; 787 MHz; 746 MHz; –; 756 MHz; FDD. 14; 788 MHz; –; 798 MHz; 758 MHz; –; 768 MHz; FDD. 15; Reserved; Reserved; FDD. 16; Reserved; Reserved; FDD. 17; 704 MHz; –; 716 MHz; 734 MHz; –; 746 MHz; FDD. 18; 815 MHz; –; 830 MHz; 860 MHz; –; 875 MHz; FDD. 19; 830 MHz; –; 845 MHz; 875 MHz; –; 890 MHz; FDD. 20; 832 MHz; –; 862 MHz; 791 MHz; –; 821 MHz; FDD. 21; 1447.9 MHz; –; 1462.9 MHz; 1495.9 MHz; –; 1510.9 MHz; FDD. 22; 3410 MHz; –; 3490 MHz; 3510 MHz; –; 3590 MHz; FDD. 231; 2000 MHz; –; 2020 MHz; 2180 MHz; –; 2200 MHz; FDD. 2417; 1626.5 MHz; –; 1660.5 MHz; 1525 MHz; –; 1559 MHz; FDD. 25; 1850 MHz; –; 1915 MHz; 1930 MHz; –; 1995 MHz; FDD. 26; 814 MHz; –; 849 MHz; 859 MHz; –; 894 MHz; FDD. 27; 807 MHz; –; 824 MHz; 852 MHz; –; 869 MHz; FDD. 28; 703 MHz; –; 748 MHz; 758 MHz; –; 803 MHz; FDD. 29; N/A; 717 MHz; –; 728 MHz; FDD2. 3015; 2305 MHz; –; 2315 MHz; 2350 MHz; –; 2360 MHz; FDD. 31; 452.5 MHz; –; 457.5 MHz; 462.5 MHz; –; 467.5 MHz; FDD. 32; N/A; 1452 MHz; –; 1496 MHz; FDD2. 33; 1900 MHz; –; 1920 MHz; 1900 MHz; –; 1920 MHz; TDD. 34; 2010 MHz; –; 2025 MHz; 2010 MHz; –; 2025 MHz; TDD. 35; 1850 MHz; –; 1910 MHz; 1850 MHz; –; 1910 MHz; TDD. 36; 1930 MHz; –; 1990 MHz; 1930 MHz; –; 1990 MHz; TDD. 37; 1910 MHz; –; 1930 MHz; 1910 MHz; –; 1930 MHz; TDD. 38; 2570 MHz; –; 2620 MHz; 2570 MHz; –; 2620 MHz; TDD. 39; 1880 MHz; –; 1920 MHz; 1880 MHz; –; 1920 MHz; TDD. 40; 2300 MHz; –; 2400 MHz; 2300 MHz; –; 2400 MHz; TDD. 41; 2496 MHz; 2690 MHz; 2496 MHz; 2690 MHz; TDD. 42; 3400 MHz; –; 3600 MHz; 3400 MHz; –; 3600 MHz; TDD. 43; 3600 MHz; –; 3800 MHz; 3600 MHz; –; 3800 MHz; TDD. 44; 703 MHz; –; 803 MHz; 703 MHz; –; 803 MHz; TDD. 45; 1447 MHz; –; 1467 MHz; 1447 MHz; –; 1467 MHz; TDD. 48; 3550 MHz; –; 3700 MHz; 3550 MHz; –; 3700 MHz; TDD. 49; 3550 MHz; –; 3700 MHz; 3550 MHz; –; 3700 MHz; TDD16. 50; 1432 MHz; -; 1517 MHz; 1432 MHz; -; 1517 MHz; TDD13. 51; 1427 MHz; -; 1432 MHz; 1427 MHz; -; 1432 MHz; TDD13. 52; 3300 MHz; -; 3400 MHz; 3300 MHz; -; 3400 MHz; TDD. 53; 2483.5 MHz; -; 2495 MHz; 2483.5 MHz; -; 2495 MHz; TDD. 54; 1670 MHz; -; 1675 MHz; 1670 MHz; -; 1675 MHz; TDD. …. 64; Reserved. 65; 1920 MHz; –; 2010 MHz; 2110 MHz; –; 2200 MHz; FDD. 66; 1710 MHz; –; 1780 MHz; 2110 MHz; –; 2200 MHz; FDD4. 67; N/A; 738 MHz; –; 758 MHz; FDD2. 68; 698 MHz; –; 728 MHz; 753 MHz; –; 783 MHz; FDD. 69; N/A; 2570 MHz; –; 2620 MHz; FDD2. 70; 1695 MHz; –; 1710 MHz; 1995 MHz; –; 2020 MHz; FDD10. 71; 663 MHz; –; 698 MHz; 617 MHz; –; 652 MHz; FDD. 72; 451 MHz; –; 456 MHz; 461 MHz; –; 466 MHz; FDD. 73; 450 MHz; –; 455 MHz; 460 MHz; –; 465 MHz; FDD. 74; 1427 MHz; –; 1470 MHz; 1475 MHz; –; 1518 MHz; FDD. 75; N/A; 1432 MHz; –; 1517 MHz; FDD2. 76; N/A; 1427 MHz; –; 1432 MHz; FDD2. 85; 698 MHz; –; 716 MHz; 728 MHz; –; 746 MHz; FDD. 87; 410 MHz; –; 415 MHz; 420 MHz; –; 425 MHz; FDD. 88; 412 MHz; –; 417 MHz; 422 MHz; –; 427 MHz; FDD. 10318; 787 MHz; –; 788 MHz; 757 MHz; –; 758 MHz; FDD. 106; 896 MHz; –; 901 MHz; 935 MHz; –; 940 MHz; FDD. NOTE 1: Band 6, 23 is not applicable NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: A UE that complies with the E-UTRA Band 65 minimum requirements in this specification shall also comply with the E-UTRA Band 1 minimum requirements. NOTE 4: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: A UE that supports E-UTRA Band 66 shall receive in the entire DL operating band NOTE 6: A UE that supports E-UTRA Band 66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_66B, CA_66C and CA_66A-66A. NOTE 7: A UE that complies with the E-UTRA Band 66 minimum requirements in this specification shall also comply with the E-UTRA Band 4 minimum requirements. NOTE 9: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. NOTE 10: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 12: A UE that complies with the E-UTRA Band 74 minimum requirements in this specification shall also comply with the E-UTRA Band 11 and Band 21 minimum requirements. NOTE 13: UE that complies with the E-UTRA Band 50 minimum requirements in this specification shall also comply with the E-UTRA Band 51 minimum requirements. NOTE 14: A UE that complies with the E-UTRA Band 75 minimum requirements in this specification shall also comply with the E-UTRA Band 76 minimum requirements. NOTE 15: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 16: This band is restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 17: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. NOTE 18: This band is restricted to NB-IoT operation only. |

#### 4.2.4.3 	Compliance template for technical performance1

| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  |  | Required value | Value(2), (3) | Require-ment met? | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test environment |  | Downlink or uplink |  |  |  |  |
| 4.2.4.3.1 Cell spectral efficiency (bit/s/Hz/cell) (4.1) | Indoor | Downlink |  | 3 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 2.25 |  | Yes  No |  |
|  | Microcellular | Downlink |  | 2.6 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 1.8 |  | Yes  No |  |
|  | Base coverage urban | Downlink |  | 2.2 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 1.4 |  | Yes  No |  |
|  | High speed | Downlink |  | 1.1 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 0.7 |  | Yes  No |  |
| 4.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) | Not applicable | Downlink |  | 15 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink |  | 6.75 |  | Yes  No |  |
| 4.2.4.3.3 Bandwidth (4.3) | Not applicable | Up to and including (MHz) |  | 40 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Scalability |  | Support of at least three band-width values(4) |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |

| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Require-ment met? | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.4 Cell edge user spectral efficiency (bit/s/Hz) (4.4) | Indoor | Downlink | 0.1 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink | 0.07 |  | Yes  No |  |
|  | Microcellular | Downlink | 0.075 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink | 0.05 |  | Yes  No |  |
|  | Base coverage urban | Downlink | 0.06 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink | 0.03 |  | Yes  No |  |
|  | High speed | Downlink | 0.04 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  |  | Uplink | 0.015 |  | Yes  No |  |
| 4.2.4.3.5 Control plane latency (ms) (4.5.1) | Not applicable | Not applicable | Less than 100 ms |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.6 User plane latency (ms) (4.5.2) | Not applicable | Not applicable | Less than 10 ms |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.7 Mobility classes (4.6) | Indoor | Uplink | Stationary, pedestrian |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  | Microcellular | Uplink | Stationary, pedestrian, vehicular up to 30 km/h |  | Yes  No |  |
|  | Base coverage urban | Uplink | Stationary, pedestrian, vehicular |  | Yes  No |  |
|  | High speed | Uplink | High speed vehicular, vehicular |  | Yes  No |  |
| 4.2.4.3.8 Mobility Traffic channel link data rates (bit/s/Hz) (4.6) | Indoor | Uplink | 1.0 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  | Microcellular | Uplink | 0.75 |  | Yes  No |  |
|  | Base coverage urban | Uplink | 0.55 |  | Yes  No |  |
|  | High speed | Uplink | 0.25 |  | Yes  No |  |

| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Require- ment met? | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.9 Intra-frequency hand-over interruption time (ms) (4.7) | Not applicable | Not applicable | 27.5 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.10 Inter-frequency handover interruption time within a spectrum band (ms) (4.7) | Not applicable | Not applicable | 40 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.11 Inter-frequency handover interruption time between spectrum bands (ms) (4.7) | Not applicable | Not applicable | 60 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.12 Inter-system handover (4.7) | Not applicable | Not applicable | Not applicable |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
| 4.2.4.3.13 Number of supported VoIP users (active users/ sector/MHz) (4.8) | Indoor | As defined in Report ITU-R M.2134 | 50 |  | Yes  No | Improves on the technology originally being submitted in ADV/8 and therefore the conclusions expressed in ITU-R M.2198 sect. 5.2.5 remains valid. |
|  | Microcellular | As defined in Report ITU-R M.2134 | 40 |  | Yes  No |  |
|  | Base coverage urban | As defined in Report ITU-R M.2134 | 40 |  | Yes  No |  |
|  | High speed | As defined in Report ITU-R M.2134 | 30 |  | Yes  No |  |
| (1) As defined in Report ITU-R M.2134. (2) According to the evaluation methodology specified in Report ITU-R M.2135. (3) Mandatory when “no” is checked, optional when “yes” is checked. (4) Refer to Report ITU-R M.2135, § 7.4.1. |  |  |  |  |  |  |

If a proponent determines that a specific question does not apply, the proponent should indicate that this is the case and provide a rationale for why it does not apply.