3GPP TSG RAN TSG Meeting #106 	RT-240018

Madrid, Spain, December 9-13th, 2024

Title:	Proposed submission to ITU-R WP4B for its recommendations on the IMT2020 Satellite component

Source: 	Thales

Type:	discussion

Document for:	discussion

Agenda Item:	ITU-R Ad-Hoc

# Introduction

This paper aims at providing an overview of the radio interface technology for the Recommendation ITU-R M.[IMT-2020-SAT.SPECS] which will contain the detailed specifications for IMT-2020 satellite component.

It is noted that “Considering that the proposed technologies in the 3GPP submission have many functionalities and specifications in common with the IMT-2020 terrestrial component technologies also submitted by 3GPP, it is suggested that the content of the radio interface overview focuses on the satellite specific information, to avoid a repetition of the detailed information already contained in Recommendation ITU-R M.2150. The description should be however sufficiently encompassing, so that the reader does not need to refer to Recommendation ITU-R M.2150 to have a sufficient information on the technology.”

The proposed overview of the radio interface technology is largely based on the following 3GPP Work item summaries

- [2], [3] for NR_NTN in respectively 17 and 18

- [4], [5] for IoT_NTN in respectively 17 and 18

In addition, it refers to the ITU-R M.2150 [1] where appropriate.

# Proposed text for the submission to ITU-R WP4B

## Overview of the satellite radio interface technology

The IMT-2020 satellite radio interface specifications, known as 5G NTN, have been developed by 3GPP and consist of IoT-NTN based on long-term evolution (LTE) and NR-NTN based on new radio (NR) as part of its Releases 17 and beyond.

5G-NTN is a set of radio interface technologies (RITs) consisting of IoT-NTN as one component RIT and NR-NTN as the other component RIT.

5G-NTN fulfils all technical performance requirements in all three selected test environments: Rural – enhanced Mobile Broadband (eMBB-s), Rural – massive Machine Type Communication (mMTC-s) and Rural – High Reliability Communications (HRC-s).

Also, 5G-NTN fulfils the service and the spectrum requirements of the IMT2020 SRI. Both component RITs, NR-NTN and IoT-NTN, utilize the frequency bands below 7.125 GHz including frequency bands identified for International Mobile Telecommunication (IMT) in the ITU Radio Regulations. In addition, the NR-NTN component RIT can also utilize the frequency bands above 7.125 GHz, i.e. above 10 GHz, including frequency bands allocated to mobile satellite services in the ITU Radio Regulations.

The complete set of standards for the satellite radio interface of IMT-2020 identified as NR-NTN and IoT-NTN includes not only the key characteristics of IMT-2020 terrestrial radio interface [1] but also the additional NTN capabilities both of which are continuing to be enhanced.

The 3GPP 5G System (5GS) also includes specifications for its non-radio aspects, such as the core network elements (the Evolved Packet Core and 5G Core Networks), security, codecs, network management, etc. These non-radio specifications are not included in the so-called “Global Core Specifications (GCS)” of IMT-2020.

### Overview of the component RIT: NR-NTN

A number of features have been introduced into the NR protocol and NG-RAN architecture enabling the support of non-terrestrial networks (NTN). NTN refers to networks, or segments of networks, using a spaceborne vehicle or an airborne vehicle for transmission, defined as follows:

- Spaceborne vehicles: Geosynchronous (GSO) and Non-Geosynchronous (NGSO) orbiting satellites. NGSO includes Low Earth Orbit at altitude approximately between 300 km and 1500 km and Medium Earth Orbit at altitude approximately between 7000 km and 25000 km.

- Airborne vehicles: High Altitude Platforms (HAPs) encompassing Unmanned Aircraft Systems (UAS) including Lighter than Air UAS (LTA), Heavier than Air UAS (HTA), all operating in altitudes typically between 8 and 50 km, quasi-stationary.

The addressed radio specifics of NTN compared to 5G terrestrial networks include

•	Delay variation, Doppler variation as well as possible Earth moving radio cells, due to the motion of space/airborne vehicles

•	Long latency due to the altitude of the space/airborne vehicles

•	Differential delay and possible multi country cell coverage due to larger radio cell size

•	Different propagation channel model

•	Different radio unit performance due to specific payload performance

Overall architecture and general aspects

As illustrated in Figure 1, non-terrestrial access is provided by means of an NTN payload, i.e. a network node on-board a satellite or HAPS, and an NTN Gateway interconnected by a feeder link, the UE accessing NTN network services through the NTN payload via a service link.

Figure 1: Overall illustration of an NTN (from TS 38.300 [4])

The NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. A gNB may serve multiple NTN payloads while an NTN payload may be served by multiple gNBs.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the NTN gNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while gNB operating with GSO satellite can provide Earth fixed cell coverage.

Timing, Synchronization and HARQ enhancements

The network broadcast ephemeris information and common Timing Advance (common TA) parameters in each NTN cell. Since NTN capable UE are expected to be all GNSS-capable, they shall acquire a valid GNSS position as well as the satellite ephemeris and common TA before connecting to an NTN cell.

To achieve uplink synchronization, before performing random access, the UE shall autonomously pre-compensate the Timing Advance, as well as the frequency Doppler shift by considering the common TA (information from the gNB), the UE position, the satellite position and satellite velocity through the satellite ephemeris. In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation. If the UE does not have a valid GNSS position and/or valid satellite ephemeris, it does not communicate with the network until both are regained. The UEs may be configured to report Timing Advance at initial access or in connected mode. In connected mode triggered reporting of the Timing Advance is supported.

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE for the uplink, the management of Doppler shift experienced over the feeder link is left to the network implementation.

To accommodate the propagation delay in NTNs, several timing relationships are enhanced by a Common Timing Advance (Common TA) and two scheduling offsets Koffset and kmac . Common TA is a configured offset that corresponds to the Round Trip Time (RTT) between the Reference Point (RP) and the NTN payload. Koffset is a configured scheduling offset that approximately corresponds to the sum of the service link RTT and the common TA. kmac is a configured offset that approximately corresponds to the RTT between the RP and the gNB.

To mitigate the impact of HARQ stalling in NTN the HARQ feedback can be disabled in the presence of ARQ re-transmissions at the RLC layer (e.g., in GSO satellite systems) and/or the number of HARQ processes for re-transmissions at the MAC layer can be increased to 32 (e.g., in NGSO satellite systems).

Mobility Management

To enable mobility in NTN, the network provides serving cell's and neighbouring cell's satellite ephemeris needed to access the target serving NTN cell in the handover command.

UE supports mobility between NTN and Terrestrial Network (i.e. from NTN to Terrestrial Network (hand-in) and from Terrestrial Network to NTN (hand-out)), but is not required to connect to both NTN and Terrestrial Network at the same time. It may also support mobility between radio access technologies based on different orbit (GSO, NGSO at different altitude).

Triggering conditions upon which UE may execute Conditional Hand-Over (CHO) to a candidate cell, have been introduced: event A4, time-based trigger condition, location-based trigger condition. The two last conditions are configured together with one of the measurement-based trigger conditions. Location is defined by the distance between UE and a reference location. Time is defined by the time between T1 and T2, where T1 is an absolute time value and T2 is a duration started at T1.

For the measurements the network can configure multiple SS/PBCH Block Measurement Timing Configuration (SMTCs) in parallel per carrier and for a given set of cells depending on UE capabilities using propagation delay difference and ephemeris information. It can also configure measurement gaps based on multiple SMTC.

The adjustment of SMTCs is possible under network control based on UE assistance information if available for connected mode and under UE control based on UE location and satellite assistance information (e.g., ephemeris, common TA parameters) for idle/inactive modes.

In the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurement in RRC_IDLE/RRC_INACTIVE. The timing and location information associated to a cell are provided via system information. They refer respectively to the time when the serving cell is going to stop serving a geographical area and to the reference location of serving cell.

A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN. The network may broadcast multiple Tracking Area Codes (TAC) per PLMN in a NR NTN cell in order to reduce the signaling load at cell edge, in particular for Earth-moving cell coverage. A TAC change in the System Information is under network control and may not be exactly synchronized with real-time illumination of beams on ground.

Regarding the UE location aspects, upon network request, after AS security is established in connected mode, a UE should report its coarse UE location information (most significant bits of the GNSS coordinates, ensuring an accuracy in the order of 2 km) to the NG-RAN if available.

Switch-over

A service link switch refers to a change of serving satellite.

A feeder link switch over is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switch over is a Transport Network Layer procedure. Both hard and soft feeder link switch over are applicable to NTN.

Service and feeder link switch overs apply mostly for the case of NGSO.

NG-RAN signaling

The Cell Identity, indicated by the gNB to the Core Network as part of the User Location Information corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported. It is used for Paging Optimization in NG interface, Area of Interest and Public Warning Services.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target radio cell as well as for RAN paging.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network. The gNB is responsible for constructing the Mapped Cell ID based on the UE location info received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

The gNB reports the broadcasted TAC(s) of the selected Public Land Mobile Network (PLMN) to the Access and Mobility Management Function (AMF) as part of UE Location Information (ULI). In case the gNB knows the UE's location information, the gNB may determine the Tracking Area Indicator (TAI) the UE is currently located in and provide that TAI to the AMF as part of ULI.

RF performances and RRM requirements

The minimum RF and performance requirements in FR1 for respectively NR User Equipment (UE) supporting satellite access operation and NR Satellite Access Node (SAN) are defined in 3GPP TS 38.101-5 and 3GPP TS 38.108.

Figure 2 illustrate the satellite access node which encompass on ground non-NTN infrastructure gNB functions, gateway and feeder link and the RF functions of the NTN payload.

Figure 2: Satellite Access Node (SAN)

The considered operating bands in frequency range FR1 are defined in Table 1:

Table 1: Satellite operating bands

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |

RF requirements of an NTN capable UE require the same RF performance as UE operating with terrestrial network.

Note that RF requirements of the SANare lower compared to the BS RF requirements of a terrestrial network .

Specific requirements for radio resource management in NTN mostly relate to delay as well as timing and frequency errors in the different procedures.

An additional set of features have been introduced into the NR and NG-RAN architecture for an optimized support of NTN with the capabilities to

- Offer increased uplink coverage performance especially when addressing commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port)

- Support of Geostationary Orbit (GSO) as well as non-Geostationary Orbit (NGSO) deployment scenarios in the [DownLink: 17.7 -20.2 GHz; UpLink: 27.5 – 30.0 GHz] frequency bands

- Support the verification by the network of the location (i.e. GNSS coordinates) reported by the UE assuming a single satellite in view in order to fulfil the regulatory requirements (e.g., Lawful intercept, emergency call, Public Warning System)

- Provide NTN-TN and NTN-NTN measurement, mobility and service continuity enhancements considering the NTN characteristics such as large propagation delay and satellite movement

- Define 30 MHz channel bandwidth for n256 and n255 for 5G NR-NTN and introduce the corresponding SAN and UE RF core requirements, limited to clause 5 (Operating Bands and Channel Arrangement) requirements for these specifications

This set of enhancements are described here after.

Uplink coverage enhancements

To improve NR uplink coverage in NTN, the following enhancements have been introduced:

- PUCCH repetition for Msg4 HARQ-ACK:

  - Supported number of transmissions are 1, 2, 4, 8.

    - If a single value from {2, 4, 8} is configured via SIB, the configured repetition factor is applied.

    - If multiple values from {1, 2, 4, 8} are configured via SIB, one of the multiple values is indicated in DAI field of DCI format 1_0 with CRC scrambled by TC-RNTI.

  - The existing mechanism on repetition slot counting is applied.

  - Frequency hopping mechanism defined for PUCCH transmission for Msg4 HARQ-ACK, in every slot is applied.

  - A RSRP threshold can be configured via SIB when the number of repetitions is configured by SIB. If the RSRP threshold is configured, UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK via Msg3 PUSCH only if measured RSRP is lower than the configured RSRP threshold. If the RSRP threshold is not configured, UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK via Msg3 PUSCH.

  - The repetition factor applied to Msg4 HARQ-ACK is used also for any PUCCH transmission before dedicated PUCCH resource is provided.

- NTN-specific PUSCH DMRS bundling enhancement that enables DMRS bundling in presence of timing drift, whereby UE maintains phase continuity considering effects of transmission delay variation between UE and uplink time synchronization reference point to enable improved channel estimation.

NR-NTN deployment in above 10 GHz bands a.k.a FR2-NTN frequency range (17300 MHz – 30000 MHz)

To support NR NTN deployment in FR2-NTN using FDD duplexing mode, the following aspects were considered:

- For operation in FR2-NTN, the value range in ms for K_offset and K-MAC shall be the same as for FR1.

- For operation in FR2-NTN, use a reference SCS of 15 kHz for the indication of K_offset and K_MAC.

- For operation in FR2-NTN, no additional MAC CE TCI application delay is introduced to facilitate mechanical beam steering with VSAT.

- For operation in FR2-NTN, for cell search procedure, Case D and Case E in TS 38.213 are used to allow FDD operation in bands defined by FR2-NTN without any update to SSB pattern.

- From RAN1 perspective, for operation in FR2-NTN, the granularity used for TA reporting is the same as corresponding to the reference subcarrier spacing applied for K_offset.

The following NTN operating bands in above 10 GHz for satellite networks (FR2-NTN) have been introduced:

| n512 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz | FDD |
| --- | --- | --- | --- |
| n511 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz | FDD |
| n510 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz | FDD |

Moreover, the following VSAT classes and types have been introduced:

| UE class | UE type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT supporting GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT supporting GSO and LEO with electronical steering antenna. |
|  | 3 | Fixed VSAT supporting LEO only with electronical steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT supporting GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT supporting GSO with electronical steering antenna. |
| Note: Assuming that UE has single beam towards one single satellite at a given time. |  |  |

Network verified UE location

Necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view have also been introduced.

For UE location verification based on multi-RTT with single satellite in NTN, at least the following UE and gNB measurements are reported: gNB receive-transmit time difference at the uplink time synchronization reference point, UE receive-transmit time difference, UE receive-transmit time difference subframe offset and DL timing drift.

The assistance information reported to the CN may include ephemeris information including accurate satellite position and velocity at the time of multi-RTT measurement, and common TA parameters (ta-Common, ta-CommonDrift, ta-CommonDriftVariant, Epoch time).

Note that network verified UE location is an optional UE feature. The verification is triggered by the CN and is up to the CN to decide when to trigger the procedure.

Mobility Management enhancements

Mobility management enhancements have been introduced to ensure service continuity covers inter-satellite, inter-gNB, inter-RAT.

For the NTN to TN mobility, the Non-Terrestrial network may broadcast NR and EUTRA TN cell coverage areas and cell information in the new SIB25. The coverage information consists in a list of geographical TN areas, with associated frequency information. UE can reduce its power consumption by skipping TN measurement based on the TN coverage information.

For the TN to NTN mobility, the TN cell can broadcast NTN neighbour satellites ephemeris via SIB 19. The UE can use ephemeris to connect to the NTN.

RACH-less handover is supported in NR NTN. It is a L3 mobility procedure that avoids a RACH procedure during the handover (between gNBs or during feeder/satellite switch), reducing random access congestion in the target cell.

Conditional handover associated to a time-based or a location-based trigger condition already introduced for quasi-Earth fixed cells is also supported for Earth-moving cells. For the location condition, the candidate cell’s reference location re-use the cell reselection procedure as the cell moves.  CondEventD2 event is introduced that includes a reference location and distance threshold for source and target cell. Moreover, for the time-based, the CHO procedure can be combined with the RACH-less HO procedure.

Figure 1 Satellite switch with re-sync

Satellite switch with-resync allows the UE to synchronize with the target satellite without handover, i.e. no L3 mobility, no PCI change, upon both hard and soft satellite switch over in the quasi-Earth fixed scenario with the same SSB frequency and the same gNB. CHO can be configured simultaneously. In the soft switch case, the capable UE can start synchronizing with the target satellite before the source satellite ends to serve the cell without required to be connected to both satellites, as long as the SSBs from the source satellite and target satellite are not overlapped in time.

Channel bandwidth up to 30 MHz in FR1

In the previous the channel bandwidth was limited to 20 MHz in FR1 bands. With this release, channel bandwidth can be set up to 30 MHz for n256 and n255 frequency bands. The corresponding SAN and UE RF core requirements have been introduced however, limited to clause 5 (Operating Bands and Channel Arrangement) requirements.

### Overview of the component RIT: IoT-NTN

The IoT-NTN Component RIT is based on E-UTRA/LTE, encompassing FDD operation and therefore is applicable for the operation with paired spectrum with NTN as follow.

The NTN enhancements refer to enhanced features necessary for the support of Bandwidth reduced Low complexity (BL) UEs, UEs in enhanced coverage and NB-IoT UEs by Non-Terrestrial Networks (NTN).

They are largely aligned with that of NR-NTN, with the exception of discontinuous coverage.

Overall architecture and general aspects:

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Non-terrestrial networks encompass platforms that provide radio access through satellites in Geosynchronous orbits (GSO) as well as Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO).

As illustrated in Figure 1, non-terrestrial access is provided by means of an NTN payload, i.e. a network node on-board a satellite, and an NTN Gateway interconnected by a feeder link, the UE accessing NTN network services through the NTN payload via a service link.

Figure 1: Overall illustration of an NTN

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over NTN is only applicable to E-UTRA connected to EPC.

Only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported.

Timing and Synchronization:

The network broadcasts ephemeris information and common Timing Advance (common TA) parameters in each NTN cell. A UE shall acquire its GNSS position as well as the satellite ephemeris and common TA before connecting to an NTN cell. To achieve uplink synchronization, before performing random access, the UE shall autonomously pre-compensate the Timing Advance, as well as the frequency Doppler shift by considering the common TA, the UE position and the satellite position through the satellite ephemeris. In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation, but the UE is not expected to perform GNSS acquisition. The UE does not perform any transmissions due to outdated satellite ephemeris, common TA or GNSS position based on timers. In connected mode, upon outdated satellite ephemeris and common Timing Advance, the UE re-acquires the broadcasted parameters and upon outdated GNSS position the UE moves to idle mode. The UEs may be configured to report Timing Advance at initial access or in connected mode. In connected mode triggered reporting of the Timing Advance is supported.

For downlink synchronization in case of NB-IoT, the two LSBs of the ARFCN are signaled in MIB for bands for which a 200 kHz channel raster is not supported, and the legacy 100 kHz raster is used. Otherwise, for bands for which a 200 kHz channel raster is supported, there is no signaling of ARFCN information in MIB.

Downlink and uplink timings are frame aligned at the uplink time synchronization reference point (RP). To accommodate the long propagation delays in NTN, the timing relationships are enhanced by the support of two scheduling offsets: Koffset and Kmac as illustrated in Figure 2:

Figure 2: Timing relationship parameters

Uplink segmented transmission is supported for uplink transmission with repetitions. The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for BL UEs and UEs in enhanced coverage and NPUSCH/NPRACH for NB-IoT from one segment to the next segment. The configuration of uplink transmission segment is indicated on SIB for initial access and can be re-configured by RRC signalling.

Discontinuous coverage and assistance information:

As a satellite moves on a specified orbit, for example in case of a NGSO satellite, the satellite beam(s) coverage area may move and cover different portions of a geographical area due to the orbital movement of the satellite. As a consequence, a UE located in the concerned geographical area may experience a situation of discontinuous coverage, due to e.g., a sparse satellite constellation deployment.

The network may broadcast assistance information relating to the serving satellite and other satellites of the constellation to enable UEs to predict upcoming satellites fly-over periods and save power during periods of no coverage. The broadcast assistance information includes SGP4 ephemeris elements based on the TLE (Two-Line Elements) sets industry standard. Additional assistance information, such as coverage footprint parameters and cell radius, may also be optionally broadcast by the network.

Predicting out of coverage and in coverage periods is up to UE implementation. When out of coverage, the UE is not required to perform Access Stratum (AS) functions.

In the Core Network, discontinuous coverage is handled by means of Tracking Area- or RAT-specific configuration of the MME such that the MME is able, via existing functionality (namely periodic TAU timer, mobile reachable timer, implicit detach timer and high latency communication), to ensure that when the UE is unreachable, a) the UE does not trigger NAS transaction or detach from the network and b) mobile-terminated data destined to the UE can be stored in the network.

Mobility Management:

With this feature, the network may broadcast more than one Tracking Area Code (TAC) per PLMN in a cell in order to reduce the signalling load at cell edge in NTN, in particular for Earth-moving cell coverage. The AS layer indicates all received TACs for the selected PLMN to the NAS layer. The network may update the UEs upon TAC removal. UEs may by UE implementation also check whether a TAC has been removed from the TACs broadcast by the network.

At the NAS layer, the UE need not trigger a Tracking Area Update due to mobility reason, if any of the broadcast TAC(s) in the cell where the UE is located is part of the UE’s Tracking Area List.

For quasi-Earth-fixed cells, timing information on when the cell is going to stop serving the area may be broadcast by the network. This may be used by the UE to start measurements on neighbour cells before the broadcast stop time of the serving cell, while the exact start of the measurements is up to UE implementation.

Radio link failure and RRC connection re-establishment are supported in NTN. To enable mobility in NTN, the network provides target cell satellite parameters needed to access the NTN cell in the handover command. Conditional handover is supported for BL UEs and UEs in enhanced coverage.

Different RAT types are introduced that allow distinction by the Core Network between existing terrestrial accesses and new non-terrestrial accesses as well as, among non-terrestrial accesses, between the different types of satellite constellations (LEO, MEO, GEO, OTHERSAT) and radio access type (i.e. WB-EUTRAN, NB-IoT and LTE-M). This allows the Core Network nodes and the HSS to identify the access a UE is using such that they are able to adjust their behavior and that of the UE accordingly (e.g. setting of NAS timers, determination and enforcement of access restrictions, etc.).

Feeder-link switch-over:

The NTN Control function determines the point in time when a feeder link switch over between two eNBs is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UE(s)' context between the two eNBs at feeder link switch over is performed by means of either S1 based or X2 based handover, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

Network-interfaces signalling aspects:

The Cell Identity in NTN corresponds to a fixed geographical area identified by a Mapped Cell ID, irrespective of the orbit of the NTN payload or of the type of the service link. For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and the Core Network (e.g. pre-configured depending on operator's policy, or based on implementation). For a BL UE or a UE in enhanced coverage or a NB-IoT UE that supports S1-U data transfer or User Plane CIoT EPS optimization, the eNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Pre-configuration of special mapped cell identifiers may be used to indicate areas outside the serving PLMN's country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE's location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.

MME(Re-)Selection by eNB:

For an RRC_CONNECTED UE, when the eNB is configured to ensure that the BL UE or the UE in enhanced coverage is using an MME that serves the country in which the UE is located. If the eNB detects that a BL UE or a UE in enhanced coverage is in a different country from that served by the serving MME, it should perform an S1 handover to change to an appropriate MME or initiate a UE Context Release Request procedure towards the serving MME (in which case the MME may decide to detach the UE).

For an RRC_CONNECTED NB-IoT UE, when the eNB is configured to ensure that the NB-IoT UE is using an MME that serves the country in which the UE is located. If the eNB detects that the UE is in a different country to that served by the serving MME, it should initiate a UE Context Release Request procedure towards the serving MME (in which case the MME may decide to detach the UE).

Verification of UE location:

The network may, according to regulatory requirements, need to enforce that the PLMN selected by the UE is allowed to operate in the geographical location where the UE is located. To this end, the MME may invoke the ULI (User Location Information) procedure during Mobility Management and Session Management procedures in order to determine the UE location. If the MME is able to determine with sufficient accuracy that it is not allowed to operate in the UE location, it may reject and/or detach the UE.

Support for E-UTRAN:

For S1 and X2 interfaces, codepoints in the RAT Restriction Information IE (in the Handover Restriction List) allow the selection of different constellation types, i.e., "LEO", "MEO", "GEO", "OTHERSAT" for satellite access.

For S1 interface, additional codepoints are added to the RAT Type IE associated with a TAC, i.e., "NBIoT-LEO", "NBIoT-MEO", "NBIoT-GEO", "NBIoT-OTHERSAT", "EUTRAN-LEO", "EUTRAN-MEO", "EUTRAN-GEO", "EUTRAN-OTHERSAT".

For S1 interface, the UE Context Reference at Source IE (eNB UE S1AP ID) is introduced in the Source eNB to Target eNB Transparent Container.

For S1 interface, a new cause value is added signaling that a UE is not within the serving area of its current PLMN.

For TAC reporting over S1, the LTE NTN TAI Information IE and the semantics description for the TAI IE are added in the User Location Information IE; the LTE NTN TAI Information IE and the semantics description for the TAI IE are added to the eNB CP RELOCATION INDICATION, HANDOVER NOTIFY, PATH SWITCH REQUEST, INITIAL UE MESSAGE, UPLINK NAS TRANSPORT, and LOCATION REPORT messages.

Other NAS protocol Aspects:

Enhancements to NAS signaling allow the UE to register to EPS core network using satellite E-UTRAN radio access technology. UICC-ME interface is extended to support network selection over satellite access and allowing to prioritize networks offering satellite access. EPS NAS re-transmission timers are extended to support longer propagation delays and response times due to extended distance between peer entities when satellite access is used. The UE supporting satellite E-UTRAN access supports also GNSS and potential uplink signaling delays to be considered in UE and network NAS implementations.

Some further enhancements to the NB-IoT and the eMTC radio interfaces were introduced to address some bottlenecks identified in performance, mobility, and coverage discontinuity aspects.

The following functionality is introduced to improve performance:

-  HARQ feedback Enabling/Disabling: HARQ feedback of downlink packets can be enabled or disabled per HARQ process by dedicated RRC signaling and/or DCI based indication to mitigate the impact of HARQ stalling in NB-IoT/eMTC NTN. Disabling HARQ feedback allows scheduling a HARQ process with a downlink packet before one HARQ RTT has elapsed since last scheduled, which allows lower layer latency and higher data rates per UE.  Disabling HARQ feedback can be configured even for NB-IoT with single HARQ process. For HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer. The (re)starting of downlink drx-inactivity timers are adjusted depending on configuration of HARQ enabling/disabling status, single or multiple TB scheduling and HARQ Ack bundling.

In the uplink two HARQ modes: HARQ mode A and HARQ mode B are introduced and configured per HARQ process basis. UL HARQ mode configuration is based on RRC signaling. For HARQ process in HARQ mode B, UE will not start the corresponding UL HARQ RTT timer. For NB-IoT NTN with single HARQ process in HARQ mode B, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission. HARQ mode B is not applicable for UL transmission using PUR. For eMTC new LCP restrictions are introduced corresponding to HARQ enabling/disabling. UE (re)starts drx-InactivityTimer accordingly based on the configured UL HARQ mode.

-  Improved GNSS operations: During long connection times and for reducing GNSS measurements impact on UE power consumption, a UE supporting improved GNSS operations can be triggered to perform or configured to autonomously perform its GNSS acquisition If ul-TransmissionExtensionEnabled is configured, a time duration starts at the end of GNSS validity duration and a UE can perform autonomously GNSS acquisition using an autonomous gap starting at the end of the time duration, or the UE can be triggered to perform GNSS acquisition before the end of the time duration. This allows a UE in RRC_CONNECTED to apply UE pre-compensation of satellite delay and Doppler to maintain UL synchronization for UL transmission in an NTN cell. While network can trigger connected UE to perform GNSS measurement using DL MAC CE, UE autonomous GNSS-triggering measurement will be configured by RRC dedicated signalling. If there is neither network trigger nor network configuration of UE autonomously GNSS measurement, UE moves to RRC_IDLE after GNSS becomes invalid. UE will report the GNSS validity duration by using a UL MAC CE and assumes the GNSS location is valid upon successful GNSS measurement. Network enables this reporting of GNSS position fix duration, in SIB2 and in dedicated signalling (for handover).

Moreover, for measurement and mobility enhancements:

-	New time-based and location-based triggers are introduced. UE can start intra/inter frequency measurement in connected mode before the specific time. A serving cell reference location and a distance threshold / radius for UE is introduced to detect when to trigger connected mode measurements. A new SIB is introduced for the network to broadcast the neighbor cell/satellite information. Satellite ids are introduced for serving and neighbor satellites. Location-based measurement initiation can also be optionally used in idle mode for cell re-selection purposes.

-	For CHO in eMTC NTN, time and location-based trigger conditions may be configured independently (i.e., without a jointly configured measurement condition.

In addition, in case of discontinuous coverage:

-	An enhancement to RRC Release is introduced. UE may directly go to RRC_IDLE after RLF is triggered if there is not enough time for the UE to finish the procedure of RRC re-establishment due to the discontinuous coverage.

-	A new cause value "Release due to discontinuous coverage" is introduced for the S1AP UE Context Release Request procedure.

Finally, for UE’s RRM core requirements the following are specified: for NB-IoT and eMTC UEs in IDLE mode, cell re-selection requirements for location-based measurement triggering; for NB-IoT and eMTC UEs in CONNECTED mode, requirements for neighbour cell measurements and the corresponding measurement triggering before RLF; for NB-IoT and eMTC UEs in CONNECTED mode, RRM requirements considering the GNSS reacquisition; for eMTC UEs in CONNECTED mode, Conditional Handover requirements for location-based and time-based events.

# References

[1]	Recommendation ITU-R M.2150-2 (12/2023), M Series: Mobile, radiodetermination, amateur and related satellite services, Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2020 (IMT-2020)

[2]	RP-221946, “Summary for NR support for Non-Terrestrial Networks (NTN) Work Item”, corresponding to the Rel-17 RAN3 led “Solutions for NR to support non-terrestrial networks (NTN)” work item

[3]	RP-240922 « Summary for NR NTN (Non-Terrestrial Networks) enhancements «, corresponding to the Rel-18 RAN2 led “NR NTN (Non-Terrestrial Networks) enhancements” work item

[4]	RP-221547 “Summary for NB-IoT/eMTC support for Non-Terrestrial Networks (NTN)”, corresponding to the Rel-17 RAN2 led “NB-IoT/eMTC support for Non-Terrestrial Networks” work item

[5]	RP-233261 “Summary for WI IoT-NTN Enhancements”, corresponding to the Rel-18 RAN2 led “IoT (Internet of Things) NTN (non-terrestrial network) enhancements” work item

END