## Compliance template for 3GPP 5G NR RIT (Release 15 and beyond)

The compliance templates from Report ITU-R M.2411 (2017) Section 5.2.4 corresponding to the proposed draft Revision for 5G[1] developed by 3GPP are provided. It includes one compliance template for 3GPP 5G SRIT (encompassing “NR” and “LTE”), and one compliance template for 3GPP 5G NR RIT.

This document provides the compliance template for the 3GPP 5G NR RIT. Given that the full context of the total terrestrial radio interface is considered to be the original submission and any previously approved updates as well as the proposed update, the tables have been updated on the basis of the 3GPP up to Release 18 specifications as per TSG Plenary #106 (Dec 2024). The proposed revision of M.2150 for the 3GPP 5G NR RIT continues to meet or improve upon the characteristics presented in Report ITU-R M.2483 sect. 5.2.2.

#### 5.2.4.1 	Compliance template for services

|  | Service capability requirements | Evaluator’s comments |
| --- | --- | --- |
| 5.2.4.1.1 | Support for wide range of services Is the proposal able to support a range of services across different usage scenarios (eMBB, URLLC, and mMTC)?: YES Specify which usage scenarios (eMBB, URLLC, and mMTC) the candidate RIT or candidate SRIT can support.(1) | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
| (1) Refer to the process requirements in IMT-2020/2. |  |  |

#### 5.2.4.2 	Compliance template for spectrum3

|  | Spectrum capability requirements |
| --- | --- |
| 5.2.4.2.1 | Frequency bands identified for IMT Is the proposal able to utilize at least one frequency band identified for IMT in the ITU Radio Regulations?: YES Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. The following frequency bands will be supported, in accordance with spectrum requirements defined by Report ITU-R M.2411-0. Introduction of other ITU-R IMT identified bands are not precluded in the future. 3GPP technologies are also defined as appropriate to operate in other frequency arrangements and bands. See table 1 for 450 – 7125 MHz (frequency range 1, FR1) and table 2 for 24250 – 71000 MHz (frequency range 2, FR2). Table 1:450 – 7125 MHz (FR1) based on table 5.2-1 in TS 38.101-1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high; Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high; Duplex Mode. n1; 1920 MHz – 1980 MHz; 2110 MHz – 2170 MHz; FDD. n2; 1850 MHz – 1910 MHz; 1930 MHz – 1990 MHz; FDD. n3; 1710 MHz – 1785 MHz; 1805 MHz – 1880 MHz; FDD. n5; 824 MHz – 849 MHz; 869 MHz – 894 MHz; FDD. n7; 2500 MHz – 2570 MHz; 2620 MHz – 2690 MHz; FDD. n8; 880 MHz – 915 MHz; 925 MHz – 960 MHz; FDD. n12; 699 MHz – 716 MHz; 729 MHz – 746 MHz; FDD. n13; 777 MHz – 787 MHz; 746 MHz – 756 MHz; FDD. n14; 788 MHz – 798 MHz; 758 MHz – 768 MHz; FDD. n18; 815 MHz – 830 MHz; 860 MHz – 875 MHz; FDD. n20; 832 MHz – 862 MHz; 791 MHz – 821 MHz; FDD. n2416; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n25; 1850 MHz – 1915 MHz; 1930 MHz – 1995 MHz; FDD. n26; 814 MHz – 849 MHz; 859 MHz – 894 MHz; FDD. n28; 703 MHz – 748 MHz; 758 MHz – 803 MHz; FDD. n2919; N/A; 717 MHz – 728 MHz; SDL. n303; 2305 MHz – 2315 MHz; 2350 MHz – 2360 MHz; FDD. n31; 452.5 MHz – 457.5 MHz; 462.5 MHz – 467.5 MHz; FDD. n34; 2010 MHz – 2025 MHz; 2010 MHz – 2025 MHz; TDD. n3810; 2570 MHz – 2620 MHz; 2570 MHz – 2620 MHz; TDD. n39; 1880 MHz – 1920 MHz; 1880 MHz – 1920 MHz; TDD. n40; 2300 MHz – 2400 MHz; 2300 MHz – 2400 MHz; TDD. n41; 2496 MHz – 2690 MHz; 2496 MHz – 2690 MHz; TDD. n48; 3550 MHz – 3700 MHz; 3550 MHz – 3700 MHz; TDD. n501; 1432 MHz – 1517 MHz; 1432 MHz – 1517 MHz; TDD. n51; 1427 MHz – 1432 MHz; 1427 MHz – 1432 MHz; TDD. n53; 2483.5 MHz – 2495 MHz; 2483.5 MHz – 2495 MHz; TDD. n54; 1670 MHz – 1675 MHz; 1670 MHz – 1675 MHz; TDD. n654; 1920 MHz – 2010 MHz; 2110 MHz – 2200 MHz; FDD. n666,7; 1710 MHz – 1780 MHz; 2110 MHz – 2200 MHz; FDD. n6719; N/A; 738 MHz – 758 MHz; SDL. n70; 1695 MHz – 1710 MHz; 1995 MHz – 2020 MHz; FDD. n71; 663 MHz – 698 MHz; 617 MHz – 652 MHz; FDD. n72; 451 MHz – 456 MHz; 461 MHz – 466 MHz; FDD. n74; 1427 MHz – 1470 MHz; 1475 MHz – 1518 MHz; FDD. n752,19; N/A; 1432 MHz – 1517 MHz; SDL. n7619; N/A; 1427 MHz – 1432 MHz; SDL. n7712; 3300 MHz – 4200 MHz; 3300 MHz – 4200 MHz; TDD. n78; 3300 MHz – 3800 MHz; 3300 MHz – 3800 MHz; TDD. n7917; 4400 MHz – 5000 MHz; 4400 MHz – 5000 MHz; TDD. n80; 1710 MHz – 1785 MHz; N/A; SUL. n81; 880 MHz – 915 MHz; N/A; SUL. n82; 832 MHz – 862 MHz; N/A; SUL. n83; 703 MHz – 748 MHz; N/A; SUL. n84; 1920 MHz – 1980 MHz; N/A; SUL. n85; 698 MHz – 716 MHz; 728 MHz – 746 MHz; FDD. n86; 1710 MHz – 1780 MHz; N/A; SUL. n89; 824 MHz – 849 MHz; N/A; SUL. n905; 2496 MHz – 2690 MHz; 2496 MHz – 2690 MHz; TDD. n919; 832 MHz – 862 MHz; 1427 MHz – 1432 MHz; FDD. n929; 832 MHz – 862 MHz; 1432 MHz – 1517 MHz; FDD. n939; 880 MHz – 915 MHz; 1427 MHz – 1432 MHz; FDD. n949; 880 MHz – 915 MHz; 1432 MHz – 1517 MHz; FDD. n958; 2010 MHz – 2025 MHz; N/A; SUL. n9715; 2300 MHz – 2400 MHz; N/A; SUL. n9815; 1880 MHz – 1920 MHz; N/A; SUL. n9922; 1626.5 MHz – 1660.5 MHz; N/A; SUL. n100; 874.4 MHz – 880 MHz; 919.4 MHz – 925 MHz; FDD. n10021; 874.4 MHz – 880 MHz; 919.4 MHz – 925 MHz; FDD. n10121; 1900 MHz – 1910 MHz; 1900 MHz – 1910 MHz; TDD. n10417,18; 6425 MHz – 7125 MHz; 6425 MHz – 7125 MHz; TDD. n105; 663 MHz – 703 MHz; 612 MHz – 652 MHz; FDD. n106; 896 MHz – 901 MHz; 935 MHz – 940 MHz; FDD. n1099; 703 MHz – 733 MHz; 1432 MHz – 1517 MHz; FDD. NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. NOTE 3: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 4: A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements. NOTE 5: Unless otherwise stated, the applicability of requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the requirements for Band n41. A UE supporting Band n90 shall also support band n41. NOTE 6: A UE that supports NR Band n66 shall receive in the entire DL operating band. NOTE 7: A UE that supports NR Band n66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_n66B and CA_n66(2A) in the current version of the specification. NOTE 8: This band is applicable in China only. NOTE 9: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 10: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. NOTE 12: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. NOTE 15: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 16: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. NOTE 17: For this band, CORESET#0 values from Table 13-5 or Table 13-6 in [8, TS 38.213] are applied regardless of the minimum channel bandwidth. NOTE 18: This band is applicable only in countries/regions designating this band for IMT licensed operation subject to country-specific conditions. NOTE 19: For SDL bands, downlink configuration for RRM performance testing is same as FDD. NOTE 20: Operating band n200 is a reserved value. NOTE 21: This band is applicable only in countries subject to ECC Decision (20)02 [16], for the FRMCS application. NOTE 22: UL operation in this band is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz.. Table 2: 24250 – 71000 MHz (FR2) based on table 5.2-1 in TS 38.101-2 NR operating band; Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL_low – FUL_high FDL_low – FDL_high; Duplex Mode. n257; 26500 MHz – 29500 MHz; TDD. n258; 24250 MHz – 27500 MHz; TDD. n259; 39500 MHz – 43500 MHz; TDD. n260; 37000 MHz – 40000 MHz; TDD. n261; 27500 MHz – 28350 MHz; TDD. n262; 47200 MHz – 48200 MHz; TDD. Note: Additional frequency bands can be introduced in the future in release independent manner. For ease of comparison, table 1 and table 2 highlights the changes with revision marks to the technology being submitted in IMT-2020/14(rev 2) characteristics template. |
| 5.2.4.2.2 | Higher Frequency range/band(s) Is the proposal able to utilize the higher frequency range/band(s) above 24.25 GHz?: YES Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. NOTE 1 – In the case of the candidate SRIT, at least one of the component RITs need to fulfil this requirement. The supported frequency bands above 24.25 GHz are provided in item 5.2.4.2.1, table 2 (FR2). |

#### 5.2.4.3 	Compliance template for technical performance3

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) | Category |  |  | Required value | Value(2) | Requirement met? | Comments (3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Usage scenario | Test environment | Downlink or uplink |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) | eMBB | Not applicable | Downlink | 20 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 10 |  | Yes |  |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) | eMBB | Not applicable | Downlink | 30 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 15 |  | Yes |  |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) | eMBB | Dense Urban – eMBB | Downlink | 100 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 50 |  | Yes |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) | eMBB | Indoor Hotspot – eMBB | Downlink | 0.3 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 0.21 |  | Yes |  |
|  |  |  | Downlink | 0.3 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 0.21 |  | Yes |  |
|  | eMBB | Dense Urban – eMBB | Downlink | 0.225 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 0.15 |  | Yes |  |
|  |  | Rural – eMBB | Downlink | 0.12 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 0.045 |  | Yes |  |
|  |  |  | Downlink | 0.12 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 0.045 |  | Yes |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) | eMBB | Indoor Hotspot – eMBB | Downlink | 9 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 6.75 |  | Yes |  |
|  |  |  | Downlink | 9 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 6.75 |  | Yes |  |
|  | eMBB | Dense Urban – eMBB | Downlink | 7.8 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 5.4 |  | Yes |  |
|  | eMBB | Rural – eMBB | Downlink | 3.3 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 1.6 |  | Yes |  |
|  |  |  | Downlink | 3.3 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 1.6 |  | Yes |  |
|  |  |  | Downlink | 3.3 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 1.6 |  | Yes |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) | eMBB | Indoor-Hotspot – eMBB | Downlink | 10 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Downlink | 10 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) | eMBB | Not applicable | Downlink | 4 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 4 |  | Yes |  |
|  | URLLC | Not applicable | Downlink | 1 |  | Yes |  |
|  |  |  | Uplink | 1 |  | Yes |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) | eMBB | Not applicable | Not applicable | 20 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  | URLLC | Not applicable | Not applicable | 20 |  | Yes |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) | mMTC | Urban Macro – mMTC | Uplink | 1 000 000 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink | 1 000 000 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
| 5.2.4.3.10 Energy efficiency (4.9) | eMBB | Not applicable | Not applicable | Capability to support a high sleep ratio and long sleep duration |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  |  |  |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
| 5.2.4.3.11 Reliability (4.10) | URLLC | Urban Macro –URLLC | Downlink | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Downlink |  |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink |  |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  | Uplink |  |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
| 5.2.4.3.12 Mobility classes (4.11) | eMBB | Indoor Hotspot – eMBB | Uplink | Stationary, Pedestrian |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  | eMBB | Dense Urban – eMBB | Uplink | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  | eMBB | Rural – eMBB | Uplink | Pedestrian, Vehicular, High speed vehicular |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) | eMBB | Indoor Hotspot – eMBB | Uplink | 1.5 (10 km/h) |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  |  | 1.5 (10 km/h) |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  | eMBB | Dense Urban – eMBB | Uplink | 1.12 (30 km/h) |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  |  | 1.12 (30 km/h) |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  | eMBB | Rural – eMBB | Uplink | 0.8 (120 km/h) |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  |  | 0.45 (500 km/h) |  | Yes |  |
|  |  |  |  | 0.8 (120 km/h) |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  |  | 0.45 (500 km/h) |  | Yes |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) | eMBB and URLLC | Not applicable | Not applicable | 0 |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) | Not applicable | Not applicable | Not applicable | At least 100 MHz |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  |  | Up to 1 GHz |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
|  |  |  |  | Support of multiple different bandwidth values(4) |  | Yes | Improves on the technology originally being submitted in IMT-2020/14 and therefore the conclusions expressed in ITU-R Report M.2483 sect. 5.2.2 remains valid. |
| (1) As defined in Report ITU-R M.2410-0. (2) According to the evaluation methodology specified in Report ITU-R M.2412-0. (3) Proponents should report their selected evaluation methodology of the Connection density, the channel model variant used, and evaluation configuration(s) with their exact values (e.g. antenna element number, bandwidth, etc.) per test environment, and could provide other relevant information as well. For details, refer to Report ITU-R M.2412-0, in particular, § 7.1.3 for the evaluation methodologies, § 8.4 for the evaluation configurations per each test environment, and Annex 1 on the channel model variants. (4) Refer to § 7.3.1 of Report ITU-R M.2412-0. |  |  |  |  |  |  |  |

[1] Developed by 3GPP as 5G, Release 15 and beyond.

If a proponent determines that a specific question does not apply, the proponent should indicate that this is the case and provide a rationale for why it does not apply.