Annex 2

Specification of the 3GPP 5G – RIT radio interface technology

Background

IMT-2020 is a system with global development activity and the IMT-2020 terrestrial radio interface specifications identified in this Recommendation have been developed by the ITU in collaboration with the GCS Proponents and the Transposing Organizations. It is noted from Document IMT-2020/20, that:

–	The GCS Proponent must be one of the RIT/SRIT Proponents for the relevant technology, and must have legal authority to grant to ITU-R the relevant legal usage rights to the relevant specifications provided within a GCS corresponding to a technology in Recommendation ITU-R M.[IMT-2020.SPECS].

–	A Transposing Organization must have been authorized by the relevant GCS Proponent to produce transposed standards for a particular technology and must have the relevant legal usage rights.

It is further noted that GCS Proponents and Transposing Organizations must also qualify appropriately under the auspices of Resolution ITU-R 9 and the ITU-R “Guidelines for the contribution of material of other organizations to the work of the Study Groups and for inviting other organizations to take part in the study of specific matters (Resolution ITU-R 9)”.

The ITU has provided the global and overall framework and requirements and has developed the Global Core Specification jointly with the GCS Proponent. The detailed standardization has been undertaken within the recognized Transposing Organizations which operate in concert with the GCS Proponent. This Recommendation therefore makes extensive use of references to externally developed specifications.

This approach was considered to be the most appropriate solution to enable completion of this Recommendation within the aggressive schedules set by the ITU and by the needs of administrations, operators and manufacturers.

This Recommendation has therefore been constructed to take full advantage of this method of work and to allow the global standardization timescales to be maintained. The main body of this Recommendation has been developed by the ITU, with each Annex containing references pointing to the location of the more detailed information.

This Annex 2 contains the detailed information developed by the ITU and “3GPP” (the GCS Proponent) and ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC (the Transposing Organizations).

Such use of referencing enables timely completion and update of the high-level elements of this Recommendation, with change control procedures, transposition, and public enquiry procedures being undertaken within the external organization. This information has generally been adopted unchanged, recognizing the need to minimize duplication of work, and the need to facilitate and support an ongoing maintenance and update process.

This general agreement, noting that the detailed information of the radio interface should to a large extent be achieved by reference to the work of external organizations, highlights not only the ITU’s significant role as a catalyst in stimulating, coordinating and facilitating the development of advanced telecommunications technologies, but also its forward-looking and flexible approach to the development of this and other telecommunications standards for the 21st century.

A more detailed understanding of the process for the development of the first release of this Recommendation may be found in Document IMT-2020/20.

## 2.1	Overview of the radio interface technology

The IMT-2020 specifications known as 5G have been developed by 3GPP and encompass NR Releases 15 and beyond.

New Radio (NR) is designed for operation in IMT spectrum and fulfils all technical performance requirements in all five selected test environments: Indoor Hotspot – enhanced Mobile Broadband (eMBB), Dense Urban – eMBB, Rural – eMBB, Urban Macro – Ultra Reliable Low Latency Communication (URLLC) and Urban Macro – massive Machine Type Communication (mMTC).

Also, NR fulfils the service and the spectrum requirements. NR utilizes the frequency bands below 7.125 GHz including frequency bands identified for International Mobile Telecommunication (IMT) in the ITU Radio Regulations. In addition, NR can also utilize the frequency bands above 7.125 GHz, i.e. above 24.25 GHz, including frequency bands identified for IMT in the ITU Radio Regulations.

The complete set of standards for the terrestrial radio interface of IMT-2020 identified as 3GPP 5G – RIT. NR − includes not only the key characteristics of IMT-2020 but also the additional capabilities of NR both of which are continuing to be enhanced.

The 3GPP 5G System (5GS) also includes specifications for its non-radio aspects, such as the core network elements (the Enhanced Packet Core (EPC) Network and 5G Core (5GC) Network), security, codecs, network management, etc. These non-radio specifications are not included in the so-called “Global Core Specifications (GCS)” of IMT-2020.

### 2.1.1	Overview of the system aspects of the NR RIT

The NR RIT represents the releases 15, 16, 17 and 18 of NR, which uses either 1) FDD operation and therefore is applicable for operation with paired spectrum or 2) TDD operation and therefore is applicable for operation with unpaired spectrum. Channel bandwidths up to 400 MHz and Carrier Aggregation over 16 component carriers are supported, yielding peak data rates up to roughly 140 Gbit/s in the downlink and 65 Gbit/s in the uplink.

#### 2.1.1.1	Overall architecture

The NG Radio Access Network (NG-RAN) includes NG-RAN nodes that support multiple radio access (e.g. NR, MR-DC of NR and E-UTRA, etc). The RIT considers the gNB as an NG-RAN node providing NR user plane and control plane protocol terminations towards the UE and connected via the NG interface to the 5GC, and the ng-eNB as NG-RAN node for Multi-Radio Dual Connectivity only.

NG-RAN nodes are interconnected by means of the interface named Xn. The gNBs and ng-eNBs are also connected by means of the NG interfaces to the 5GC, more specifically to the Access and Mobility Management Function (AMF) by means of the NG-C interface and to the User Plane Function (UPF) by means of the NG-U interface.

The NG-RAN architecture is illustrated in Fig. 28 below.

Figure 28

Overall Architecture

#### 2.1.1.2	Radio protocol architecture

##### 2.1.1.2.1	User Plane (UP)

Figure 29 shows the protocol stack for the User Plane, where Service Data Adaptation Protocol (SDAP), PDCP, RLC and MAC sublayers (terminated in gNB on the network side) perform the functions listed in § 2.1.1.5.

Figure 29

User Plane protocol stack

##### 2.1.1.2.2	Control Plane

Figure 30 below shows the protocol stack for the control plane, where:

−	PDCP, RLC and MAC sublayers (terminated in gNB on the network side) perform the functions listed in § 2.1.1.5;

−	RRC (terminated in gNB on the network side) performs the functions listed in § 2.1.1.6;

−	Non-Access Stratum (NAS) control protocol (terminated in AMF on the network side) performs the functions listed in 3GPP TS 23.501, for instance: authentication, mobility management, security control.

Figure 30

Control Plane protocol stack

##### 2.1.1.2.3	Integrated access and Backhaul (IAB) architecture

From NR Rel-16, Integrated access and backhaul (IAB) enables wireless relaying in NG-RAN. The relaying node, referred to as the IAB-node, supports access and backhauling via NR. The terminating node of NR backhauling on the network side is referred to as the IAB-donor, which represents a gNB with additional functionality to support IAB. Backhauling can occur via a single or via multiple hops. The IAB architecture is shown in Fig. 31.

Figure 31

IAB architecture; a) IAB-node using SA mode with 5GCN; b) IAB-node using EN-DC

Mobile IAB introduces the mobile IAB-node, which is a RAN node that provides NR access links to UEs and an NR backhaul link to a parent node, and that can conduct physical mobility across the RAN area. The mobile IAB-node includes a mobile IAB-MT and a mobile IAB-DU. Mobile IAB supports the same functionality as IAB unless explicitly specified.

##### 2.1.2.2.4	Network-Controlled Repeaters (NCR) architecture

A Network-Controlled Repeater node, referred to as NCR-node, is an RF repeater that enables wireless amplifying-and-forwarding functionality in NG-RAN. The NCR-node is capable of receiving and applying side control information from a gNB with additional functionality to support Network-Controlled Repeater.

The NCR-node comprises an NCR-MT and an NCR-Fwd. The NCR-MT is an entity supporting a subset of the UE functionality that communicates with the gNB to receive side control information via a control link based on the NR Uu interface. The NCR-Fwd is the function performing amplifying-and-forwarding of signals between gNB and UE via the NCR-Fwd backhaul link and NCR-Fwd access link, respectively. The NCR-node is modelled as depicted in Figure 4.9.1-1.

Figure 4.9.1-1: Conceptual model of network-controlled repeater.

#### 2.1.1.3	Multi-Radio Dual Connectivity (MR-DC)

NG-RAN supports Multi-Radio Dual Connectivity (MR-DC) operation whereby a UE in the RRC_CONNECTED state is configured to utilise radio resources provided by two distinct schedulers, located in two different NG-RAN nodes connected via a non-ideal backhaul, one providing NR access and the other one providing either E-UTRA or NR access. In MR-DC, one NG-RAN node acts as a Master Node (MN) and another NG-RAN node acts as a Secondary Node (SN).

NR may also be used as part of MR-DC configuration when combined with E-UTRA, within either an NG-RAN or E-UTRAN. For the case of NR-NR Dual Connectivity (NR-DC), a UE is connected to one gNB that acts as a MN and another gNB that acts as a SN. The master gNB is connected to the 5GC via the NG interface and to the secondary gNB via the Xn interface. The secondary gNB might also be connected to the 5GC via the NG-U interface.

#### 2.1.1.4	Physical layer

##### 2.1.1.4.1	Waveform, numerology and frame structure

The downlink transmission waveform is conventional OFDM using a cyclic prefix. The uplink transmission waveform is conventional OFDM using a cyclic prefix with a transform precoding function performing DFT spreading that can be disabled or enabled.

Figure 32

Transmitter block diagram for CP-OFDM with optional DFT-spreading

The numerology is based on exponentially scalable sub-carrier spacing f = 2µ × 15 kHz with µ = {0,1,3,4,5,6} for Primary Synchronization Signal (PSS), Secondary Synchronization Signal (SSS) and PBCH and µ = {0,1,2,3,5,6} for other channels. Normal Cyclic Prefix (CP) is supported for all sub-carrier spacings, Extended CP is supported for µ = 2. 12 consecutive sub-carriers form a Physical Resource Block (PRB). Up to 275 PRBs are supported on a carrier.

TABLE 4

Supported transmission numerologies

| µ | f = 2µ × 15 [kHz] | Cyclic prefix | Supported for data | Supported for synch |
| --- | --- | --- | --- | --- |
| 0 | 15 | Normal | Yes | Yes |
| 1 | 30 | Normal | Yes | Yes |
| 2 | 60 | Normal, Extended | Yes | No |
| 3 | 120 | Normal | Yes | Yes |
| 4 | 240 | Normal | No | Yes |
| 5 | 480 | Normal | Yes | Yes |
| 6 | 960 | Normal | Yes | Yes |

The UE may be configured with one or more bandwidth parts on a given component carrier, of which only one can be active at a time, as described in § 2.1.1.5.10. The active bandwidth part defines the UE’s operating bandwidth within the cell’s operating bandwidth. For initial access, and until the UEʼs configuration in a cell is received, the initial bandwidth part detected from system information is used.

Downlink and uplink transmissions are organized into frames of 10 ms duration, consisting of ten 1 ms subframes. Each frame is divided into two equally sized half-frames of five subframes. The slot duration is 14 symbols with Normal CP and 12 symbols with Extended CP, and scales in time as a function sub-carrier spacing in use so that there is always an integral number of slots in a subframe.

Timing Advance (TA) is used to adjust the uplink frame timing relative to the downlink frame timing.

Figure 33

Uplink-downlink timing relation

Operation on both paired and unpaired spectrum is supported.

##### 2.1.1.4.2	Downlink

###### 2.1.1.4.2.1	Downlink transmission scheme

Demodulation Reference Signal (DMRS) based spatial multiplexing is supported for Physical Downlink Shared Channel (PDSCH). Up to 8, 12, 16 and 24 orthogonal DL DMRS ports are supported for type 1, type 2, enhanced type 1 and enhanced type 2 DMRS respectively. Up to 8 orthogonal DL DMRS ports per UE are supported for Single-User MIMO (SU-MIMO) and up to 4 orthogonal DL DMRS ports per UE are supported for Multi-User MIMO (MU-MIMO). The number of SU-MIMO code words is one for 1-4 layers transmissions and two for 5-8 layers transmissions.

The DMRS and corresponding PDSCH are transmitted using the same precoding matrix and the UE does not need to know the precoding matrix to demodulate the transmission. The transmitter may use different precoder matrix for different parts of the transmission bandwidth, resulting in frequency selective precoding. The UE may also assume that the same precoding matrix is used across a set of Physical Resource Blocks (PRBs) denoted Precoding Resource Block Group (PRG).

Transmission durations from 2 to 14 symbols in a slot with a single PDSCH is supported.

Aggregation of multiple slots with Transport Block (TB) repetition is supported.

From Rel-16, enhancements for DL/UL MIMO are introduced, including enhancements on multi-Transmission Reception Points (TRPs) or multi-panel transmission improving reliability and robustness with both ideal and non-ideal backhaul.

###### 2.1.1.4.2.2	Physical-layer processing for physical downlink shared channel

The downlink physical-layer processing of transport channels consists of the following steps:

−	TB CRC attachment;

−	code block segmentation and code block CRC attachment;

−	channel coding: Low Density Parity Check (LDPC) coding;

−	physical-layer hybrid-ARQ processing;

−	rate matching;

−	scrambling;

−	modulation: QPSK, 16QAM, 64QAM, 256QAM and 1024QAM;

−	layer mapping;

−	mapping to assigned resources and antenna ports.

The UE may assume that at least one symbol with demodulation reference signal is present on each layer in which PDSCH is transmitted to a UE, and up to three additional DMRS symbols can be configured by higher layers.

Phase tracking RS may be transmitted on additional symbols to aid receiver phase tracking.

###### 2.1.1.4.2.3	Physical downlink control channels

The Physical Downlink Control Channel (PDCCH) can be used to schedule DL transmissions on PDSCH and UL transmissions on PUSCH, where the Downlink Control Information (DCI) on PDCCH includes:

−	downlink assignments containing at least modulation and coding format, resource allocation, and HARQ information related to DL-SCH;

−	uplink scheduling grants containing at least modulation and coding format, resource allocation, and HARQ information related to UL-SCH.

In addition to scheduling, PDCCH can be used for

−	activation and deactivation of configured PUSCH transmission with configured grant;

−	activation and deactivation of PDSCH semi-persistent transmission;

−	notifying one or more UEs of the slot format;

−	notifying one or more UEs of the PRB(s) and OFDM symbol(s) where the UE may assume no transmission is intended for the UE;

−	transmission of Transmit Power Control (TPC) commands for PUCCH and PUSCH;

−	transmission of one or more TPC commands for Sounding Reference Signal (SRS) transmissions by one or more UEs;

−	Switching a UE’s active bandwidth part;

−	Initiating a random access procedure;

−	Indicating the UE(s) to monitor the PDCCH during the next occurrence of the DRX on-duration;

−	In IAB context, indicating the availability for soft symbols of an IAB-DU;

−	Triggering one shot HARQ-ACK codebook feedback.

A UE monitors a set of PDCCH candidates in the configured monitoring occasions in one or more configured COntrol REsource SETs (CORESETs) according to the corresponding search space configurations.

A CORESET consists of a set of PRBs with a time duration of 1 to 3 OFDM symbols. The resource units Resource Element Groups (REGs) and Control Channel Elements (CCEs) are defined within a CORESET with each CCE consisting a set of REGs. Control channels are formed by aggregation of CCEs. Different code rates for the control channels are realized by aggregating different number of CCEs. Interleaved and non-interleaved CCE-to-REG mapping are supported in a CORESET.

The PDCCH repetition is operated by using two search spaces which are explicitly linked by configuration provided by the RRC layer, and are associated with corresponding CORESETs. For PDCCH repetition, two linked search spaces are configured with the same number of candidates, and two PDCCH candidates in two search spaces are linked with the same candidate index. When PDCCH repetition is scheduled to a UE, an intra-slot repetition is allowed and each repetition has the same number of CCEs and coded bits, and corresponds to the same DCI payload.

Polar coding is used for PDCCH.

Each resource element group carrying PDCCH carries its own DMRS.

QPSK modulation is used for PDCCH.

###### 2.1.1.4.2.4	Synchronization signal and PBCH block

The Synchronization Signal and PBCH block (SSB) consists of primary and secondary synchronization signals (PSS, SSS), each occupying 1 symbol and 127 subcarriers, and PBCH spanning across three OFDM symbols and 240 subcarriers, but on one symbol leaving an unused part in the middle for SSS as show in Fig. 34. For the 3 MHz channel bandwidth, the PBCH is further equally punctured from both edges to span 144 subcarriers. The possible time locations of SSBs within a half-frame are determined by sub-carrier spacing, and the periodicity of the half-frames where SSBs are transmitted is configured by the network. During a half-frame, different SSBs may be transmitted in different spatial directions (i.e. using different beams, spanning the coverage area of a cell).

Within the frequency span of a carrier, multiple SSBs can be transmitted. The Physical Cell Identifiers (PCIs) of SSBs transmitted in different frequency locations do not have to be unique, i.e. different SSBs in the frequency domain can have different PCIs. However, when an SSB is associated with a Remaining Minimum System Information (RMSI), the SSB corresponds to an individual cell, which has a unique NR Cell Global Identifier (NCGI). Such an SSB is referred to as a Cell-Defining SSB (CD-SSB). A PCell is always associated to a CD-SSB located on the synchronization raster.

When an SSB is not associated with an RMSI, the SSB is referred to as a non-Cell Defining SSB (NCD-SSB), which can be used to perform Radio Link Monitoring (RLM), Beam Failure Detection (BFD), and RRM measurements and measurements for RA resource selection inside the active DL BWP when the active BWP does not contain the CD-SSB. A UE may be configured with multiple SSBs provided that each BWP is configured with at most one SSB (CD-SSB or NCD-SSB).

Figure 34

Time-frequency structure of SSB

Polar coding is used for the PBCH.

The UE may assume a band-specific sub-carrier spacing for the SSB unless a network has configured the UE to assume a different sub-carrier spacing.

PBCH symbols carry their own frequency-multiplexed DMRS.

QPSK modulation is used for PBCH.

###### 2.1.1.4.2.5	Physical layer procedures

2.1.1.4.2.5.1	Link adaptation

Link adaptation (Adaptive Modulation and Coding (AMC)) with various modulation schemes and channel coding rates is applied to the PDSCH. The same coding and modulation is applied to all groups of resource blocks belonging to the same L2 Protocol Data Unit (PDU) scheduled for one user within one transmission duration and within a MIMO codeword.

For channel state estimation purposes, the UE may be configured to measure CSI-RS and estimate the downlink channel state based on the CSI-RS measurements. The UE reports the estimated channel state back to the gNB to be used in link adaptation.

2.1.1.4.2.5.2	Power control

Downlink power control can be used.

2.1.1.4.2.5.3	Cell search

Cell search is the procedure by which a UE acquires time and frequency synchronization with a cell and detects the Cell ID of that cell. NR cell search is based on the primary and secondary synchronization signals, and PBCH DMRS, located on the synchronization raster.

2.1.1.4.2.5.4	HARQ

Asynchronous Incremental Redundancy Hybrid ARQ is supported. The gNB provides the UE with the HARQ-ACK feedback timing either dynamically in the DCI or semi-statically in an RRC configuration.

The UE may be configured to receive code block group-based transmissions where retransmissions may be scheduled to carry only a sub-set of all the code blocks of a TB.

2.1.1.4.2.5.5	Reception of SIB1

The Master Information Block (MIB) on the PBCH provides the UE with parameters (e.g. CORESET#0 configuration) for monitoring of the PDCCH for scheduling PDSCH that carries the System Information Block 1 (SIB1). The PBCH may also indicate that there is no associated SIB1, in which case the UE may be pointed to another frequency from where to search for an SSB that is associated with a SIB1 as well as a frequency range where the UE may assume no SSB associated with SIB1 is present. The indicated frequency range is confined within a contiguous spectrum allocation of the same operator in which SSB is detected.

##### 2.1.1.4.3	Uplink

###### 2.1.1.4.3.1	Uplink transmission scheme

Two transmission schemes are supported for PUSCH: codebook-based transmission and non-codebook-based transmission.

For codebook-based transmission, the gNB provides the UE with a transmit precoding matrix indication in the DCI. The UE uses the indication to select the PUSCH transmit precoder from the codebook. For non-codebook-based transmission, the UE determines its PUSCH precoder based on the wideband SRS Resource Indicator (SRI) field from the DCI.

DMRS based spatial multiplexing is supported for PUSCH. For a given UE, up to four-layer transmissions or up to eight-layer are supported. Up to 8, 12, 16, and 24 orthogonal UL DMRS ports are supported for type 1, type 2, enhanced type 1, and enhanced type 2 DMRS respectively. The number of codewords is one for 1 to 4 layer transmission and two for 5 to 8 layer transmission. When transform precoding is used, only a single MIMO layer transmission is supported.

Transmission durations from 1 to 14 symbols in a slot with a single PUSCH is supported.

Aggregation of multiple slots with TB repetition is supported.

Two types of frequency hopping are supported, intra-slot frequency hopping, and in case of slot aggregation, inter-slot frequency hopping.

The PUSCH may be scheduled with DCI on the PDCCH, or a semi-static configured grant may be provided over RRC, where two types of operation are supported:

−	The first PUSCH is triggered with a DCI, with subsequent PUSCH transmissions following the RRC configuration and scheduling received on the DCI, or

−	The PUSCH is triggered by data arrival in the UE’s transmit buffer and the PUSCH transmissions follow the RRC configuration.

###### 2.1.1.4.3.2	Physical-layer processing for physical uplink shared channel

The uplink physical-layer processing of transport channels consists of the following steps:

−	Transport Block CRC attachment;

−	code block segmentation and Code Block CRC attachment;

−	channel coding: LDPC coding;

−	physical-layer HARQ processing;

−	rate matching;

−	scrambling;

−	modulation: π/2 BPSK (with transform precoding only), QPSK, 16QAM, 64QAM and 256QAM;

−	layer mapping, transform precoding (enabled/disabled by configuration), and pre-coding;

−	mapping to assigned resources and antenna ports.

The UE transmits at least one symbol with Demodulation Reference Signal on each layer on each frequency hop in which the PUSCH is transmitted, and up to three additional DMRS symbols can be configured by higher layers.

Phase tracking RS may be transmitted on additional symbols to aid receiver phase tracking.

###### 2.1.1.4.3.3	Physical uplink control channel

The physical uplink control channel (PUCCH) carries the Uplink Control Information (UCI) from the UE to the gNB. Five formats of PUCCH exist, depending on the duration of PUCCH and the UCI payload size.

−	format #0: short PUCCH of 1 or 2 symbols with small UCI payloads of up to two bits with UE multiplexing capacity of up to 6 UEs with 1-bit payload in the same PRB;

−	format #1: long PUCCH of 4 to 14 symbols with small UCI payloads of up to two bits with UE multiplexing capacity of up to 84 UEs without frequency hopping and 36 UEs with frequency hopping in the same PRB;

−	format #2: short PUCCH of 1 or 2 symbols with large UCI payloads of more than two bits with no UE multiplexing capability in the same PRBs;

−	format #3: long PUCCH of 4 to 14 symbols with large UCI payloads with no UE multiplexing capability in the same PRBs;

−	format #4: long PUCCH of 4 to 14 symbols with moderate UCI payloads with multiplexing capacity of up to 4 UEs in the same PRBs.

The short PUCCH format of up to two UCI bits is based on sequence selection, while the short PUCCH format of more than two UCI bits frequency-multiplexes the UCI and the DMRS. The long PUCCH formats time-multiplex the UCI and DMRS. Frequency hopping is supported for long PUCCH formats and for short PUCCH formats of duration of two symbols. Short and long PUCCH formats can be repeated over multiple slots or sub-slots, where the repetition factor is either indicated dynamically in the DCI or semi-statically in an RRC configuration. UCI multiplexing in the PUSCH is supported when UCI and PUSCH transmissions coincide in time, either due to transmission of a UL-SCH transport block or due to triggering of A-CSI transmission without UL-SCH transport block. In addition, HARQ-ACK multiplexing of a certain priority in PUSCH of a different priority is supported when HARQ-ACK and PUSCH transmissions coincide in time, either due to transmission of a UL-SCH transport block or due to triggering of A-CSI transmission without UL-SCH transport block:

−	UCI carrying HARQ-ACK feedback with 1 or 2 bits is multiplexed by puncturing the PUSCH;

−	In all other cases the UCI is multiplexed by rate matching the PUSCH.

UCI consists of the following information:

−	CSI;

−	ACK/NAK;

−	Scheduling request.

Simultaneous transmission of PUCCH and PUSCH associated with different priorities on cells of different bands in a PUCCH group is supported, where UCI multiplexing in the PUCCH associated with a priority in combination of UCI multiplexing in a PUSCH associated with a different priority is supported if the UCI multiplexed on PUSCH is of same priority as the PUSCH.

Simultaneous transmission of PUCCH and PUSCH associated with same priority on cells of different bands in a PUCCH group is supported.

QPSK and π/2 BPSK modulation can be used for long PUCCH with more than 2 bits of information, QPSK is used for a short PUCCH with more than 2 bits of information and BPSK and QPSK modulation can be used for a long PUCCH with up to 2 information bits.

Transform precoding is applied to a long PUCCH.

Channel coding used for uplink control information is described in Table 5.

TABLE 5

Channel coding for uplink control information

| Uplink control information size including CRC, if present | Channel code |
| --- | --- |
| 1 | Repetition code |
| 2 | Simplex code |
| 3-11 | Reed Muller code |
| >11 | Polar code |

###### 2.1.1.4.3.4	Random access

Random access preamble sequences of different lengths are supported. The sequence length 839 is applied with subcarrier spacings of 1.25 and 5 kHz, and the sequence length 139 is applied with subcarrier spacings of 15, 30, 60, 120, 480 and 960 kHz, sequence length of 571 is applied with subcarrier spacings of 30, 120, and 480 kHz, and sequence length 1151 is applied with subcarrier spacings of 15 and 120 kHz respectively. Sequence length 839 supports unrestricted sets and restricted sets of Type A and Type B, while sequence lengths 139, 571 can be used for operation with only with subcarrier spacings of 120 kHz and 480 kHz and sequence lengths of 1151 support unrestricted sets only with subcarrier spacings of 120 kHz.

Multiple PRACH preamble formats are defined with one or more PRACH OFDM symbols, and different cyclic prefixes and guard times. The PRACH preamble configuration to use is provided to the UE in the system information.

The UE calculates the PRACH transmit power for the retransmission of the preamble based on the most recent estimated pathloss and the power ramping counter value.

The system information provides information for the UE to determine the association between the SSB and the RACH resources. The Reference Signal Received Power (RSRP) threshold for SSB selection for RACH resource association is configurable by the network.

###### 2.1.1.4.3.5	Physical layer procedures

2.1.1.4.3.5.1	Link adaptation

Four types of link adaptation are supported as follows:

−	adaptive transmission bandwidth;

−	adaptive transmission duration;

−	transmission power control;

−	adaptive modulation and channel coding rate.

For channel state estimation purposes, the UE may be configured to transmit SRS that the gNB may use to estimate the uplink channel state, and then use the estimate in link adaptation.

2.1.1.4.3.5.2	Uplink power control

The gNB determines the desired uplink transmit power and provides uplink transmit power control commands to the UE. The UE uses the provided uplink transmit power control commands to adjust its transmit power.

2.1.1.4.3.5.3	Uplink timing control

The gNB (including IAB-DU and IAB-donor-DU) determines the desired Timing Advance setting and provides that to the UE (or IAB-MT). The UE/IAB-MT uses the provided TA to determine its uplink transmit timing relative to the UE’s/IAB-MTs observed downlink receive timing.

An IAB-node may support additional modes for uplink timing:

–	The IAB-MT uses the provided TA plus a provided additional offset to determine its uplink transmission timing, to facilitate parent node’s IAB-MT Rx / IAB-DU Rx multiplexing;

–	The IAB-MT aligns its uplink transmission timing to that of the collocated IAB-DU downlink transmission timing, to facilitate IAB-MT Tx / IAB-DU Tx multiplexing of this IAB-node.

The IAB-node uplink timing mode is indicated by the parent node via MAC-CE.2.1.1.4.3.5.4HARQ

Asynchronous Incremental Redundancy Hybrid ARQ is supported. The gNB schedules each uplink transmission and retransmission using the uplink grant on the DCI.

The UE may be configured to transmit code block group-based transmissions where retransmissions may be scheduled to carry only a sub-set of all the code blocks of a transport block.

##### 2.1.1.4.4	Carrier Aggregation (CA)

In Carrier Aggregation (CA), two or more Component Carriers (CCs) are aggregated. A UE may simultaneously receive or transmit on one or multiple CCs depending on its capabilities:

−	a UE with single timing advance capability for CA can simultaneously receive and/or transmit on multiple CCs corresponding to multiple serving cells sharing the same timing advance (multiple serving cells grouped in one Timing Advance Group (TAG));

−	a UE with multiple timing advance capability for CA can simultaneously receive and/or transmit on multiple CCs corresponding to multiple serving cells with different timing advances (multiple serving cells grouped in multiple TAGs). NG-RAN ensures that each TAG contains at least one serving cell;

−	a non-CA capable UE can receive on a single CC and transmit on a single CC corresponding to one serving cell only (one serving cell in one TAG).

CA is supported for both contiguous and non-contiguous CCs. When CA is deployed, frame timing and System Frame Number (SFN) are aligned across cells that can be aggregated. The maximum number of configured CCs for a UE is 16 for DL and 16 for UL. From Rel-16, cells with unaligned frame boundary can also be aggregated.

Multi-cell PDSCH/PUSCH scheduling with a single DCI is supported in order to reduce the control overhead for scheduling data on multiple cells in intra-band or inter-band multi-carrier operation.

##### 2.1.1.4.5	Supplementary uplink

In conjunction with a UL/DL carrier pair (FDD band) or a bidirectional carrier (TDD band), a UE may be configured with additional, Supplementary Uplink (SUL). SUL differs from the aggregated uplink in that the UE may be scheduled to transmit either on the supplementary uplink or on the uplink of the carrier being supplemented, but not on both at the same time.

##### 2.1.1.4.6	Uplink Tx switching

In uplink CA or SUL, a UE configured with uplink Tx switching can have Tx chain(s) dynamically from one uplink band or two uplink bands to another uplink band or two uplink bands for enabling up to 2Tx UL transmission in one uplink band or simultaneous UL transmissions in two uplink bands at a time.

##### 2.1.1.4.7	Group transmission for Multicast and Broadcast Services (MBS) delivery

Resource efficient delivery of multicast/broadcast services is introduced for services of Point-To-Multipoint nature, such as for example public safety and mission critical services, V2X applications, IPTV, live video, software delivery over wireless and IoT applications.

For broadcast communication service, the same service and the same specific content data are provided simultaneously to all UEs in a geographical area. A broadcast communication service is delivered to the UEs using a broadcast session. A UE can receive a broadcast communication service in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state.

For multicast communication service, the same service and the same specific content data are provided simultaneously to a dedicated set of UEs. A multicast communication service is delivered to the UEs using a multicast session. A UE can receive a multicast communication service in RRC_CONNECTED state with mechanisms such as Point-To-Point (PTP) and/or Point-To-Multipoint (PTM) delivery. HARQ feedback/retransmission can be applied to both PTP and PTM transmission.

##### 2.1.1.4.8	Transport channels

The physical layer offers information transfer services to MAC and higher layers. The physical layer transport services are described by how and with what characteristics data are transferred over the radio interface.

Downlink transport channel types are:

1	Broadcast Channel (BCH) characterised by:

−	fixed, pre-defined transport format;

−	requirement to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different BCH instances.

2	Downlink Shared Channel (DL-SCH) characterised by:

−	support for HARQ;

−	support for dynamic link adaptation by varying the modulation, coding and transmit power;

−	possibility to be broadcast in the entire cell;

−	possibility to use beamforming;

−	support for both dynamic and semi-static resource allocation;

−	support for UE discontinuous reception (DRX) to enable UE power saving;

−	support for group transmission for MBS delivery.

3	Paging Channel (PCH) characterised by:

−	support for UE discontinuous reception (DRX) to enable UE power saving (DRX cycle is indicated by the network to the UE);

−	requirement to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different PCH instances;

−	mapped to physical resources which can be used dynamically also for traffic/other control channels.

Uplink transport channel types are:

1	Uplink Shared Channel (UL-SCH) characterised by:

−	possibility to use beamforming;

−	support for dynamic link adaptation by varying the transmit power and potentially modulation and coding;

−	support for HARQ;

−	support for both dynamic and semi-static resource allocation.

2	Random Access Channel(s) (RACH) characterised by:

−	limited control information;

−	collision risk.

Sidelink transport channel types are:

1	Sidelink broadcast channel (SL-BCH) characterised by:

−	pre-defined transport format.

2	Sidelink shared channel (SL-SCH) characterised by:

−	support for unicast transmission, groupcast transmission and broadcast transmission;

−	support for both UE autonomous resource selection and scheduled resource allocation by NG-RAN;

−	support for both dynamic and semi-static resource allocation when UE is allocated resources by the NG-RAN;

−	support for HARQ;

−	support for dynamic link adaptation by varying the transmit power, modulation and coding;

−	support for SL discontinuous reception (SL DRX) to enable UE power saving.

#### 2.1.1.5	Layer 2

##### 2.1.1.5.1	Overview

The layer 2 of NR is split into the following sublayers: Medium Access Control (MAC), Radio Link Control (RLC), Packet Data Convergence Protocol (PDCP) and Service Data Adaptation Protocol (SDAP). Figures 35 and 36 depict the Layer 2 architecture for downlink and uplink, where:

−	the physical layer offers to the MAC sublayer transport channels;

−	the MAC sublayer offers to the RLC sublayer logical channels;

−	the RLC sublayer offers to the PDCP sublayer RLC channels;

−	the PDCP sublayer offers to the SDAP sublayer radio bearers;

−	the SDAP sublayer offers to 5GC QoS flows;

−	Comp. refers to header compression or uplink data compression;

−	Segm. refers to segmentation;

−	control channels (BCCH, PCCH are not depicted for clarity).

NOTE – The gNB may not be able to guarantee that a L2 buffer overflow will never occur. If such overflow occurs, the UE may discard packets from the L2 buffer.

Figure 35

Downlink layer 2 structure

Figure 36

Uplink layer 2 structure

Similar to LTE, Radio bearers are categorized into two groups: DRBs for UP data and SRBs for CP data.

##### 2.1.1.5.2	MAC Sublayer

###### 2.1.1.5.2.1	Services and Functions

The main services and functions of the MAC sublayer are:

−	mapping between logical channels and transport channels;

−	multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;

−	scheduling information reporting;

−	error correction through HARQ (one HARQ entity per cell in case of CA);

−	priority handling between UEs by means of dynamic scheduling;

−	priority handling between logical channels of one UE by means of logical channel prioritisation;

−	padding.

A single MAC entity can support multiple numerologies, transmission timings and cells. Mapping restrictions in logical channel prioritisation control which numerology(ies), cell(s), and transmission timing(s) a logical channel can use.

###### 2.1.1.5.2.2	Logical channels

Different kinds of data transfer services as offered by MAC. Each logical channel type is defined by what type of information is transferred. Logical channels are classified into two groups: control channels and traffic channels. Control channels are used for the transfer of control plane information only:

−	Broadcast Control Channel (BCCH): a downlink channel for broadcasting system control information.

−	Paging Control Channel (PCCH): a downlink channel that carries paging messages.

−	Common Control Channel (CCCH): channel for transmitting control information between UEs and network. This channel is used for UEs having no RRC connection with the network.

−	Dedicated Control Channel (DCCH): a point-to-point bi-directional channel that transmits dedicated control information between a UE and the network. Used by UEs having an RRC connection.

Traffic channels are used for the transfer of user plane information only:

−	Dedicated Traffic Channel (DTCH): point-to-point channel, dedicated to one UE, for the transfer of user information. A DTCH can exist in both uplink and downlink.

The following logical channels are used for MBS delivery:

−	MTCH: A point-to-multipoint downlink channel for transmitting MBS data of either multicast session or broadcast session from the network to the UE;

−	DTCH: A point-to-point channel defined for transmitting MBS data of a multicast session from the network to the UE;

−	MCCH: A point-to-multipoint downlink channel used for transmitting MBS broadcast control information associated to one or several MTCH(s) from the network to the UE.

###### 2.1.1.5.2.3	Mapping to Transport Channels

In Downlink, the following connections between logical channels and transport channels exist:

−	BCCH can be mapped to BCH;

−	BCCH can be mapped to DL-SCH;

−	PCCH can be mapped to PCH;

−	CCCH can be mapped to DL-SCH;

−	DCCH can be mapped to DL-SCH;

−	DTCH can be mapped to DL-SCH.

The following connections between logical channels and transport channels for group transmission exist:

−	MCCH can be mapped to DL-SCH;

−	MTCH can be mapped to DL-SCH.

In Uplink, the following connections between logical channels and transport channels exist:

−	CCCH can be mapped to UL-SCH;

−	DCCH can be mapped to UL- SCH;

−	DTCH can be mapped to UL-SCH.

###### 2.1.1.5.2.4	HARQ

The HARQ functionality ensures delivery between peer entities at Layer 1. A single HARQ process supports one TB when the physical layer is not configured for downlink/uplink spatial multiplexing, and when the physical layer is configured for downlink/uplink spatial multiplexing, a single HARQ process supports one or multiple TBs.

##### 2.1.1.5.3	RLC sublayer

###### 2.1.1.5.3.1	Transmission modes

The RLC sublayer supports three transmission modes:

−	Transparent Mode (TM);

−	Unacknowledged Mode (UM);

−	Acknowledged Mode (AM).

The RLC configuration is per logical channel with no dependency on numerologies and/or transmission durations, and ARQ can operate on any of the numerologies and/or transmission durations the logical channel is configured with.

For SRB0, paging and broadcast system information, TM mode is used. For other SRBs AM mode used. For DRBs, either UM or AM mode are used.

###### 2.1.1.5.3.2	Services and functions

The main services and functions of the RLC sublayer depend on the transmission mode and include:

−	transfer of upper layer PDUs;

−	sequence numbering independent of the one in PDCP (UM and AM);

−	error Correction through ARQ (AM only);

−	segmentation (AM and UM) and re-segmentation (AM only) of RLC SDUs;

−	reassembly of SDU (AM and UM);

−	duplicate Detection (AM only);

−	RLC SDU discard (AM and UM);

−	RLC re-establishment;

−	protocol error detection (AM only).

###### 2.1.1.5.3.3	ARQ

The ARQ within the RLC sublayer has the following characteristics:

−	ARQ retransmits RLC SDUs or RLC SDU segments based on RLC status reports;

−	polling for RLC status report is used when needed by RLC;

−	the RLC receiver can also trigger a RLC status report after detecting a missing RLC SDU or RLC SDU segment.

##### 2.1.1.5.4	PDCP sublayer

###### 2.1.1.5.4.1	Services and functions

The main services and functions of the PDCP sublayer are:

−	transfer of data (user plane or control plane);

−	maintenance of PDCP SNs;

−	header compression and decompression using the ROHC protocol;

−	header compression and decompression using EHC protocol;

−	compression and decompression of uplink PDCP SDUs: DEFLATE based UDC only;

−	ciphering and deciphering;

−	integrity protection and integrity verification;

−	timer based SDU discard;

−	for split bearers, routing;

−	duplication;

−	reordering and in-order delivery;

−	out-of-order delivery;

−	duplicate discarding.

Since PDCP does not allow COUNT to wrap around in DL and UL, it is up to the network to prevent this from happening (e.g. by using a release and add of the corresponding radio bearer or a full configuration).

##### 2.1.1.5.5	SDAP Sublayer

The main services and functions of SDAP are:

−	mapping between a QoS flow and a data radio bearer;

NOTE:	When remapping an XR QoS flow carrying PDU sets from one DRB to another all SDAP SDUs belonging to a PDU Set should be mapped to the same DRB.

−	marking QoS Flow ID (QFI) in both DL and UL packets.

A single protocol entity of SDAP is configured for each individual PDU session.

##### 2.1.1.5.6	L2 data flow

An example of the Layer 2 Data Flow is depicted on Fig. 37, where a transport block is generated by MAC by concatenating two RLC PDUs from RBx and one RLC PDU from RBy. The two RLC PDUs from RBx each corresponds to one IP packet (n and n+1) while the RLC PDU from RBy is a segment of an IP packet (m).

Figure 37

Data flow example

NOTE – H depicts the headers and subheaders.

##### 2.1.1.5.7	Carrier Aggregation (CA)

With CA, the multi-carrier nature of the physical layer is only exposed to the MAC layer for which one HARQ entity is required per serving cell as depicted on Figs 38 and 39 below:

−	In both uplink and downlink, there is one independent HARQ entity per serving cell and one transport block is generated per assignment/grant per serving cell in the absence of spatial multiplexing. Each transport block and its potential HARQ retransmissions are mapped to a single serving cell.

Figure 38

Layer 2 structure for DL with CA configured

Figure 39

Layer 2 structure for UL with CA configured

##### 2.1.1.5.8	Dual Connectivity (DC)

When the UE is configured with a SCG, the UE is configured with two MAC entities: one MAC entity for the MCG and one MAC entity for the SCG.

##### 2.1.1.5.9	Supplementary uplink

In case of Supplementary Uplink (SUL), the UE is configured with two ULs for one DL of the same cell, and uplink transmissions on those two ULs are controlled by the network to avoid overlapping PUSCH/PUCCH transmissions in time. Overlapping transmissions on PUSCH are avoided through scheduling while overlapping transmissions on PUCCH are avoided through configuration (PUCCH can only be configured for only one of the two ULs of the cell). In addition, initial access is supported in each of the uplink.

##### 2.1.1.5.10	Bandwidth Adaptation (BA)

With Bandwidth Adaptation, the receive and transmit bandwidth of a UE need not be as large as the bandwidth of the cell and can be adjusted: the width can be ordered to change (e.g. to shrink during period of low activity to save power); the location can move in the frequency domain (e.g. to increase scheduling flexibility); and the subcarrier spacing can be ordered to change (e.g. to allow different services). A subset of the total cell bandwidth of a cell is referred to as a Bandwidth Part (BWP) and BA is achieved by configuring the UE with BWP(s) and telling the UE which of the configured BWPs is currently the active one.

Figure 40 below describes a scenario where three different BWPs are configured:

−	BWP1 with a width of 40 MHz and subcarrier spacing of 15 kHz;

−	BWP2 with a width of 10 MHz and subcarrier spacing of 15 kHz;

−	BWP3 with a width of 20 MHz and subcarrier spacing of 60 kHz.

Figure 40

BA example

##### 2.1.2.5.11	Multiple Transmit/Receive Point Operation

In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from four TRPs, providing better coverage, reliability and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH.

#### 2.1.1.6	Radio Resources Control (RRC)

##### 2.1.1.6.1	Services and functions

The main services and functions of the RRC sublayer include:

−	broadcast of System Information related to AS and NAS;

−	paging initiated by 5GC or NG-RAN;

−	establishment, maintenance and release of an RRC connection between the UE and NG-RAN including:

•	addition, modification and release of Carrier Aggregation;

•	addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR;

−	security functions including key management;

−	establishment, configuration, maintenance and release of Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs);

−	Mobility functions including:

•	handover and context transfer;

•	UE cell selection and reselection and control of cell selection and reselection;

•	inter-RAT mobility;

−	QoS management functions;

−	UE measurement reporting and control of the reporting;

−	detection of and recovery from radio link failure;

−	NAS message transfer to/from NAS from/to UE.

In order to enhance mobility robustness and performance, additional NR mobility enhancements are introduced in 3GPP Rel-16. User data interruption is reduced during handover to 0 ms by dual-active-protocol-stack handover. In addition, the robustness during handover is improved by a conditional handover.

In order to further reduce the mobility latency, additional NR mobility enhancements are introduced in 3GPP Rel-18. The L1/L2-triggered mobility procedure (LTM), where the cell switch is based on L1 measurements and triggered by MAC CE, enables the early UL and DL synchronization to target cell and the RACH-less handover, resulting in the minimized mobility interruption time.

##### 2.1.1.6.2	Protocol states

RRC supports the following states which can be characterised as follows:

−	RRC_IDLE:

•	Public Land Mobile Network (PLMN) selection;

•	broadcast of system information;

•	cell re-selection mobility;

•	paging for mobile terminated data is initiated by 5GC;

•	DRX for core network paging configured by NAS.

−	RRC_INACTIVE:

•	PLMN selection;

•	broadcast of system information;

•	cell re-selection mobility;

•	paging is initiated by NG-RAN (RAN paging);

•	RAN-based notification area (RNA) is managed by NG-RAN;

•	DRX for RAN paging configured by NG-RAN;

•	5GC – NG-RAN connection (both CP and UP) is established for UE;

•	the UE AS context is stored in NG-RAN and the UE;

•	NG-RAN knows the RNA which the UE belongs to;

•	Transfer of MBS multicast/broadcast data to the UE over MRB(s);

•	Transfer of unicast data and/or signalling to/from the UE over radio bearers configured for Small Data Transmission (SDT).

−	RRC_CONNECTED:

•	5GC – NG-RAN connection (both CP and UP) is established for UE;

•	the UE AS context is stored in NG-RAN and the UE;

•	NG-RAN knows the cell which the UE belongs to;

•	transfer of unicast data to/from the UE;

•	network controlled mobility including measurements.

### 2.1.2	Support of verticals

The NR RIT supports a diverse set of mobile broadband (eMBB) services and other so-called “verticals”, including URLLC, Industrial IoT, Automotive/V2X, Private Networks (NPN), Reduced Capability (RedCap) NR devices and others. NR RIT supports in-band coexistence with NB-IoT and eMTC.

For optimal support of specific verticals, the NR RIT has been designed, or enhanced, with certain key features, or set of features. Taking few verticals as examples, a short summary of relevant NR RIT capabilities is provided below.

#### 2.1.2.1	Ultra-Reliable and Low Latency Communications (URLLC) and Industrial IoT (IIoT)

For support of Ultra-Reliable and Low Latency Communications services, some of the main features supported by the NR RIT are:

−	Logical Channel Priority (LCP) restrictions

−	Packet duplication with DC or CA

−	New QCI table for block error rate 10-5

−	Physical layer short transmission time interval (TTI).

From Rel-16 onwards, URLLC and Industrial IoT use cases are further facilitated by NR PDCP duplication enhancements:

−	Prioritization/multiplexing enhancements

−	NR Time Sensitive Communications (TSC) related enhancements, e.g. Ethernet header compression, and

−	Precise time information delivery.

−	Network timing synchronization status monitoring

−	RAN feedback for adaptation of Burst Arrival Time and Periodicity

#### 2.1.2.2	Vehicle-to-everything (V2X)

From Rel-16, NR RIT includes support of Vehicle-to-everything (V2X), mainly by means of NR sidelink communication over the PC5 interface, partly leveraging what was defined for E-UTRA V2X sidelink communication.

The architecture supporting the PC5 interface is illustrated in Fig. 41, for the overall NG-RAN (showing both NR and EUTRA). Sidelink transmission and reception over the PC5 interface are supported when the UE is inside NG-RAN coverage, irrespective of which RRC state the UE is in, and when the UE is outside NG-RAN coverage.

Figure 41

NG-RAN architecture supporting the PC5 interface

NR sidelink communication can support three types of transmission modes (in the access stratum):

−	Unicast, characterized by:

•	support of at least one PC5-RRC connection between peer UEs;

•	transmission and reception of control information and user traffic between peer UEs in sidelink;

•	support of sidelink feedback, for HARQ and link adaptation;

•	support of RLC AM;

•	Detection of radio link failure for the PC5 unicast connection.

−	Groupcast, characterized by:

•	transmission and reception of user traffic among UEs belonging to a group in sidelink;

•	support of distance/range based sidelink HARQ feedback;

•	support of connection-less groupcast, and application managed groupcast.

−	Broadcast, characterized by transmission and reception of broadcast user traffic among UEs in sidelink.

−	Sidelink discovery

•	The UE may perform NR sidelink discovery while in-coverage or out-of-coverage for non-relay operation.

NR sidelink communication may also be used to support other services than V2X, depending on service requirements.

#### 2.1.2.3	Sidelink Relay

Sidelink relay support 5G ProSe UE-to-Network Relay (U2N Relay) function to provide connectivity to the network for U2N Remote UE(s). Both L2 and L3 U2N Relay architectures are supported. The L3 U2N Relay architecture is transparent to the serving RAN of the U2N Relay UE, except for controlling sidelink resources.

#### 2.1.2.4	Reduced Capability (RedCap) NR devices

A RedCap UE has reduced capabilities with the intention to have lower complexity with respect to non-RedCap UEs. It is mandatory for a RedCap UE to support 20 MHz maximum UE channel bandwidth in FR1 and 100 MHz in FR2. An eRedCap UE has further reduced capabilities with the intention to have lower complexity with respect to RedCap UEs. It is mandatory for an eRedCap UE to support reduced DL/UL peak data rate of 10 Mbps, with or without reduced baseband bandwidth of 5 MHz for unicast PDSCH/PUSCH in FR1.

CA, MR-DC, DAPS, CPAC (Conditional PSCell Addition/Change) and IAB related capabilities are not supported by RedCap UEs.

#### 2.1.2.5	Positioning services

The DL Positioning Reference Signals (DL PRS) are defined to facilitate support of different positioning methods such as DL-TDOA, DL-AoD, multi-RTT through the following set of UE measurements DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference. Besides DL PRS signals, UE can use SSB and CSI-RS for RRM (RSRP and RSRQ) measurements for E-CID type of positioning.

The periodic, semipersistent and aperiodic transmission of Rel-15 SRS is defined for gNB UL RTOA, UL SRS-RSRP, UL-AoA measurements to facilitate support of UL TDOA and UL AoA positioning. The periodic, semipersistent and aperiodic transmission of SRS for positioning is defined for gNB UL RTOA, UL SRS-RSRP, UL-AoA, gNB Rx-Tx time difference measurements to facilitate support of UL TDOA, UL AoA and multi-RTT positioning methods.

The Sidelink Positioning Reference Signal (SL-PRS) is defined to facilitate Ranging/Sidelink positioning methods, such as SL-TOA, SL-TDOA, SL-RTT, SL-AoA, through the following set of measurements SL-PRS RSRP, RSRPP, RTOA, RSTD, Rx-Tx timing difference and AoA.

LPHAP (Low Power High Accuracy Positioning) is supported which includes: (i) enhancement of SRS for positioning configurations in RRC_INACTIVE state for UL and UL+DL positioning based on SRS for positioning validity area to avoid frequent reconfiguration of SRS for positioning upon serving cell changes; (ii) introduction of (e)DRX cycles longer than 10.24 seconds in RRC_INACTIVE state; (iii) alignment of (e)DRX cycles in RRC_INACTIVE state and DL PRS configurations, and (iv) support of measurements on DL PRS resources in RRC_IDLE state with reporting of the corresponding measurements when in RRC_CONNECTED state.

#### 2.1.3.6	eXtended Reality Services

The support of eXtended Reality (XR) services that require high data rate and low latency communications.

##### 2.1.3.6.1	Awareness

XR-Awareness relies on QoS flows, PDU Sets, Data Bursts and traffic assistance information.

##### 2.1.3.6.2	Power Saving

Most XR video frame rates (15, 30, 45, 60, 72, 90 and 120 fps) correspond to periodicities that are not an integer (66.66, 33.33, 22.22, 16.66, 13.88, 11.11 and 8.33 ms respectively). The gNB may configure a DRX cycle expressed in rational numbers so that the DRX cycle matches those periodicities, e.g. for the traffic with a frame rate of 60 fps, the network may configure the UE with a DRX cycle of 50/3 ms.

Configured grants may be configured without the need for the UE to wake up to monitor possible grants for UL retransmissions of configured grants, thus increasing the number of power saving opportunities for the UE.

##### 2.1.3.6.3	Capacity

The following enhancements for configured grant-based PUSCH transmission are introduced:

-	Support of multiple CG PUSCH transmission occasions within a single period of a CG configuration;

-	Indication of unused CG PUSCH occasion(s) of a CG configuration with Uplink Control Information multiplexed in CG PUSCH transmission of the CG configuration.

For Layer 2 enhancements.

In order to enhance the scheduling of uplink resources for XR, the following improvements are introduced:

-	One additional buffer size table to reduce the quantisation errors in BSR reporting (e.g. for high bit rates):

-	Delay Status Report (DSR) of buffered data via a dedicated MAC CE:

-	Reporting of uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow by the UE via UE Assistance Information. In case target gNB receives the burst arrival time from source gNB during the handover preparation procedure, the target gNB may use it by considering the SFN offset of the source gNB.

When the PSIHI indicates that all PDUs of the PDU Set are needed for a QoS flow, as soon as one PDU of a PDU set is known to be lost, the remaining PDUs of that PDU Set can be considered as no longer needed by the application and may be subject to discard operation at the transmitter to free up radio resources.

##### 2.1.3.6.4	Non-Homogeneous support of PDU set based handling in NG-RAN

During a handover from a gNB supporting PDU Set based handling to another gNB, the source gNB signals the PDU Set Information over Xn-U if the target node has signalled the support of PDU Set based handling in the Xn Handover Request Acknowledge message.

#### 2.1.3.7	ECN marking for L4S and congestion information exposure

In order to support ECN marking for L4S at gNB, SMF provides ECN marking request per QoS flow level to the gNB as part of PDU Session Resource Management procedure. If the gNB supports ECN marking, it provides the status indication back to the SMF which is used by the SMF. During Xn Handover Preparation procedure, source gNB provides the ECN marking request to target gNB.

#### 2.1.3.8	Support for Aerial UE Communication

NR connectivity for UEs capable of Aerial communication is supported via the following functionalities:

-	subscription-based Aerial UE identification and authorization;

-	altitude reporting based on the measurement event(s) where the UE's altitude has crossed a network-configured reference altitude threshold;

-	altitude-dependent configurations which apply only to specific altitude regions;

-	interference detection based on a measurement reporting that is triggered when a configured number of cells (i.e. larger than one) fulfils the triggering criteria simultaneously;

-	signalling of flight path information from UE to NG-RAN and from the source gNB to target gNB during handover;

-	location information reporting, including UE's horizontal and vertical velocity;

-	transmitting of BRID and DAA messages via PC5 interface.

#### 2.1.3.9	Support for Air to Ground Networks

In Air-to-ground (ATG) network, an NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000m and take-off/landing altitudes down to 3000m.

CA and DC are not supported for ATG in this release of the specification.

#### 2.1.3.10	Support of AI/ML for NG-RAN

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

#### 2.1.3.11 Non-Public Network

A Non-Public Network (NPN) is a network for non-public use, which includes Stand-Alone NPN (SNPN) and Public Network Integrated NPN (PNI-NPN)，where:

- A Stand-Alone NPN (SNPN) is a network deployed for non-public use which does not rely on network functions provided by a PLMN. An SNPN is identified by a PLMN ID and a Network Identifier (NID) broadcast.

- A Public Network Integrated NPN (PNI-NPN) is a network deployed for non-public use which relies on network functions provided by a PLMN. In PNI-NPN, a Closed Access Groups (CAG) identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG. A CAG is identified by a CAG identifier broadcast.

Developed by 3GPP Proponent as “5G, Release 15 and beyond – NR RIT”.

In 3GPP terminology, the term Evolved-UMTS Terrestrial Radio Access (E-UTRA) is also used to signify the LTE radio interface.