## 2.2	Detailed specification of the radio interface technology

Detailed specifications described in this Annex are developed around a “Global Core Specification” (GCS), which is related to externally developed materials incorporated by specific references for a specific technology. The process and use of the GCS, references, and related notifications and certifications are found as Document IMT-2020/20.

The IMT-2020 standards contained in this section are derived from the global core specification for 3GPP 5G-RIT contained at IMT-2020 Global Core Specifications. The following notes apply to the sections below:

1)	The identified Transposing Organizations should make their reference material available from their website.

2)	This information was supplied by the Transposing Organizations and relates to their own deliverables of the transposed global core specification.

Section 2.2.1 contains titles and synopses of the Global Core Specification of IMT-2020 radio interface technology entitled 3GPP 5G-RIT and the related hyperlinks to the transposed standards.

The specific 3GPP specifications of the Global Core Specifications (GCS) for IMT-2020 and 5G that are being transposed in § 2.2.1 are summarized in Table 6:

TABLE 6

3GPP specifications in § 2.2.1 that are to be transposed

| PART A List of Specifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| 37.xxx Series | 38.100 Series | 38.200 Series | 38.300 Series | 38.400 Series |
| TS 37.104 TS 37.105 TS 37.113 TS 37.114 TS 37.320 TS 37.324 TS 37.340 TS 37.355 TS 37.460 TS 37.461 TS 37.462 TS 37.466 TS 37.470 TS 37.471 TS 37.472 TS 37.473 TS 37.480 TS 37.481 TS 37.482 TS 37.483 | TS 38.101-1 TS 38.101-2 TS 38.101-3 TS 38.104 TS 38.106 TS 38.113 TS 38.114 TS 38.124 TS 38.133 TS 38.174 TS 38.175 | TS 38.201 TS 38.202 TS 38.211 TS 38.212 TS 38.213 TS 38.214 TS 38.215 | TS 38.300 TS 38.304 TS 38.305 TS 38.306 TS 38.307 TS 38.314 TS 38.321 TS 38.322 TS 38.323 TS 38.331 TS 38.340 TS 38.351 TS 38.355 | TS 38.401 TS 38.410 TS 38.411 TS 38.412 TS 38.413 TS 38.414 TS 38.415 TS 38.420 TS 38.421 TS 38.422 TS 38.423 TS 38.424 TS 38.425 TS 38.455 TS 38.460 TS 38.461 TS 38.462 TS 38.463 TS 38.470 TS 38.471 TS 38.472 TS 38.473 TS 38.474 |
| PART B Specification versions to be used |  |  |  |  |
| The specific versions of the 3GPP specifications that are to be used for the transpositions of those specifications listed in Table 6 are provided in the link below: Click here for the direct link to the GCS material |  |  |  |  |

### 2.2.1	Titles and synopses of the global core specification and the transposed standards

#### 2.2.1.1	Introduction

The standards documents referenced below, as transposed from the relevant 3GPP specifications, are provided by the identified Transposing Organizations as the transposed sets of standards for the terrestrial radio interface of IMT-2020 identified as 5G and includes not only the key characteristics of IMT-2020 but also the additional capabilities of 5G both of which are continuing to be enhanced.

#### 2.2.1.2	Radio Layer 1

##### 2.2.1.2.1	TS 38.201

NR; Physical layer; General description

This document provides a general description of the physical layer of NR radio interface. This document also describes the document structure of the 3GPP physical layer specifications, i.e. TS 38.200 series.

##### 2.2.1.2.2	TS 38.202

NR; Services provided by the physical layer

This document is a technical specification of the services provided by the physical layer of 5G-NR to upper layers.

##### 2.2.1.2.3	TS 38.211

NR; Physical channels and modulation

This document describes the physical channels and signals for 5G-NR.

##### 2.2.1.2.4	TS 38.212

NR; Multiplexing and channel coding

This document specifies the coding, multiplexing and mapping to physical channels for 5G NR.

##### 2.2.1.2.5	TS 38.213

NR; Physical layer procedures for control

This document specifies and establishes the characteristics of the physical layer procedures for control operations in 5G-NR.

##### 2.2.1.2.6	TS 38.214

NR; Physical layer procedures for data

This document specifies and establishes the characteristics of the physicals layer procedures of data channels for 5G-NR.

##### 2.2.1.2.7	TS 38.215

NR; Physical layer measurements

This document describes the physical layer measurements for NR.

#### 2.2.1.3	Radio Layers 2 & 3

##### 2.2.1.3.1	TS 37.320

Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2

This document provides an overview and overall description of the minimization of drive tests functionality. The document describes functions and procedures to support collection of UE-specific measurements for MDT using Control Plane architecture, for both UTRAN and E-UTRAN. Details of the signalling procedures for single-RAT operation are specified in the appropriate radio interface protocol specification. Network operation and overall control of MDT is described in OAM specifications.

##### 2.2.1.3.2	TS 37.324

Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Service Data Adaptation Protocol (SDAP) specification

This document specifies the Service Data Adaptation Protocol (SDAP) for a UE with connection to the 5G-CN.

##### 2.2.1.3.3	TS 37.340

NR; Multi-connectivity; Overall description; Stage-2

This document provides an overview of the multi-connectivity operation using E-UTRA and NR radio access technologies. Details of the network and radio interface protocols are specified in companion specifications of the 36 and 38 series.

##### 2.2.1.3.4	TS 37.355

LTE Positioning Protocol (LPP)

This document contains the definition of the LTE Positioning Protocol (LPP) for the radio access technologies E-UTRA/LTE and NR.

##### 2.2.1.3.5	TS 38.300

NR; NR and NG-RAN Overall description; Stage-2

This document provides an overview and overall description of the NG-RAN and focuses on the radio interface protocol architecture of NR connected to 5GC (E-UTRA connected to 5GC is covered in the 36 series). Details of the radio interface protocols are specified in companion specifications of the 38 series.

##### 2.2.1.3.6	TS 38.304

NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state

This document specifies the Access Stratum (AS) part of the UE procedures in RRC_IDLE state (also called Idle mode) and RRC_INACTIVE state. The non-access stratum (NAS) part of Idle mode procedures and processes is specified in TS 23.122.

This document specifies the model for the functional division between the NAS and AS in a UE.

This document applies to all UEs that support at least NR Radio Access, including multi-RAT UEs as described in 3GPP specifications, in the following cases:

−	When the UE is camped on a NR cell;

−	When the UE is searching for a cell to camp on;

NOTE:	When the UE is camped on or searching for a cell to camp on belonging to other RATs, the UE behaviour is described in the specifications of the other RATs.

##### 2.2.1.3.7	TS 38.305

NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN

This document specifies the stage 2 of the UE Positioning function of NG-RAN which provides the mechanisms to support or assist the calculation of the geographical position of a UE. UE position knowledge can be used, for example, in support of Radio Resource Management functions, as well as location-based services for operators, subscribers, and third-party service providers. The purpose of this stage 2 specification is to define the NG-RAN UE Positioning architecture, functional entities and operations to support positioning methods. This description is confined to the NG-RAN Access Stratum. It does not define or describe how the results of the UE position calculation can be utilised in the Core Network (e.g. LCS) or in NG-RAN (e.g. RRM).

UE Positioning may be considered as a network-provided enabling technology consisting of standardised service capabilities that enable the provision of location applications. The application(s) may be service provider specific. The description of the numerous and varied possible location applications which are enabled by this technology is outside the scope of this document. However, clarifying examples of how the functionality being described may be used to provide specific location services may be included.

This stage 2 specification covers the NG-RAN positioning methods, state descriptions, and message flows to support UE Positioning.

##### 2.2.1.3.8	TS 38.306

NR; User Equipment (UE) radio access capabilities

This document defines the NR UE Radio Access Capability Parameters.

##### 2.2.1.3.9	TS 38.307

NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band

This document specifies requirements for UEs supporting release independent features such as additional NR operating bands and power classes on top of TS 38.101 and TS 38.133.

##### 2.2.1.3.10	TS 38.314

NR; Layer 2 measurements

This document contains the description and definition of the measurements performed by NR or the UE that are transferred over the standardised interfaces in order to support NR radio link operations, radio resource management (RRM), network operations and maintenance (OAM), minimization of drive tests (MDT) and self-organising networks (SON).

Only the differences relative to TS 28.552 are specified in this specification.

##### 2.2.1.3.11	TS 38.321

NR; Medium Access Control (MAC) protocol specification

This document specifies the NR MAC protocol.

##### 2.2.1.3.12	TS 38.322

NR; Radio Link Control (RLC) protocol specification

This document specifies the NR Radio Link Control (RLC) protocol for the UE – NR radio interface.

##### 2.2.1.3.13	TS 38.323

NR; Packet Data Convergence Protocol (PDCP) specification

This document provides the description of the Packet Data Convergence Protocol (PDCP).

##### 2.2.1.3.14	TS 38.331

NR; Radio Resource Control (RRC); Protocol specification

This document specifies the Radio Resource Control protocol for the radio interface between UE and NG-RAN.

The scope of this document also includes:

−	the radio related information transported in a transparent container between source gNB and target gNB upon inter gNB handover;

−	the radio related information transported in a transparent container between a source or target gNB and another system upon inter RAT handover.

−	the radio related information transported in a transparent container between a source eNB and target gNB during E-UTRA-NR Dual Connectivity.

##### 2.2.1.3.15	TS 38.340

NR; Backhaul Adaptation Protocol (BAP) specification

This document provides description of the Backhaul Adaptation Protocol (BAP).

##### 1.2.1.3.16	TS 38.351

NR; Sidelink Relay Adaptation Protocol (SRAP) Specification

This document provides description of the Sidelink Relay Adaptation Protocol (SRAP).

##### 1.2.1.3.16	TS 38.355

NR; Sidelink Positioning Protocol (SLPP); Protocol Specification

The present document specifies the Sidelink Positioning Protocol (SLPP) for the interface between UEs and between UE and LMF.

#### 2.2.1.4	Architecture

##### 2.2.1.4.1	TS 37.460

Iuant interface: General aspects and principles

This document is an introduction to the 3GPP TS 37.46x series of Technical Specifications that define the Iuant Interface. The Iuant interface is applicable for UTRAN, E-UTRAN and NG-RAN. In this specification UTRAN, E-UTRAN and NG-RAN are denoted as “RAN”, whereas the corresponding network entities Node B, eNB, en-gNB and NG-RAN node are denoted as “RAN Node”. The logical Iuant interface is an interface internal to the RAN Node and defined to reside between the implementation specific O&M function and the RET antennas and between the implementation specific O&M function and the TMA control unit function.

##### 2.2.1.4.2	TS 37.461

Iuant interface: Layer 1

This document specifies the standards allowed to implement layer 1 on the Iuant interface for UTRA, E-UTRA and NR.

The specification of transmission delay requirements and O&M requirements are not in the scope of this document.

##### 2.2.1.4.3	TS 37.462

Iuant interface: Signalling transport

This document specifies the signalling transport related to RETAP and TMAAP signalling to be used across the Iuant interface for UTRAN, E-UTRAN and NG-RAN. In this specification UTRAN, E-UTRAN and NG-RAN are denoted as “RAN”, whereas the corresponding network entities Node B, eNB, en-gNB and NG-RAN node are denoted as “RAN Node”. The logical Iuant interface is an interface internal to the RAN Node and defined to reside between the implementation specific O&M function and the RET antennas and between the implementation specific O&M function and the TMA control unit function.

##### 2.2.1.4.4	TS 37.466

Iuant interface: Application part

This document is an introduction to the 3GPP TS 37.46x series of Technical Specifications that define the Iuant Interface. The Iuant interface is applicable for UTRAN, E-UTRAN and NG-RAN. In this specification UTRAN, E-UTRAN and NG-RAN are denoted as “RAN”, whereas the corresponding network entities Node B, eNB, en-gNB and NG-RAN node are denoted as “RAN Node”. The logical Iuant interface is an interface internal to the RAN Node and defined to reside between the implementation specific O&M function and the RET antennas together with the TMAs control unit function of the RAN Node.

This document is applicable for UTRAN, E-UTRAN and NG-RAN and specifies the Remote Electrical Tilting Application Part (RETAP) and the Tower Mounted Amplifier Application Part (TMAAP). In this specification UTRAN, E-UTRAN and NG-RAN are denoted as “RAN”, whereas the corresponding network entities Node B, eNB, en-gNB and NG-RAN node are denoted as “RAN Node”. RETAP supports the functions of the Iuant interface between the implementation specific O&M transport function and the RET Antenna Control unit function, TMAAP supports the functions of the Iuant interface between the implementation specific O&M transport function and the TMA control function.

##### 2.2.1.4.5	TS 37.470

W1 interface; General aspects and principles

This document is an introduction to the 3GPP TS 37.4xx series of technical specifications that define the W1 interface. The W1 interface provides means for interconnecting a ng-eNB-CU and a ng-eNB-DU of a ng-eNB within a NG-RAN.

##### 2.2.1.4.6	TS 37.471

W1 interface; Layer 1

This document specifies the standards allowed to implement Layer 1 on the W1 interface. The W1 interface provides means for interconnecting a ng-eNB-CU and a ng-eNB-DU of a ng-eNB within a NG-RAN.

The specification of transmission delay requirements and O&M requirements are not in the scope of this document.

##### 2.2.1.4.7	TS 37.472

W1 interface; Signalling transport

This document specifies the standards for Signalling Transport to be used across the W1 interface. The W1 interface provides means for interconnecting a ng-eNB-CU and a ng-eNB-DU of a ng-eNB within a NG-RAN. This document describes how the W1AP signalling messages are transported over W1.

##### 2.2.1.4.8	TS 37.473

W1 interface; Application Protocol (W1AP)

This document specifies the 5G radio network layer signalling protocol for the W1 interface. The W1 interface provides means for interconnecting a ng-eNB-CU and a ng-eNB-DU of a ng-eNB within an NG-RAN. The W1 Application Protocol (W1AP) supports the functions of W1 interface by signalling procedures defined in this document. W1AP is developed in accordance with the general principles stated in TS 38.401 and TS 37.470.

##### 1.2.1.4.9	TS 37.480

E1 general aspects and principles

This document is an introduction to the 3GPP TS 37.48x series of technical specifications that define the E1 interface. The E1 interface provides means for interconnecting a gNB-CU-CP and a gNB-CU-UP of a gNB-CU within an NG-RAN, or for interconnecting a gNB-CU-CP and a gNB-CU-UP of an en-gNB within an E-UTRAN, or for interconnecting an eNB-CP and an eNB-UP of an eNB within an E-UTRAN, or for interconnecting an ng-eNB-CU-CP and an ng-eNB-CU-UP of an ng-eNB-CU within an NG-RAN.

##### 1.2.1.4.10	TS 37.481

E1 layer 1

This document specifies the standards allowed to implement layer 1 on the E1 interface.

##### The specification of transmission delay requirements and O&M requirements are not in the scope of this document.1.2.1.4.11	TS 37.482

E1 signalling transport

This document specifies the standards for Signalling Transport to be used across the E1 interface. The E1 interface provides means for the interconnection of gNB-CU-CP and gNB-CU-UP within the NG-RAN architecture (TS 38.401), or for the interconnection of ng-eNB-CU-CP and ng-eNB-CU-UP within the NG-RAN architecture (TS 38.401), or for the interconnection of eNB-CP and eNB-UP within the E-UTRAN architecture (TS 36.401).

##### 1.2.1.4.12	TS 37.483

E1 Application Protocol (E1AP)

This document specifies the 5G radio network layer signalling protocol for the E1 interface. The E1 interface provides means for interconnecting a gNB-CU-CP and a gNB-CU-UP of a gNB within an NG-RAN, or for interconnecting a gNB-CU-CP and a gNB-CU-UP of an en-gNB within an E-UTRAN, or for interconnecting an eNB-CP and an eNB-UP of an eNB within an E-UTRAN, or for interconnecting an ng-eNB-CU-CP and an ng-eNB-CU-UP of an ng-eNB within an NG-RAN. The E1 Application Protocol (E1AP) supports the functions of E1 interface by signalling procedures defined in this document. E1AP is developed in accordance to the general principles stated in TS 38.401 and TS 37.480.

##### 2.2.1.4.13	TS 38.401

NG-RAN; Architecture description

This document describes the overall architecture of the NG-RAN, including interfaces NG, Xn and F1 interfaces and their interaction with the radio interface.

##### 2.2.1.4.14	TS 38.410

NG-RAN; NG general aspects and principles

This document is an introduction to the 3GPP TS 38.41x series of technical specifications that define the NG interface for the interconnection of an NG-RAN node to the 5GC (5G Core Network).

##### 2.2.1.4.15	TS 38.411

NG-RAN; NG layer 1

This document specifies the standards allowed to implement layer 1 on the NG interface.

The specification of transmission delay requirements and O&M requirements are not in the scope of this document.

##### 2.2.1.4.16	TS 38.412

NG-RAN; NG signalling transport

This document specifies the standards for signalling transport to be used across NG interface. NG interface is a logical interface between the NG-RAN and the 5GC. This document describes how the NGAP signalling messages are transported over NG.

##### 2.2.1.4.17	TS 38.413

NG-RAN; NG Application Protocol (NGAP)

This document specifies the radio network layer signalling protocol for the NG interface. The NG Application Protocol (NGAP) supports the functions of the NG interface by signalling procedures defined in this document. NGAP is developed in accordance with the general principles stated in TS 38.401 and TS 38.410.

##### 2.2.1.4.18	TS 38.414

NG-RAN; NG data transport

This document specifies the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the NG interface.

##### 2.2.1.4.19	TS 38.415

NG-RAN; PDU session user plane protocol

This document specifies the PDU Session user plane protocol being used over the NG-U, Xn-U and N9 interfaces. Applicability to other interfaces is not precluded.

##### 2.2.1.4.20	TS 38.420

NG-RAN; Xn general aspects and principles

This document is an introduction to the TSG RAN TS 38.42x series of Technical Specifications that define the Xn interface. It is an interface for the interconnection of two NG-RAN nodes within the NG-RAN architecture (TS 38.401).

##### 2.2.1.4.21	TS 38.421

NG-RAN; Xn layer 1

This document specifies the standards allowed to implement layer 1 on the Xn interface.

The specification of transmission delay requirements and O&M requirements are not in the scope of this document.

##### 2.2.1.4.22	TS 38.422

NG-RAN; Xn signalling transport

This document specifies the standards for signalling transport to be used across Xn interface. The Xn interface provides means for interconnecting two NG-RAN nodes. The Xn interface is a logical interface between two nodes of the NG-RAN. This document describes how the XnAP signalling messages are transported over Xn.

##### 2.2.1.4.23	TS 38.423

NG-RAN; Xn Application Protocol (XnAP)

This document specifies the radio network layer signalling procedures of the control plane between NG-RAN nodes in NG-RAN. XnAP supports the functions of the Xn interface by signalling procedures defined in this document. XnAP is developed in accordance with the general principles stated in TS 38.401 and TS 38.420.

##### 2.2.1.4.24	TS 38.424

NG-RAN; Xn data transport

This document specifies the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the Xn interface.

##### 2.2.1.4.25	TS 38.425

NG-RAN; NR user plane protocol

This document specifies the NR user plane protocol functions used within NG-RAN and, for EN-DC, within E-UTRAN. NR user plane protocol functions may reside in nodes terminating either the X2-U (for EN-DC) or the Xn-U or the F1-U interface.

##### 2.2.1.4.26	TS 38.455

NG-RAN; NR Positioning Protocol A (NRPPa)

This document specifies the control plane radio network layer signalling procedures between a NG-RAN node and the LMF. NRPPa supports the concerned functions by signalling procedures defined in this document.

##### 2.2.1.4.27	TS 38.460

NG-RAN; E1 general aspects and principles

This document is an introduction to the 3GPP TS 38.46x series of technical specifications that define the E1 interface. The E1 interface provides means for interconnecting a gNB-CU-CP and a gNB-CU-UP of a gNB-CU within an NG-RAN, or for interconnecting a gNB-CU-CP and a gNB-CU-UP of an en-gNB within an E-UTRAN.

##### 2.2.1.4.28	TS 38.461

NG-RAN; E1 layer 1

This document specifies the standards allowed to implement layer 1 on the E1 interface.

The specification of transmission delay requirements and O&M requirements are not in the scope of this document.

##### 2.2.1.4.29	TS 38.462

NG-RAN; E1 signalling transport

This document specifies the standards for Signalling Transport to be used across the E1 interface. The E1 interface provides means for the interconnection of gNB-CU-CP and gNB-CU-UP within the NG-RAN architecture (TS 38.401).

##### 2.2.1.4.30	TS 38.463

NG-RAN; E1 Application Protocol (E1AP)

This document specifies the 5G radio network layer signalling protocol for the E1 interface. The E1 interface provides means for interconnecting a gNB-CU-CP and a gNB-CU-UP of a gNB within an NG-RAN, or for interconnecting a gNB-CU-CP and a gNB-CU-UP of an en-gNB within an E-UTRAN. The E1 Application Protocol (E1AP) supports the functions of E1 interface by signalling procedures defined in this document. E1AP is developed in accordance to the general principles stated in TS 38.401 and TS 38.460.

##### 2.2.1.4.31	TS 38.470

NG-RAN; F1 general aspects and principles

This document is an introduction to the 3GPP TS 38.47x series of technical specifications that define the F1 interface. The F1 interface provides means for interconnecting a gNB-CU and a gNB-DU of a gNB within an NG-RAN, or for interconnecting a gNB-CU and a gNB-DU of an en-gNB within an E-UTRAN.

##### 2.2.1.4.32	TS 38.471

NG-RAN; F1 layer 1

This document specifies the standards allowed to implement Layer 1 on the F1 interface. The F1 interface provides means for interconnecting a gNB-CU and a gNB-DU of a gNB within an NG-RAN, or for interconnecting a gNB-CU and a gNB-DU of an en-gNB within an E-UTRAN.

The specification of transmission delay requirements and O&M requirements are not in the scope of this document.

##### 2.2.1.4.33	TS 38.472

NG-RAN; F1 signalling transport

This document specifies the standards for Signalling Transport to be used across the F1 interface. The F1 interface provides means for interconnecting a gNB-CU and a gNB-DU of a gNB within an NG-RAN, or for interconnecting a gNB-CU and a gNB-DU of an en-gNB within an E-UTRAN. This document describes how the F1AP signalling messages are transported over F1.

##### 2.2.1.4.34	TS 38.473

NG-RAN; F1 Application Protocol (F1AP)

This document specifies the 5G radio network layer signalling protocol for the F1 interface. The F1 interface provides means for interconnecting a gNB-CU and a gNB-DU of a gNB within an NG-RAN, or for interconnecting a gNB-CU and a gNB-DU of an en-gNB within an E-UTRAN. The F1 Application Protocol (F1AP) supports the functions of F1 interface by signalling procedures defined in this document. F1AP is developed in accordance with the general principles stated in TS 38.401 and TS 38.470.

##### 2.2.1.4.35	TS 38.474

NG-RAN; F1 data transport

This document specifies the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the F1 interface. The F1 interface provides means for interconnecting a gNB-CU and a gNB-DU of a gNB within an NG-RAN, or for interconnection a gNB-CU and a gNB-DU of an en-gNB within an E-UTRAN.

#### 2.2.1.5	Radio-frequency aspects

##### 2.2.1.5.1	TS 37.104

NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception

This document establishes the minimum RF characteristics of E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS). Requirements for multi-RAT and single-RAT operation of MSR BS are covered in this document. The requirements in this document for E-UTRA and UTRA single-RAT operation of MSR BS are also applicable to E-UTRA and UTRA multi-carrier capable single-RAT BS. Requirements for GSM BS that are only single-RAT capable are not covered.

##### 2.2.1.5.2	TS 37.105

Active Antenna System (AAS) Base Station (BS) transmission and reception

This document establishes the RF characteristics, the RF minimum requirements and minimum performance requirements for E-UTRA AAS Base Station (BS), the FDD mode of UTRA AAS Base Station (BS), the 1,28 Mchip/s TDD mode of UTRA AAS Base Station (BS) in single RAT and any MSR AAS Base Station (BS) implementation of these RATs.

##### 2.2.1.5.3	TS 37.113

NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)

This document covers the assessment of E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Stations and associated ancillary equipment in respect of Electromagnetic Compatibility (EMC). This document specifies the applicable test conditions, performance assessment and performance criteria for E-UTRA, UTRA and GSM/EDGE Base Stations and associated ancillary equipment in one of the following categories: (i) Multi-Standard Radio (MSR) Base Stations for E-UTRA, UTRA and GSM/EDGE meeting the requirements of TS 37.104, with conformance demonstrated by compliance to TS 37.141; (ii) Base Stations for E-UTRA meeting the requirements of TS 36.104, with conformance demonstrated by compliance to TS 36.141; (iii) Base Stations for UTRA FDD meeting the requirements of TS 25.104, with conformance demonstrated by compliance to TS 25.141; (iv) Base Stations for UTRA TDD meeting the requirements of TS 25.105, with conformance demonstrated by compliance to TS 25.142; (v) Base Stations for GSM/EDGE meeting the requirements of TS 45.005, with conformance demonstrated by compliance to TS 51.021. The environment classification used in this document refers to the environment classification used in IEC 61000-6-1 and IEC 61000-6-3.

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

##### 2.2.1.5.4	TS 37.114

Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)

This document covers the assessment of E-UTRA, UTRA and Multi-Standard Radio (MSR) Active Antenna Systems Base Stations in respect of Electromagnetic Compatibility (EMC).

This document specifies the applicable test conditions, performance assessment and performance criteria for E-UTRA and UTRA Base Stations and associated ancillary equipment in one of the following categories:

–	Active Antenna System Base Station for E-UTRA, UTRA and MSR meeting the requirements of 3GPP TS 37.105, with conformance demonstrated by compliance to 3GPP TS 37.145.

The scope of this document is AAS BS with TAB connectors for every transceiver unit at the Transceiver Array Boundary. Requirement, procedures and values of an AAS Base Station without TAB connectors are not included in this document and are FFS.

The environment classification used in this document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 and IEC 61000-6-3.

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

##### 2.2.1.5.5	TS 38.101-1

NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

This document establishes the minimum RF requirements for NR User Equipment (UE) operating on frequency Range 1.

##### 2.2.1.5.6	TS 38.101-2

NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

This document establishes the minimum RF requirements for NR User Equipment (UE) operating on frequency Range 2.

##### 2.2.1.5.7	TS 38.101-3

NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

This document establishes the minimum RF requirements for NR User Equipment (UE) Interworking operation with other radios. This includes but is not limited to additional requirements for carrier aggregation or NR dual connectivity between Range 1 and Range 2 and additional requirements due to NR non-standalone (NSA) operation mode with E-UTRA.

##### 2.2.1.5.8	TS 38.104

NR; Base Station (BS) radio transmission and reception

This document establishes the minimum RF characteristics and minimum performance requirements of NR and NB-IoT operation in NR in-band Base Station (BS).

##### 1.2.1.5.9	TS 38.106

NR repeater radio transmission and reception

This document establishes the minimum RF characteristics of NR Repeater.

##### 2.2.1.5.10	TS 38.113

NR; Base Station (BS) ElectroMagnetic Compatibility (EMC)

This document covers the assessment of NR Base Station (BS) and ancillary equipment in respect of Electromagnetic Compatibility (EMC).

This document specifies the applicable test conditions, performance assessment and performance criteria for base stations and associated ancillary equipment in the following categories:

−	BS equipped with antenna connectors or TAB connectors which are possible to be terminated during EMC testing, meeting the BS type 1-C and BS type 1-H RF requirements of TS 38.104, with conformance demonstrated by compliance to TS 38.141-1.

−	BS not equipped with antenna connectors nor TAB connectors, i.e. with antenna elements radiating during the EMC testing, meeting the BS type 1-O and BS type 2-O RF requirements of TS 38.104, with conformance demonstrated by compliance to TS 38.141-2.

The scope of this document is twofold:

−	Requirements, procedures and values of a BS with antenna connectors or TAB connectors,

−	Requirements, procedures and values of a BS without antenna connectors, nor TAB connectors.

The environment classification used in this document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 and IEC 61000-6-3.

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

##### 1.2.1.5.11	TS 38.114

NR; Repeaters ElectroMagnetic Compatibility (EMC)

This document covers the assessment of NR repeater and ancillary equipment in respect of Electromagnetic Compatibility (EMC).

This document specifies the applicable requirements, procedures, test conditions, performance assessment and performance criteria for NR repeater and associated ancillary equipment in the following categories:

-	NR repeater equipped with antenna connectors which are possible to be terminated during EMC testing, meeting the repeater type 1-C RF requirements of TS 38.106, with conformance demonstrated by compliance to TS 38.115-1.

-	NR repeater not equipped with antenna connectors, i.e. with antenna elements radiating during the EMC testing, meeting the repeater type 2-O RF requirements of TS 38.106, with conformance demonstrated by compliance to TS 38.115-2.

The environment classification used in this document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 and IEC 61000-6-3.

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

##### 2.2.1.5.12		TS 38.124

NR; Electromagnetic compatibility (EMC) requirements for mobile terminals and ancillary equipment

This document establishes the essential EMC requirements for “3rd generation” digital cellular mobile terminal equipment and ancillary accessories in combination with a 3GPP NR user equipment (UE).

The equipment conforming to the requirements laid out in this document and used in its intended electromagnetic environment in accordance with the manufacturer’s instructions

−	shall not generate electromagnetic disturbances at a level which may interfere with the intended operation of other equipment;

−	has an adequate level of intrinsic immunity to electromagnetic disturbances to operate as intended;

This document specifies the applicable EMC tests, methods of measurement, frequency ranges, applicable limits and minimum performance criteria for all types of NR UE(s) and their accessories. NR base station equipment operating within network infrastructure is outside the scope of this document. However, this document does cover mobile and portable equipment that is intended to be operated in a fixed location while connected to the AC mains. NR base station equipment operating within network infrastructure is covered by the technical specification TS 38.113.

Requirements for the radiated emission from the enclosure port of integral antenna equipment and ancillaries are included in this document. Technical specifications for conducted emissions from the antenna connector are found in the 3GPP specifications for the radio interface, e.g. TS 38.xyz, for the effective use of the radio spectrum.

Requirements for the radiated emissions from the enclosure port and ancillaries cover two cases:

−	UE equipment supporting operations in a frequency range for which antenna connectors are available (i.e. for operations in Frequency Range 1 as defined in e.g. TS 38.101-1 for the radio interface);

−	UE equipment supporting operations in a frequency range for which only integral antennas may be available (i.e. for operations in Frequency Range 2 as defined in e.g. TS 38.101-2 for the radio interface).

The immunity requirements are selected to ensure an adequate level of compatibility for apparatus in residential, commercial, light industrial and vehicular environments. The levels, however, do not cover extreme cases, which may occur in any location but with low probability of occurrence.

Compliance of radio equipment to the requirements of this document does not signify compliance to any requirement related to the use of the equipment (i.e. licensing requirements).

Compliance to the requirements of this document does not signify compliance to any safety requirement. However, any temporary or permanent unsafe condition caused by EMC is considered as non-compliance.

##### 2.2.1.5.13		TS 38.133

NR; Requirements for support of radio resource management

This document specifies requirements for support of Radio Resource Management for the FDD and TDD modes of New Radio (NR). These requirements include requirements on measurements in NR and the UE as well as requirements on node dynamical behaviour and interaction, in terms of delay and response characteristics.

##### 1.2.1.5.14	TS 38.174

NR; Integrated Access and Backhaul (IAB) radio transmission and reception

This document establishes the minimum RF characteristics, minimum radio resource management (RRM) requirements, RRM test cases and minimum performance requirements of NR Integrated access and backhaul (IAB).

##### 1.2.1.5.15	TS 38.175

NR; Integrated Access and Backhaul (IAB) Electromagnetic Compatibility (EMC)

This document covers the assessment for NR Integrated access and backhaul (IAB) node and associated ancillary equipment in respect of Electromagnetic Compatibility (EMC).

This document specifies the applicable test conditions, performance assessment and performance criteria for NR Integrated access and backhaul (IAB) node and associated ancillary equipment.

The environment classification used in this document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1  and IEC 61000-6-3.

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

### 2.2.2	Other specifications

Other specifications addressing radio and device testing, but not part of the GCS, are listed in this section.

Information on system and core network specifications can be found in the 3GPP web site for a complete system perspective. These system and core network specifications address the network, terminal, and service aspects required to provide an integrated mobility solution including aspects such as user services, connectivity, interoperability, mobility and roaming, security, codecs and media, operations and maintenance, charging, etc.

All the 3GPP specifications can be found at the following link: https://www.3gpp.org/specifications/specification-numbering. 3GPP specifications are reviewed and updated after each Technical Specification Group Plenary meeting (held every year in March, June, September and December).

#### 2.2.2.1	TS 37.141

E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS).

#### 2.2.2.2	TS 37.144

User Equipment (UE) and Mobile Station (MS) GSM, UTRA and E-UTRA over the air performance requirements

This document establishes over the air antenna minimum requirements for user equipment (UE) and mobile station (MS).

Handheld UE requirements are defined for roaming bands for the speech position (beside the head and beside the head and hand) and hand phantom browsing mode position. Laptop mounted equipment requirements are defined for roaming bands for the data transfer position (laptop ground plane phantom). Laptop embedded equipment requirements are defined for roaming bands for the data transfer position (free space).

All bands are potential roaming bands, and the requirements for roaming bands shall therefore be fulfilled for all bands supported by a UE/MS.

Requirements for operating bands are dependent on how the network has been built and are thus operator specific and cannot be specified here. Recommended performance values for operating bands (Annex B) are however included in this specification for information. It should be recognised that the ability to meet the recommended performance values depends on the number of frequency bands supported by the UE/MS.

#### 2.2.2.3	TS 37.145-1

Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: conducted conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for Single RAT E-UTRA, UTRA and Multi-Standard Radio (MSR) UTRA and EUTRA Active Antenna System (AAS) Base Station (BS). These have been derived from, and are consistent with the E-UTRA, UTRA AAS BS specification defined in 3GPP TS 25.104. The technical specification is in two parts: part 1 (this document) covers conducted requirements and part 2 covers radiated requirements.

#### 2.2.2.4	TS 37.145-2

Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for Single RAT E-UTRA, UTRA and Multi-Standard Radio (MSR) UTRA and EUTRA Active Antenna System (AAS) Base Station (BS). These have been derived from, and are consistent with the E-UTRA, UTRA AAS BS specification defined in 3GPP TS 25.104. The technical specification is in two parts, part 1 covers conducted requirements and part 2 (this document) covers radiated requirements.

#### 2.2.2.5	TS 37.171

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA); User Equipment (UE) performance requirements for RAT-Independent Positioning Enhancements

This document establishes the minimum performance requirements for RAT-Independent Positioning Enhancements (e.g. MBS positioning technology) for FDD or TDD mode of UTRA and E-UTRA for the User Equipment (UE).

#### 2.2.2.6	TS 38.101-4

NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

This document establishes the minimum performance requirements for NR User Equipment (UE).

#### 2.2.2.7	TS 38.141-1

NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for NR Base Station (BS) Type 1-C and Type 1-H. These have been derived from, and are consistent with the conducted requirements for BS Type 1-C and BS Type 1-H in NR BS specification defined in TS 38.104.

−	A BS type 1-C only has conducted requirements, so it requires compliance to this specification only.

−	A BS type 1-H has both conducted and radiated requirements, so it requires compliance to the applicable requirements of this specification and TS 38.141-2.

−	BS type 1-O and BS type 2-O have only radiated requirements, so they require compliance to TS 38.141-2 only.

#### 2.2.2.8	TS 38.141-2

NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for NR Base Station (BS) type 1-H, BS type 1-O and BS type 2-O. These have been derived from, and are consistent with the radiated requirements for BS type 1-H, BS type 1-O and BS type 2-O in BS specification defined in TS 38.104.

−	A BS type 1-C only has conducted requirements, so it does not require compliance to this specification.

−	A BS type 1-H has both conducted and radiated requirements, so it requires compliance to the applicable requirements of this specification and TS 38.141-1.

−	BS type 1-O and BS type 2-O have only radiated requirements, so they require compliance to this specification only.

#### 1.2.2.9	TS 38.151

NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

This document establishes the Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs operating on frequency Range 1 and frequency Range 2, for NR standalone (SA) and NR non-standalone (NSA) operation mode. The corresponding test methodologies are also presented in the Annex of this Technical Specification.

#### 2.2.2.10	TS 38.171

NR; Requirements for support of Assisted Global Navigation Satellite System (A-GNSS)

This document establishes the minimum requirements for both UE based and UE assisted FDD or TDD A-GNSS terminals which have NG-RAN access via gNB (in SA NR, NR-DC or NE-DC NR operation mode) or via ng-eNB (in EN-DC operation mode) and which are supporting A-GNSS in 5GS via LPP between UE and LMF as described in TS 38.305.

#### 1.2.2.11	TS 38.176-1

NR; Integrated Access and Backhaul (IAB) conformance testing; Part 1: Conducted conformance testing

This document establishes the minimum RF characteristics and minimum performance requirements of NR Integrated access and backhaul (IAB).

This document specifies the Radio Frequency (RF) test methods and conformance requirements for NR Integrated access and backhaul (IAB) type 1-H. These have been derived from, and are consistent with the conducted requirements for NR Integrated access and backhaul (IAB) type 1-H in NR Integrated access and backhaul specification defined in TS 38.174.

An IAB type 1-H has both conducted and radiated requirements so it requires compliance to the applicable requirements of this document and TS 38.176-2.

IAB type 1-O and IAB type 2-O have only radiated requirements so they require compliance to TS 38.176-2 only.

#### 1.2.2.12	TS 38.176-2

NR; Integrated Access and Backhaul (IAB) conformance testing; Part 2: Radiated conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for NR Integrated access and backhaul: IAB type 1-H, IAB type 1-O and IAB type 2-O. These have been derived from, and are consistent with the radiated requirements for IAB type 1-H, IAB type 1-O and IAB type 2-O in BS specification defined in TS 38.174.

A IAB type 1-H has both conducted and radiated requirements, so it requires compliance to the applicable requirements of this document and TS 38.176-1.

IAB type 1-O and IAB type 2-O have only radiated requirements, so they require compliance to this document only.

#### 2.2.2.13	TS 37.571-1

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 1: Conformance test specification

This document specifies the procedures for the conformance test of the measurement requirements for FDD mode of UTRA and FDD or TDD mode of E-UTRA for the User Equipment (UE) that supports one or more of the defined positioning methods. These positioning methods are for UTRA: Assisted Global Positioning System (A-GPS), Assisted Global Navigation Satellite Systems (A-GNSS) and for E-UTRA: Assisted Global Navigation Satellite System (A-GNSS), Observed Time Difference of Arrival (OTDOA), Enhanced Cell ID (ECID).

Tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the “Test applicability” part of the test.

The Implementation Conformance Statement (ICS) pro-forma could be found in the 3rd part of this document.

#### 2.2.2.14	TS 37.571-2

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance

This document specifies the protocol conformance testing for the 3rd Generation UTRAN and E-UTRAN User Equipment (UE) supporting UE positioning.

This is the second part of a multi-part test specification. The following information can be found in this part:

–	the overall protocol conformance test structure;

–	the protocol conformance test configurations;

–	the conformance requirement and reference to the core specifications;

–	the test purposes; and

–	a brief description of the test procedure, the specific test requirements and short message exchange table.

The Implementation Conformance Statement (ICS) pro-forma could be found in the 3rd part of this document.

This document is valid for UE supporting UE positioning implemented according to 3GPP releases starting from Release 99 up to the Release indicated on the cover page of this document.

#### 2.2.2.15	TS 37.571-3

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 3: Implementation Conformance Statement (ICS)

This document provides the ICS proforma for 3rd Generation UTRAN and E-UTRAN User Equipment (UE) supporting UE positioning, in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-1 and ISO/IEC 9646-7.

This document also specifies a recommended applicability statement for the test cases included in 3GPP TS 37.571-1 and 3GPP TS 37.571-2. These applicability statements are based on the features implemented in the UE.

Special conformance testing functions can be found in 3GPP TS 34.109 for UTRA and 3GPP TS 36.509 for E-UTRA. The common test environments are included in 3GPP TS 34.108 for UTRA and in 3GPP TS 36.508 for E-UTRA.

This document is valid for UE supporting UE positioning implemented according to 3GPP releases starting from Release 99 up to the Release indicated on the cover page of this document.

#### 2.2.2.16	TS 37.571-4

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 4: Test suites

This document specifies the protocol and signalling conformance testing in TTCN for the UE:

–	A-GPS at the UTRA Uu interface;

–	LTE positioning at the LTE-Uu interface;

–	A-GNSS at the UTRA Uu interface.

The following TTCN test specification and design considerations can be found in this document:

–	Test system architecture;

–	Test models and ASP definitions;

–	Test methods and usage of communication ports definitions;

–	Test configurations;

–	Design principles and assumptions;

–	TTCN styles and conventions;

–	Partial PIXIT proforma;

–	Test suites in TTCN-2 and TTCN-3;

–	The Test Suites designed and implemented in this document are based on the test specifications in prose in 3GPP TS 37.571-2;

–	The applicability of the individual test cases is specified in the test ICS proforma specification in 3GPP TS 37.571-3.

#### 2.2.2.17	TS 37.571-5

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 5: Test scenarios and assistance data

This document specifies the test scenarios and assistance data required for the conformance test for FDD or TDD mode of UTRA and E-UTRA for the User Equipment (UE) that supports one or more of the defined positioning methods. For UTRA these are Assisted Global Positioning System (A-GPS) and Assisted Global Navigation Satellite System (A-GNSS). For E-UTRA these are A-GNSS, Observed Time Difference of Arrival (OTDOA) and Enhanced Cell ID (ECID).

#### 2.2.2.18	TS 38.508-1

5GS; User Equipment (UE) conformance specification; Part 1: Common test environment

This document defines the test environment for the 5G System.

This specification covers all aspects, including NG-RAN, 5GC and interworking between 5GS and EPS used for conformance tests of User Equipment (UE).

#### 2.2.2.19	TS 38.508-2

5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma

This document provides the Implementation Conformance Statement (ICS) proforma for 5G New Radio (NR) User Equipment (UE), in compliance with the relevant requirements.

Special conformance testing functions can be found in 3GPP TS 38.509 and 3GPP TS 36.509 and the common test environments are included in 3GPP TS 38.508-1 and 3GPP TS 36.508.

This document is valid for UE implemented according to 3GPP Releases starting from Release 15 up to the Release indicated on the cover page of this document.

#### 2.2.2.20	TS 38.509

5GS; Special conformance testing functions for User Equipment (UE)

This document defines for User Equipment (UE) those special functions and their activation/deactivation methods that are required in the UE for conformance testing purposes when the UE is connected to the 5G System (5GS) via its radio interface(s).

The document also describes the operation of these special functions when the 5GS capable UEs are connected via a non-5GS system e.g. E-UTRA FDD or TDD system.

Depending on the 5GS system’s architecture some relevant for the UE for conformance testing special functions may be defined in TS 36.509.

#### 2.2.2.21	TS 38.521-1

NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 standalone

This document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain RF characteristics for frequency Range 1 as part of the 5G-NR.

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the “definition and applicability” part of the test.

For example, only Release 15 and later UE declared to support 5G-NR shall be tested for this functionality. In the event that for some tests different conditions apply for different releases, this is indicated within the text of the test itself.

#### 2.2.2.22	TS 38.521-2

NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 standalone

This document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain RF characteristics for frequency Range 2 as part of the 5G-NR.

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the “definition and applicability” part of the test.

For example, only Release 15 and later UE declared to support 5G-NR shall be tested for this functionality. In the event that for some tests different conditions apply for different releases, this is indicated within the text of the test itself.

#### 2.2.2.23	TS 38.521-3

NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

This document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain RF characteristics for carrier aggregation between Range 1 and Range 2 and additional requirements due to NR non-standalone (NSA) operation mode with E-UTRA.

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the “definition and applicability” part of the test.

For example, only Release 15 and later UE declared to support 5G-NR shall be tested for this functionality. In the event that for some tests different conditions apply for different releases, this is indicated within the text of the test itself.

#### 2.2.2.24	TS 38.521-4

NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance

This document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain performance requirements as part of 5G-NR.

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the “definition and applicability” part of the test.

For example, only Release 15 and later UE declared to support 5G-NR shall be tested for this functionality. In the event that for some tests different conditions apply for different releases, this is indicated within the text of the test itself.

#### 2.2.2.25	TS 38.522

NR; User Equipment (UE) conformance specification; Applicability of radio transmission, radio reception and radio resource management test cases

This document provides the Implementation Conformance Statement (ICS) proforma for 5G New Radio (NR) User Equipment (UE), in compliance with the relevant requirements.

This document specifies the recommended applicability statement for the test cases included in 3GPP TS 38.521-1, TS 38.521-2, TS 38.521-3, TS 38.521-4 and TS 38.533. These applicability statements are based on the features implemented in the UE.

Special conformance testing functions can be found in 3GPP TS 38.509 and the common test environments are included in 3GPP TS 38.508-1. Common implementation conformance statement (ICS) proforma can be found in 3GPP TS 38.508-2.

#### 2.2.2.26	TS 38.523-1

5GS; User Equipment (UE) conformance specification; Part 1: Protocol

This document specifies the protocol conformance testing for the 3GPP UE connecting to the 5G System (5GS) via its radio interface(s).

The following information can be found in this document (first part of a multi-part test specification):

−	the overall test structure;

−	the test configurations;

−	the conformance requirement and references to the core specifications;

−	the test purposes; and

−	a brief description of the test procedure, the specific test requirements and short message exchange table.

The applicability of the individual test cases is specified in the ICS proforma specification (3GPP TS 38.523-2). The Test Suites are specified in part 3 (3GPP TS 38.523-3).

#### 2.2.2.27	TS 38.523-2

5GS; User Equipment (UE) conformance specification; Part 2: Applicability of protocol test cases

This document provides the applicability of protocol test cases proforma for 5G New Radio (NR) User Equipment (UE), in compliance with the relevant requirements.

This document specifies the recommended applicability statement for the test cases included in 3GPP TS 38.523-1 and 3GPP TS 38.523-3. These applicability statements are based on the features implemented in the UE.

Special conformance testing functions can be found in 3GPP TS 38.509 and 3GPP TS 36.509 and the common test environments are included in 3GPP TS 38.508-1 and 3GPP TS 36.508.

#### 2.2.2.28	TS 38.523-3

5GS; User Equipment (UE) conformance specification; Part 3: Protocol Test Suites

This document specifies the protocol and signalling conformance testing in TTCN-3 for the 3GPP UE connecting to the 5G System (5GS) via its radio interface(s).

The following TTCN test specification and design considerations can be found in this document:

−	the test system architecture;

−	the overall test suite structure;

−	the test models and ASP definitions;

−	the test methods and usage of communication ports definitions;

−	the test configurations;

−	the design principles and assumptions;

−	TTCN styles and conventions;

−	the partial PIXIT proforma;

−	the test suites.

The Test Suites designed in the document are based on the test cases specified in prose in 3GPP TS 38.523-1. The applicability of the individual test cases is specified in 3GPP TS 38.523-2.

#### 2.2.2.29	TS 38.533

NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)

This document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain requirements for support of RRM (Radio Resource Management) as part of the 5G New Radio (5G-NR). This document covers NR Range 1, NR Range 2 and Interworking.

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the “test applicability” part of the test.

#### 2.2.2.30	TS 34.229-1

Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

This document specifies the protocol conformance testing for the User Equipment (UE) supporting the Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP).

This is the first part of a multi-part test specification. The following information can be found in this part:

−	the overall test structure;

−	the test configurations;

−	the conformance requirement and reference to the core specifications;

−	the test purposes; and

−	a brief description of the test procedure, the specific test requirements and short message exchange table.

The following information relevant to testing can be found in accompanying specifications:

−	the applicability of each test case.

#### 2.2.2.31	TS 34.229-2

Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) specification

This document provides the Implementation Conformance Statement (ICS) proforma for 3rd Generation User Equipment (UE) supporting the Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP), in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-7 and ETSI ETS 300 406.

#### 2.2.2.32	TS 34.229-3

Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract test suite (ATS)

This document specifies the protocol conformance testing in TTCN for the 3GPP User Equipment (UE) at the Gm interface.

This document is the 3rd part of a multi-part test specification, 3GPP TS 34.229. The following TTCN test specification and design considerations can be found in this document:

−	the overall test suite structure;

−	the testing architecture;

−	the test methods and PCO definitions;

−	the test configurations;

−	the design principles, assumptions, and used interfaces to the TTCN tester (System Simulator);

−	TTCN styles and conventions;

−	the partial PIXIT proforma;

−	the TTCN files for the mentioned protocols tests.

The Abstract Test Suites designed in the document are based on the test cases specified in prose (3GPP TS 34.229-1).

#### 2.2.2.33	TS 34.229-5

Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)

This document specifies the protocol conformance testing for the User Equipment (UE) supporting the Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP) when using the 5G System (5GS).

This is the fifth part of a multi-part test specification. The following information can be found in this part:

−	the overall test structure;

−	the test configurations;

−	the conformance requirement and reference to the core specifications;

−	the test purposes; and

−	the test procedure.

The following information relevant to testing can be found in accompanying specifications:

−	Implementation Conformance Statement (ICS) pro-forma and the applicability of each test case.

______________

A “GCS” (Global Core Specification) is the set of specifications that defines a single RIT, an SRIT, or a component RIT within an SRIT.

The following identified Transposing Organizations have provided their transposed sets of standards information contained in this section:

–	Association of Radio Industries and Businesses (ARIB)

–	Alliance for Telecommunications Industry Solutions (ATIS)

–	China Communications Standards Association (CCSA)

–	European Telecommunications Standards Institute (ETSI)

–	Telecommunications Standards Development Society, India (TSDSI)

–	Telecommunications Technology Association (TTA)

–	Telecommunication Technology Committee (TTC).