3GPP TSG RAN Meeting #100	RP-231055

Taipei, June 12-14, 2023

Agenda item:	9.2.2 Study on Ambient IoT in RAN

Source: 	Sierra Wireless

Title: 	Discussion on Ambient IoT

Document for:	Discussion and decision

# Introduction

A new study item on ambient IoT [1] has been approved in the RAN#97e meeting. In this contribution, we give our views on the following topics:

Deployment Scenarios

Energy Storage

Coverage Requirements

Coverage Analysis

Maximum message size

Latency

Demodulation capabilities

Carrier Wave Information

SID Scope

TR Conclusions and Recommendations Proposals

Note: the agreements, conclusions and WA made in RAN #98e and RAN #99 are in the appendix I and II.

# Deployment Scenarios

The following deployment scenarios in [2] have not been filled in yet. This section includes text proposals for the TR. Note that the red underlines are the difference between FLS in [5] and this proposal:

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) Note: topology (3) where either the uplink or the downlink coverage is extended |
|  | Spectrum | Licensed FDD, or Licensed TDD or unlicensed (for topology (2) and (3)) |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

A solution using unlicensed bands (especially lower bands 868MHz,915MHz, or 2.4GHz) could be practical to provide indoor coverage from an outdoor base station even with topologies 1 and thus should not be excluded.

Deployment 3: Device indoors, UE-based reader UE indoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | No base station in the topology |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | FFS |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B Note: a device C could also support such deployment |

Title in TR should match FLS in [5]

Deployment 4: Device outdoors, basestation outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD ,Licensed TDD, or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

A solution using unlicensed bands (especially lower bands 868MHz,915MHz, or 2.4GHz) could be practical to provide outdoor coverage and thus should not be excluded.

Deployment 5: Device outdoors, UE outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | No base station in the topology |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | FFS |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

No changes needed from FLS in [5]

# Energy Storage

- The following editor’s note describes the open issues in section 4.3 WRT to energy storage values E1 and E2:

Editor's note: Values of E1 and E2 are FFS, and it is possible that E1 = E2, in which case storage 2 and 3 could be replaced by a single description such as "limited energy storage".

- Editor's note: FFS whether to include device function; whether to include a target maximum power consumption for each device; whether/how to describe what stored energy is used for; if combinations of these devices are considered

The problem with defining E1 and E2 is that the required energy storage is highly dependent on application specific variables, such as “how much data needs to be sent” and “how often”. It will also depend on how much energy is being harvested to restore the energy storage.

The worst-case amount of energy used to send data can already be calculated based on the agreed Max Power Consumption and Min Data Rate:

- Energy =  Max power consumption * Min Data Rate * Amount of data to send

- For example, if 1000bits need to be sent for Device C, then energy usage and require energy storage is

- (1 or 10mW) * (0.1 kbps) *1000 = 10-100 mW*S

One way to define E1 and E2 is in terms of transmission time or number of messages the device could transmit at worst-case coverage:

E1 equals energy required to transmit 1 message of  maximum message size at worse case coverage

E2 equals energy required to transmit > 1 message of maximum sized message at worse case coverage

# Coverage Requirements

The following SA1 coverage requirements per use case are indicated in the following table:

| Scenario | Communication Range |
| --- | --- |
| Automated warehousing | 30m indoors |
| Medical instruments inventory management and positioning | 50m indoors, 200m Outdoor |
| Medical instruments online modification | 50m indoors |
| Smart Grids | 50-200m (typically Outdoors) |
| Non-Public Network for Logistics | <30m indoors |
| Smart Home | 10-30m |
| Airport Terminal/Shipping Port | FFS |
| Remote Lost Item (Finding) | 10m |
| Location Services (indoor) | NA |
| Ranging (finding items w/in a home) | 10m |
| Machine Room Supervision | 30m indoors |
| Shopping Mall Positioning | 10m |
| Automated Supply Distribution | 600,000 m^2 |
| Activation/Deactivation | NA |
| Forest Fire Monitoring | 150m |
| Smart Agriculture | 30-100m |

Based on the above table, the following observations can be made:

The SA1 outdoor use cases require a maximum coverage of 200m.

The SA1 indoor use cases require a maximum coverage of  50m.

Given we want A-IOT to support as many use cases as possible, 200m is a good practical coverage target.

The outdoor UL and DL target coverage from a A-IOT device to an intermediate node, assisting node or BS should be 200m.

The indoor UL and DL target coverage from a A-IOT device to an intermediate node, assisting node or BS should be 50m.

For at least device type A, the carrier wave source needs to be close enough to the A-IOT device to power it.  SA1 does not provide any input on this requirement. SA1 does provide indoor coverage requirements so if Device type A is only designed to support indoor use cases then we can use the SA1 indoor coverage requirements for this purpose.

For Device type A and B, the carrier wave coverage target (i.e., the distance from the carrier wave source to the A-IOT device) should 50m.

# Coverage Analysis

This section provides analysis of what are possible or feasible coverage distances given certain assumptions which are provide in Appendix I.

## Device A:  Distance from carrier wave source to A-IoT Device A:

If only RF energy harvesting is used by a type A device, it needs to be close enough to the carrier wave such that it can harvest enough energy to operate (this is also called the wake-up point). The amount of energy it needs will depend on the implement. The current state of the art requires a carrier wave of at least -30dBm [3], where most commercial RFID devices are around -20dBm; we evaluate between -20 and -35dBm. Since this distance will be small, it is unlikely that a macro or micro-BS will be close enough to be a carrier waver source, thus only UEs were considered to be carrier wave sources. UEs typically have TX power maximums of +23 or +26 dBm so these values were used for evaluation.

| Min Rx Carrier Wave (Wake-up point) | Carrier wave TX Power | Distance from Carrier Wave Source (m) |
| --- | --- | --- |
| -20 dBm | +23, +26 dBm | 2.2, 2.7 |
| -25 dBm | +23, +26 dBm | 3.0, 3.6 |
| -30 dBm (current state of the Art) | +23, +26 dBm | 4.0, 5.0 |
| -35 dBm | +23, +26 dBm | 6.3, 8.9 |
| -40 dBm | +23, +26 dBm | 11.2, 15.9 |

Note: This measurement does not apply to Device B or C as it assumes Device B and C are powered from the ES.

Given the current state of the art technology, the carrier wave source must be within 5m to wake-up/activate a device type A. This is not within the coverage target of 50m.

## Device A:  UL distance A-IoT device to intermediate/assisting node or BS:

This section includes an analysis of potential supported distances when a device type A receives different power levels from a carrier wave source and then backscatters the signal to an intermediate or assisting node, micro-BS, or macro BS. The range of the carrier wave source received powers analyzed was -40 to -20 dBm, which corresponds to the wake-up point range analyzed above. For reference, the distances, assuming a +23dB UE is sending the carrier waver, are also included in the table.

| Receiver Type | Carrier Wave Power Rcvd at A-Iot Device | Distance from carrier wave to Device | Distance to a Receiver (m) |
| --- | --- | --- | --- |
| Inter/Assisting Node | -40, -30, -20 dBm | 11.2m, 4.0m, 2.2m | 10.4, 19.0, 34.8 |
| Micro Base Station | -40, -30, -20 dBm | 11.2m, 4.0m, 2.2m | 43.7, 126, 239 |
| Macro Base Station | -40, -30, -20 dBm | 11.2m, 4.0m, 2.2m | 71.3, 213, 495 |

Given the current state of the art wake-up point of -30dBm, the following observation can be made:

It is NOT feasible for device type A to support UL range of >50m to intermediate/assisting nodes.

It is feasible for device type A to support outdoor UL range >200m to macro-BS assuming carrier wave source is within 4m.

## Device B:  UL distance device to intermediate/assisting node or BS:

This section includes an analysis of possible UL distances covered by a device type B through backscattering a signal to various receiver types with 10, 20, and 30 dB of reflection amplifier gain. The range of the carrier wave source received power analyzed was -53,-40, -30 dBm. The -53dBm value was chosen as this results in a 50m range, which is the indoor coverage requirement from SA1. For reference, the distances, assuming a +23dB UE is sending the carrier waver, are also included in the table.

| Receiver Type | Reflection Amplifier Gain (dB) | Carrier Wave Rcvd Power at Device (dBm) | Distance from carrier wave to Device | Distance to a Receiver (m) |
| --- | --- | --- | --- | --- |
| Inter/Assisting Node | 10 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 12.5, 27.4, 50.2 |
| Micro Base Station | 10 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 60.5, 190, 338 |
| Macro Base Station | 10 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 100, 393, 700 |
| Inter/Assisting Node | 20 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 22.9, 50.2, 91.8 |
| Micro Base Station | 20 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 160, 338, 601 |
| Macro Base Station | 20 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 293, 700, 1246 |
| Inter/Assisting Node | 30 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 41.8, 91.8, 168 |
| Micro Base Station | 30 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 284, 601, 1068 |
| Macro Base Station | 30 | -53,-40, -30 dBm | 50m, 11.2m, 4.0m | 589, 1246, 2215 |

It is feasible for Device B with gain of >30dB to support 50m range from a carrier wave source and UL range of 50m to intermediate/assisting nodes.

It is feasible for Device B with gain of 30dB to support 50m range from a carrier wave source and UL range of >200m to micro-BS nodes.

It is feasible for Device B with gain of 20dB to support 50m range from a carrier wave source and UL range of >200m to macro-BS nodes.

## Device C:  UL distance device to intermediate/assisting node or BS:

In this section, we examine the potential UL coverage of device type C. In contrast to device types A and B, device type C does not employ backscatter-based communication in the UL. Consequently, this device type can transmit higher power levels. Based on the agreed maximum current of device type C is 1-10mW, transmit power levels of 0, 5, and 10 dBm were analyzed. The same set of receiver types were analyzed.

| Receiver Type | PA power (dBm) | Distance to a Receiver (m) |
| --- | --- | --- |
| Inter/Assisting Node | +0, +5 , +10, dBm | 168, 227, 308 |
| Micro Base Station | +0, +5 , +10, dBm | 1069, 1425, 1900 |
| Macro Base Station | +0, +5 , +10, dBm | 2215, 2954, 3940 |

It is feasible for Device C with 0 dBm PA to support UL range of 200m to micro-BS or macro-BS

It is feasible for Device C with a +5dBm PA to support UL range of 200m to an intermediate/assisting node.

## Device A/B/C:  DL distance from intermediate/assisting node or BS to device A/B/C:

It is assumed that all three types of devices can support demodulation. Additionally, it is assumed that the ability to demodulate is required for operation, thus this section includes an analysis of DL coverage for all three device types. The analysis in this section includes four distinct transmitter types: a UE /intermediate node/assisting node (+23 and +26dBm), a micro-BS, and a macro BS. The A-IOT receiver is modeled using four different receiver sensitivity values of -25, -50, -75, -100 dBm. Since the receiver sensitivity is a function of the complexity (i.e., noise figure), in theory, any device type could support any receiver sensitivity but given the complexity constraints agreed, the follow typical sensitivity ranges per device type are most likely:

Device A sensitivity : -25, -50 dBm

Device B sensitivity: -50, -75 dBm

Device C sensitivity: -75, -100 dBm

| Transmitter Types | Receiver Sensitivity (dBm) | Distance to Device (m) |
| --- | --- | --- |
| UE/Intermediate/Assisting Node 23dBm Tx Power | -25, -50, -75, -100 | 3.3, 15.1, 68.3, 310 |
| UE/Intermediate/Assisting Node 26dBm TX Power | -25, -50, -75, -100 | 4.0, 18.1, 81.9, 371 |
| Micro Base Station | -25, -50, -75, -100 | 0, 31.8, 270, 1139 |
| Macro Base Station | -25, -50, -75, -100 | 0, 70.4, 656, 2768 |

Notes: It is assumed that the A-IOT protocol will ensure that demodulation and backscatter are not needed simultaneously (i.e., device first demodulates, then backscatters).

It is feasible for A-IOT devices with receiver sensitivity of  -75dBm to support DL ranges of >50m to intermediate/assisting node, micro-BS or macro-BS.

It is feasible for A-IOT devices with receiver sensitivity of  -75dBm to support DL ranges of >200m to micro-BS or macro-BS.

It is feasible for A-IOT devices with receiver sensitivity of  -100dBm to support DL ranges of >200m to an intermediate/assisting node.

# Maximum message size

This section discusses possible text proposals and proposals which can be used to populate section “5.5. Maximum message size (or maximum ‘TB’ size)” of the TR.

The purpose of section 5.5 in the TR is to provide some requirements for RAN WG level to further study and/or do protocol development.  Clearly the TB size should not be discussed at the RANPL level as this is a very detailed protocol choice and should be made by RAN WG.

The Maximum TB size shall not be specified in this RAN PL TR

What is useful input to RAN WG is what expected application-level traffic sizes are to be expect so that is the focus of the discussion in this section.

UL Message Sizes Considerations:

For most applications, the UL message size will be fairly small in the order of 100 Bytes including application-level security headers (e.g. DTLS headers).  However,  during security key exchanges, the messages which can include certificates, will be much larger, in the range of 1000 Bytes.  Since support of standardized E2E security mechanism (e.g. TLS, DTLS, HTTPS) is required, the ability to send up to 1000 Byte efficiently should be supported. However this does NOT mean the TB size needs to be 1000 Bytes – clearly that is much too large. The RAN WG could consider an efficient fragmentation mechanism - a mechanism to break up large messages and into smaller messages (i.e. transport blocks) for transport.

DL Message Size Considerations:

Similar to the UL, for most applications, the DL message size will be fairly small in the order of 100 Bytes including application-level security headers.  However, another important aspect of security for commercial systems is to be able to support security patches or firmware updates over the air (OTA). In all commercial (i.e. non-retail) IoT applications, the ability to update the stack and application with security patches OTA is a hard requirement and often a reason why a wireless technology is not chosen.  Supporting OTA security patches will require the ability to efficiency support larger blocks of transfers of 100’s of KBytes.

Based on the above, discussion the following text proposal for include in the TR is proposed

Text proposal for section 5.5. Maximum message size (or maximum ‘TB’ size)

5.5. Maximum message size (or maximum ‘TB’ size)

Application messages sizes in the UL can infrequently be up to [1000] Bytes (e.g. during security exchanges) but normally are in the range of [10-100] Bytes. Application messages sizes in the DL can infrequently be 100’s of Kbytes (e.g. during firmware downloads) but normally are in the range of [10-100] Bytes. An efficient fragmentation solution to support application messages should be consider.

# Latency

This section discusses possible text proposals to populate section “5.6. Latency” of the TR. The purpose of this section in the TR is to provide some requirements and guidance to RAN WGs to further study and/or do protocol development/evaluation thus the discussion in this section is focused on providing a TP for that purpose.

Latency is often defined as control plane latency and user plane latency where control plane latency is the time it takes to send (DO) or receive a message (DT) starting from a low power state (e.g. idle mode) and where user plane latency is the time it takes to send or receive a message from a higher power state (e.g. connected mode). It’s unclear whether user plane latency will apply so no need to specify requirements around that now. Control plane latency needs to be further broken down into DO or DT control plane latency.

DT Control Plane Latency:

Device terminated (DT) messaging is required for all representative use cases: Inventory, Sensors, Positioning, Command. The use case requirements for DT latency vary greatly and thus a range needs to be supported to provide devices with the ability to trade-off DT latency (i.e. sleep times) for battery life (i.e., similar to eDRX’s flexibility) so the maximum latency of 1 HR should be supported. The most stringent DT latency comes from the Command user case where minimum latency of  [hundreds of ms] is required (i.e. from SA1 [4] 5.2 Medical Instruments Inventory – 1 sec,  5.14 Shopping Center = 0.5sec, 5.17 Device activation and deactivation, 5.26 Elderly Health Care – 1 sec, 5.30 Controller in Smart Agriculture – hundreds of ms).

DO Control Plane Latency:

Autonomous device originated (DO-A) or device triggered device originated (DO-DTT) is required for all four use cases: Inventory, Sensors, Positioning, Command. The use case requirements for DO latency also vary significantly where minimum latency of  [hundreds of ms] is required

Given the above discussion the following TP for the TR is proposed:

TP for TR

The device-terminated and device-originated target minimum latency is hundreds of msec.

To support DRX, the device-terminated latency is selectable by the device to a maximum of 1HR.

# Demodulation capabilities

From following proposal is from the FLS in [5]:

Proposal 5.2-3-v2: Any Device A, B, C can receive/demodulate data from the network. FFS which data, e.g. control plane, user plane.

This is a good clarification to add to the TR with a minor wording change of “network” to “assisting/intermediate nodes or BS” and where the FFS is clarified to mean both control plane and user plane data.

Add to TR -  Device A, B, C can receive/demodulate both control plane and user plane data from UE’s, assisting nodes, intermediate nodes, or BS.

# Carrier Wave Information

The following text is the ONLY text included in the TR [2] WRT carrier waves:

4.2.1	Connectivity topologies

The following connectivity topologies for Ambient IoT networks and devices are defined for the purposes of the study. In all these topologies, the Ambient IoT device may be provided with a carrier wave from other node(s) either inside or outside the topology. The links in each topology may be bidirectional or unidirectional.

The carrier wave is used to power on the A-IoT device and is used  for backscattering so is a critical part of the system. The text above has very minimal information for such a critical part of the system.  Listing possible formats, sources, and duplexing options would be helpful.

The carrier wave options at least includes the following:

Capture in TR that carrier waves have at least the following formats, sources and duplexing options should be captured in the TR:

Formats:

Unmodulated carrier wave

Modulated carrier wave

Sources:

UL transmission by a UE

DL transmission by gNB/eNB/Relay/IAB

Non-3GPP device (e.g. wifi, BT, TV signal,…)

Duplexing:

Full duplex - carrier wave in the same band as the backscattered signal

FDD - carrier wave in a different band as the backscattered signal

# SID Scope

The SID[1] has the following objective:

  - “Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported.

NOTE: This is not to require a detailed WG-level of analysis”

Given the above requirement, the editor created the following section in the TR:

6	 Comparison and assessment

Editor’s note: This clause will analyze and compare which deployment scenario(s) are identified for each (representative) use case and the trade-offs between them, and identify assumptions on required functionality, as well as assess the RAN design targets for the {deployment scenario, use case} associations.

From SID [1], this TR is expected to be approved by RAN #100 – this meeting.

Since the design targets have not yet been agreed, itis simply not feasible to complete this analysis during this meeting. As always, we have two options – remove the objective from the SID or delay the completion of the study. Given the analysis will not be a RAN WG analysis, it will be difficult to draw definitive conclusions based the results even if it was completed, so the proposal is to remove the objective from the SID.

Remove the following objective from the SID

“Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported”

# Technical Report Conclusions and Recommendations

This section discusses possible conclusions and recommendation that can be added to section “7	Conclusions and recommendations” of the TR.

Based on the coverage analysis in this paper, there is a small feasible range between the carrier wave source and type A devices so the following conclusion is proposed:

Conclusion: If only RF energy harvesting is used, companies’ analysis’ show that device type A will likely not meet indoor nor outdoor coverage targets. A more methodical and systematic RAN WG is needed to verify this conclusion.

Based on our coverage analysis for device type B and C, the following conclusion is proposed:

Conclusion: If only RF energy harvesting is used, companies’ analysis’ show that device type B and C can meet both indoor and outdoor coverage targets. A more methodical and systematic RAN WG is needed to verify this conclusion.

Since support for software security patches is required, the device would need NV RAM (MRAM or flash) support and the following conclusion can be made WRT to complexity:

Conclusion: If software security patches are supported, the device type A targeted complexity requirement is not feasible.

Since both device type A and B are based on a backscattering and where device type C is not, the following conclusion can be made:

Conclusion: Device types A and B could operate using a similar protocol, but device type C would require a different protocol. Standards development effort for each protocol would be similar.

The complexity/cost of all three types of devices is targeted to be at least orders of magnitude lower than NB-IOT. The lower complexity devices are envisioned to reduce the total cost of ownership but if the cost of service is too high, a low TCO is not achieved. Given this, the following proposal for conclusion is made:

Conclusion: To provide a low total cost of ownership, the system should prioritize solutions and operating bands which provide a low cost for service.

Leveraging the 3GPP system can be a great differentiator against already mature and established solutions such as RFID/NFC for Device type A, active RF ID for device type B, and UWB/Zigbee/BLE/Wi-SUN/DECT 2020/LoRa for Device type C. Given this the following proposal is made:

Future Rel 19 RAN WG studies should priorities connection topologies (i.e., topology 1,2,3), which leverage the 3GPP system (Assisting/Intermediate nodes and BS)

As with most 3GPP RAN PL studies, the study should start with a wide scope, and hence the agreement to include these three device types is correct. However, the amount of protocol specification work and development work should be considered for future work. As already concluded, device type A and B can share a protocol where device type C cannot. It is unclear how a device type C will be differentiated against already existing technologies (e.g. BLE, NFC, and UWB) that are already well commercialize within UE’s and support connection topology 2,3,4. Given this discussion the following proposal for a recommendation is made:

Future Rel 19 RAN WG studies should prioritize the study of device types A and B

# Observations and Proposals

- Deployment Scenarios:

Proposal:  Include TP proposals from section 2 into the TR

- Energy Storage:

- E1 equals energy required to transmit 1 message of  maximum message size at worse case coverage

E2 equals energy required to transmit > 1 message of maximum sized message at worse case coverage

- Coverage Requirements:

- The SA1 outdoor use cases require a maximum coverage of 200m.

The SA1 indoor use cases require a maximum coverage of  50m.

The outdoor UL and DL target coverage from a A-IOT device to an intermediate node, assisting node or BS should be 200m.

The indoor UL and DL target coverage from a A-IOT device to an intermediate node, assisting node or BS should be 50m.

For Device type A and B, the carrier wave coverage target (i.e., the distance from the carrier wave source to the A-IOT device) should 50m.

- Coverage feasibility observations:

Given the current state of the art technology, the carrier wave source must be within 5m to wake-up/activate a device type A. This is not within the coverage target of 50m.

It is NOT feasible for device type A to support UL range of >50m to intermediate/assisting nodes.

It is feasible for device type A to support outdoor UL range >200m to macro-BS assuming carrier wave source is within 4m.

It is feasible for Device B with gain of >30dB to support 50m range from a carrier wave source and UL range of 50m to intermediate/assisting nodes.

It is feasible for Device B with gain of 30dB to support 50m range from a carrier wave source and UL range of >200m to micro-BS nodes.

It is feasible for Device B with gain of 20dB to support 50m range from a carrier wave source and UL range of >200m to macro-BS nodes.

It is feasible for Device C with 0 dBm PA to support UL range of 200m to micro-BS or macro-BS

It is feasible for Device C with a +5dBm PA to support UL range of 200m to an intermediate/assisting node.

It is feasible for A-IOT devices with receiver sensitivity of  -75dBm to support DL ranges of >50m to intermediate/assisting node, micro-BS or macro-BS.

It is feasible for A-IOT devices with receiver sensitivity of  -75dBm to support DL ranges of >200m to micro-BS or macro-BS.

It is feasible for A-IOT devices with receiver sensitivity of  -100dBm to support DL ranges of >200m to an intermediate/assisting node.

Maximum message size

The Maximum TB size shall not be specified in this RAN PL TR

Text proposal for section 5.5. Maximum message size (or maximum ‘TB’ size)

5.5. Maximum message size (or maximum ‘TB’ size)

Application messages sizes in the UL can infrequently be up to [1000] Bytes (e.g. during security exchanges) but normally are in the range of [10-100] Bytes. Application messages sizes in the DL can infrequently be 100’s of Kbytes (e.g. during firmware downloads) but normally are in the range of [10-100] Bytes. An efficient fragmentation solution to support application messages should be consider

Latency

TP for TR

The device-terminated and device-originated target minimum latency is hundreds of msec.

To support DRX, the device-terminated latency is selectable by the device to a maximum of 1HR.

Demodulation capabilities

Add to TR -  Device A, B, C can receive/demodulate both control plane and user plane data from UE’s, assisting nodes, intermediate nodes, or BS.

Carrier Wave Information

Capture in TR that carrier waves have at least the following formats, sources and duplexing options should be captured in the TR:

Formats:

Unmodulated carrier wave

Modulated carrier wave

Sources:

UL transmission by a UE

DL transmission by gNB/eNB/Relay/IAB

Non-3GPP device (e.g. wifi, BT, TV signal,…)

Duplexing:

Full duplex - carrier wave in the same band as the backscattered signal

FDD - carrier wave in a different band as the backscattered signal

SID Scope

Remove the following objective from the SID

“Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported”

TR Conclusions and Recommendations Proposals

Conclusion: If only RF energy harvesting is used, companies’ analysis’ show that device type A will likely not meet indoor nor outdoor coverage targets. A more methodical and systematic RAN WG is needed to verify this conclusion.

Conclusion: If only RF energy harvesting is used, companies’ analysis’ show that device type B and C can meet both indoor and outdoor coverage targets. A more methodical and systematic RAN WG is needed to verify this conclusion.

Conclusion: If software security patches are supported, the device type A targeted complexity requirement is not feasible.

Conclusion: Device types A and B could operate using a similar protocol, but device type C would require a different protocol. Standards development effort for each protocol would be similar.

Conclusion: To provide a low total cost of ownership, the system should prioritize solutions and operating bands which provide a low cost for service.

Future Rel 19 RAN WG studies should priorities connection topologies (i.e., topology 1,2,3), which leverage the 3GPP system (Assisting/Intermediate nodes and BS)

Future Rel 19 RAN WG studies should prioritize the study of device types A and B

# References

RP-223396,  SID- Study on Ambient IoT.

RP-230419 TR 38.848 v0.1.0 Study on Ambient IoT (Internet of Things) CMCC

S1-231333 3GPP TR 22.840 V1.2.0

S1-220192, New SID: Study on Ambient power-enabled Internet of Things.

RP-230801 FLS - Results of offline discussions on ambient IoT 	Huawei

Griffin, J. D., & Durgin, G. D. (2009). Complete link budgets for backscatter-radio and RFID systems. IEEE Antennas and Propagation Magazine, 51(2), 11-25.

3GPP TR 38.901 V17.0.0

3GPP TR 38.104 V18.0.0

3GPP TR 38.101 V18.0.0

# Appendix I:  Coverage Analysis Assumptions

The following assumptions were made when calculating coverage feasibility in Section 5:

| Common simulation parameters |  |
| --- | --- |
| Carrier frequency | 2.4 GHz |
| Maximum BER | 1% |
| Modulation | ASK |
| A-IoT device BW | 15 kHz |
| A-IoT device antenna gain | -3 dBi |
| Receiver minimum SNR | 4.32 dB |
| UL and DL Fade margin | 7dB |

Backscatter-communication based link budget models were derived for devices A and B based on the methodology presented in [6]. It was assumed that device B has sufficient energy harvesting for sufficient reception and for reflection amplification, therefore only Carrier Wave SRC to Device distance was analyzed for device A.

## Carrier Wave Source to A-IoT

The following values were used when calculating the distance between a carrier wave source and a device type A:

| Carrier Wave Assumptions |  |  |
| --- | --- | --- |
| Source | Height (m) | Transmission power (dB) |
| Carrier Wave | 1.5 | 23, 26 |

| Device A Receiver Assumptions |  |  |
| --- | --- | --- |
| Receiver model | Height (m) | Wake-up Point |
| Device A | 1.5 | -20, -25, -30, -35, -40 dBm |

The NLOS indoor hotspot (In-H) path loss model was used based on [7].

## Uplink coverage assumptions

The following values were used when calculating the UL coverage analysis for device types A and B:

| Device A & B (i.e., Backscattering) Assumptions |  |  |  |
| --- | --- | --- | --- |
| Device type | Device height (m) | Modulation index | Reflection amplification (dB) |
| Device A | 1.5 | 0.25 | 0 |
| Device B | 1.5 | 1 | 20, 30, 40 |

The following values were used when calculating the coverage analysis for device type C:

| Device C Assumptions |  |  |
| --- | --- | --- |
|  | Transmission power (dBm) | Device height (m) |
| Device C | 0, 10, 20 | 1.5 |

UL link budgets were calculated using three different receiver infrastructures aligning with connectivity topologies 1-4.

| Receiver assumptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Infrastructure | Antenna Gain (dBm) | Noise Figure (dB) | Infrastructure height (m) | Sensitivity (dBm) |
| Assisting/intermediate Node | 0 | 5 | 1.5 | -115.9 |
| Micro | 17 | 10 | 10 | -110.9 |
| Macro | 17 | 5 | 20 | -115.9 |

Note: Sensitivity calculated assuming a temperature of 290K, ASK modulation, and 1% BER.

The Urban Macro (UMa) path loss model was used based on [7].

## Downlink Coverage Assumptions

The following values were used when calculating the DL coverage analysis for device types A, B, and C:

| Transmitters Assumptions |  |  |  |
| --- | --- | --- | --- |
| Source | Transmission power (dB) | Transmitted power in 15kHz | Infrastructure height (m) |
| UE/Intermediate Node/Assisting Node | 23, 26 | 23, 26 | 1.5 |
| Micro BS | 35 | 16.76* | 10 |
| Macro BS | 46 | 27.76* | 20 |

*Note: Transmitted power calculated assumes 10dB of power spectral density boosting for both Micro and Macro base stations and 10MHz system bandwidth.

| Receiver Assumptions |  |  |
| --- | --- | --- |
| Transmitter model | Noise figure (dB) | Receiver Sensitivity |
| Device A | 100.9, 95.9, | -25, -50 |
| Device B | 95.9, 90.9 | -50, -75 |
| Device C | 85.9 | -100 |

The Urban Macro (UMa) path loss model was used based on [7].

# Appendix II: Agreements and WA in RAN #98e

Proposal 4-3b-v2:

Agree that for basestation deployments (when present), “Coexistence with existing 3GPP

technologies” can be:

Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type

Deployed on new sites without an assumption of an existing 3GPP deployment

Proposal 5-1a-extended, in section 6.1.6 (Energy harvesting text)

The following text is included in TR 38.848, with precise location up to later decision, unless there is an

objection to letting the precise location be decided later, in which case the text is included in an Annex

of the TR.

“Companies have reported the following energy sources for energy harvesting in literature: RF,

solar/light, piezoelectric (kinetic/vibration), electromagnetic, electrostatic, heat/thermal,

thermoelectric, magnetic, wind/water, acoustic, etc.”

Proposal 5-3b-v2: the following set of Ambient Iot devices are considered in the SI:

Device A: No energy storage, no harvesting ambient sources, no independent signal generation, i.e.

backscattering transmission

Device B: Has energy storage from harvesting ambient sources, no independent signal generation, i.e.

backscattering transmission. Use of stored energy can include amplification for reflected signals

Device C: Has energy storage from harvesting ambient sources, has independent signal generation,

i.e. active RF component for transmission

FFS: Whether to include device function

FFS: Whether to include a target maximum power consumption for each device

FFS: Whether/how to describe what stored energy is used for (in addition to the statement for Device B)

FFS: if combination of these devices will be considered.

Working assumption:

Proposal 5-2a-extended: Make a working assumption for this framework to categorize energy storage:

Storage 1: no storage at all

Storage 2: Up to E1 joules

Storage 3: Up to E2 joules

FFS: In RAN#99 value(s) of E1, E2 and it is possible that E1=E2, in which case we have only two storage

categories. Note in this case that storage 2 and 3 could be replaced by a single description such as ”limited

energy storage”, instead.

Proposal 4-4v2: Define the following topologies as starting points for the next meeting:

  - Topology (1): BS <-> Ambient IoT device

    - NOTE 1: Includes the possibility of BS Rx and BS Tx in different BSs

  - Topology (2): BS <-> Intermediate node <-> Ambient IoT device

    - NOTE 1: Intermediate node can be relay, IAB, UE, repeater, etc. which is capable of ambient IoT

  - Topology (3): BS <-> assisting node <-> Ambient IoT device <-> BS

    - NOTE 1: Assisting node can be relay, IAB, UE, repeater, etc. which is capable of ambient IoT

    - FFS: If the two BS can be different

  - Topology (4): UE <-> Ambient IoT device

  - FFS: Topology (5) UE <-> Ambient IoT device <-> {BS or UE}

NOTE: For potential topology (5), discuss its relation with other topologies, its necessity, etc. in RAN#99.

NOTE: For all topologies: The Ambient IoT device may be provided with carrier wave from another node(s) either inside or outside the topology

NOTE: For all topologies: The links in each topology may be bidirectional or unidirectional

FFS: Whether to consider combination of different topologies in the study.

FFS: BS, UE, or assisting UE could be multiple BSs, UEs or assisting UEs, respectively.

Proposal 2, in section 3.6 (Handling of SA1 use cases)

Define the groups of Grouping A as follows, as a start point:

Indoor

Outdoor

Indoor/outdoor

Define the groups of Grouping B as follows, as a start point:

Inventory

Sensors

Positioning

Command

Whether to incorporate Grouping A and Grouping B according to Approach 1 (include both separately) or Approach 2 (Group first by A, and second by B) will be decided in RAN#99.

Mapping of SA1 use cases to the groups of each grouping will be discussed in the next meeting, including whether RAN needs to attempt that mapping, or only has to define the groups.

Conclusion 3-2  and Proposal 3-1 in section 4.6 (Deployment scenarios table)

FFS: whether/which rows can be indicated with more than one value, e.g. Environment = Indoor/Outdoor

FFS: Possible values for each characteristic row (see following questions)

- FFS: Whether device characteristic is added to the table.

Proposal 4-1v2, in section 5.1.6 (Environment)

‘Environment of device’ can be ‘indoor’, ‘outdoor’, ‘indoor or outdoor’.

FFS: Whether to further describe other aspects, e.g. the size of the served area.

Proposal 4-2 in section 5.2.5 (Basestation characteristic)

Proposal 4-2: Agree that ‘basestation characteristic’ can be: macro-cell based deployment, micro-cell based

deployment, pico-cell base deployment, or none.

FFS: If/how it is necessary to reflect mobile IAB basestation.

Proposal 4-3a in section 5.3.5 (on in-band, guard-band, standalone band)

Proposal 4-3a: Agree that the study considers Ambient IoT deployment in-band to NR, in guardband of NR, and standalone band from NR, and FFS: relationship to deployment scenarios.

Note: Prioritization among them can be discussed in later meetings.

Proposal 4-5, in section 5.5.5 (Spectrum)

Proposal 4-5: Agree that spectrum in a deployment scenario is: licensed FDD, licensed TDD, unlicensed.

Note: Further discuss if the study should apply any limitations to the cases for which unlicensed spectrum is studied.

Conclusion 4-6 in section 5.6.5 (Traffic assumption)

FFS: whether the TR will describe different types of device-terminated traffic, e.g. Device-Terminated command and Device-Terminated reporting trigger, and whether to describe relationships between device-originated and device-terminated traffic, etc.

# Appendix III: Agreements and WA in RAN #99

RP-230795	Results of offline discussions on ambient IoT	Huawei

Proposal 6-1 (offline consensus): Agree to set at least the design targets below in Ambient IoT in the RAN SI.

- Device power consumption

- Device complexity

- Coverage

- Data rate

- Maximum message size (or maximum ‘TB’ size)

- Latency

- Positioning accuracy

- Connection/device density

- Device speed (FFS absolute or relative or both)

Proposal 3.5 (offline consensus): The following 5 deployment scenarios are studied and captured in the TR:

- Note1: all the detailed values (with or without square brackets) in the tables are to be further discussed

- Note2: the location of assisting or intermediate node, where applicable for the corresponding topologies, would need to be defined additionally.

Deployment 1: Device indoors, base station indoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico- cell BS |
|  | Connectivity topology | Topology (1),[(2)],[(3)] |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2) |
|  | Spectrum | Licensed FDD, or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 3: Device indoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory [indoor sensor] indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Deployment 4: Device outdoors, Outdoor base station

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 5: Device outdoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory [outdoor sensor] outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Proposal 6-3a-v2 (Wed offline consensus): Device design target for power consumption during transmitting/receiving is:

- [Device A ≤ 10 μW] or [Device A ≤ 1 μW]

- Device A ≪ Device B < Device C, or Device A ≤ Device B < Device C

- Device C ≤ 1 mW to ≤ 10 mW

Proposed working assumption (Wed offline consensus): Device complexity design target is:

- Device A: Comparable to UHF RFID ISO18000-6C (EPC C1G2)

- Device A < Device B < Device C

- Device C: Orders-of-magnitude lower than NB-IoT

Proposal 6-3d-v2 (Wed offline consensus): User experienced data rate design target is, at least for the uplink:

- Maximum  not less than: 5 kbps

- Minimum  not less than 0.1 kbps

Deployment scenarios

Proposal: Add device characteristics into the deployment scenario table, with possible values of ‘Device A’, ‘Device B’, ‘Device C’.

Proposal: All rows in the deployment scenario tables can be indicated with more than one value.

Connectivity topologies

Proposal: Do not include Topology (5) in the study.

- Note: it is understood this topology can be realized via combinations of other topologies

RP-230801	Results of offline discussions on ambient IoT (part2)	Huawei

proposal 3.4-v2 endorsed

Proposal 3.4-v2 (WED EVE offline consensus):

Traffic assumption of Device-Originated (DO) is further clarified as including Device-Originated autonomous (DO-DOA) and Device-Terminated triggered (DO-DTT), in the study.

- Note: this makes no assumption on whether an Ambient-IoT device may have only Tx capability

proposal 3.5#2 endorsed

Proposal 3.5#2 (WED EVE offline): The deployment scenario tables are updated as shown below.

Deployment 1 (WED EVE offline consensus): Device indoors, base station indoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico- cell BS |
|  | Connectivity topology | Topology (1),[(2)],[(3)] |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) Note: topology (3) where either the uplink or the downlink coverage is extended |
|  | Spectrum | Licensed FDD, or Licensed TDD or unlicensed (for topology (2) and (3)) |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 3: Device indoors, UE indoors based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory [indoor sensor] indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | No base station in the topology |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | FFS-[Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] Note: a device C could also support such deployment |

Deployment 4: Device outdoors, Outdoor base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 5: Device outdoors, UE outdoors based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory [outdoor sensor] outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | No base station in the topology |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | FFS-[Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

proposal 4.3 v2 endorsed

Proposal 4.3-v2 (WED EVE offline consensus):

BS, UE, or assisting node or intermediate node could be multiple BSs or UEs, respectively. The mixture of indoor and outdoor placement of such nodes is regarded as a network implementation choice. Account would need to be taken of potential impact on device or node complexity. In the connectivity topologies,

- This does not imply the existence of multi-hop assisting or intermediate nodes

- Capture this above statement in TR 38.848,

No need to re-express any agreed topologies to account for this above statement.

proposal 5.2-1 endorsed

Proposal 5.2-1 (WED EVE offline consensus):

The following FFS points from the RAN#98e agreement don’t need further action:

- FFS: Whether/how to describe what stored energy is used for (in addition to the statement for Device B)

- FFS: if combination of these devices will be considered