3GPP TSG-RAN WG3 #120		                                         R3-232607

Incheon, Republic of Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1014 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TS 38.423 on RB Set Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Lenovo, Qualcomm, Nokia, Nokia Shanghai Bell, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-5-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification, the description of the start RB index of the first RB set in RB Set Configuration is “The start RB index of the first RB set is the lowest index of RB of the IAB-DU cell.”. However, the meaning of “the lowest index of RB of the IAB-DU cell” is not clear and would lead to misunderstanding. RAN3 discussed two potential understandings in RAN3#118 meeting and sent an LS (R3-226781) to RAN1 for confirmation. In RAN3#119bis-e meeting, the reply LS from RAN1 was received in R3-231105 with the following answer: Answer: RAN1 has discussed the above question and would like to clarify that the modified understanding 2 as below is correct: - The start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is the RB index of the lowest common RB with the reference SCS, which overlaps with the lowest usable RB across all SCS-specific carriers provided by the NR Carrier List IE for this cell.. Based on the reply LS from RAN1, the description for the start RB index of the first RB set in the RB Set Configuration IE needs to be corrected to reflect the latest RAN1 agreement. |
| Summary of change: |  | Add the following text in the description for the start RB index of the first RB set: “If NR Carrier List IE(9.3.1.137) is provided, the start RB index of the first RB set is the RB index of the lowest common RB with the SCS provided by RB Set Configuration IE, which overlaps with the lowest usable RB across all SCS-specific carriers provided by the NR Carrier List IE for the IAB-DU cell. Otherwise, the start RB of the first RB set is aligned with point A for the IAB-DU cell”. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact under functional point of view. The impact can be considered isolated because the change affects only the IAB related procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The meaning of “the lowest index of RB of the IAB-DU cell” in Number of RB Sets IE is not clear and would lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.97 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev 1: - Correct R3-2xxxxxx to R3-231105. Rev 2: - Resubmission in RAN3#120 meeting. |
| --- | --- |

-------------------------------------------Start of change-------------------------------------------

#### 9.2.2.97	RB Set Configuration

This IE contains the configuration for up to M non-overlapping RB sets for a given gNB-DU cell, used for frequency domain resource allocation. This IE is only applicable if the gNB-DU is an IAB-DU. The maximum value of M is 8.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. |
| RB Set Size | M |  | ENUMERATED (rb2, rb4, rb8, rb16, rb32, rb64) | Number of PRBs in each RB set. If the RB sets of IAB-DU H/S/NA resource configuration do not cover the entire carrier bandwidth, the remaining RBs not part of an RB set configuration are considered as included in the last RB set. |
| Number of RB Sets | M |  | INTEGER(1.. maxnoofRBsetsPerCell) | Number of configured RB sets. The RB sets are contiguous and non-overlapping. If NR Carrier List IE(9.2.2.63) is provided, the start RB index of the first RB set is the RB index of the lowest common RB with the SCS provided by RB Set Configuration IE, which overlaps with the lowest usable RB across all SCS-specific carriers provided by the NR Carrier List IE for the IAB-DU cell. Otherwise, the lowest subcarrier of the start RB of the first RB set is aligned with point A for the IAB-DU cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per IAB-DU cell. Value is 8. |

-------------------------------------------End of change-------------------------------------------