3GPP TSG RAN Meeting #100	RP-231186

Taipei, June 12-14, 2023

Source:	China Telecom, University of Electronic Science and Technology of China

Title:	Discussion on Ambient IoT

Agenda Item:	9.2.2

Document for:	Discussion and decision

Introduction

In RAN# 99 meeting, issues on ambient IoT were discussed [1][2]. Use cases from SA1 outcomes were categorized and mapped to the RAN level use cases, environment deployment scenarios are further confirmed and RAN design targets are further discussed. The ambient IoT communication study has got the rudiment.

In this contribution, aspects of ambient IoT will be further discussed. In section 2, deployment scenarios are further discussed, including coexistence problem, deployment placement and spectrum. In section 3, views on the device characteristics are illustrated. In section 4, consideration of the RAN design targets is provided. And in section 5, feasibility & functionality are further described.

Deployment scenarios

  - Coexistence

The description of co-existence is heatedly discussed in the last meeting. when base station deployment is present, co-site and new site are utilized to distinguish whether the deployed base station reuses the legacy 3GPP infrastructure or a new 3GPP base station is needed. For the new site, the existing base station and backscatter receiver are located in a different place, and the interference analysis of co-site and new site is different as well. When base station deployment is absent, user equipment such as a smartphone is regarded as a reader for Ambient IoT, sidelink communication enhancement is considered for the deployment scenario and UE enhancement is the key enhancement point, which is quite different from the dimension considered when the base station is present.

Observation 1: The dimension considered for whether the UE is regarded as an Ambient IoT reader is quite different.

If the base station is absent, the description of coexistence with the existing 3GPP technology should be considered from several perspectives. On one hand, since the traditional UE is regarded as the Ambient IoT reader, the carrier transmitter and signal receiver will be integrated, which means the transceiver and the traditional UE are located in the same geographical location. From this perspective, deployment difference does not exist between the legacy UE and the new ‘reader’ UE, and only the description of ‘co-equipment’ can be utilized for such a deployment state. On the other hand, the ‘new’ UE reader needs corresponding new functions to be added, such as carrier sending, information receiving and demodulating which may be different based on different traffic assumptions, i.e., DO/DT, so the new technologies, such as the newly defined frame structure, signal process, etc, will be introduced based on them. From this perspective, the coexistence with existing 3GPP technologies can be described as ‘new/legacy 3GPP technologies’. However, it seems not appropriate to define a deployment classification based on spec difference. Therefore, it is really hard to find the most appropriate description for the new ‘reader’ UE and the legacy UE. To keep the discussion ongoing, a description of ‘co-equipment’ for the coexistence with the existing 3GPP technology seems pretty fine at this stage.

Proposal 1: For the description of ‘Coexistence with existing 3GPP technologies’ when the base station is absent, ‘co-equipment’ need to be prioritized.

  - Placement

According to the previous discussion, the frequency domain location for Ambient IoT transmission and reception should be confirmed. Three placements are considered as candidate positions, which are in band, guard band and standalone respectively. For the in band, existing OFDM frequency/time resource can be utilized for Ambient IoT communication, performance will be guaranteed and no more additional channel requirements need to be met, such as EVM or ACLR. Especially for the lower band like 2.1GHz or 700MHz, a better coverage range can be achieved in the current Ambient IoT deployment. Besides, utilization of the existing OFDM system will introduce co-existence problems with the NR legacy signals, signal structure and corresponding carrier waveform design may need further study. Considering the current study of LP-WUS is still ongoing and some identified problems are similar, solutions can be reused as a starting point to reduce the study workload.

Observation 2: In band placement has some advantages in the coverage performance.

Observation 3: Solutions on the LP-WUS can be reused as a starting point to reduce the study workload.

For the guard band, frequency resource between the operator band will be utilized, and such placement does not affect the transmission of the existing operator bands and frequency domain resources will be effectively saved. However, the guard band size is quite limited for the current spectrum. As shown in the following table [3], if 2.1GHz FDD (20MHz) and 3.5GHz TDD (100MHz) are utilized, the guard band size is at most 2RB, which may be limited for the potential coverage performance. Besides, the guard band deployment will be challenging for the corresponding RAN4 metric, especially for the ACLR. Hence, the prioritization of guard band deployment should be carefully considered.

Observation 4: The guard band deployment is more limited by the frequency resource and ACLR metric.

Table 5.3.3-1: Minimum guardband (kHz) (FR1)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

For standalone deployment, no co-existence problem is caused and specific spectrum resources will be utilized for the Ambient IoT communication. Best transmission and latency performance can be achieved, and the impact on the existing operator band is minimal. However, standalone deployment needs a dedicated new band, which is difficult in the FR1 spectrum where most of the spectrum resource has been occupied and inevitably vacating work has to be done. Therefore, standalone deployment is hard to be applied in the current FR1 spectrum.

Observation 5: Standalone deployment is hard to be applied in the current FR1 spectrum.

Therefore, the above three deployment methods have their own pros and cons, placement should be carefully considered and selected based on scenario characteristics. Considering the current study of LP-WUS is still ongoing and some solutions can be reused for the in band deployment, in band deployment can be slightly prioritized in the potential placement.

Proposal 2: In band deployment can be slightly prioritized in the potential placement.

  - Spectrum

In the last meeting, the deployment spectrum in each scenario is discussed. For the unlicensed band, the transmission power is limited and the interference is more serious compared with the licensed band, causing a huge performance degradation and is hard to be solved in the current technology. Meanwhile, the method of LBT in the unlicensed band is not effective for resource allocation, if passive or semi-passive devices are deployed in the existing scenario, a competition mechanism is needed for the initial access of multiple devices, and power consumption will be also increased due to continuous monitoring of LBT for the access attempt. Hence, the unlicensed band has some limitations on performance and should be deprioritized.

Observation 6: The unlicensed band has some limitations on the transmission performance.

For the licensed band, FDD is definitely having a better coverage range performance than the TDD band due to its lower carrier frequency and more transmission resource, i.e., all slots can be utilized for uplink/downlink specially. Moreover, uplink-to-downlink conversion needs accurate clock timing and synchronization performance. Since Ambient IoT device is simpler than the traditional device, such problem will affect the transmission performance and lower the coverage range. Considering TDD band has a larger bandwidth than FDD which may be beneficial for coverage enhancement solutions, the TDD band can be preserved at this stage, and FDD band can be prioritized if needed.

Observation 7: FDD band has a better coverage range performance than the TDD band.

Proposal 3: TDD band can be preserved at this stage, and FDD band can be prioritized if needed.

Proposal 4: Unlicensed band should be deprioritized.

Device categorization

Ambient IoT device descriptions were studied in the last meeting, devices are characterized according to their energy storage capacity and the RF signals capability of their transmissions. The following candidate categorization method should be downselected:

Proposal 5.1-1-v3: The TR captures energy storage levels as: - Option 1: Two levels - Storage 1: Without energy storage - Storage 2: With limited energy storage - Option 2: Three levels - Storage 1: Without energy storage - Storage 2 and 3: Up to E1/E2 joules - For this case, 2 exemplary traffic models are chosen, and E1, E2 calculated accordingly. - Model 1: power = per-device design target, rate = 5 kbps, message size = {as agreed under design target}. - Model 2: power = per-device design target, traffic = continuously ON until end of an inventory process. - Option 3: Status quo, i.e. TR reports the three storage levels, and that E2 > E1. (If no agreement, this option results).

Two aspects need to be considered for the energy storage capacity. On the one hand, from the perspective of business requirements, the energy storage capacity is calculated according to different traffic models with different design targets, thus more than two energy storage capacities will be obtained based on their coupled traffic models. On the other hand, from the point of hardware implementation, the energy storage capacity depends on different implementation methods and can be further divided into different levels. The capacity values are related to energy sources, conversion efficiency, etc., and is hard to acquire a constant one. Therefore, no matter which aspects are considered, the exact capacity values are hard to be unified and should be further confirmed at the WG-level.

For the description captured in TR, since device A has the capacity of energy storing while device B and C are not able to store energy, a general description of option 1 is sufficient to illustrate their characteristics.

Proposal 5: The two-level description ‘Without energy storage’ and ‘With limited energy storage’ is prioritized to be captured in TR.

RAN design target

  - Device power consumption

In the last meeting, device power consumption is discussed and the following proposal has been achieved for further discussion:

Proposal 6-3a-v2 (Wed offline consensus): Device design target for power consumption during transmitting/receiving is: - [Device A ≤ 10 μW] or [Device A ≤ 1 μW] - Device A ≪ Device B < Device C, or Device A ≤ Device B < Device C - Device C ≤ (1 to 10) mW, with possibility to strive for hundreds of μW. Device C ≤ 1 mW to ≤ 10 mW

Since the complexity of device is proposed to refer to UHF RFID ISO18000-6C (EPC C1G2), the power consumption can be as a reference as well, which means a range of less than 1μW ~10μw can be considered. Similarly, the power consumption of device C can be described based on the same method, i.e., a range of less than 1mw~10mW. For device B, power consumption level should not be regarded much higher than device A. Compared with deivice A, device B has the function of energy storage, and the stored energy can be used to amplify reflected signals, so the power consumption of device B is naturally larger than or equal to device A. In addition, considering both device A and B work by backscatter technology, there exists the same energy consumption situation, so the power consumption difference should not have a large gap. Therefore, the description of ‘≪’ is not reasonable, the description of ‘Device A ≤ Device B < Device C’ is better.

Observation 8: The description of ‘≪’ is not reasonable, description of ‘Device A ≤ Device B < Device C’ is better

Proposal 6: Prioritize the description of ‘Device A ≤ Device B < Device C’

  - Latency

In the last meeting, the latency design target is also discussed, and the following proposal has been achieved:

## Proposal 6-3f-v3: Latency is defined as the end-to-end latency. ## FFS: Value/values ## FFS: Whether to define a minimum and a maximum latency, or only one ## FFS: Whether to differentiate latency among DO and DT traffic. ## Assume that RAN WGs would refine this overall target into e.g. user plane latency, control plane latency, or etc.

For the RAN level latency, public network mainly focused on the average latency, and private network pays more attention to the latency under a certain cumulative probability such as 99.99% sample points less than 50ms, then different considerations on the Ambient IoT network are different again. For the Ambient IoT network, a maximum latency should be confirmed due to the effect on the receiving time windows design, and detection performance will be affected as well, thus at least a maximum latency should be defined. Finally, considering one-way latency or bidirectional latency will be different based on DO or DT traffic type, the differentiating latency among DO and DT traffic is necessary.

Proposal 7: At least the maximum latency should be defined

Proposal 8: Differentiating latency among DO and DT traffic is necessary.

  - Device speed

For device speed, the impact of two types of movement speed should be considered for the Ambient IoT network. One is the absolute speed, which can be regarded as the typical ‘UE speed’, the QoS level should be guaranteed no matter how large the speed is and what type of the current network. That means, the access to the ambient IoT network quality should be acceptable regardless of whether in a plane or on a high-speed railway. Hence, mobility management should be great enough. Another is the relative speed, which is the relative movement speed of the device to the reader. From the perspective of mobile communication, relative speed is more meaningful since the reader may also be moveable and the accurate Doppler frequency shift is reflected only by relative speed. Therefore, the consideration of relative speed is necessary. Moreover, positioning accuracy of the Ambient IOT device will also be affected by both absolute and relative speed, both of the two speed types need to be considered.

Proposal 9: Both absolute speed and relative speed should be considered for the device speed.

Feasibility & functionality

  - Functionalities

In the last meeting, the following potential assumed RAN functionalities were found in the submitted papers：

- Security (authentication, encryption, data integrity, authorization) - Positioning/localization/ranging - Support of channel access regulations associated with unlicensed spectrum. - Coexistence with legacy systems, devices, and network deployments - Energy harvesting signals, and/or use of legacy signals for energy harvesting - Identification and management of devices - Possibility of CN connection, including for sporadic and opportunistic small data between device and core - Suggest to send LS to SA on feasibility of solutions with/without CN - Backscattering modulation - Envelope detection in receiver - Non-OFDM waveform in DL, robust against low-accuracy receiver architecture - Compact protocol layers design - Anti-collision methods / random access procedure / tag-reaction load distribution - Interference mitigation (intra-reader, inter-reader, reader – cellular network) - For topology (1) and (2), gNB/UE/intermediate node may need limited full-duplex capability - Mobility management procedures - Communicate with all, a subset, or one of the Ambient IoT devices present - Activation, deactivation of ambient IoT device - NW configuration of signals and channels for communication with Ambient IoT devices - Control of when tag reflects/reacts to a received signal when addressed - DRX - Synchronization scheme robust against low-accuracy receiver architecture - Coverage enhancement techniques with low device complexity

These above 22 functionalities are divergent and better to further categorize these functions so that a more clear enhancement framework can be referred to, i.e., potentially required functionality in the physical layer, in the high-level protocol, or in other great classification methods.

Proposal 10: Different functions of Ambient IOT are better to be further categorized, i.e., potentially required functionality in the physical layer, in the high-level protocol, or in other great classification methods.

Conclusion

In this contribution, we discuss the issues of ambient IoT, and have the following proposals:

Observation 1: The dimension considered for whether the UE is regarded as an Ambient IoT reader is quite different.

Observation 2: In band placement has some advantages in the coverage performance.

Observation 3: Solutions on the LP-WUS can be reused as a starting point to reduce the study workload.

Observation 4: The guard band deployment is more limited by the frequency resource and ACLR metric.

Observation 5: Standalone deployment is hard to be applied in the current FR1 spectrum.

Observation 6: The unlicensed band has some limitations on the transmission performance.

Observation 7: FDD band has a better coverage range performance than the TDD band.

Observation 8: The description of ‘≪’ is not reasonable, description of ‘Device A ≤ Device B < Device C’ is better

Proposal 1: For the description of ‘Coexistence with existing 3GPP technologies’ when the base station is absent, ‘co-equipment’ need to be prioritized.

Proposal 2: In band deployment can be slightly prioritized in the potential placement.

Proposal 3: TDD band can be preserved at this stage, and FDD band can be prioritized if needed.

Proposal 4: Unlicensed band should be deprioritized.

Proposal 5: The two-level description ‘Without energy storage’ and ‘With limited energy storage’ is prioritized to be captured in TR.

Proposal 6: Prioritize the description of ‘Device A ≤ Device B < Device C’

Proposal 7: At least the maximum latency should be defined

Proposal 8: Differentiating latency among DO and DT traffic is necessary.

Proposal 9: Both absolute speed and relative speed should be considered for the device speed.

Proposal 10: Different functions of Ambient IOT are better to be further categorized, i.e., potentially required functionality in the physical layer, in the high-level protocol, or in other great classification methods.

- Reference

- RP-230795, Results of offline discussions on ambient IoT, 3GPP TSG-RAN Meeting #99, Rotterdam, Netherlands, March 20-23, 2023.

- RP-230801, Results of offline discussions on ambient IoT (part 2), 3GPP TSG-RAN Meeting #99, Rotterdam, Netherlands, March 20-23, 2023.

- TS 38.104 v18.0.0, Base Station (BS) radio transmission and reception (Release-18).