# 3GPP TSG-RAN WG4 Meeting # 107         	                                                    R4-2309865

# Incheon, Korea, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.113 | CR | 0123 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS 37.113 MSR base station test configuration and performance criteria R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2023-5-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Note 2 and Note 3 for CS7 is missing in current version test configuration. 35MHz and 45MHz channel bandwidth are missing in NR performance criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Note 2 and Note 3 to CS7 test configuration. Add 35MHz and 45MHz channel bandwidth into NR performance criteria. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Note 2 and Note 3 will be missing, then no optional for NB-IoT in band and guard band. The performance criteria for 35MHz and 45MHz channel bandwidth would not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 6.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------Start of text changes -------------

## 4.5	BS test configurations

The present clause defines the BS test configurations that shall be used for demonstrating conformance. This is specified in table 4.5-1, 4.5-1a, and 4.5-1b for multi-RAT capable MSR Base Stations, in table 4.5-2 for single-RAT capable BS and in table 4.5-3 for multi-band capable BS. For other BS supporting more than one RAT (other than MSR BS), table 4.5-2 applies separately for each RAT supported.

The test configurations apply according to the declared RAT Capability Set (CS) of the MSR Base Station according to subclause 4.7 of TS 37.141 [11] and the Band Category of the declared operating band (BC1, BC2 or BC3), as listed in the heading of each table.

The test configurations (TCx) are defined in TS 37.141 [11], subclause 4.8.

For a BS declared to be capable of contiguous operation only, the test configuration(s) in tables 4.5-1 and 4.5-2 denoted by a "C" shall be used for testing.

For a BS declared to be capable of contiguous and non-contiguous operation and where the parameters in the manufacture's declaration according to subclause 4.7.2 of TS 37.141 [11] are identical for contiguous and non-contiguous operation, the test configurations denoted by "CNC" shall be used.

For a BS declared to be capable of contiguous and non-contiguous operation and where the parameters in the manufacture's declaration according to subclause 4.7.2 of TS 37.141 [11] are not identical for contiguous and non-contiguous operation, the test configurations denoted by "C/NC" shall be used for testing.

For a BS declared to support NB-IoT operating in-band, the test configuration(s) in table 4.5-1 and 4.5-2 denoted by "NI" shall be used for testing.

For a BS declared to support NB-IoT operating in guard band, the test configuration(s) in table 4.5-1 and 4.5-2 denoted by "NG" shall be used for testing.

For a BS declared to support NB-IoT operating in guard band and in-band, the test configuration(s) in table 4.5-1 and 4.5-2 denoted by "NG" or/and "NI" shall be used for testing.

For a BS declared to support NB-IoT standalone, the test configuration(s) in table 4.5-1a, 4.5-1b and 4.5-2 and entries that refer to single-RAT specifications shall be used for testing.

For immunity tests:

-	The communication link for the RAT(s) listed in the table shall be established according to subclause 4.2.

-	Tests for ports relating to the RAT(s) supported shall be performed according to subclause 4.1.

Table 4.5-1: Test configurations for single-band Multi-RAT capable MSR BS (CS3-6)

| Capability Set | UTRA + E-UTRA. NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) (CS 3) |  |  | GSM+ UTRA (CS 4) | GSM + E-UTRA, NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) (CS 5) | GSM + UTRA + E-UTRA, (CS 6) | GSM+UTRA/ E-UTRA, UTRA+ E-UTRA, NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) (CS7) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC2 | BC2 | BC2 | BC2 |
| Emission tests | C: TC3a CNC: NTC3 C/NC: TC3a and NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a and NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: NTC4a C/NC: TC4a and NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b and NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c and NTC4c | C: (TC4a, TC3a) (Note 4), TC4b CNC: (NTC4a, NTC3) (Note 4), NTC4b C/NC: (TC4a, NTC4a,TC3a, NTC3) (Note 4), TC4b,NTC4b NI: TC15,(TC16) (Note 4) NG: TC18, (TC19) (Note 4) |
| Immunity tests | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC5a CNC: NTC5a C/NC: TC5a, NTC5a | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 | C: TC5b CNC: NTC5c, C/NC: TC5b, NTC5c | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 |
| NOTE 1: Void NOTE 2: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE 3: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. NOTE 4: For Band 3, the test configuration is only applicable if UTRA is declared to be supported in Band 3. |  |  |  |  |  |  |  |

------------Start of next changes -------------

### 6.1.5	NR performance criteria

The throughput in table 6.1.5-1 is stated relative to the maximum throughput of the FRC. The maximum throughput for an FRC is equal to the [payload size * the number of uplink subframes per second].

The BS Uplink and Downlink paths shall each meet the performance criteria defined in table 6.1.5-1 during the test. If the Uplink and Downlink paths are evaluated as a one loop, then the criterion is two times the throughput reduction shown in table 6.1.5-1. After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1.5-1: NR BS performance criteria for continuous phenomena for BS

| NR channel bandwidth as defined in TS 38.104 section 5.3.2-1 [35] (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate (as in TS 38.104 [35], annex A.1) | Performance criteria (Note1, Note 2) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 | Throughput > 95 %, No loss of service |
| 10, 15 | 30 | G-FR1-A1-2 |  |
| 10, 15 | 60 | G-FR1-A1-3 |  |
| 20 to 50 | 15 | G-FR1-A1-4 |  |
| 20 to 100 | 30 | G-FR1-A1-5 |  |
| 20 to 100 | 60 | G-FR1-A1-6 |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

-------------End of changes -------------