3GPP TSG-RAN WG4 Meeting # 107	R4-2309900

Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0356 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.141-1, Add sweep time for true RMS detection mode for receiver spurious emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The sweep time for true RMS detection mode for unwanted emissions was agreed, but the sweep time for true RMS detection mode for receiver spurious emssions is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add sweep time for true RMS detection mode for receiver spurious emssion in sub-clause 7.6.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sweep time for true RMS detection mode for receiver spurious emission is not clear, which will affect measurement accuracy of receiver spurious emission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.6	Receiver spurious emissions

7.6.1	Definition and applicability

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the antenna connector (for BS type 1-C) or at the TAB connector (for BS type 1-H). The requirements apply to all BS with separate RX and TX antenna connectors / TAB connectors.

NOTE:	In this case for FDD operation the test is performed when both TX and RX are ON, with the TX antenna connectors / TAB connectors terminated.

For antenna connectors / TAB connectors supporting both RX and TX in TDD, the requirements apply during the transmitter OFF period. For antenna connectors / TAB connectors supporting both RX and TX in FDD, the RX spurious emissions requirements are superseded by the TX spurious emissions requirements, as specified in clause 6.6.5.

For RX-only multi-band connectors, the spurious emissions requirements are subject to exclusion zones in each supported operating band. For multi-band connectors that both transmit and receive in operating band supporting TDD, RX spurious emissions requirements are applicable during the TX OFF period, and are subject to exclusion zones in each supported operating band.

For BS type 1-H manufacturer shall declare TAB connector RX min cell groups (D.33). Every TAB connector of BS type 1-H supporting reception in an operating band shall map to one TAB connector RX min cell group, where mapping of TAB connectors to cells/beams is implementation dependent.

The number of active receiver units that are considered when calculating the conducted RX spurious emission limits (NRXU,counted) for BS type 1-H is calculated as follows:

NRXU,counted = min(NRXU,active , 8 × Ncells)

NRXU,countedpercell is used for scaling of basic limits and is derived as NRXU,countedpercell = NRXU,counted / Ncells, where Ncells is defined in clause 6.1.

NOTE:	NRXU,active is the number of actually active receiver units and is independent to the declaration of Ncells.

7.6.2	Minimum requirement

The minimum requirements for BS type 1-C and BS type 1-H are in TS 38.104 [2], clause 7.6.2.

7.6.3	Test purpose

The test purpose is to verify the ability of the BS to limit the interference caused by receiver spurious emissions to other systems.

7.6.4	Method of test

7.6.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band operation, see clause 4.9.1,

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.9.1.

7.6.4.2	Procedure

The minimum requirement is applied to all connectors under test,

For BS type 1-H where there may be multiple TAB connectors they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.4.4. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the connector under test to measurement equipment as shown in annex D.2.6 for BS type 1-C and in annex D.4.4 for BS type 1-H.

2)	For FDD operation, for separate RX only connectors with single carrier operation set the connector under test to transmit at manufacturers declared rated carrier output power (Prated,c,AC or Prated,c,TABC, D.21). Channel set-up shall be according to NR-FR1-TM 1.1.

For FDD operation, for separate RX only connectors declared to be capable of multi-carrier and/or CA operation (D.15-D.16) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.

For separate RX only connectors declared to be capable of NB-IoT operation in NR in-band (D.41), test shall be performed using N-TM according to clause 4.9.2.2.9.

For TDD connectors capable of transmit and receive ensure the transmitter is OFF.

3)	Set the measurement equipment parameters as specified in table 7.6.5.1-1.

The measurement device characteristics shall be:

-	Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.3-1.

4)	Measure the spurious emissions over each frequency range described in table 7.6.5.1-1.

In addition, for a multi-band connector, the following steps shall apply:

5)	For multi-band connector and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

<End of Change 1>