3GPP TSG-RAN WG5 Meeting #100	R5-234222

Toulouse, France, 21st – 25th Aug, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0224 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of R16 NR Positioning Support default test conditions in TS 37.571-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN#90-e, WI of UE Conformance Test Aspects - NR Positioning Support has been approved, related contents should be introduced into TS 37.571-5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the specification scope to include release 16 NR positioning methods. - Added Release-16 NR positioning methods associated abbreviations - Added description of DL-AoD and DL-TDOA test scenario and assistance data - Added sub-test 25 UE supporting GNSS (Rel-15 onwards) in GNSS scenarios and assistance data - Added sub-test 23 UE supporting MBS (Rel-16 onwards) in MBS scenarios - Added ReferencePoint and RelativeLocation information elements for UE-based DL-TDOA signal tests - Added ReferencePoint and RelativeLocation information elements for UE-based DL-AoD signal tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to Release-16 NR positioning methods signal test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 3, 4, 6, 8, 11, 12(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

1	Scope

The present document specifies the test scenarios and assistance data required for the conformance and minimum performance tests for FDD or TDD mode of UTRA, E-UTRA and NR for the User Equipment (UE) that supports one or more of the defined positioning methods. For UTRA these are Assisted Global Positioning System (A-GPS) and Assisted Global Navigation Satellite System (A-GNSS). For E-UTRA these are A-GNSS, Observed Time Difference of Arrival (OTDOA), Enhanced Cell ID (ECID), Wireless Local Area Network (WLAN), Metropolitan Beacon Systems (MBS) and Bluetooth. For NR these are A-GNSS, Observed Time Difference of Arrival (LTE) (OTDOA (LTE)), Enhanced Cell ID (LTE) (ECID (LTE)), Wireless Local Area Network (WLAN), Metropolitan Beacon Systems (MBS), Bluetooth, NR Enhanced Cell-ID (NR-ECID), Multiple-Round Trip Time (Multi-RTT), Downlink Angle-of-Departure (DL-AoD) and Downlink Time Difference Of Arrival (DL-TDOA).

The present document also specifies the GNSS scenario files for the test scenarios defined in TS 36.508 for V2X and aerial testing.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[3]	Void

[4]	Void

[5]	Void

[6]	3GPP TS 37.571-1: “User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance”.

[7]	3GPP TS 37.571-2: “User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance”.

[8]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[9]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[10]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[11]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[12]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[13]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[14]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[15]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[16]	3GPP TS 25.331: “Radio Resource Control (RRC); Protocol specification”

[17]	STANAG 4294: NATO STANAG 4294. Navstar Global Positioning System (GPS) System Characteristics.

[18]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[19]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[20]	3GPP TS 36.508: "Common test environments for User Equipment (UE) conformance testing".

[21]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[22]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[23]	Bluetooth Special Interest Group (SIG): “Bluetooth Core Specification v4.2”, December 2014.

[24]	3GPP TS 38.508-1: "User Equipment (UE) conformance specification; Part 1: Common test environment".

[25]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", China Satellite Navigation Office, December 2019.

[26]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December 2017.

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 36.101 [2], TS 36.104 [18] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Horizontal Dilution Of Precision (HDOP): measure of position determination accuracy that is a function of the geometrical layout of the satellites used for the fix, relative to the receiver antenna

3.2	Symbols

For the purposes of the present document, the following symbols apply:

B1I	BeiDou B1I navigation signal with carrier frequency of 1561.098 MHz.

B1C	BeiDou B1C navigation signal with carrier frequency of 1575.420 MHz.

E1	Galileo E1 navigation signal with carrier frequency of 1575.420 MHz.

E5	Galileo E5 navigation signal with carrier frequency of 1191.795 MHz.

E6	Galileo E6 navigation signal with carrier frequency of 1278.750 MHz.

G1	GLONASS navigation signal in the L1 sub-bands with carrier frequencies 1602 MHz ± k  562.5 kHz.

G2	GLONASS navigation signal in the L2 sub-bands with carrier frequencies 1246 MHz ± k  437.5 kHz.

k		GLONASS channel number, k = -7…13.

L1 C/A	GPS or QZSS L1 navigation signal carrying the Coarse/Acquisition code with carrier frequency of 1575.420 MHz.

L1C	GPS or QZSS L1 Civil navigation signal with carrier frequency of 1575.420 MHz.

L2C	GPS or QZSS L2 Civil navigation signal with carrier frequency of 1227.600 MHz.

L5	GPS or QZSS L5 navigation signal with carrier frequency of 1176.450 MHz.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-Galileo	Assisted-Galileo

A-GANSS	Assisted- Galileo and Additional Navigation Satellite Systems

A-GLONASS	Assisted- GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (English: Global Navigation Satellite System)

A-GNSS	Assisted Global Navigation Satellite System

A-GPS	Assisted - Global Positioning System

AP	Access Point

AWGN	Additive White Gaussian Noise

BDS	BeiDou Navigation Satellite System

C/A	Coarse/Acquisition

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-TDOA	Downlink Time Difference Of Arrival

DUT	Device Under Test

ECEF	Earth Centred, Earth Fixed

ENB	Evolved Node B

EN-DC	E-UTRA-NR Dual Connectivity

E-UTRA 	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For further study

GANSS	Galileo and Additional Navigation Satellite Systems

GEO	Geostationary Earth Orbit

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (English: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GSS	GNSS System Simulator

HDOP	Horizontal Dilution Of Precision

ICD	Interface Control Document

ICS	Implementation Conformance Statement

IS	Interface Specification

LOS	Line Of Sight

LPP	LTE Positioning Protocol

MBS	Metropolitan Beacon System

Multi-RTT	Multi-Round Trip Time

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NG-RAN	NextGen Radio Access Network

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NR E-CID	NR Enhanced Cell ID (positioning method)

PPM	Parts per million

PRS	Positioning Reference Signal

QZSS	Quasi-Zenith Satellite System

RRC	Radio Resource Control

SBAS	Space Based Augmentation System

SCC	Secondary Component Carrier

SFN	System Frame Number

SS	System simulator

SV	Space Vehicle

SV ID	Space Vehicle Identification

TDD	Time Division Duplex

TOD	Time Of Day

TOW	Time of Week TTFF	Time To First Fix

UE	User Equipment

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

4	General

4.1	GPS and GNSS orbital model information, assistance data and assistance data files

The following subclauses 5 and 6 define the GPS and GNSS orbital model information, the assistance data and the assistance data files (subclause 5 only) for the test cases as follows:

Subclause 5.1: data for UTRA A-GPS Signalling test cases defined in TS 37.571-2 [7] subclauses 6.1.1 to 6.1.3.

Subclause 5.2: data for UTRA A-GPS Minimum Performance test cases defined in TS 37.571-1 [6] subclause 5.

Subclause 6.1: data for UTRA, E-UTRA and NR A-GNSS Signalling test cases defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9.

Subclause 6.2: data for UTRA, E-UTRA and NR A-GNSS Minimum Performance test cases defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

For subclause 5 the orbital model information is defined and where appropriate is given in Yuma format in .txt files for each scenario in the appropriate data file specified in Annex A.

For subclause 6 the orbital model information is defined and where appropriate is given in Rinex navigation data file format or Yuma format in .txt files for each scenario in the appropriate data file specified in Annex B.

For subclause 5, where the assistance data is fixed or is not required on a per-satellite basis, then it is defined in the following subclauses. Where assistance data is required on a per-satellite basis, or where the values of the data also vary with time then it is specified in comma-separated-variable files in the appropriate data file specified in Annex A. These files specify the values to be used for each satellite, indexed by satellite PRN or SV ID, and, where applicable, the values to be used indexed by both time and satellite PRN or SV ID.

For subclause 6, the assistance data is defined in the following subclauses.

For the aerial GNSS scenarios defined in TS 36.508 [20], the orbital model information is defined and where appropriate is given in Yuma format in .txt files for each scenario in the appropriate data file specified in Annex B.

4.2	OTDOA assistance data

The following subclause 7 defines the OTDOA assistance data for the test cases as follows:

Subclause 7.1: data for OTDOA Signalling test cases defined in TS 37.571-2 [7].

Subclause 7.2: data for OTDOA Measurement test cases defined in TS 37.571-1 [6].

Subclause 7.3: data for OTDOA Measurement test cases for Carrier Aggregation defined in TS 37.571-1 [6].

4.3	MBS scenario and assistance data

The following subclause 8 defines the MBS scenario and assistance data for the test cases as follows:

Subclause 8.1: scenario data for MBS signalling test cases defined in TS 37.571-2 [7].

Subclause 8.2: scenario data for MBS performance test cases defined in TS 37.571-1 [6].

Subclause 8.3: assistance data for MBS performance test cases defined in TS 37.571-1 [6] and signalling test cases defined in TS 37.571-2 [7].

4.4	WLAN scenario and assistance data

The following subclause 9 defines the WLAN scenario and assistance data for the test cases as follows:

Subclause 9.1: scenario data for WLAN signalling test cases defined in TS 37.571-2 [7].

Subclause 9.2: scenario data for WLAN performance test cases defined in TS 37.571-1 [6].

Subclause 9.3: assistance data for WLAN signalling test cases defined in TS 37.571-2 [7].

4.5	Bluetooth scenario data

The following subclause 10 defines the Bluetooth scenario for the test cases as follows:

Subclause 10.1: scenario data for Bluetooth signalling test cases defined in TS 37.571-2 [7].

4.6	DL-TDOA scenario data

The following subclause 11 defines the DL-TDOA scenario for the test cases as follows:

Subclause 11.1.2: scenario data for DL-TDOA performance test cases defined in TS 37.571-1 [6].

Subclause 11.2: common scenario data for DL-TDOA performance test cases defined in TS 37.571-1 [6] and signalling test cases defined in TS 37.571-2 [7].

4.7	DL-AoD scenario data

The following subclause 12 defines the DL-AoD scenario for the test cases as follows:

Subclause 12.1: common scenario data for DL-AoD performance test cases defined in TS 37.571-1 [6] and signalling test cases defined in TS 37.571-2 [7].

< Unchanged sections omitted >

6	GNSS information

6.1	GNSS Scenarios and Assistance Data for Assisted GNSS signalling tests

6.1.1	General

This subclause defines the GNSS scenario and the associated assistance data that shall be used where required for UTRA, E-UTRA and NR Assisted GNSS signalling tests defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9.

In all cases the Assistance Data is given in the two necessary formats, RRC format for TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and LPP format for TS 37.571-2 [7] subclauses 7 and 9. Other information is also given separately for TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9 where it differs between the subclauses.

The satellite simulator shall generate all the UE supported GNSS satellite signals defined in subclause 6.1.2 and/or shall provide assistance data dependent on the UE capabilities defined in subclause 6.1.3. Note that some tests require assistance data to be provided even though satellite signals are not required.

The A-GNSS signalling test cases may include several sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined below. In some cases the detailed assistance data content defined in subclause 6.1.3 depends on the particular sub-test case.

Table 6.1.1-1: Sub-Test Case Number Definition for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS only |
| 2 | UE supporting A-Galileo only |
| 3 | UE supporting A-GPS and Modernized GPS only |
| 4 | UE supporting A-GPS and A-GLONASS only |
| 8 | UE supporting A-GPS and A-Galileo only |
| 9 | UE supporting A-BDS only |
| 10 | UE supporting A-GPS and A-BDS only |

Table 6.1.1-2: Sub-Test Case Number Definition for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | Void |
| 2 | Void |
| 3 | Void |
| 4 | Void |
| 7 | UE supporting GNSS(1) and OTDOA |
| 8 | Void |
| 9 | Void |
| 10 | Void |
| 15 | UE supporting GNSS(1) |
| 25 | UE supporting GNSS(1) (Rel-15 onwards) |
| Note 1: The GNSS combination of GPS, GLONASS, Galileo or BDS supported by the UE |  |

The term SV ID used in this subclause is defined as the satellite PRN for GPS, as Code Number for Galileo, as the satellite Slot Number for GLONASS and as the Ranging Code Number for BDS.

6.1.2	GNSS Scenario

Table 6.1.2-0: GNSS Scenarios to be used

| Scenarios | Condition |
| --- | --- |
| GNSS scenarios from 2020 with the navigation data files defined in Table 6.1.2-1 and Table 6.1.2-2 and the SV IDs defined in Tables 6.1.2-2A to 6.1.2-4 | px_GnssScenario2012 = FALSE |
| GNSS scenarios from 2012 with the navigation data files defined in Table 6.1.2-5 and Table 6.1.2-6 and the SV IDs defined in Table 6.1.2-7 and Table 6.1.2-8 | px_GnssScenario2012 = TRUE |
| Note: the GNSS scenarios from 2012 may be used until September 2023. |  |

If px_GnssScenario2012 = FALSE, the following GNSS scenario shall be used. The assistance data specified in the following subclauses is consistent with this GNSS scenario:

-	Rinex navigation data files: the required file(s) in the GNSS orbital data sig zip file specified in Annex B are given below.

Table 6.1.2-1: Rinex navigation data files for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | Rinex navigation datafile(s) |
| --- | --- |
| 1 | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| 2 | Sig GNSS Galileo 2020_9_17 Rinex.txt |
| 3 | Sig GNSS GPS 2020_9_17 Rinex.txt |
| 4 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| 8 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS Galileo 2020_9_17 Rinex.txt |
| 9 | Sig GNSS BDS 2020_9_17 Rinex.txt |
| 10 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS BDS 2020_9_17 Rinex.txt |

Table 6.1.2-2: Rinex navigation data files for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | Rinex navigation datafile(s) (1) |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15, 25 | GPS | Sig GNSS GPS 2020_9_17 Rinex.txt |
|  | GLONASS | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
|  | Galileo | Sig GNSS Galileo 2020_9_17 Rinex.txt |
|  | BDS | Sig GNSS BDS 2020_9_17 Rinex.txt |
| Note 1: Where the UE supports more than one GNSS then all the relevant Rinex navigation data files are used |  |  |

-	UE location and Reference location: Static at latitude: 35 degrees 44 minutes 39.432 seconds north, longitude: 139 degrees 40 minutes 48.633 seconds east, (Tokyo Japan), height: = 300m.

-	Nominal start time: 17th September 2020 23:40:00  (GPS time).

-	The visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given in Table 6.1.2-2A.

Table 6.1.2-2A: SV IDs of Visible satellites

| GNSS | SV IDs of Visible satellites |
| --- | --- |
| GPS | 3, 4, 6, 17, 19, 28 |
| GLONASS | 3, 4, 5, 10, 18, 19 |
| Galileo | 3, 5, 13, 15, 21, 27 |
| BDS | 38, 40, 42, 43, 59, 60 |

-	For BDS, the satellite types are given in Table 6.1.2-2B

Table 6.1.2-2B: BDS satellite types

| BDS Satellite type | SV IDs of Satellites |
| --- | --- |
| GEO | 59, 60 |
| IGSO | 38, 40 |
| MEO | 42, 43 |

-	The visible satellites to be simulated in each sub-test case are given  in Table 6.1.2-3 and Table 6.1.2-4

Table 6.1.2-3: Satellites to be simulated for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 3, 4, 5, 10, 18, 19 (GLONASS) |
| 2 | 3, 5, 13, 15, 21, 27 (Galileo) |
| 3 | 3, 4, 6, 17, 19, 28 (GPS) (Note) |
| 4 | GPS: 3, 4, 6, 28. GLONASS: 5, 16, 18, 19. |
| 8 | GPS: 3, 4, 6, 28. Galileo: 3, 5, 13, 21. |
| 9 | 38, 40, 42, 43, 59, 60 (BDS) |
| 10 | GPS: 3, 4, 6, 28. BDS: 38, 40, 59, 60. |
| Note: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. |  |

Table 6.1.2-4: Satellites to be simulated for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | SV IDs of Satellites to be simulated |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15, 25 (Note) | GPS | 3, 4, 6, 17, 19, 28 |
|  | GLONASS | 3, 4, 5, 10, 18, 19 |
|  | Galileo | 3, 5, 13, 15, 21, 27 |
|  | BDS | 38, 40, 42, 43, 59, 60 |
| Note: For this sub-test the satellite simulator shall generate all the GNSS signals supported by the UE for all the simulated satellites. |  |  |

If px_GnssScenario2012 = TRUE, the following GNSS scenario shall be used. The assistance data specified in the following subclauses is consistent with this GNSS scenario:

-	Yuma Almanac data: the required file(s) in the GNSS data sig zip file specified in Annex B are given below.

Table 6.1.2-5: Yuma / AGL Almanac data files for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | Yuma / AGL file(s) |
| --- | --- |
| 1 | Sig GNSS 1-1 AGL.txt |
| 2 | Sig GNSS 1-2 Yuma.txt |
| 3 | Sig GNSS 1-3 Yuma.txt |
| 4 | Sig GNSS 1-3 Yuma.txt and Sig GNSS 1-1 AGL.txt |
| 8 | Sig GNSS 1-3 Yuma.txt and Sig GNSS 1-2 Yuma.txt |
| 9 | Sig GNSS 1-9 Yuma.txt |
| 10 | Sig GNSS 1-3 Yuma.txt and Sig GNSS 1-9 Yuma.txt |

Table 6.1.2-6: Yuma / AGL Almanac data files for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | Yuma / AGL file(s) (1) |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15, 25 | GPS | Sig GNSS 1-3 Yuma.txt |
|  | GLONASS | Sig GNSS 1-1 AGL.txt |
|  | Galileo | Sig GNSS 1-2 Yuma.txt |
|  | BDS | Sig GNSS 1-9 Yuma.txt |
| Note 1: Where the UE supports more than one GNSS then all the relevant Yuma / AGL data files are used |  |  |

-	UE location and Reference location:

Static at latitude: 35 degrees 44 minutes 39.432 seconds north, longitude: 139 degrees 40 minutes 48.633 seconds east, (Tokyo Japan 2012), height: = 300m.

-	Nominal start time:

1st January 2012 00:31:00 (GPS time).

-	Visible satellites simulated are given below

Table 6.1.2-7: Satellites to be simulated for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 3, 4, 9, 10, 18, 20 |
| 2 | 5, 10, 11, 18, 19, 20 |
| 3 | 1, 11, 17, 20, 23, 28 (Note) |
| 4 | GPS: 1, 17, 20, 28. GLONASS: 3, 10, 18, 20 |
| 8 | GPS: 1, 17, 20, 28. Galileo: 5, 10, 11, 18. |
| 9 | 1, 2, 7, 18, 21, 27 |
| 10 | GPS: 1, 17, 20, 28. BDS: 1, 7, 18, 21. |
| Note: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. |  |

Table 6.1.2-8: Satellites to be simulated for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | SV IDs of Satellites to be simulated |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15, 25 (Note) | GPS | 1, 11, 17, 20, 23, 28 |
|  | GLONASS | 3,4,9,10,18,20 |
|  | Galileo | 5,10,11,18,19,20 |
|  | BDS | 1,2,7,18,21,27 |
| Note: For this sub-test the satellite simulator shall generate all the GNSS signals supported by the UE for all the simulated satellites. |  |  |

-	Ionospheric model: see values in subclause 6.1.3

-	The levels of the simulated satellites shall all be at -125dBm +/- 6dB

< Unchanged sections omitted >

8	MBS information

8.1	Scenario for MBS signalling tests

8.1.1	Introduction

This clause defines the Metropolitan Beacon System (MBS) scenario that shall be used where required for E-UTRA and NR MBS signalling tests defined in TS 37.571-2 [7] clauses 7 and 9.

The beacon simulator shall generate all the UE supported MBS beacon signals defined in subclause 8.1.2.

The MBS sub-test case is identified by a Sub-Test Case Number 12 for Rel-13 only, Sub-Test Case Number 16 for Rel-14 onwards and Sub-Test Case Number 23 for Rel-16 onwards as defined in Table 8.1.1-1.

Table 8.1.1-1: Sub-Test Case Number Definition for TS 37.571-2 clauses 7 and 9

| Sub-Test Case Number | Supported Positioning Method |
| --- | --- |
| 12 | UE supporting MBS (Rel-13 only) |
| 16 | UE supporting MBS (Rel-14 onwards) |
| 23 | UE supporting MBS (Rel-16 onwards) |
| NOTE: Metropolitan Beacon System (MBS) is a specific type of Terrestrial Beacon System (TBS) [21] |  |

8.1.2	MBS Signalling Scenario

The following MBS scenario shall be used:

-	Simulated UE location for Rel-13 only: not applicable, for Rel-14 onwards: Latitude: 37.787528º, Longitude: -122.40337	13º, Altitude: 13 m

-	The levels of the simulated beacons shall all be at the power levels shown in Table 8.1.2-1 +/- 6dB

Table 8.1.2-1: General test parameters for the beacons to be simulated for TS 37.571-2 clauses 7 and 9

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Number of Beacons |  | 4 | Beacons 1 to 4. Transmitted in the first four consecutive beacon slots in the MBS beacon transmission period. Other slots contain no simulated beacons. |
| Centre Frequency | MHz | 925.977 |  |
| RF Channel |  | AWGN |  |
| MBS Beacon Configuration |  | TB1 (2 MHz) | For details see [21] |
| MBS Packet Type |  | Type 2 | For details see [21] |
| Beacon PN Code | Integer | Chosen for each beacon from the PN code list for TB1 | Each of the 4 beacons uses a different PN code. For details see [21] |
| Transmit power | dBm | -90 |  |

Table 8.1.2-2: MBS Beacon Payload fields and code phase delay for the beacons to be simulated for TS 37.571-2 clauses 7 and 9

| Beacon | MBS Tx ID [21] | Slot Index [21] | All Other fields [21] | Code phase delay (ms) |
| --- | --- | --- | --- | --- |
| 1 | Equal to Slot number | Equal to Slot number | min value (bit_value = 0) | 0 |
| 2 | Equal to Slot number | Equal to Slot number | min value (bit_value = 0) | 0 |
| 3 | Equal to Slot number | Equal to Slot number | min value (bit_value = 0) | 0 |
| 4 | Equal to Slot number | Equal to Slot number | min value (bit_value = 0) | 0 |
| Note: bit_value is the conversion of the binary number represented by the corresponding bits in the payload to decimal. |  |  |  |  |

8.2	Scenario for MBS performance tests

The Metropolitan Beacon System (MBS) scenario that shall be used for E-UTRA and NR MBS performance tests is defined in TS 37.571-1 [6] clause 11.

8.3	MBS Assistance Data (Release 14 onwards)

8.3.1	Introduction

This subclause defines the Metropolitan Beacon System (MBS) Assistance Data that shall be used where required for the MBS measurement tests defined in TS 37.571-1 [6] clause 11 and E-UTRA and NR MBS signalling tests defined in TS 37.571-2 [7] clauses 7 and 9.

8.3.2	MBS Almanac Assistance Data for signalling tests

MBS almanac assistance data is used in the MBS UE-based signalling tests as indicated in TS 37.571-2 [7]. The following fields shall be included in the MBS almanac assistance data (MBS-AlmanacAssistance-r14).

Table 8.3.2-1: MBS-AlmanacAssistance-r14 values

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| transmitterID-r14 | Integer | Any value 0 to 32777 | Rel-14 onwards |
| transmitterLatitude-r14 | degrees | See Table 8.3.2-2 | Rel-14 onwards |
| transmitterLongitude-r14 | degrees | See Table 8.3.2-2 | Rel-14 onwards |
| transmitterAltitude-r14 | meters | See Table 8.3.2-2 | Rel-14 onwards |
| timeCorrection-r14 | ns | See Table 8.3.2-2 | Rel-14 onwards |

Table 8.3.2-2: MBS-AlmanacAssistance-r14 beacon details

| Beacon number | Latitude | Longitude | Altitude | Time Correction |
| --- | --- | --- | --- | --- |
| Beacon 1 | 37.78670 | -122.40324 | 92.22 | 0 |
| Beacon 2 | 37.79132 | -122.39822 | 130.79 | 0 |
| Beacon 3 | 37.78915 | -122.40676 | 52.49 | 0 |
| Beacon 4 | 37.78554 | -122.39800 | 66.70 | 0 |

8.3.3	MBS Acquisition Assistance Data for signalling and measurement tests

MBS acquisition assistance data is used in the MBS signalling tests as indicated in TS 37.571-2 [7] clauses 7 and 9 and the MBS measurement tests as indicated in TS 37.571-1 [6] clause 11. The following fields shall be included in the MBS acquisition assistance data (MBS-AcquisitionAssistance-r14).

Table 8.3.3-1: MBS-AcquisitionAssistance-r14 values

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| transmitterID-r14 | Integer | Any value 0 to 32777 | Rel-14 onwards |
| mbsConfiguration-r14 |  | tb1 for TB1 beacon parameters and tb2 for TB2 beacon parameters | Rel-14 onwards |
| pnCodeIndex-r14 | Integer | Any value 1 to 128 | Rel-14 onwards |
| freq-r14 | Hz | 925977000 for tb1 or 924442000 for tb2 | Rel-14 onwards |

< Unchanged sections omitted >

11	DL-TDOA information

11.1	DL-TDOA Assistance data for DL-TDOA measurement tests

11.1.1	General

This subclause defines the DL-TDOA assistance data that shall be used for the DL-TDOA measurement tests defined in TS 37.571-1 [6].

11.1.2	DL-TDOA Assistance Data

This subclause defines the DL-TDOA assistance data elements which shall be provided to the UE in the DL-TDOA measurement tests defined in TS 37.571-1 [6].

Table 11.1.2-1: Sequence data values for 13 instances of sequence for test case 14.2.1

| Cell | Value physCellId | Value cellidentity |  | Value PRS muting info | Value PRS RE offset | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |  |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘01’ | 0 | 0 |  |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘10’ | 1 | 23 |  |
| Dummy cell | 7 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘01’ | 0 | 46 |  |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘10’ | 1 | 0 |  |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘01’ | 0 | 23 |  |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘10’ | 1 | 15 |  |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘01’ | 0 | 0 |  |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘10’ | 1 | 23 |  |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘01’ | 0 | 0 |  |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘10’ | 1 | 46 |  |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘01’ | 0 | 0 |  |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘10’ | 1 | 23 |  |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘01’ | 0 | 23 |  |

Table 11.1.2-2: Sequence data values for 6 instances of sequence for test case 14.2.2 for the first PFL

| Cell | Value physCellId | Value cellidentity |  | Value PRS muting info | Value PRS RE offset | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |  |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘01’ | 0 | 0 |  |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘10’ | 1 | 23 |  |
| Dummy cell | 7 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘01’ | 0 | 46 |  |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘10’ | 1 | 0 |  |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘01’ | 0 | 23 |  |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘10’ | 1 | 15 |  |

Table 11.1.2-3: Sequence data values for 7 instances of sequence for test case 14.2.2 for the second PFL

| Cell | Value physCellId | Value cellidentity |  | Value PRS muting info | Value PRS RE offset | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |  |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘01’ | 0 | 0 |  |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘10’ | 1 | 23 |  |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘01’ | 0 | 0 |  |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘10’ | 1 | 46 |  |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘01’ | 0 | 0 |  |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘10’ | 1 | 23 |  |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘01’ | 0 | 23 |  |

11.2	ReferencePoint and RelativeLocation

This subclause defines the reference point and relative location information elements which shall be provided to the UE in the UE-based DL-TDOA measurement tests defined in TS 37.571-1 [6] and signalling tests defined in TS 37.571-2 [7].

Table 11.2-1: ReferencePoint

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReferencePoint-r16 ::= SEQUENCE { |  |  |  |
| referencePointGeographicLocation-r16 CHOICE { |  |  |  |
| ha-location3D-r16 SEQUENCE { |  |  |  |
| degreesLatitude-r15 | 852891909 | 35.744287 |  |
| degreesLongitude-r15 | 1666449411 | 139.680176 |  |
| altitude-r15 | 38400 | 300 m |  |
| uncertaintySemiMajor-r15 | 2 |  |  |
| uncertaintySemiMinor-r15 | 2 |  |  |
| orientationMajorAxis-r15 | 0 |  |  |
| horizontalConfidence-r15 | 68 |  |  |
| uncertaintyAltitude-r15 | 2 |  |  |
| verticalConfidence-r15 | 68 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2-2: RelativeLocation for NR Cell 2

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RelativeLocation-r16 ::= SEQUENCE { |  |  |  |
| milli-arc-second-units-r16 | mas30 |  |  |
| height-units-r16 | m |  |  |
| delta-latitude-r16 SEQUENCE { |  |  |  |
| delta-Latitude-r16 | 250 |  |  |
| coarse-delta-Latitude-r16 | 0 |  |  |
| } |  |  |  |
| delta-longitude-r16 SEQUENCE { |  |  |  |
| delta-Longitude-r16 | 250 |  |  |
| coarse-delta-Longitude-r16 | 0 |  |  |
| } |  |  |  |
| delta-height-r16 SEQUENCE { |  |  |  |
| delta-Height-r16 | 0 |  |  |
| coarse-delta-Height-r16 | 0 |  |  |
| } |  |  |  |
| locationUNC-r16 SEQUENCE { |  |  |  |
| horizontalUncertainty-r16 | 2 |  |  |
| horizontalConfidence-r16 | 68 |  |  |
| verticalUncertainty-r16 | 2 |  |  |
| verticalConfidence-r16 | 68 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2-3: RelativeLocation for NR Cell 3

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RelativeLocation-r16 ::= SEQUENCE { |  |  |  |
| milli-arc-second-units-r16 | mas30 |  |  |
| height-units-r16 | m |  |  |
| delta-latitude-r16 SEQUENCE { |  |  |  |
| delta-Latitude-r16 | 250 |  |  |
| coarse-delta-Latitude-r16 | 0 |  |  |
| } |  |  |  |
| delta-longitude-r16 SEQUENCE { |  |  |  |
| delta-Longitude-r16 | 250 |  |  |
| coarse-delta-Longitude-r16 | 0 |  |  |
| } |  |  |  |
| delta-height-r16 SEQUENCE { |  |  |  |
| delta-Height-r16 | 0 |  |  |
| coarse-delta-Height-r16 | 0 |  |  |
| } |  |  |  |
| locationUNC-r16 SEQUENCE { |  |  |  |
| horizontalUncertainty-r16 | 2 |  |  |
| horizontalConfidence-r16 | 68 |  |  |
| verticalUncertainty-r16 | 2 |  |  |
| verticalConfidence-r16 | 68 |  |  |
| } |  |  |  |
| } |  |  |  |

12	DL-AoD information

12.1	ReferencePoint and RelativeLocation

The reference point and relative location information elements which shall be provided to the UE in the UE-based DL-AoD measurement tests defined in TS 37.571-1 [6] and signalling tests defined in TS 37.571-2 [7] are the same as those defined in clause 11.2.