3GPP TSG RAN Meeting #101		                          RP-231780

Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.12.1

Source:			OPPO

Title:				Discussion on normative work of LP-WUS in Rel19

Document for:	Discussion & Decision

Introduction

In this contribution, we discuss the WG study results of Low Power Wake Up Signal and the conclusion of the study. We also consider how the following work will be done in the Rel19.

Low Power WUR/WUS study

RAN1 study progress

Basically, aspect of LP-WUS waveform candidates, receivers, content, different RRC modes, measurement, procedure, and coverage were studied.

During those RAN1 meetings, there have been many discussions on LP WUR architectures. Three main types of receiver architectures for LP-WUR have been identified.

- Architecture with RF envelope detection

- Heterodyne architecture with IF envelope detection

- Homodyne/zero-IF architecture with baseband envelope detection

The discussion on L1 signal design and procedure for low power WUS have figured out several waveform generation methods, which are named as OOK1~4, FSK1/2 and OFDMA-based.

The details of the receiver architectures associated to waveform generation of LP-WUS were discussed, and some observations on the receiver architectures were agreed during past RAN1 meetings.

- Receiver architectures for OOK

- Parallel receiver architectures for FSK

- Receiver architectures for FSK with frequency to amplitude conversion

- Receiver architecture for OFDMA-based signals/channels

Based on the WUS/WUR options, different designs have been discussed.

In the evaluation methodologies, the LP-WUS study has included power models for Low-power Radio and Main Radio, traffic characteristics, coverage targets, hardware assumption and other necessary models. Those WUS solutions has been thoroughly evaluated in terms of performance, power saving gain, latency impact, overhead as described in the WID.

Study in RAN2/RAN4

RAN2 has also covered the higher layer issues including different coverage LR/MR, RRM measurement, DRX, and other procedure issues in different RRC modes. RAN4 also discussed the RF related issue including, the ACS and ASCS value, guard RB numbers and others. Some initial agreements have been made.

The potential normative works

The evaluations of LP-WUS/WUR shown that the power saving gain can be achieved under certain configurations, e.g. RRM relaxation and others. Power saving can be reached by waveform options of OOK1~4, FSK1/2 and OFDMA-based. The latency impact is small with introduction of the wake-up mechanism. And the overhead is small compared to the overall system resources. Although the gains can be different for those waveform options, we can still consider to support LP-WUS/WUR in Rel19.

Proposal1. We propose to have followed bullets in the potential LP-WUS work item for Rel19.

Specify physical layer structure and waveform for LP-WUS, which can be coexisted with NR normal signals in RAN1.

Specify physical layer structure and waveform for periodically transmitted LP-SS in RAN1.

Study and specify necessary physical layer and higher layer procedure for wake-up, RRM measurement by LR and MR in RAN1 and RAN2.

Introducing further RRM relaxation by MR in RAN2 and RAN4.

Specify the ACS and ASCS requirement for LP-WUS/LP-SS, and RB numbers for LP-WUS ASCS in RAN4.

The above scope is actually dependent to each other. The waveform selection would determine the LP-SS design, since the LP-SS is more fundamental to waveforms sensitive to the timing/frequency error. If multiple waveforms selected, the wake-up procedure should take them into account. Considering the LP-WUS is relatively new design, the workload would be very high if we still have too many candidates in the scopes. The further selection of the candidates will anyway be carried out. In the current evaluation, the results are already too diversified for cross-comparison. Different waveforms will be applied with different radio resources, due to the quite different signal generations.

It is also should be noted that RAN2 and RAN4 works could be much clearer with small number of fundamental waveforms. In the study phase, the assessments have to covered different options.

In the study stage, it was also observed that some of the wave form can outperform others. Thus, it would be better to select at least one category of waveform to help for other bullets of scope. We consider OOK1 and OOK4 can be the starting point. To facilitate the Rel19 work, RAN can decide the waveforms in the work item description.

Proposal 2: For waveform of both LP-WUS and LP-SS, OOK1 and OOK4 can be the starting point for designing physical layer structure.

Conclusion

Based on the discussion, we have the flowing proposals for the possible work in LP-WUS in Rel19:

Proposal1. We propose to have followed bullet in the potential LP-WUS work item for Rel19.

Specify physical layer structure and waveform for LP-WUS, which can be coexisted with NR normal signals in RAN1.

Specify physical layer structure and waveform for periodically transmitted LP-SS in RAN1.

Study and specify necessary physical layer and higher layer procedure for wake-up, RRM measurement by LR and MR in RAN1 and RAN2.

Introducing further RRM relaxation by MR in RAN2 and RAN4.

Specify the ACS and ASCS requirement for LP-WUS/LP-SS, and RB numbers for LP-WUS ASCS in RAN4.

Proposal 2: For waveform of both LP-WUS and LP-SS, OOK1 and OOK4 can be the starting point for designing physical layer structure.

Reference

- RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#97e, September 2022.

- TR38.869v031, “Study on low-power wake up signal and receiver for NR”, vivo