3GPP TSG RAN meeting #101			   			            RP-232002

Bangalore, India, September 11-15, 2023

Agenda item:	9.9

Source:	Qualcomm Incorporated

Title:	Views on release-independence for NR NTN bands

Document for:	Approval

# Introduction

NR NTN bands n256 and n255 were introduced in Rel-17 TS 38.101-5. TS 38.307 is listed as one of the impacted existing TSs/TRs in the WID of solutions for NR to support non-terrestrial networks (NTN) [1] which is to introduce NR NTN bands in a release-independent manner.

In RAN4#107 meeting, the CR on TS 38.307 was submitted to introduce Rel-17 NR NTN bands in Rel-17 TS 38.807 as the release-independent requirements [2]. However, no consensus was reached regarding whether to create a new specification or reuse TS 38.307 for NR NTN release-independent requirements and whether to make the changes from Rel-17 or Rel-18 specification. It was suggested by companies to further discuss these two issues in RAN plenary meeting.

In RAN#100, the paper on how to implement release-independent NR NTN bands was discussed in [3] and the following agreement was captured in the meeting report of RAN#100.

“RAN VC (AT&T): FR1 starting with REL-17 and FR2 starting with REL-18; will come back on Wed on the question whether to use 38.307 also for NTN aspects”.

Per the above meeting notes in RAN#100, it shows that FR1 NR NTN bands shall be specified in release-independent manner from Rel-17 and FR2 NR NTN bands shall be specified in a release-independent manner from Rel-18. While whether to create a new specification or reuse TS 38.307 is still open.

This paper further provides our views on release-independence for NR NTN bands.

# Discussion

## Create a new specification or reuse TS 38.307

TS 38.307 specifies requirements for NR UEs supporting release-independent features such as additional NR operating bands and power classes on top of TS 38.101-1/2/3/4 and TS 38.133. For NR FR1 NTN, the UE RF and demodulation requirements are specified in TS 38.101-5, and RRM requirements are specified in TS 38.133. For TS 38.101-5, the skeleton is almost the same as TS 38.101-1 and many clauses refer to TS 38.101-1 directly. Therefore, it is straightforward to specify the requirements of release-independent requirements for NR NTN UEs in TS 38.307.

In RAN#100, one option was proposed to create a new specification of NR NTN feature for ITU-R reasons. From our perspective, it is not necessary to create a new specification of NR NTN for ITU-R submission since RAN4 has already used separate specifications for NR NTN. And there were no issues for ITU-R submission with separate NR FR1 UE RF specification, i.e., TS 38.101-1 and NR FR2 UE RF specification, i.e., TS 38.101-2 which are both included in TS 38.307.

In addition, if a new specification is created to introduce the release-independent features for NR NTN, that means 3GPP will have to create a separate specification for IoT NTN release-independent features as well. It will result in a lot of changes to the TS 36.307 in which release independence requirements for IoT NTN have already been specified [3-4].

For the above reasons, we have the following proposal:

Propose 1: Specify NR NTN release-independent features in TS 38.307.

Proposal 1 of reusing TS 38.307 will include both FR1 and FR2 NR NTN bands in the TS 38.307. Following the agreements from RAN#100, FR1 bands shall be specified starting from Rel-17 TS 38.307 and FR2 bands shall be specified starting from Rel-18 TS 38.307.

Observation 1: With proposal 1 of reusing TS 38.307, both FR1 and FR2 NR NTN bands will be included in the TS 38.307. FR1 bands shall be specified starting from Rel-17 TS 38.307 and FR2 bands shall be specified starting from Rel-18 TS 38.307.

## How to introduce release independent NR NTN bands in TS 38.307

NR NTN bands n256 and n255 were specified in Rel-17 TS 38.101-5. NR NTN operating bands, UE power class and UE channel bandwidth should be release-independent from Rel-17 per [1]. As specified in TS 38.307, the following rule is regarding how to implement TS 38.307 when a new feature is introduced in the latest release of release independent which is also in line with the agreements in RAN#100 [4] (shown in Clause 1 of this paper).

“When a new feature is introduced only the latest release of release independent spec needs to be updated. The latest release of release independent spec refers to the release which the new feature is introduced in. If an RF feature introduced in the same release as the release which the feature is independent from, (i.e. M=N), the common UE RF requirements table in annex B.4 is specified from release N+1, otherwise the common UE RF requirements table is specified from release N.”

Therefore, we have the following proposal on how to introduce FR1 NR NTN bands release independence from Rel-17 in TS 38.307:

Proposal 2: Include Rel-17 FR1 NR NTN bands, power class and UE channel bandwidth from Rel-17 TS 38.307. The draft CR for FR1 NR NTN bands release independent is captured in Annex.

Proposal 3: Include Rel-18 FR2 NR NTN bands, power class and UE channel bandwidth from Rel-18 TS 38.307.

# Conclusion

In this paper, we provide our views on release-dependence for NR NTN bands. The following observation and proposals are made:

Propose 1: Specify NR NTN release-independent features in TS 38.307.

Observation 1: With proposal 1 of reusing TS 38.307, both FR1 and FR2 NR NTN bands will be included in the TS 38.307. FR1 bands shall be specified starting from Rel-17 TS 38.307 and FR2 bands shall be specified starting from Rel-18 TS 38.307.

Proposal 2: Include Rel-17 FR1 NR NTN bands, power class and UE channel bandwidth from Rel-17 TS 38.307. The draft CR for FR1 NR NTN bands release independent is captured in Annex.

Proposal 3: Include Rel-18 FR2 NR NTN bands, power class and UE channel bandwidth from Rel-18 TS 38.307.

# References

- RP-221169, “Revised WID: solutions for NR to support non-terrestrial networks (NTN)”, Thales, 3GPP RAN#96

- R4-2309249, “CR on TS 38.307 for NR NTN bands release independent”, Qualcomm, 3GPP RAN4#107

- RP-231214, “Views on release-independent frequency bands for NR NTN”, Qualcomm, 3GPP RAN#100

- RP-231502, “Draft Meeting Report for TSG RAN meeting #100”, 3GPP RAN#101

- RP-223546, “CR to TS 36.307: release independence requirements introduction for NTN IoT, Rel-17”, Huawei, HiSilicon, Mediatek, 3GPP RAN#98e

- R4-2302450, “CR to TS 36.307: release independence requirements introduction for IoT NTN, Rel-18”, Huawei, HiSilicon, Mediatek, RAN4#106

- R4-2312892, “[NR_NTN_solutions-Core] CR on TS 38.307 for NR NTN bands release independent”, Qualcomm, RAN4#108

# Annex: Draft CR on TS 38.307 for NR FR1 NTN bands release independent

====================Start of the changes=========================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: UE performance requirements".[6]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[7]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[8]	3GPP TS 38.101-5: "NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

====================End of the changes=========================

====================Start of the changes=========================

# 9	Release independent features for NR NTN operating in frequency range 1

## 9.1	Additional NR NTN operating bands and NTN UE power classes for frequency range 1

Requirements for a Rel-17 NTN UE for additional NR NTN operating bands power classes and NTN UE channel bandwidth in frequency range 1 compared to TS 38.101-5 of Rel-17 [8] are introduced via this clause.

Table 9.1-1: NR NTN operating bands

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| NR NTN bands | FDD | Rel-17 |  |

Table 9.1-2: NR NTN UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 3 | FDD | Rel-17 |  |

Table 9.1-3: NR NTN UE channel bandwidth

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- |
| UE channel bandwidth | FDD | Rel-17 |  |

======================End of the changes=======================