3GPP TSG RAN Meeting #101	RP-232054

Bangalore, India, September 11 - 15, 2023

Agenda Item:	8A.2.8

Source: 	CATT

Title: 	Discussion on the Scope of Rel-19 XR evolution WI

Document for: 	Discussion and Decision

Introduction

According to the conclusion in RAN Rel-19 workshop, XR evolution has been included in the list of Rel-19 topics endorsed in [1]. In this contribution, the scope of Rel-19 XR evolution WI is discussed from our perspective.

Discussion

Support of multi-modal communication service

The support of multi-modal communication service has been specified in SA1 and SA2 specifications [2][3] since Rel-18. As per SA1 and SA2 specifications, the multi-modal application is characterized by the correlated data flows (e.g. haptic and video service data) that need to be treated jointly at the NW and/or UE side, and the overall requirement of these data flows needs to be satisfied together. Nevertheless, since not any specification work specific to multi-modality support is possibly carried out by RAN WGs in Rel-18 XR, transmission performance for multi-modal service cannot be sufficiently guaranteed by the current RAN specifications. Specifically, RAN is unable to be aware of the correlated data flows, due to the lack of related information from 5GC, so that these correlated data flows, as in the legacy, are currently still treated independently in RAN. As a result, with no effective means to treat the inter-flow independency, the overall transmission performance of the correlated data flows cannot be ensured.

In addition, as a distinctive requirement specified in [2], synchronous transmission among these correlated data flows is required to guarantee the user experience for the multi-modal application (as cited below from [2]).

Table 1: Typical synchronization thresholds for immersive multi-modality VR applications

| Media components | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

RAN currently has not means to support such synchronous transmission among the correlated data flows in NR Uu either, resulting in potential user experience degradation with disordered data flow processing.

All above means that even with Rel-18 XR feature implemented, there are still insufficient mechanisms for RAN to support multi-modal services as per SA1/2 requirements.

Observation 1: SA1 and SA2 have already specified support of multi-modal communication services since Rel-18, but there has been no consideration on how to support the related characteristics/requirements in Rel-18 RAN XR WI (e.g. data flow correlation, inter-flow synchronization requirements, etc.). As a result, multi-modal service transfer may still not be well supported by RAN with required performance.

Therefore, enhancement to support multi-modal service transmission ought to be one main objective in RAN WI for Rel-19 XR evolution. At the first place, the awareness/identification of multi-modal services should be enabled at RAN and/or UE AS. Then, since the data flows are finally operated as QoS flows which are mapped to the radio bearers at RAN/UE AS for transmission, it is necessary to discuss the QoS flow mapping and DRB maintenance for the correlated data flows, regarding e.g. how to cope with the inter-flow/inter-DRB dependency to enable the joint treatment. Afterward, the synchronous transmission of correlated data flows should be supported by potential enhancements on gNB scheduling and/or UE multiplexing (e.g. sync-based LCP mechanism, more UE info reporting for scheduling, etc.). We think this objective may prioritize the single UE case, and have not found clear motivation/impact to support the multiple UE case.

To this end, it is proposed to study and specify, if needed, the enhancements to support multi-modal services, with the potential scope as follows from our perspective. It should be noted that coordination with SA2 would be needed on the specific QoS requirements (e.g. sync requirements) for multi-modal data flows, and this may be conducted during a study phase to justify the necessity/feasibility for further normative work on this objective. Also note that this discussion may be considered together with the PDU set concept which was specifically introduced for XR services in Rel-18.

Proposal 1: Study and if needed, specify enhancements to support multi-modal service transfer in RAN with following objectives:

- RAN/AS awareness of the correlated data flows associated with a multi-modal application; [RAN2, RAN3]

- QoS flow mapping and DRB maintenance considering the correlated data flows; [RAN2]

- Synchronous transmission of the correlated data flows for one UE [and multiple UEs] (e.g. scheduling enhancements). [RAN2, RAN1]

NOTE: This work may need coordination with SA WGs, and may be carried out taking PDU set handling into consideration.

XR-specific mobility enhancement

Whether to include an objective to support XR-specific mobility was discussed during the discussion on Rel-18 XR SI scope. Finally, this topic has not been included in either the Rel-18 XR SI/WI or mobility enhancement WI.

However, XR-specific characteristics and requirements (especially PDU-set parameters [3]) were introduced by SA2 and related PDU set specific enhancements are being introduced in RAN in Rel-18 accordingly.

TS 23.501 ### 5.7.7 PDU Set QoS Parameters #### 5.7.7.1 General PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. At least one PDU Set QoS Parameter shall be sent to the NG-RAN to enable PDU Set based QoS handling. The following PDU Set QoS Parameters are specified: 1. PDU Set Delay Budget (PSDB). 2. PDU Set Error Rate (PSER). 3. PDU Set Integrated Handling Information (PSIHI). The QoS Profile may include the PDU Set QoS Parameters described in this clause (see clause 5.7.1.2). The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile. If the NG-RAN receives PDU Set QoS Parameters and supports them, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters as described in this clause. […] #### 5.37.5.2 PDU Set Information and Identification To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above. The PDU Set Information comprises: - PDU Set Sequence Number. - Indication of End PDU of the PDU Set. - PDU Sequence Number within a PDU Set. - PDU Set Size in bytes. - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow. […]

To this end, we think adaptive enhancements with the consideration of PDU-set/data burst level characteristics/QoS parameters during mobility procedures should be supported to improve user experience. Although for Rel-18 XR WI there are some issues related to this aspect initially identified in RAN3 [4], not any conclusions have been made till now, and there has been not even any discussion carried out in RAN2. As a result, it is uncertain whether this topic can really be done within the Rel-18 time framework, considering the limited progress till now and that mobility enhancement is not within the scope of Rel-18 XR WI anyway.

Therefore, to satisfy the user experience for XR during mobility procedure, we propose to support PDU-set specific enhancements during handover in Rel-19 XR evolution (if not supported in Rel-18), including e.g., PDU set info transfer between RAN nodes, service continuity w.r.t. PDU set handling, etc.

Proposal 2: Specify the enhancements to support PDU set handling during handover (if not done in Rel-18 XR WI), including:

- PDU set related info transfer between RAN nodes; [RAN3, RAN2]

- Service continuity w.r.t. PDU set handling (e.g. loss less delivery). [RAN2, RAN3]

Potential RAN impacts by SA WGs

The initial draft of Rel-19 SID on XRM Ph2 was endorsed in [5] by SA2. It is envisioned that some objectives may lead to potential RAN impacts (e.g. alternative QoS and notification control enhancements, FEC enhancements, QoS flow mapping for multiplexed data flow, etc.), which should also be considered as part of the Rel-19 XR evolution work in RAN. Particularly, after the normative phase starts in SA2, RAN XR evolution WI may need to take potential SA2 progress into account, and identify necessary RAN impacts that need to be made to support related SA2 design.

Proposal 3: Rel-19 XR evolution WI should also take into account the necessary RAN impacts resulting from Rel-19 SA2 WI/SI on XRM Ph2.

Conclusion

According to the analysis in section 2, we provide below observations and proposals:

Observation 1: SA1 and SA2 have already specified support of multi-modal communication services since Rel-18, but there has been no consideration on how to support the related characteristics/requirements in Rel-18 RAN XR WI (e.g. data flow correlation, inter-flow synchronization requirements, etc.). As a result, multi-modal service transfer may still not be well supported by RAN with required performance.

Proposal 1: Study and if needed, specify enhancements to support multi-modal service transfer in RAN with following objectives:

- RAN/AS awareness of the correlated data flows associated with a multi-modal application; [RAN2, RAN3]

- QoS flow mapping and DRB maintenance considering the correlated data flows; [RAN2]

- Synchronous transmission of the correlated data flows for one UE [and multiple UEs] (e.g. scheduling enhancements). [RAN2, RAN1]

NOTE: This work may need coordination with SA WGs, and may be carried out taking PDU set handling into consideration.

Proposal 2: Specify the enhancements to support PDU set handling during handover (if not done in Rel-18 XR WI), including:

- PDU set related info transfer between RAN nodes; [RAN3, RAN2]

- Service continuity w.r.t. PDU set handling (e.g. loss less delivery). [RAN2, RAN3]

Proposal 3: Rel-19 XR evolution WI should also take into account the necessary RAN impacts resulting from Rel-19 SA2 WI/SI on XRM Ph2.

Reference

- RWS-230488	RAN Chair's summary of Rel-19 workshop	RAN Chairman

- TS 22.261	Service requirements for the 5G system; Stage 1 V19.2.0

- TS 23.501	System architecture for the 5G System (5GS) 18.1.0

- R3-234613	Summary of Offline Discussion on CB: # R18XR

- S2-2310035	New SID on 5GS XRM Ph2	Nokia, Nokia Shanghai Bell