3GPP TSG RAN Meeting #101						RP-232094

Bangalore, India, September 11-15, 2023

Source: 	Apple

Title: 	Views on the scope of sub-band full duplex WI

Agenda Item: 	9.2.4

Document for:	Discussion and Decision

# Background

In Rel-18, a new SI on evolution of NR duplex operation was approved at RAN#94-e [1], and later revised at RAN#96 [2], to identify the feasibility and solutions of duplex evolution to provide enhanced UL coverage, reduced uplink latency and improved uplink system capacity, for NR TDD operations in unpaired spectrum. At RAN1#14, the study item on evolution of NR duplex operation was completed from RAN1 perspective, although no recommendation was made to proceed with a normative work. The results and conclusions of this study were summarized in TR 38.858 [3] which covers the evaluation assumptions, performance results and potential enhancements for SBFD and dynamic TDD. While RAN4 is still studying the feasibility of full-duplex operation at the BS and non-coexistence scenarios, the following conclusions are captured in Section 13, TR 38.858 [3]:

SBFD operation at gNB for UEs was studied under the following assumptions,

  - SBFD operation within a TDD carrier,

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies, and

  - Up to one UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier.

RAN1 concluded SBFD operation Alt 4 is feasible for RRC_CONNECTED state from the RAN1 specification perspective, where SBFD operation Alt 4 assumes

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side from RAN1 specification point of view.

To support SBFD operation Alt 4 for RRC_CONNECTED state, RAN1 identified the following potential specification impact for SBFD-aware UE:

  - Indication of time and frequency domain locations of SBFD subbands to UEs

  - UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols

In this contribution, we provide our views on the potential work item scope for NR duplex operation at the gNB in Rel-19.

# Proposed Work Item Scope

At this stage, given that RAN4 has not completed the study yet, the proposed scope for work item can only consider the recommendations from RAN1 as summarized in section 1 above. Any Work Item Description on evolution of NR duplex operation will require an update after RAN4 completes the study.

## 2.1	SBFD slot configuration to SBFD-aware UEs in RRC_CONNECTED

A SBFD-aware UE is a UE which is capable to support enhanced signalling and procedure that may be required for SBFD operation at the gNB (e.g., enhanced CLI measurement and report, etc). Such a UE must be in RRC_CONNECTED to indicate its capability to the NW. RAN1 conclusion is clear that indication of time and frequency domain locations of SBFD slots/sub-bands is semi-static and there is no intention to dynamically adapt sub-band sizes, given that such dynamic indication may result gNB-to-gNB and/or UE-to-UE CLI. In addition, given that the objective and focus of Rel-18 study was to enhance uplink performance in a DL heavy TDD slot configuration, SBFD slot indication shall be limited to symbols indicated as DL or flexible in TDD-UL-DL-ConfigCommon,

Therefore, the following work item objective is proposed.

Proposal 1: Specify signaling and procedures to semi-statically indicate SBFD slots and sub-bands to a SBFD-aware UE in RRC_CONNECTED within a TDD carrier [RAN1/RAN4] - Requirements for transition points from non-SBFD symbols to SBFD symbols (and vice versa) within a TDD UL/DL pattern period - One UL sub-band with center frequency aligned with configured DL and UL BWP pair - No dynamic adaptation of UL/DL sub-band sizes

## 2.2	Enhancements to support SBFD operation at the gNB

For SBFD aware UEs, couple of enhancements regarding transmission and reception was studied during the SI phase, given that UE is assumed a half-duplex UE. For example, for a channel/signal with repetitions, new specification may be needed when different repetitions span both SBFD and non-SBFD slots and/or go beyond the allocated sub-band boundaries. Depending on the specific channel/signal, the proposed solution can vary from completely dropping the repetition occasion to perform rate-matching and/or partially dropping. The scope of such signalling enhancement should be limited to channels/signals that are semi-statically configured, like CG-PUSCH, periodic CSI-RS, etc. In other words, for dynamic indications without repetitions, it is not expected that the allocation in time/frequency goes beyond the indicated sub-band and/or symbol types.  In addition, PDCCH should be deprioritize from such enhancements since PDCCH can be configured by scheduler to be monitored only within DL sub-band boundaries. In lack of strong justification for PDCCH, new design for PDCCH just increases UE complexity and unnecessarily demands specification effort.

Therefore, the following work item objective is proposed.

Proposal 2: Specify signaling and procedures for transmissions and receptions that go beyond allocated sub-bands and/or different symbol types [RAN1] - De-prioritize PDCCH enhancement

Rel-16 specified UE-to-UE CLI measurement although the procedure is L3 based and not responsive to fast change of CLI interference profile. Within the study item phase in Rel-18, enhancement to CLI management, measurement and reporting for both gNB-to-gNB and UE-to-UE inter-subband co-channel CLI were discussed.  Examples of such possible signalling enhancements include L1/L2 based UE-to-UE CLI measurement/report and coordinated scheduling by exchange of semi-static SBFD configurations among gNBs.

Therefore, to enhance CLI measurement and reporting, the following work item objective is proposed.

Proposal 3: Specify the enhancements and procedures for CLI management and L1/L2 based CLI measurement and reporting [RAN1/RAN4/RAN3] - In-band measurement is prioritized over inter-subband measurements - Current CSI measurement/report procedure is the baseline. - For UE-to-UE CLI management, strive to reuse existing signaling and procedure to manage CLI (like UL power control mechanism, etc) - For gNB-to-gNB CLI management, strive to reuse signaling and procedure that was specified in R17 eIAB (like DL power adjustment, indication of desired/prohibited beams, etc)

# Summary

In this contribution, we provided our view on the RAN1 scope of evolution of NR duplex operation Work Item for Rel-19. The following Work Item Objectives are proposed.

1. Specify signaling and procedures to semi-statically indicate SBFD slots and sub-bands to a SBFD-aware UE in RRC_CONNECTED within a TDD carrier [RAN1/RAN4]

-	Requirements for transition points from non-SBFD symbols to SBFD symbols (and vice versa) within a TDD UL/DL pattern period

-	One UL sub-band with center frequency aligned with configured DL and UL BWP pair

-	No dynamic adaptation of UL/DL sub-band sizes

2. Specify signaling and procedures for transmissions and receptions that go beyond allocated sub-bands and/or different symbol types [RAN1]

-     De-prioritize PDCCH enhancement

3. Specify the enhancements and procedures for CLI management and L1/L2 based CLI measurement and reporting [RAN1/RAN4/RAN3]

- In-band measurement is prioritized over inter-subband measurements

- Current CSI measurement/report procedure is the baseline.

- For UE-to-UE CLI management, strive to reuse existing signaling and procedure to manage CLI (like UL power control mechanism, etc)

- For gNB-to-gNB CLI management, strive to reuse signaling and procedure that was specified in R17 eIAB (like DL power adjustment, indication of desired/prohibited beams, etc)

# References

- RP-213591, New SI: Study on evolution of NR duplex operation, CMCC

- RP-221352, "Revised SID: Study on evolution of NR duplex operation".

- R1-23xxxx, "3GPP TR 38.858, Study on Evolution of NR Duplex Operation (R18)".