3GPP TSG-RAN Meeting # 101	RP-232108

# Bangalore, India, September 11-14, 2023

Source: 	Apple

Title: 	On dynamic configuration of type 2 UE

Agenda Item:	9.3.4.10

Document for:	Discussion

# Introduction

During the past meetings, RAN4 discussed the signalling aspects for UE supporting intra-band non-collocated CA/EN-DC, but no consensus has been reached [1-4]. To ensure timely completion of this WI, this paper will address the open issue for R18 WI Support of intra-band non-collocated EN-DC NR-CA deployment [5].

# Discussion

### 2.1 Type 2 UE capability

RAN4 agreed to introduce Type 2 UE to support intra-band non-collocated CA combination in n77/n78. From hardware perspective, Type 2 UE can be implemented by splitting 4 RF chains of Type-1 UE into 2x2 chains and each set of 2 RF chains can be optimized in gain/timing for receiving one of the CCs involved. Based on the hardware capability assumption as such, it is understood that Type 2 UE can fall back to work as Type 1 UE when collocation condition is met. So, the following apply for Type 1 and Type 2 UE.

Type 1 UE is characterized by:

- Capable of handling 6dB power imbalance and 3us MRTD between CCs (type 1 capability).

- Support maximum 4MIMO layer per CC (type 1 capability).

Type 2 UE is characterized by:

- Capable of handling 25dB power imbalance and 33us MRTD between CCs (type 2 capability).

- Support maximum 2MIMO layer per CC in non-collocated scenario (type 2 capability).

- Support maximum 4MIMO layer per CC in collocated scenario (type 1 capability).

It means a Type 1 UE supports legacy requirements for intra-band collocated scenario and a Type 2 supports both legacy requirement for intra-band collocated scenario and intra-band non-collocated scenario 25dB power imbalance & 33us MRTD requirement. Type 2 UE can work as Type 1 UE in non-collocated scenario and work as Type 2 UE in non-collocated scenario depending on the hardware configuration.

Observation 1: Type 1 UE supports legacy requirements for intra-band collocated scenario and a Type 2 supports both legacy requirement for intra-band collocated scenario and 25dB power imbalance & 33us MRTD requirement.

Observation 2: From hardware capability perspective, Type 2 UE can work as Type 1 UE in non-collocated scenario and work as Type 2 UE in non-collocated scenario through hardware configuration.

RAN4 agreed to introduce a new UE capability for intra-band non-collocated CA in R18 [intraBandNonColocatedCA-r18] for type 2 UE. It is proposed to provide RAN2 a clear description of the capability from RAN4 perspective since this is RAN4 driven feature. Otherwise, there might be confusion on the relevant RF requirements in RAN2 and further LS back and forth between RAN2 and RAN4 for further technical clarification.

Proposal 1: It is proposed to agree the following capability definition for Type 2 UE for intra-band non-collocated CA.

Table 2.1-1: Capability for Type 2 UE

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandNonColocatedCA-r18 Indicates the UE supports TDD-TDD intra-band non-collocated CA operation with NR carrier aggregation MRTD according to Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated CA in 7.10A in 38.101-1 in additional to the RF requirements for intra-band collocated CA. If the capability is not reported, the UE only supports the RF requirements for TDD-TDD intra-band collocated CA, e.g. NR carrier aggregation MRTD according to Table 7.6.4-1 in 38.133 [5] and the RF requirements for intra-band CA RF requirements except for 7.10A in 38.101-1. | BC | No | TDD only | FR1 only |

### 2.2 Dynamic configuration for UE supporting both type 1 and 2

The motivation discussion on dynamic configuration of  UE which supports both Type 1 and 2 is that for CA_n77(2A), CA78(2A), EN-DC_42-n77/n78, there are operators who will deploy CA/DC in non-collocated manner and there are also operators who will deploy CA/DC in collocated manner. For UE supporting both type 1 and 2, it would be expected that UE Rx chain can be configured according to the network deployment condition. More specifically, UE should configure its hardware to support maximum 4MIMO layer per CC when collocated condition is met and maximum 2MIMO layer per CC when collocated condition is not met. Network indication is needed to enable such dynamic configuration.

The following 2 candidate solutions have been discussed in RAN4 for several meetings [2, 3]. Majority of the companies think that Solution#1 is clean and robust solution. However, no consensus has been reached due to objection from the proponent of Solution#2. The main concern for Soluiton#2 is that it may cause different understanding between UE and network on configuration of type 1 and type 2. E.g. when PDSCH-ServingCellConfig.maxMIMO-Layers is set to 2, the UE can either understand that it will remain as type 1 with up to 2L/CC with 4Rx per CC or reconfigure its Rx chain to type 2 with 2L/CC with 2Rx per CC. Solution#2 will also cause confusion to the interpretation of current signaling PDSCH-ServingCellConfig.maxMIMO-Layers. Further, it is the majority view that the robustness of this solution is largely impacted by RSRP/RTD measurement accuracy.

  - Solution 1: Introduce new signaling indication during RRC Reconf.

On receiving such indication when Scell is configured, the UE will assume the CCs to be aggregated are deployed in non-collocated manner and configure its RF to type 2, e.g. 2Rx/CC supporting maximum 2layer/CC

  - Solution 2: Reuse the existing RRC Reconf.

The idea of Solution#2 is to reuse existing PDSCH-ServingCellConfig.maxMIMO-Layers in RRC Reconfiguration to indicate whether the UE should be configured as Type 1 or Type 2. This solution will rely on UE measurement like RSRP, RTD and CSI feedback.

Table 2.2-1: Implicit UE type distinction with RRC reconfiguration message

| RRC IE | value | UE’s interpretation |
| --- | --- | --- |
| PDSCH-ServingCellConfig.maxMIMO-Layers | 4 | Type 1 UE (4 layers per CC) |
| PDSCH-ServingCellConfig.maxMIMO-Layers | 2 | Type 2 UE (2 layers per CC) |

Observation 3: Solution 2 (Reuse the existing RRC Reconfiguration) will cause confusion in interpretation of current signaling as well as different understanding in type 2 UE configuration between network and UE.

Proposal 2: Solution 1 (Introduce new signaling indication during RRC Reconf) is adopted to support Type 2 UE reconfiguration.

Proposal 3: If this issue cannot be concluded in RAN, further discuss in RAN4 Oct meeting and make decision based on majority view.

# 3. Conclusion

In this contribution, we shared the status for RAN4 discussion on dynamic configuration of type 2 UE for intra-band non-collocated EN-DC NR-CA deployment. The following observation and proposals are concluded.

Observation 1: Type 1 UE supports legacy requirements for intra-band collocated scenario and a Type 2 supports both legacy requirement for intra-band collocated scenario and 25dB power imbalance & 33us MRTD requirement.

Observation 2: From hardware capability perspective, Type 2 UE can work as Type 1 UE in non-collocated scenario and work as Type 2 UE in non-collocated scenario through hardware configuration.

Proposal 1: It is proposed to agree the following capability definition for type 2 UE for intra-band non-collocated CA.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandNonColocatedCA-r18 Indicates the UE supports TDD-TDD intra-band non-collocated CA operation with NR carrier aggregation MRTD according to Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated CA in 7.10A in 38.101-1 in additional to the RF requirements for intra-band collocated CA. If the capability is not reported, the UE only supports the RF requirements for TDD-TDD intra-band collocated CA, e.g. NR carrier aggregation MRTD according to Table 7.6.4-1 in 38.133 [5] and the RF requirements for intra-band CA RF requirements except for 7.10A in 38.101-1. | BC | No | TDD only | FR1 only |

Observation 3: Solution 2 (Reuse the existing RRC Reconfiguration) will cause confusion in interpretation of current signaling as well as different understanding in type 2 UE configuration between network and UE.

Proposal 2: Solution 1 (Introduce new signaling indication during RRC Reconf) is adopted to support Type 2 UE reconfiguration.

Proposal 3: If this issue cannot be concluded in RAN, further discuss in RAN4 October meeting and make decision based on majority view.

A LS draft is also provided in [6] for discussion if RAN2 should be involved.

# 4. Reference

[1] R4-2310017, Topic summary for [107][134] NonCol_intraB, KDDI, RAN4#107

[2] R4-2311300, WF on NonCol_intraB, KDDI, RAN4#107

[3] R4-2314216, Topic summary for [108][134] NonCol_intraB, KDDI, RAN4#108

[4] R4-2314906, WF on UE RF requirements for NonCol_intraB, KDDI, RAN4#108

[5] RP-231066, Support of intra-band non-collocated EN-DC NR-CA deployment, KDDI, RAN#100

[6] R4-232109, LS on signaling support for type 2 UE, Apple, RAN#101