3GPP TSG RAN Meeting #101									RP-232116

Bangalore, September 11-15, 2023

Agenda item:	8A.2.15

Source: 	Huawei, HiSilicon

Title: 	High Accuracy Timing Services in Rel-19

Document for:	Discussion/Decision

- Introduction

High accuracy timing service (HATS) is considered to be an essential technology for 3GPP to open for wide use cases including IIOT, Smart Grid, Finance, GPS/GNSS backup, and it has been studied and developed in RAN1/2/3 since Rel-16. In this contribution, we provide our considerations on HATS in Rel-19.

- Discussion

2.1	Motivation

High accuracy timing was first studied in Rel-16 in support of TSC synchronization for IIOT with the target E2E synchronicity of 1us where Uu budget is ~900ns, which is illustrated in the figure 1 below.

Figure 1. Accuracy timing for TSC synchronization

In Rel-17 and continued Rel-18, accuracy timing has been further enhanced to support more use cases beyond Rel-16 TSN, e.g. UL timing where the sync master is located in the UE/End-station side, and GPS/GNSS backup. Propagation delay compensation (PDC) was introduced to improve the timing accuracy, especially for out-door scenario, e.g. Smart Grid with 1us E2E requirement, and synchronization status awareness has been agreed to improve the timing resiliency when GPS/GNSS is not available or reliable.

From the perspective of high accuracy timing being a commercial service (i.e. HATS), we observe that more requirements for various use cases need to be addressed to help the deployments in practice for Smart Grid and IIOT. For instance, SA1 Rel-18 has adopted the stringent requirement of 5GS synchronicity budget with 250ns-1us for Smart Grid [1], as illustrated in Figure 2. It is noted that numerous power end-stations are connected to enable complicated digital operations, e.g. sampled values in IEC 61850-9-2, where the UE-side time distribution accuracy can be up to 750ns and cannot be ignored. In addition, the impact of challenging radio environment and mobility requirement of mobile robots/AGV to the timing performance should be also considered in Rel-19. Therefore, a more holistic E2E approach can be considered for HATS in Rel-19 for accuracy enforcement and timing performance enhancements.

Figure 2. Power distribution substation for Smart Grid (IEC 61850)

Table 5.6.2-1: Clock synchronization service performance requirements for 5G System

| User-specific clock synchronicity accuracy level | Number of devices in one communication group for clock synchronisation | 5GS synchronicity budget requirement (note 1) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 1 | up to 300 UEs | ≤ 900 ns | ≤ 100 m x 100 m | - Motion control (A.2.2.1) - Control-to-control communication for industrial controller (A.2.2.2) |
| 2 | up to 300 UEs | ≤ 900 ns | ≤ 1,000 m x 100 m | - Control-to-control communication for industrial controller (A.2.2.2) |
| 3 | up to 10 UEs | < 10 µs | ≤ 2,500 m2 | - High data rate video streaming |
| 3a | up to 100 UEs | < 1 µs | ≤ 10 km2 | - AVPROD synchronisation and packet timing |
| 4 | up to 100 UEs | < 1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |
| 4a | up to 100 UEs | < 250 ns to 1 µs | < 20 km² | Smart Grid: IEC 61850-9-2 Sampled Values |
| 4b | up to 100 UEs | <10-20 µs | < 20 km² | Smart Grid: IEC 61850-9-2 Sampled Values – Power system protection in digital substation |
| 4c | 54/km² (note 2) 78/km2 (note 3) | < 10 µs | several km² | Smart Grid: Intelligent Distributed Feeder Automation (A.4.4.3) |
| 4d | up to 100 UEs | <1 ms | < 20 km² | Smart Grid: IEC 61850-9-2 Sampled Values – Event reporting and Disturbance recording |
| 5 | up to 10 UEs | < 50 µs | 400 km | - Telesurgery (A.6.2) and telediagnosis (A.6.3) |
| NOTE 1: The clock synchronicity requirement refers to the clock synchronicity budget for the 5G system, as described in Clause 5.6.1. NOTE 2: When the distributed terminals are deployed along overhead line, about 54 terminals will be distributed along overhead lines in one square kilometre. The resulting power load density is 20 MW/km2. NOTE 3: When the distributed terminals are deployed in power distribution cabinets, there are about 78 terminals in one square kilometre. The resulting power load density is 20 MW/km2, |  |  |  |  |

2.2	Objectives

- 2.2.1 Timing accuracy enforcement

In earlier E2E performance analysis of RTT-based PDC method, propagation delay is well-designed in the specification, which is measured and compensated by RTT-based PDC via [T1’, T2’, T3’ and T4’], i.e. the delay spans from gNB antenna and UE antenna, as shown in Figure 3. The UE-side delay/inaccuracy is not carefully considered and it is simply assumed that there is only one TSN end-station outside 5G system and adopt 50-100ns for calculation. However, considering the commercial use cases and diverse hardware embedded at the UE, this part is one noticeable component of E2E synchronicity. For instance, as illustrated in Figure 2, the (g)PTP sync packet can pass by at maximum 15 hop end-stations, thereby causing the time distribution delay/inaccuracy to be up to 750ns (PTP instance [1-5.1.8.2] time error is +/-50ns).

Figure 3. Illustration of E2E synchronicity

Therefore, it is required for the UE to indicate the related UE-side timing distribution accuracy to gNB for a holistic E2E synchronicity budget balance and more efficient radio resource utilization for providing the timing service. The existing delay measurement and compensation mechanism can be further enhanced by taking the UE indication into account for timing accuracy enforcement.

Proposal 1: To introduce the indication from the the UE of the related UE-side timing distribution accuracy to gNB for E2E timing accuracy enforcement.

Proposal 2: To enhance the E2E delay measurement and compensation by taking the UE indication into account.

With respect to HATS, the challenging radio condition is also important factor related to the timing accuracy. For power distribution substations whose sites are normally surrounded by buildings in urban areas, it is a typical NLOS radio environment which is subject to multi-paths at the detection. Currently, only the first arrival path (FAP) for both UL and DL are used for RTT-based PDC calculation where the UE reports its RX-TX time difference. In LOS environment, FAP can be simply assumed to be aligned between UL and DL because of channel reciprocity. However, in NLOS environment, it is prone to cause misalignment of FAP between DL and UL thereby causing synchronicity errors. According to our observations on the dataset of practical NLOS radio channels in Figure 5, the misalignment error can be up to 80ns for the unaligned UL and DL path. Thus, the existing UE RX-TX time difference report used in RTT-based PDC should be extended by considering the multi-path information report to align the UL and DL paths used for RTT-based PDC in order to enforce the timing accuracy.

Figure 4. Deployment of power substation in NLOS radio environment

Figure 5. Evaluation results of misalignment of UL and DL FAP

Proposal 3: To extend the UE RX-TX time difference by considering multi-path to align the DL and UL paths for RTT-based PDC calculation.

- 2.2.2 Mobility support

Mobility enhances flexibility in the manufacturing process as specified in TS 22.104, which is important to 5GS integration with TSN. However, the sync master change due to UE’s mobility could cause considerable interruption time and accuracy loss for the timing service performance in 5GS. The sync master change can be possible in the scenario of multiple working clock domains where each working clock domain has a sync master. For instance, as shown in Figure 6, when robot (UE#4) moves to a production line to participate in a task, it needs to sync with domain#2 while originally it was associated with domain #1. Accordingly, the gNB re-issues the timing for UE#2 based on domain#2.

Figure 6. Merging of multiple working clock domains

Therefore, the impact of mobility to the timing performance due to sync master change should be considered in Rel-19 and one possible direction is 5GS awareness of working clock domain change to timely react to the sync master change during the mobility.

Proposal 4: To introduce the 5GS awareness of working clock domain change to optimize the timing service performance due to UE’s mobility.

2.3	Potential scope of WI

Considering the various requirements from practical use cases, UE-side timing distribution accuracy, challenging radio condition of NLOS environment and mobility are identified as important factors impacting the timing performance. Thus, a more holistic E2E approach should be considered for HATS in Rel-19 for accuracy enforcement and timing performance enhancements. The potential scope of a WI can be as follows:

- Timing accuracy enforcement - To further enhance E2E delay measurement and compensation for timing accuracy enforcement [RAN2, RAN1, RAN4] - The UE indicates the related UE-side timing distribution accuracy to gNB for E2E timing accuracy enforcement. - E2E delay measurement and compensation is enhanced by taking the UE indication into account. - NOTE1: The UE-side timing distribution (in)accuracy can be up to 15hops/750ns as specified in TS 22.104. - To mitigate the prorogation delay errors in NLOS for timing accuracy enforcement [RAN2, RAN1] - The UE RX-TX time difference is extended to consider multi-path to align the DL and UL paths for RTT-based PDC calculation. - Mobility support across working clock domains - 5GS awareness of working clock domain change to optimize the timing service performance due to UE’s mobility [RAN3, RAN2] - NOTE2: SA2 coordination is needed.

Proposal 5: High accuracy timing service (HATS) should be considered as a RAN2-led WI in Rel-19 by taking proposal 1-4 into account.

- Conclusion

In this paper, we provide our views on HATS in Rel-19 with the following proposals:

Proposal 1: To introduce the indication from the the UE of the related UE-side timing distribution accuracy to gNB for E2E timing accuracy enforcement.

Proposal 2: To enhance the E2E delay measurement and compensation by taking the UE indication into account.

Proposal 3: To extend the UE RX-TX time difference by considering multi-path to align the DL and UL paths for RTT-based PDC calculation.

Proposal 4: To introduce the 5GS awareness of working clock domain change to optimize the timing service performance due to UE’s mobility.

Proposal 5: High accuracy timing service (HATS) should be considered as a RAN2-led WI in Rel-19 by taking proposal 1-4 into account.

- References

3GPP TS 22.104