3GPP TSG RAN Meeting #101	RP-232122

Bangalore, India, September 11th -15th, 2023

Agenda Item:	8A.2.9

Source:	Huawei

Title:	AI/ML for NG-RAN in Rel-19

Document for:	Discussion, Decision

# 1	Introduction

In the last Rel-19 workshop [1], most of companies addressed their outlook on AI/ML for NG-RAN in Rel-19, including the remaining open issues from Rel-18 and possible new use cases. In this paper, we further discuss general considerations on AI/ML for NG-RAN in Rel-19, and provide corresponding proposals.

# 2	Discussion

## 2.1	General Description

As the 1st release of 5G-Advanced in 3GPP, Rel-18 was agreed to establish a new WI: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, with the aim to address Network Energy Saving, Load Balancing, Mobility Optimization use cases. As part of this WI, the following principles have been established:

- AI/ML algorithms and models are out of 3GPP scope;

- details of AI/ML model's performance feedback are out of 3GPP scope;

- base on existing RAN architecture, data security and privacy should be respected;

- signalling procedures between network nodes (mainly on Xn and F1 so far) used for the exchange of AI/ML related information (e.g., input, output, feedback, …) are use case and data type agnostic.

These principles above achieved in Rel-18 provided a good starting point for discussion of AI/ML use cases, hence we think they should continue to be applied in Rel-19.

AI/ML principles achieved in Rel-18 shall still apply to Rel-19.

According to the latest status of Rel-18 WI, good progress has been achieved, but some topics will not be pursued in Rel-18, e.g. continuous MDT reporting from a certain UE for AI/ML models’ (re-)training in the OAM, AI/ML impacts over the NG interface, etc. In our understanding, some of these topics not introduced in Rel-18 should be within the scope of NG-RAN AI/ML in Rel-19.

Observation 1: There are some topics that have not been pursued in the Rel-18 WI which should be potential Rel-19 topics for NG-RAN AI/ML.

In the last Rel-19 workshop, according to the summary from RAN chair in RWS-230488 [1], it is suggested that NG-RAN AI/ML should be one candidate of RAN3-lead topics in Rel-19. Most of companies also proposed possible new use cases in Rel-19 to enhance UE service experience, such as AI/ML-based QoE (Quality of Experience), DC (Dual Connectivity), Slicing, CCO (Coverage and Capacity Optimization), etc. Huawei also proposed three new AI/ML-enabled use cases, including RACH Optimization, DC mobility optimization and CCO, as candidates [2]. But we would like also that Rel-19 NG-RAN AI/ML should at least consider Rel-18 leftovers, and potential new use cases:

Rel-19 NG-RAN AI/ML should consider two parts: Rel-18 leftovers as well as potential new use cases which benefit from using inference-based techniques.

## 2.2	New Use Cases

### 2.2.1	RACH Optimization

It is a common knowledge that the improper RACH configuration may cause many problems (e.g. access conflict), which may further lead to RACH access delay, RACH failures or false alarms in neighbour cells, etc. In order to identify potential RACH resource allocation problems, 3GPP introduced the RACH optimization mechanism (Figure 1).

Figure 1. RACH optimization

The existing RACH optimization mechanism is supported by UE reported information being available at the NG-RAN node and by PRACH parameters’ exchange between NG-RAN nodes, to mitigate the RACH configuration conflicts between neighbour nodes. Obviously, this mechanism mainly focusses on solving RACH problems through history information, e.g. updating the RACH configuration according to historic RACH report to reduce the RACH delay. In other words, the network currently is not able to foresee potential RACH problems and take proper measures in advance.

Figure 2. Inference-based RACH optimization

Therefore, it is desirable to improve the performance of the existing RACH optimization mechanism. The main principle, shown in Figure 2, is that RACH reports could be enhanced to include more information (e.g., success and failure with corresponding RACH parameters), such enhanced information is used by the NG-RAN node (say, gNB1) for RACH problem prediction/RACH configuration recommendation, by also considering the current neighbours’ RACH configurations. The proposed inference-based RACH optimization will have some new standard impacts, e.g. additional information exchange over network interfaces, such as the optimized RACH configuration based on prediction.

It is proposed RAN3 consider inference-based RACH Optimization as one of potential use cases in Rel-19.

### 2.2.2	DC mobility optimization

The current discussion of inference-based UE mobility enhancement in Rel-18 focuses on optimizing UE handovers across cells within the same single next-hop target NG-RAN node. For example, the source NG-RAN node predicts the cell-based UE trajectory which the UE is expected to follow (i.e., the list of target cells where it will likely be handed over) so as to optimize handover resources in advance as well as reduce UE handover delay and failure probability.

Rel-18 mainly considers the scenario where a UE is connected to a single NG-RAN node, and does not consider the DC scenario. In case of DC (Figure 3), the current procedures for SN, e.g. SN Change, are mainly based on MN-configured UE measurement results. In addition, both MN and SN can trigger procedures affecting the SN.

Figure 3. DC deployment scenario

However, there are some DC-specific issues, e.g., how to improve the overall performance and service experience of the UE via optimization of air interface resources from both the MN and the SN simultaneously. Another example is the concurrent triggering of SN procedures from both the MN and SN: in case the MN decides that a SN Change procedure is not required, but the same SN Change procedure is initiated by the SN, should the SN Change be performed? In other words, if MN and SN simultaneously trigger different procedures towards the SN, it is necessary to evaluate which one should be performed.

Figure 4. Inference-based DC

Therefore, a mechanism is needed to address these issues above, and the introduction of an inference-based DC framework could be an option (Figure 4). The main principle is to enable interaction between MN and SN by exchanging respective inference-based predictions and subsequent recommended mobility actions. In a specific example, by taking predicted UE trajectories inferenced by both the MN and SN into consideration, such inference results and corresponding recommended mobility decisions, e.g. SN Change/Addition/Release, can be exchanged and used to determine which action to perform. It can be seen that the proposed inference-based DC framework has some standard impacts due to the additional information exchange over network interfaces, e.g. recommended SN Change/Addition/Release. In addition, we think RAN3 should consider NR-DC scenario as a starting point.

It is proposed RAN3 consider inference-based NR-DC as one of potential use cases in Rel-19.

### 2.2.3	Coverage and Capacity Optimization

In the current specification, the NG-RAN node can perform CCO (Coverage and Capacity Optimization) self-adjusting and it notifies neighboring NG-RAN nodes of the adjustment result, as shown in Figure 5. In this way, the neighboring NG-RAN nodes (say, gNB2) can also perform corresponding adjustment and, other than reporting whether they have accepted the adjustment from the source node (gNB1), they can provide their own corresponding adjustment results to the source NG-RAN node, thereby improving the overall cell coverage capability.

Figure 5. Illustration of CCO mechanism

As could be seen from the figure above, each NG-RAN node can be configured with alternative coverage configurations by OAM, and the CCO configuration is usually adjusted when there is a coverage problem detected by a certain NG-RAN node or a notification of CCO configuration adjustment from a neighbor NG-RAN node is received. That is, with this existing mechanism, the coverage and capacity problem cannot be foreseen and prevented in advance.

Figure 6. Inference-based CCO

Therefore, the performance of the existing CCO mechanism could be further improved via AI/ML. As shown in Figure 6, AI/ML could be used to predict cell capacity and coverage issues in the NG-RAN node and to derive corresponding recommended adjustment; such predicted CCO information are then exchanged among NG-RAN nodes along with predicted timing information at which the recommended capacity and coverage adjustments should be performed. The proposed inference-based CCO optimization framework is expected to have standard impacts due to the need of additional information exchange over network interfaces, e.g. the predicted CCO configuration, as well as the reason, timing information and advantages to promptly adopt the new recommended CCO configuration.

It is proposed RAN3 consider inference-based CCO as one of potential use cases in Rel-19.

- We noticed also from the contributions submitted during last meetings, it is possible that many potential new use cases would be proposed, hence, in such case it is reasonable to start a study phase to evaluate and identify proper new use cases before normative work.

A study phase is needed to evaluate and identify proper new use cases before normative work.

# 3	Conclusion

Based on the discussion in this paper, we make the following observations and proposals.

Observation 1: There are some topics that have not been pursued in the Rel-18 WI which should be potential Rel-19 topics for NG-RAN AI/ML.

- AI/ML principles achieved in Rel-18 shall still apply to Rel-19.

- Rel-19 NG-RAN AI/ML should consider two parts: Rel-18 leftovers as well as potential new use cases which benefit from using inference-based techniques.

- It is proposed RAN3 consider inference-based RACH Optimization as one of potential use cases in Rel-19.

- It is proposed RAN3 consider inference-based NR-DC as one of potential use cases in Rel-19.

- It is proposed RAN3 consider inference-based CCO as one of potential use cases in Rel-19.

- A study phase is needed to evaluate and identify proper new use cases before normative work.

# 4. Reference

- RWS-230488, RAN Chair’s Summary of Rel-19 Workshop.

- RWS-230188, AI/ML for NG-RAN in Rel-19, Huawei, HiSilicon