3GPP TSG RAN Meeting #101	RP-232412

Bengaluru, India, September 11-15, 2023

Source:	Huawei, HiSilicon

Title:	pCR for TR 38.848 on Feasibility assessment and required functionalities for Ambient IoT

Agenda item:	9.2.2

Document for:	Agreement

- Introduction

This pCR proposes text to add feasibility assessments and required functionalities of Ambient IoT into TR 38.848 v0.2.0.

Rapporteur note: The comment boxes identifying the references would be removed by the TR editor.

- Proposed changes

# <Unchanged text is omitted>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.840: "Study on Ambient power-enabled Internet of Things".

[X1]	"A Battery-Free Tag for Wireless Monitoring of Heart Sounds", 2009 Sixth International Workshop on Wearable and Implantable Body Sensor Networks, June 2009

[X2]	"A Survey of Low-Power Transceivers and Their Applications", IEEE Circuits and Systems Magazine (Vol. 15), third quarter 2015.

[X3]	http://www.winlab.rutgers.edu/~crose/322_html/envelope_detector.html.

[X4]	Liu, V., Talla, V., & Gollakota, S. (2014). Enabling instantaneous feedback with full-duplex backscatter. Proceedings of the 20th Annual International Conference on Mobile Computing and Networking MobiCom ’14.

[X5]	D. A. Loku Galappaththige, et. al., "Link Budget Analysis for Backscatter-Based Passive IoT", IEEE Access, vol. 10, pp. 128890-128922, 2022.

[X6]	"A 2.4 GHz Interferer-Resilient Wake-Up Receiver Using A Dual-IF Multi-Stage N-Path Architecture", IEEE Journal of Solid-State Circuits (Vol. 51), Sept. 2016

[X7]	"An 802.11ba-based wake-up radio receiver with Wi-Fi transceiver integration", IEEE J. Solid-State Circuits (Vol. 55), May 2020.

[X8]	Kimionis, J., Georgiadis, A., Collado, A., & Tentzeris, M. M. (2014). Enhancement of RF Tag Backscatter Efficiency With Low-Power Reflection Amplifiers. IEEE Transactions on Microwave Theory and Techniques, 62(12), 3562–3571.

[X9]	J. Bae and H. Yoo, "A low energy injection-locked FSK transceiver with frequency-to-amplitude conversion for body sensor applications," 2010 Symposium on VLSI Circuits, 2010, pp. 133-134, doi: 10.1109/VLSIC.2010.5560325.

[X10]	K. Tang et al., "A 75.3 pJ/b Ultra-Low Power MEMS-Based FSK Transmitter in ISM-915 MHz Band for Pico-IoT Applications," 2021 IEEE International Symposium on Circuits and Systems (ISCAS), 2021, pp. 1-4, doi: 10.1109/ISCAS51556.2021.9401715

[X11]	M. S. Jahan, J. Langford and J. Holleman, "A low-power FSK/OOK transmitter for 915 MHz ISM band," 2015 IEEE Radio Frequency Integrated Circuits Symposium (RFIC), 2015, pp. 163-166, doi: 10.1109/RFIC.2015.7337730.

[X12]	RP-222644, "Revised SID: Study on low-power Wake-up Signal and Receiver for NR", vivo, September 2022.

[X13]	F. Amato et. al., “Tunneling RFID Tags for Long-Range and Low-Power Microwave Applications”, IEEE Journal of Radio Frequency Identification, vol. 2, no. 2, pp. 93-103, June 2018.

[X14]	RP-231559 “Views on Ambient IoT (Rel-18 SI)”, Qualcomm, RAN#101, September 2023.

[X15]	RP-231598 “Discussion on Ambient IoT”, Semtech (Sierra Wireless) , RAN#101, September 2023.

[X16]	RP-231617 “Ambient IoT deployment feasibilities”, Nokia, Nokia Shanghai Bell, RAN#101, September 2023.

[X17]	RP-231627 “Input to Study on Ambient IoT in RAN”, Ericsson, RAN#101, September 2023.

[X18]	RP-231810 “Remaining issues on Ambient IoT SI”, vivo, RAN#101, September 2023.

[X19]	RP-231848 “Further consideration on ambient IoT for RAN”, OPPO, RAN#101, September 2023.

[X20]	RP-231915 “Further discussion on Ambient IoT in RAN”, Spreadtrum Communications, RAN#101, September 2023.

[X21]	RP-232284 “Feasibility analysis on coverage of A-IoT ZTE”, Sanechips, RAN#101, September 2023.

[X22]	RP-232408 “Discussion on Feasibility assessment and required functionalities for Ambient IoT”, Huawei, HiSilicon, RAN#101, September 2023.

[X23]	C. Xu et. al, "Practical Backscatter Communication Systems for Battery-Free Internet of Things: A Tutorial and Survey of Recent Research," IEEE Signal Processing Magazine, vol. 35, no. 5, pp. 16-27, Sept. 2018.

[X24]	Chenyang Li , Lingfei Mo, and Dongkai Zhang, “Review on UHF RFID Localization Methods”, IEEE Journal of Radio Frequency Identification, VOL. 3, NO. 4, DECEMBER 2019.

[X25]	Deep Convolutional Neural Network for Passive RFID Tag Localization Via Joint RSSI and PDOA Fingerprint Features, IEEE Access (Vol. 9), Jan. 2021.

[X26]	A Multi-tag Cooperative Localization Algorithm Based on Weighted Multidimensional Scaling for Passive UHF RFID, IEEE Internet of Things Journal (Vol. 6), Mar. 2019.

[X27]	Lam K H, Cheung C C, Lee W C, “RSSI-based LoRa localization systems for large-scale indoor and outdoor environments”, IEEE Transactions on Vehicular Technology, VOL. 68, No. 12, December 2019.

[X28]	Sallouha H, Chiumento A, Pollin S, “Localization in long-range ultra-narrow band IoT networks using RSSI”, 2017 IEEE International Conference on Communications (ICC), IEEE, May 2017

[X29]	Xingqin Lin, Johan Bergman, Fredrik Gunnarsson, etc., “Positioning for the Internet of Things: A 3GPP Perspective”, IEEE Communications Magazine, VOL. 55, NO. 12, DECEMBER 2017.

# 6	 Comparison and assessment

## 6.1	Feasibility assessment

### 6.1.1	Device power consumption

Feasibility of power consumption for Device A at ≤1 μW level has been reported by reference to [X1], [X2], [X3], [X4], and by reference to [X5] at ≤10 μW level. Feasibility of power consumption for Device C at ≤1 mW level has been reported by reference to [X6], [X7], [X8], [X9], [X10], [X11], and by reference to the Rel-18 SI on low-power wake up signal/receiver at ≤10 mW level [X12]. For Device B, which adds energy storage to Device A and has been described as also potentially including (without limitation), e.g. reflection amplification, feasibility of power consumption in the order of hundreds of uW has been reported by reference to [X13].

### 6.1.2	Device complexity

Feasibility assessment for this aspect was presented on a qualitative basis by companies describing exemplary waveform, and/or transmitter, and/or receiver architectures which, according to their analysis, would satisfy the device power consumption design target. It was also observed that the amount of energy storage could affect device complexity. Examples of considered waveforms include OOK/FSK, and ASK. Examples of considered transmitter architectures for Device A/B include those based on backscattering technology, and receiver architectures based on envelope detection, while for Device C, very low power consumption heterodyne/homodyne architectures were reported as satisfying the complexity design target.

On the other hand, aspects such as the memory required for security, authentication, etc., and hardware used for encryption processing would add to the complexity of the device. There were also discussions of the quality required in circuitry such as energy harvesting, backscattering, and PAs which, as their efficiency increases, tend to have higher complexity.

Detailed analysis of designs which meet the device complexity requirement is considered to require WG-level technical expertise.

### 6.1.3	Coverage

Feasibility of a coverage target was assessed by sources differently for Devices A, B, C. In addition to typical elements comprising a 3GPP coverage evaluation, such as BS/intermediate/assistant node transmit power and receive sensitivity, device receiver sensitivity, propagation losses, etc. (which vary according to the specific calculation method), aspects particular to Ambient IoT devices were added. These include:

-	Device A/B backscattering activation power threshold

-	Device B amplification (if any)

-	Device A/B reflection loss

-	Distance from carrier wave source to Ambient IoT device, for Device A/B

-	Power of carrier wave source and/or incident power at Device A/B from carrier wave source

Although different evaluation methodologies were adopted by sources [X14] – [X22], the coverage of Device A is reported as less than Device B (with or without amplification), which is less than Device C. The coverage of Ambient IoT devices relying on backscattering technology was reported to be larger when the carrier wave source is closer to the Ambient IoT device, due to the higher incident power on the device.

### 6.1.4	User experienced data rate

Full user-experienced data rate assessment was noted as requiring WG-level technical expertise, and detailed assumptions on air interface design. For RAN-level purposes, companies used simplified approximants. Peak data rate was one such approximant, where companies reported values above 5 kbps to be achievable according to various sets of assumptions including sufficient bandwidth, and in one case noted that the components assumed by reference to [X23] would need further investigation whether the device power consumption target could be simultaneously met. One source [X14] estimated data rate by accounting for device charging time and operation time, resulting in data rates 0.14 – 2.24 kbps.

### 6.1.5	Maximum message size

This design target does not require detailed feasibility assessment in RAN, since it primarily imposes requirements on technical design of the WGs. Where companies studied feasibility of this design requirement, it was by reference to e.g. RFID which supports message sizes larger than [X: the design target]. It was also observed that the different Devices could be regarded as feasible for different maximum message sizes, if they are envisioned for different use cases.

### 6.1.6	Latency

Feasibility of latency was reported typically by comparing a message size to a data rate, for example 5 kbps / 1000 bit = 200 ms latency for the largest message size at the target peak rate. Feasibility would also depend on a consideration of signalling procedures, possible random access-like procedure [and, if included in the definition of latency, time to store enough energy in Device B or C to transmit the message].

### 6.1.7	Positioning accuracy

Feasibility assessment for this aspect has been reported by reference to technologies of a similar complexity level, such as UHF RFID achieving 2-3 m accuracy in [X24], [X25], [X26] indoors, and ultra-narrow IoT achieving from several tens to 100 or 150 m in [X27], [X28], [X29] outdoors. It is also observed that Device A and B need to have a carrier wave source in an appropriate distance to be able to transmit signals for positioning.

### 6.1.8	Connection/device density

Feasibility of the connection/device density targets was discussed from the basis of assuming cell access procedures, and device addressing. It was discussed as applying a requirement on the design of such procedures, identities, etc. rather than being a matter for feasibility assessment before starting such designs.

### 6.1.9	Moving speed of device

This design target does not require detailed feasibility assessment in RAN, since it primarily imposes requirements on technical design of the WGs. The feasibility aspects that were reported related to the impact of Doppler shift or channel coherence on the possible types of modulation contemplated by companies for especially Device A and B. It was observed, without limiting RAN WG design scope, that at least for low-order non-OFDM modulation, Doppler impacts at the low moving-speed level implied in Clause 5.9 would likely be acceptable, although the impact on transmission times potentially a multiple of the coherence time should also be studied in RAN WGs.

## 6.2 Required RAN functionality

The assumptions on required functionality have been studied on the basis of supporting certain RAN design targets as well as other requirements. At least the following potential functionalities are identified in different sets, respectively, according to the purpose of each functionality assumed to be mainly used for. An entry in the tables below neither implies nor precludes RAN specification impact.

Required RAN functionality set #1: for supporting RAN design target

| Design target | Functionality |
| --- | --- |
| Device power and complexity | - Ultra-low power transceiver / Device architecture - Transmitting based on backscattering (including carrier waving provision for backscattering) for Device A and Device B - Receiving based on envelope detection - Low-complexity waveform / modulation (e.g. non-OFDM) / coding / signal / channel / synchronization scheme, if applicable to Device, robust to frequency error and timing error - Compact protocol stack and signaling procedure |
| Coverage | - Coverage enhancement techniques with low device complexity (e.g., forward error correction, enough receiver sensitivity and transmitted power, reflection gain enhancement), if applicable to Device |
| User experienced data rate | - Compact protocol stack and signaling procedure - Potential schemes such as flexible MCS, resource allocation, multiple access methods |
| Maximum message size | - Compact protocol stack and signaling procedure - Signal/channel design which can deliver the maximum message size |
| Latency | - Access mechanisms and signaling procedures which allow meeting the latency target |
| Positioning accuracy | - Positioning method(s) applicable to the connectivity topologies, for ultra-low power consumption and complexity device, including potential new positioning method |
| Connection/Device density | - Efficient anti-collision schemes e.g. contention-based or contention-free random access scheme, lightweight collision detection or avoidance mechanism in the uplink transmission, and mechanisms to flexibly control the collision rate. |
| Moving speed of device | - Physical layer design (low-order modulation e.g. ASK or BPSK etc., reference signal etc. and others) robust to the appropriate ranges of Doppler shift |

Required RAN functionality set #2: for supporting other requirements

| Requirement | Functionality |
| --- | --- |
| Device management | - Identification, activation/deactivation, and management of Ambient IoT devices - Ability to communicate with all, a subset, or one of the Ambient IoT devices |
| Security* | - Authentication, encryption, data integrity, authorization) |
| Mobility | - Mobility management (at least nomadic) for active device at least in IDLE mode |
| Interference management and coexistence | - Interference management/coordination scheme - Potential full duplex capability, including self-interference suppressing, may be required for gNB/UE to communicate with Device A and Device B, if carrier wave transmission and backscatter reception is performed simultaneously by the same gNB/UE. - Coexistence with existing network infrastructure, and possibility to reuse existing network deployments |
| CN connectivity | - RAN functionality for Ambient IoT to support CN, with possibility of potential lightweight protocol stack architecture and simplified signaling process |
| Forward compatibility | - Operation of the Ambient IoT device is agnostic to RAN connectivity topology - Unified design for Device A and Device B, strive for common design between Device A/B and Device C where appropriate |

*Note: This does not necessarily mean security has RAN impact, further study is needed.

The required functionalities may not all be addressed in the same Release.

# 7	Conclusions and recommendations

# <Unchanged text is omitted>