3GPP TSG-RAN WG1 Meeting #114	R1-2308525

# Toulouse, France, August 21st – August 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0487 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on PDCCH Monitoring Adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, OPPO, Moderator (MediaTek) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the case of active DL BWP change by RRC configuration, UE PDCCH monitoring behavior regarding Rel-17 SSSG switching and/or skipping of PDCCH monitoring is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE PDCCH monitoring behavior for active BWP change by the expiration of bwp-InactivityTimer as specified in clause 10.4 of TS 38.213 is reused, i.e., - UE resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell when UE is in a PDCCH skipping duration, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP - UE monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell, if the UE is provided searchSpaceGroupIdList-r17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB and UE may miss data scheduled due to inconsistent understanding on UE PDCCH monitoring behavior. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.4	Search space set group switching and skipping of PDCCH monitoring

<Unchanged part omitted>

If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer or by RRC configuration, the UE

- resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell when UE is in a PDCCH skipping duration, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP

- monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell, if the UE is provided searchSpaceGroupIdList-r17.

<Unchanged part omitted>