3GPP TSG RAN Meeting #101

# Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.1

Source:	CEWiT, IIT-M, IIT-H, IIT-K

Title:	AI/ML Air Interface

Document for:		Discussion and decision

# Introduction

In RAN#94e, the Rel-18 SID for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface was approved [1], the objective of this study item is to study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

# Agreements of AI/ML for air interface

In RAN1#114 meeting, evaluation performance of CSI compression using two-sided AI/ML models under various scenarios were agreed and their performance gains over the benchmark were noted. Then, the methods of performing model monitoring for two-sided models were discussed. In potential specification impact discussion, the model pairing information were proposed by companies for observation.

In CSI prediction, time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. In agreements, the evaluation performance of single sided AI/ML model for CSI prediction under different scenarios were agreed. Then, the potential specification impact on data collection and model monitoring for functionality based  were discussed. Furthermore, different benchmark schemes (i.e., the nearest historical CSI without prediction, non-AI/ML based CSI prediction and collaboration level x AI/ML based CSI prediction approach) were reached to evaluate the performance of time domain CSI prediction using UE sided model.

Based on the observations from Meeting RAN1 #112bis-e for AI/ML based positioning enhancements, on the evaluation of Label-free model monitoring methods companies (vivo (R1-2302481), CATT (R1-2302699), MediaTek (R1-2303340), Ericsson (R1-2302335), Nokia (R1-2302632)) showed  the feasibility of label-free model monitoring methods. Hence, there is a need for further investigation on Label free model monitoring methods as well as label free positioning  methods.

In this contribution, we provide our views on AI/ML for CSI enhancements and Positioning.

# Evaluation results of AI/ML for air interface

According to the evaluation results (Till RAN1 #114) above, it is observed that CSI compression using two-sided model achieves a good performance over benchmark scheme. The performance gain can be achieved when precoding vectors can be considered as model input.

According to the evaluation results above (Till RAN1 #114), it is observed that time domain based CSI prediction using UE sided model can achieve performance gains over different benchmark schemes. The performance gain is when raw channel matrix is used as model input. Concerning on the time limitation in RAN1, the discussion on time domain CSI prediction should reuse the high-level principle of other one-sided model sub-cases and Rel-18 MIMO enhancement for CSI prediction.

Observation 1: AI/ML based CSI compression achieves performance gain over the benchmark Rel-16 E.Type -II codebook.

In the study item, evaluations of AI/ML based CSI compression achieved performance gain in data traffic over the benchmark of Release 16 Enhanced Type-II codebook.

Observation 2: AI/ML based CSI compression can achieve decent performance over different antenna configuration, carrier frequency, deployment scenario etc.

Observation 3: Time domain based CSI prediction using UE-sided model can achieve performance gains over different benchmark schemes .

In RAN1 #114 meeting, evaluation results of AI/ML based CSI prediction achieved moderate performance gains over  traditional methods(Kalman filter, nearest historic CSI, autoregressive based prediction methods).

Proposal 1:For CSI enhancements using AI/ML, CSI prediction can be proceeded with WI. Work item has to include Data Collection aspects, Fallback mechanism model monitoring and reporting.

In RAN1 #114 meeting, the potential specification impacts of data collection for CSI prediction were discussed. The AI/ML based CSI prediction using a single sided model at UE, has less complexity. The methodology for data-collection, monitoring and signaling aspects can be studied.

Proposal 2: For CSI enhancements using AI/ML, CSI compression using two sided model can proceed with WI.

Proposal 3: To study new potential use-cases in RAN1 like joint CSI compression and prediction using AI/ML

To consider the outcome of Rel18 MIMO study for CSI enhancements for medium/high velocity conditions and  consider them to be the benchmark for the joint compression and prediction use-case.

Observation 4: Study on direct AI/ML and AI/ML assisted positioning is done on various deployment scenario and generalization aspects in different deployment scenario along with timing synchronization and SNR mismatch in Release 18.

Proposal 4: Based on the evaluation result  of AI/ML based positioning of Release 18 study, we should proceed for the Work Item phase.

Proposal 5:  Label free positioning methods and model monitoring methods are to be studied further.

# Conclusion

In this contribution, we provide our views on AI/ML for CSI enhancements and positioning. We have the following observation and proposal:

Observation 1: AI/ML based CSI compression achieves performance gain over the benchmark Rel-16  E.Type-II codebook.

Observation 2: AI/ML based CSI compression can achieve decent performance over different antenna configuration, carrier frequency, deployment scenario etc.

Observation 3: Time domain CSI prediction using UE sided model can achieve performance gains over different benchmark schemes (i.e., the nearest historical CSI without prediction, non-AI/ML based CSI prediction and collaboration level x AI/ML based CSI prediction approach).

Proposal 1: For CSI enhancements using AI/ML, CSI prediction can be proceeded with WI. Work item has to include Data Collection aspects, Fallback mechanism model monitoring and reporting.

Proposal 2: CSI compression using two sided model can proceed with WI.

Proposal 3: To study new potential use-cases in RAN1 like joint CSI compression and prediction using AI/ML.

Observation 4: Study on direct AI/ML and AI/ML assisted positioning is done on various deployment scenario and generalization aspects in different deployment scenario along with timing synchronization and SNR mismatch in Release 18.

Proposal 4: Based on the evaluation result  of AI/ML based positioning of Release 18 study, we should proceed for the Work Item phase.

Proposal 5:  Label free positioning methods and model monitoring methods are to be studied further.

# 4. Reference

[1] Chair's notes RAN1#112

[2] Chair's notes RAN1#112bis-e

[3] Chair's notes RAN1#113

[4] Chair's notes RAN1#114