3GPP TSG-RAN5 Meeting #100	R5-234713

# Toulouse, France, 21st – 25th Aug. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2382 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of A-MPR requirements for UEs supporting higher power limit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Power_Limit_CA_DC-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 38.101-1, if UEs support higherPowerLimit-r17 for PC2 inter-band CA configurations, PPowerClass,CA is replaced by the sum of PPowerClass for each cell (i.e. 27.8dBm). Thus, the A-MPR test requirements will be different from that of UEs not supporting higher power limit. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the A-MPR test requirements for CA_n3A-n78A for UEs supporting higher power limit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UEs supporting higher power limit can't be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document should be implemented after CR 2379 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A.3.1	UE additional maximum output power reduction for CA (2UL CA)

Editor’s note: This clause is complete for AMPR testing. But the following aspects are either missing for not yet determined:

FFS is left in Test applicability since there are no requirements for 6.5A.2.3 Additional Spectrum Emission mask for CA and 6.5A.3.3 Additional Spurious Emissions for CA in In Release-15.

6.2A.3.1.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the CA maximum output power as specified in Table 6.2A.1.0.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2. Outer and inner allocation notation used in clause 6.2A.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2A.3.1.2	Test applicability

The requirements of this test apply in test case 6.5A.2.3 Additional Spectrum Emission mask for CA for network signalling values FFS to all types of NR UE release 15 and forward.

The requirements of this test apply in test case 6.5A.3.3 Additional Spurious Emissions for CA for network signalling values FFS to all types of NR UE release 15 and forward.

6.2A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in 6.2A.3.0.

6.2A.3.1.4	Test description

6.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2A.3.0.3-1, the power limits specified in subclause 6.2A.4.1.3 apply.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 6.2A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2A.3.1.4.1-1: Test Configuration Table for inter-band CA (network signalling value NS_47/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] clause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range for PCC and SCC PCC specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] clause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest,Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL | UL configuration |  |  |  |  |  |  |
|  | configuration | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 | N/A for A-MPR | Default | 30 | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | testing | Default | 30 | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | Default | 30 | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.4.1-2: Test Configuration Table for inter-band CA (network signalling value NS_100/NS_01)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest NRB_agg for PCC and SCC Highest NRB_agg for PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest,Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Inner Full |
| 2 |  | CP-OFDM 256QAM | CP-OFDM 256QAM |  | Outer Full | Outer Full |
| 3 |  | DFT-s-OFDM QPSK | CP-OFDM 256QAM |  | Inner Full | Outer Full |

<Unchanged Skipped>

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance (Additional Maximum Power Reduction (A-MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78 and CA_n8-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23 (22.72) | 3 (52) | 25+TT | 20-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1c: UE Power Class 2 with supporting higherPowerLimit-r17 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 27.8 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8TT (23.3-TT2) |
| 2 | 27.8 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 29.8+TT | 16.3-TT |
| 3 | 27.8 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 | 29.8+TT | 21.6-TT (20.6+-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Changes>