3GPP TSG-RAN WG4 Meeting #111	R4-2408878

# Fukuoka, Japan, May 20th –24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4539 | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Perf) CR on modification on the filed name with LEO in NTN related features in Rel-17 perf requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#125bis, an LS(R2-2403858) on usage of LEO or NGSO was sent to inform RAN4 that two RAN4 related NTN features with “LEO” in the field name are also applicable to NGSO (not just LEO). Based on this guidance, the decriptions in tne performance requirements related to these two features need to be correspondingly changed in RAN4 spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the field name as below <1> Rename enhancedMeasurementLEO to enhancedMeasurementNGSO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN4 related NTN features with ‘LEO’(enhancedMeasurementLEO) are not applicable to other satellite types belonging to NGSO |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

A.14.1	RRC_IDLE state mobility

A.14.1.1	Cell reselection to FR1 intra-frequency NR case

A.14.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for satellite access specified in clause 4.2C.2.3.

A.14.1.1.2	Test Parameters

The test scenario comprises of 2 cells on 1 NR carrier configured each in a different satellite as given in tables A.14.1.1.2-1, A.14.1.1.2-2 and A.14.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.14.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.14.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| Final condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| RF Channel Number |  |  | 1 |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 (NOTE 1) | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 15 (NOTE 1) | T3 needs to be defined so that cell re-selection reaction time is taken into account. |
| NOTE 1: If the test is performed in a NGSO configuration, and the scaling factor Kmulti_SMTC defined in clause 4.2C.2.3 is greater than 1, according to UE capabilities, the duration of times T2 and T3 shall be scaled for the same factor to allow the UE to complete the cell reselection within the duration of the test case. |  |  |  |  |

Table A.14.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | SSC.1 for GSO test SSC.2 for NGSO test |  |  | NSC.1 for GSO test NSC.2 for NGSO test |  |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| SSB configuration |  | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
| SMTC configuration |  | #1: SMTC.1 for Cell 1 #2: SMTC.4 for Cell 2 |  |  | #1: SMTC.1 for Cell 1 #2: SMTC.4 for Cell 2 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 16 | 13 | 16 | -infinity | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -82 | -infinity | -82 | -85 |
| Io | dBm/9.36 MHz | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 60 |  |  | 60 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |

A.14.1.1.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than:

34 s if Kmulti_SMTC  is  equal to 1 (see note on Table A.14.1.1.2-2); or

66 s if Kmulti_SMTC is equal to 2.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than:

8 s if Kmulti_SMTC  is  equal to 1 (see note on Table A.14.1.1.2-2); or

14.5 s if Kmulti_SMTC is equal to 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Kmulti_SMTC *Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Kmulti_SMTC *Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2C.2.3-1 in clause 4.2C.2.3

Tevaluate, NR_ intra	See Table 4.2C.2.3-1 in clause 4.2C.2.3

Kmulti_SMTC  is described in clause 4.2C.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20ms period and 80 ms period, respectively.

If Kmulti_SMTC = 1, Kmulti_SMTC *Tevaluate, NR_Intra + TSI-NR = 7.68 s; allow 8 s. And Kmulti_SMTC *Tdetect, NR_ intra + TSI-NR = 33.28 s, allow 34s.

If K_multi_SMTC = 2,  Kmulti_SMTC *Tevaluate, NR_Intra + TSI-NR = 14.08 s; allow 14.5 s. And Kmulti_SMTC *Tdetect, NR_ intra + TSI-NR = 65.28 s, allow 66s.

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements

A.14.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for satellite access specified in clause 4.2C.2.3.

A.14.1.2.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.14.1.2.2-1, A.14.1.2.2-2 and A.14.1.2.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. The flag enhancedMeasurementNGSO-r17 should be set.

Table A.14.1.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.14.1.2.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| Final condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| RF Channel Number |  |  | 1 |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 (NOTE 1) | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 15 (NOTE 1) | T3 needs to be defined so that cell re-selection reaction time is taken into account. |
| NOTE 1: If the test is performed in a NGSO configuration, and the scaling factor Kmulti_SMTC defined in clause 4.2C.2.3 is greater than 1, according to UE capabilities, the duration of times T2 and T3 shall be scaled for the same factor to allow the UE to complete the cell reselection within the duration of the test case. |  |  |  |  |

Table A.14.1.2.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | SSC.1 for GSO test SSC.2 for NGSO test |  |  | NSC.1 for GSO test NSC.2 for NGSO test |  |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| SSB configuration |  | SSB.1 FR1 |  |  | SSB.X FR1 |  |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 #2: SMTC.7 for Cell 2 |  |  | #1: SMTC.2 for Cell 1 #2: SMTC.7 for Cell 2 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 16 | 13 | 16 | -infinity | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -82 | -infinity | -82 | -85 |
| Io | dBm/9.36 MHz | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 60 |  |  | 60 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| PDSCH RMC |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 16 | 13 | 16 | -infinity | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -82 | -infinity | -82 | -85 |
| Io | dBm/9.36 MHz | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 60 |  |  | 60 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |

A.14.1.2.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than:

11 s if Kmulti_SMTC  is  equal to 1 (see note on Table A.14.1.2.2-2); or

20 s if Kmulti_SMTC is equal to 2.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than:

6 s if Kmulti_SMTC  is  equal to 1 (see note on Table A.14.1.2.2-2); or

9 s if Kmulti_SMTC is equal to 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Kmulti_SMTC *Tdetect, NR_Intra_enh + TSI-NR, and to an already detected cell can be expressed as: Kmulti_SMTC *Tevaluate, NR_ intra_enh + TSI-NR,

Where:

Tdetect, NR_Intra_enh	See Table 4.2C.2.3-2 in clause 4.2C.2.3

Tevaluate, NR_ Intra_enh	See Table 4.2C.2.3-2 in clause 4.2C.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20ms period and 80 ms period, respectively.

If Kmulti_SMTC = 1, Kmulti_SMTC *Tevaluate, NR_Intra_enh + TSI-NR = 5.12 s; allow 6 s. And Kmulti_SMTC *Tdetect, NR_ Intra_enh + TSI-NR = 10.24 s, allow 11 s.

If K_multi_SMTC = 2,  Kmulti_SMTC *Tevaluate, NR_Intra_enh + TSI-NR = 8.96 s; allow 9 s. And Kmulti_SMTC *Tdetect, NR_Intra_enh + TSI-NR = 19.2 s, allow 20s.

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection

A.14.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for satellite access specified in clause 4.2C.2.3.

A.14.1.3.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.14.1.a3.2-1, A.14.1.a3.2-2 and A.14.1.a3.2-3. The test consists of two successive time periods, with time duration of T1 and T2, respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. t-Service broadcasted in SIB19 of Cell 1 is set to the time point that is 36s after start of T2.

Table A.14.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.14.1.3.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| RF Channel Number |  |  | 1 |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 (NOTE 1) | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| NOTE 1: If the test is performed in a NGSO configuration, and the scaling factor Kmulti_SMTC defined in clause 4.2C.2.3 is greater than 1, according to UE capabilities, the duration of time T2 shall be scaled for the same factor to allow the UE to complete the cell reselection within the duration of the test case. |  |  |  |  |

Table A.14.1.3.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | SSC.1 for GSO test SSC.2 for NGSO test |  | NSC.1 for GSO test NSC.2 for NGSO test |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.1 for Cell 1 and Cell 2 |  | #1: SMTC.1 for Cell 1 and Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | -3.11 | -infinity | 2.79 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity | 16 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity | -82 |
| Io | dBm/9.36 MHz | -53.94 | -52.21 | Same as parameters specified in Cell 1 columns- |  |
| Treselection | s | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 40 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

A.14.1.3.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than :

34 s if Kmulti_SMTC  is  equal to 1 (see note on Table A.14.1.3.2-2); or

66 s if Kmulti_SMTC is equal to 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Kmulti_SMTC *Tdetect, NR_Intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2C.2.3-1 in clause 4.2C.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20ms period and 80 ms period, respectively.

If Kmulti_SMTC = 1, Kmulti_SMTC *Tdetect, NR_ intra + TSI-NR = 33.28 s, allow 34s.

If Kmulti_SMTC = 2, Kmulti_SMTC *Tdetect, NR_ intra + TSI-NR = 65.28 s, allow 66s.

A.14.1.4	Location-based measurement initiation to FR1 intra-frequency NR cell reselection

A.14.1.4.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for satellite access specified in clause 4.2C.2.3.

A.14.1.4.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.14.1.a4.2-1, A.14.1.a4.2-2 and A.14.1.a4.2-3. The test consists of two successive time periods, with time duration of T1 and T2, respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

At 4s after the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SIB19 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m.

Table A.14.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.14.1.4.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| RF Channel Number |  |  | 1 |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 (NOTE 1) | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| NOTE 1: If the test is performed in a NGSO configuration, and the scaling factor Kmulti_SMTC defined in clause 4.2C.2.3 is greater than 1, according to UE capabilities, the duration of time T2 shall be scaled for the same factor to allow the UE to complete the cell reselection within the duration of the test case. |  |  |  |  |

Table A.14.1.4.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | SSC.1 for GSO test SSC.2 for NGSO test |  | NSC.1 for GSO test NSC.2 for NGSO test |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.1 for Cell 1 and Cell 2 |  | #1: SMTC.1 for Cell 1 and Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | -3.11 | -infinity | 2.79 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity | 16 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity | -82 |
| Io | dBm/9.36 MHz | -53.94 | -52.21 | Same as parameters specified in Cell 1 columns- |  |
| Treselection | s | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 40 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

A.14.1.4.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than:

34 s if Kmulti_SMTC  is  equal to 1 (see note on Table A.14.1.3.2-2); or

66 s if Kmulti_SMTC is equal to 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Kmulti_SMTC *Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Kmulti_SMTC *Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2C.2.3-1 in clause 4.2C.2.3

Tevaluate, NR_ intra	See Table 4.2C.2.3-1 in clause 4.2C.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20ms period and 80 ms period, respectively.

If Kmulti_SMTC = 1, Kmulti_SMTC *Tdetect, NR_ intra + TSI-NR = 33.28 s, allow 34s.

If Kmulti_SMTC = 2, Kmulti_SMTC *Tdetect, NR_ intra + TSI-NR = 65.28 s, allow 66s.

# <End of change 1>