3GPP TSG RAN Meeting #105	RP-241887

# Melbourne, Australia, Sep 9th – 12th, 2024

# Title: 	Motivation for Iridium Half Duplex SAN for NB-IoT NTN

# Source:	Iridium Satellite LLC

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	10.1.2

Release	Rel-19

# Introduction

This paper provides details about the need for Half Duplex functionality on network level for NB-IoT NTN, to accompany RP-241889 - the request for a new Work Item.

# Background

A 3GPP NB-IoT based service is to be hosted on a current in-orbit LEO constellation (66 active Satellite Vehicles, plus spares) operated by Iridium.  This constellation works at ~1.6 GHz (ITU-R Mobile Satellite Service allocated band) worldwide. The satellites were recently refreshed, and the constellation has an expected longevity until at least 2035.

This constellation provides global coverage including over polar regions and oceans.  It is fully cross-linked in space, designed as a fully regenerative constellation, with multiple feeder links around the globe ensuring that real-time connectivity between all Satellite Vehicles and ground gateways. These ground gateways will be interfaced with a 3GPP core network allowing connectivity with other terrestrial networks. The in-orbit satellite payloads are fully software defined, allowing the update of the software and firmware on the constellation to enable the required NB-IoT functionalities.

Today, Iridium operates a collection of proprietary services using the current constellation with a healthy revenue stream.  The intention is to add a new NB-IoT-based service to its constellation. To accomplish this, a part of the Iridium’s 10 MHz MSS spectrum will be carved out and dedicated to host NB-IoT anchor carrier(s) for this new service.  An example of this is shown in purple and identified with bold arrow in the diagram to the right. The boundary between legacy services and the new NB-IoT services can be dynamically shifted and hence the NB-IoT service capacity may be progressively increased.  Candidates for this additional capacity are indicated in the figure above in green and identified using light arrows to some of them.

# Market Perspectives

There is a quick-to-market opportunity to deploy this feature over an existing space segment (e.g. Iridium) and provide truly global IoT-NTN service coverage, including the often-overlooked polar and maritime regions.

This new NB-IoT NTN service offer could be marketed primarily as an inbound roaming offering accessed through traditional MNO carriers. When a UE no longer has service from their terrestrial network (TN), the device can roam onto the non-terrestrial network (NTN).

With this new NB-IoT NTN service, three main applications are expected to be brought to the market:

- IoT messaging to support tracking, status reporting, infrastructure monitoring

- Lifeline and rescue services

- SMS to support person-to-person messaging

Several different classes of device may be considered from this service.  Traditional cellular IoT devices may benefit from having a solution that works when out of terrestrial coverage and simplify the logistics for IoT devices.  In addition, smartphones and wearables can also benefit from this service.

For all the above-mentioned reasons, the adoption from a variety of market segments can be addressed in cooperation with MNOs:

- Energy production

- Shipping

- Critical Infrastructure monitoring

- Automotive

- Maritime

- Aviation

- Smartphone and wearables

# The need for further enhancements for 3GPP defined IoT-NTN

Given that most NB-IoT systems are deployed using FDD procedures, it is recommended to consider further enhancements to the IoT-NTN standard with devices operating in FDD mode.

Observation 1: Further enhancements to “IoT-NTN” assuming FDD UE operation to be considered

## Improving the Energy Efficiency

To bring NB-IoT to an NTN constellation, one needs to consider the limited energy budget on board the satellites.  Typically, LEO-based space vehicles have on-board batteries that provide the power for the system and are generally recharged via solar panels on every orbit, where they spend half their time in the shadow of the Earth.  To avoid degrading the constellation lifespan, any NTN system may need to reduce the per-burst RF energy & orbit-average RF energy used to transmit the anchor carrier.  In fact, this requires network energy savings schemes that may also be useful for terrestrial systems.

This approach to reducing orbit-average energy consumption is shown here by using ‘selective activation’ parameter, that signifies transmit and/or receive in only one radio frame within a multi frame period (e.g. N radio frames) (1: N selective activation).  This parameter is referred to as the selective activation periodicity (sa-Periodicity) and is measured in radio frames.

The UE will need to know the configuration details of the Selective Activation to find the anchor carrier.  As such, the details of the selective activation cannot be communicated explicitly over the system information available on the anchor carrier.  Several options are available to communicate this to the UE, including:

- Determining this information in the UE through measurements/observation

- Defining this information as part of the band definition and inform the UE through configuration

The mechanisms to determine/communicate this parameter to the UE and to indicate “usable” System Frames and Sub-Frames can be determined during the standardization process.

SFN  …

SFN  …

SFN  …

SFN  …

SFN N+2

SFN N+1

SFN   N

SFN  …

SFN 3

SNF 2

SFN 1

DL:

SFN 0

sa-Periodicity

Observation 2: Further enhancements on network energy saving to be considered to “IoT-NTN”

# Link budget analysis

The link budget analysis has been provided below for an 8.2-degree elevation angle to satellite (as seen by the UE) for L-band, with the following reference parameters for the existing Iridium LEO@780km deployment:

1/ Scenario investigated:

| Orbit | Farthest Beam Center Elevation | Beam Edge Elevation Angle to Satellite (seen from UE) |
| --- | --- | --- |
| LEO@780km | 20.7 deg | 8.2 deg |

2/ Satellite parameters:

| Payload Characteristics for DL Transmissions: |  |
| --- | --- |
| Equivalent satellite antenna aperture (This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811) | 1.13 m |
| Satellite EIRP density (dBW/MHz) | 37.13 |
| Satellite Tx max Gain (dBi) | 23 |
| Payload Characteristics for UL Transmissions: |  |
| Equivalent satellite antenna aperture (This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811) | 1.13 m |
| G/T(dB/K) | -3.5 |
| Satellite Rx max Gain (dBi) | 23 |

3/ UE parameters:

| UE PC3 | Tx UL = 23 dBm |
| --- | --- |
| UE antenna gain (dBi) | 0 |
| NF (dB) | 7 |

Further, LEO@780km link budget analysis (evaluation of CNR) for both DL and UL (with previous SAN/UE parameters) has been represented below:

| LEO@780km | DL (Satellite to UE) | UL (UE to satellite) |
| --- | --- | --- |
| Frequency [GHz] | 1.60 | 1.60 |
| Tx: DL EIRP [dBm/MHz]/UL [dBm] | 67.13 | 23.00 |
| Rx: G/T [dB/K] | -31.62 | -3.50 |
| Free space path loss [dB] | 164.36 | 164.36 |
| Atmospheric loss [dB] | 0.10 | 0.10 |
| Shadow fading margin [dB] | 3.00 | 3.00 |
| Scintillation Loss [dB] | 2.20 | 2.20 |
| Polarization loss [dB] | 3.00 | 3.00 |
| Additional losses [dB] | 0.00 | 0.00 |
| CNR [dB] for 180 kHz BW | 1.44 | -7.12 |
| CNR [dB] for 90 kHz BW | N/A | -4.11 |
| CNR [dB] for 45 kHz BW | N/A | -1.10 |
| CNR [dB] for 30 kHz BW | N/A | 0.66 |
| CNR [dB] for 15 kHz BW | N/A | 3.68 |
| CNR [dB] for 3.75 kHz BW | N/A | 9.70 |

The results indicate good CNR values which may be covered already by different sets of parameters provided in TR 36.763.

Observation 3: A new reference scenario compared to the ones in TR 36.763 may be considered for a potential normative work on the proposed enhancements

# Conclusions

A new NB-IoT NTN service can be offered over an existing in orbit LEO constellation providing true global coverage in complement with terrestrial coverage provided by MNOs across the world.

This would require some enhancements discussed in the previous clauses:

Observation 1: Further enhancements to “IoT-NTN” assuming FDD UE operation to be considered

Observation 2: Further enhancements on network energy saving to be considered to “IoT-NTN”

Observation 3: A new reference scenario compared to the ones in TR 36.763 may be considered for a potential normative work on the proposed enhancements

A link budget analysis has also been provided for information.

Considering the rather limited enhancements needed to enable this new service that can be made commercially available worldwide in a very short term, and the benefits for the “IoT-NTN” eco system, 3GPP is invited to consider this work as part of the Release 19 work program in the September 2024 RAN Plenary.

Proposal 1: 3GPP to consider further enhancements to “IoT-NTN” in Rel-19 work plan at RAN#105 in September 2024