3GPP TSG-RAN Meeting #105	RP-241912

Melbourne, Australia, Sept 9th – 13th, 2024

Agenda Item:	9.3.1.3

Source:		InterDigital Inc.

Title:	Data collection for the training of a UE-sided model

Document for:	Discussion

# 1. Introduction

The Rel-19 WID on AI/ML for NR Air Interface [1] includes the objective to specify support on necessary signalling/mechanism(s) for LCM to facilitate data collection for UE-sided models as part of the AI/ML general framework (except for the purpose of CN/OAM/OTT collection of UE-sided model training data):

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models

RAN2-125bis discussed data collection for the training of a UE-sided model without reaching consensus [2].

RAN2-127 discussed the topic following an email discussion [3] aiming to create common understanding of the different options [4]. The outcome of the discussion, including multiple remaining FFS, can be found in the table in [5]. During the discussion, RAN2 could not reach consensus on related key requirements [6].

This contribution discusses requirements for data collection for the training of UE-side models, which may then be used to select the option to progress as part of the normative work in RAN2. The contribution also discusses a possible LS to get feedback from SA2 on specific options and remaining FFS.

# 2. Collection of training data for UE-side models

Multiple options in [5] have been discussed that may be loosely referred to as follows i.e.,

- option 1a/b – termination at application level (UP tunnel) with variants 1A (OTT) and 1B (server)

- option 2 – termination in CN domain

- option 3 – termination in OAM domain

RAN2-127 discussions highlighted two key questions related to collecting training date for UE-sided models :

- How should data collected for training of UE models be transferred from the UE over Uu?

More specifically, should the network have means to control the timing and amount of training data collected for UE-side model training when using resources over the Uu interface.

- What part, if any, of the message format(s) carrying collected data over Uu should be standardized?

More specifically, should transfer of data over Uu use a specified procedure including a message format with standardized contents, or a characterisation thereof, for the collected data. Additionally, should there be mean for UE implementation to include proprietary information i.e., information that may only be parsed or decoded by proprietary means.

As discussed in R2-2407413 [6], the collection of data for UE-side model training should be designed for efficiency and scalability towards increasing number of use cases in the future while ensuring that data relevant for training is indeed being collected while minimizing collection of data that may not be relevant when transferred over Uu.

For example, the UEs from which the data is being collected must have the measurement configuration by the network (i.e., RAN/CN). The network is responsible for configuring UEs to report CQI, best beam information, and location information, and thus it is in the unique position to determine feasible and useful data collection configurations. For example, the network might avoid configurations that are overly burdensome, e.g., significantly different RS configurations per UE for BM or overly dense RS configurations for CQI, BM, and positioning. The only way to avoid requiring RS configurations with high overhead to support AI/ML functionalities is for models to be trained with reasonable datasets which resemble reasonable RS transmission configurations from the network.

As discussed in R2-2407413 [6], the amount of data collected may correspond to large amounts of data, which amount if transferred over Uu should be managed as part of the overall system load. Data collection without network awareness or control could easily lead to overcollection of data simply to over engineer models or otherwise having data collected that may ultimately not be used for model training.

There may be further risks that even when configured with the required measurement configuration relevant to data collection for UE-side model training, a UE may not perform the measurements in some cases. For example, to save UE battery, a UE is not required to perform neighbouring cell measurements if the serving cell’s signal level is above the s-measure (if configured), even if the UE has been provided with intra or inter-frequency measurement configurations.

Observation 1:	Unless properly specified, collection of training data for UE-side model may lead to unnecessary collection of data that is not relevant, or may lead to relevant data not being collected, for a given use case.

The total amount of data collected that could be transferred over the Uu interface is also an important consideration even when only relevant data is collected efficiently. Increasing amounts of data may be expected assuming that more use cases where AIML is applicable are deployed over time. Scheduling, traffic management and admission control may become more challenging if the transfer of collected data over Uu is transparent to the network, thereby increasing the possible risks towards network load, congestion events and overall system performance degradation for other services active in concerned cells where such data gets transferred – impacting even services of UEs not involved in the data collection process.

Observation 2: 	Unless properly specified, transfer of training data collected for UE-side models may lead to arbitrarily large amounts of data being untimely transferred over Uu e.g., in period of high system load.

If the network is part of the pipeline for transferring the collected data, then network control of the data collection process implies that the network is responsible for routing the collected data to a network-controlled entity or to a third-party entity via a network entity. In short, the network would be in control of the flow of collected data, which would be up to network implementation.

Considering the above observations, we propose the following which may be used as a requirement on data collection for the training of UE-side models:

Proposal 1: 	Procedure(s) and signaling mechanism(s) for collection of training data for UE-side model for R19 are specified such that the network can control the transfer of data over Uu interface including the procedure(s) to start, schedule and terminate the transfer of collected data.

As explained earlier, relevant data for a given use case should be efficiently collected for UE-side model training. It may however further be desirable to include additional data that may only be decodable and processible by proprietary means.

Observation 3: 	The procedure(s) and protocol(s) used to transfer training data for UE-side model can be designed to meet both the needs for standardized information and the needs for exchanging proprietary information.

Proposal 2: 	Procedure(s) and message format(s) for transferring data collected for training of UE-sided models for R19 are specified such to support both:

    - standardized fields for collected training data for UE-side models; and

    - transparent container(s) of variable size to transfer data that may only be decodable or processible by proprietary means.

Procedure(s) and protocols may be designed to configure whether proprietary information may be transferred.

Finally, we make the following observations related to the discussions in RAN2 [2][3][4][5]:

Observation 4: 	RAN2 discussed several options that possibly involve network functions, protocol layers or functionalities that are outside of RAN2 scope.

Observation 5: 	There was broad understanding and agreement in RAN2 by the majority of companies that it is feasible to meet the requirements of proposals 2 and 3 above with both options 2 and 3 given proper specifications work is done.

Observation 6: 	The majority of companies agreed in RAN2 that network control of the transfer of collected data (e.g., controllability) and use of standardized procedures and messages (e.g., visibility) cannot be achieved with option 1b without a Service Level Agreement (SLA).

Consequently, we propose to ask feedback to SA regarding the feasibility of option 1b in light of the requirements of proposals 1 and 2:

Proposal 3: 	Send an LS to SA2 (cc RAN2 and RAN1) requesting SA2 to evaluate if solution 1b can meet the requirements of proposals 1 and 2 above.

# 4. Conclusion

The following observations and proposals are made regarding collection of training data for UE-side models:

Observation 1:	Unless properly specified, collection of training data for UE-side model may lead to unnecessary collection of data that is not relevant, or may lead to relevant data not being collected, for a given use case.

Observation 2: 	Unless properly specified, transfer of training data collected for UE-side models may lead to arbitrarily large amounts of data being untimely transferred over Uu e.g., in period of high system load.

Proposal 1: 	Procedure(s) and signaling mechanism(s) for collection of training data for UE-side model for R19 are specified such that the network can control the transfer of data over Uu interface including the procedure(s) to start, schedule and terminate the transfer of collected data.

Observation 3: 	The procedure(s) and protocol(s) used to transfer training data for UE-side model can be designed to meet both the needs for standardized information and the needs for exchanging proprietary information.

Proposal 2: 	Procedure(s) and message format(s) for transferring data collected for training of UE-sided models for R19 are specified such to support both:

    - standardized fields for collected training data for UE-side models; and

    - transparent container(s) of variable size to transfer data that may only be decodable or processible by proprietary means.

Observation 4: 	RAN2 discussed several options that possibly involve network functions, protocol layers or functionalities that are outside of RAN2 scope.

Observation 5: 	There was broad understanding and agreement in RAN2 by the majority of companies that it is feasible to meet the requirements of proposals 2 and 3 above with both options 2 and 3 given proper specifications work is done.

Observation 6: 	The majority of companies agreed in RAN2 that network control of the transfer of collected data (e.g., controllability) and use of standardized procedures and messages (e.g., visibility) cannot be achieved with option 1b without a Service Level Agreement (SLA).

Proposal 3: 	Send an LS to SA2 (cc RAN2 and RAN1) requesting SA2 to evaluate if solution 1b can meet the requirements of proposals 1 and 2 above.

# 5. References

[1]	RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, December 2023

[2]	RAN2-116bis Chairman Notes, Changsha, China, April 2024

[3]	R2-2404476, Report of [POST125bis][020][AI/ML PHY] UE side data collection, MediaTek Inc., RAN2-126, Fukuoka, Japan, May 2024

[4]	R2-2407773, Report of [AT127][021][AIML PHY] UE side data collection, InterDigital, Maastricht, Netherlands, August 2024

[5]	R2-2407785, Data collection table modified online, RAN2 Chair, Maastricht, Netherlands, August 2024

[6]	R2-2407413, Data collection for the training of UE-sided model, InterDigital, Ericsson, Nokia, T-Mobile USA Inc., BT PLC, Deutsche Telekom, Verizon, NTT Docomo, Charter Communications Inc., Maastricht, Netherlands, August 2024

[7]	R2-2407209, RAN2 inputs to TR 38.843, Ericsson, RAN2-127, Maastricht, Netherlands, August 2024

[8]	TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Release 18, 2023/12