3GPP TSG RAN Meeting #105							RP-242030

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.1.3

Source:	Ericsson

Title:	Checkpoint for AI/ML PHY

Document for:	Discussion, Decision

# 1	Background

The Rel-19 WID on AI/ML PHY has checkpoints as follows in RAN#105 (Sept ’24):

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

  - For CSI prediction (UE-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

- Testability and interoperability [RAN4]:

  - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects considering use-case specifics

    - Generalization aspects

    - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.)

    - UE processing capability and limitations

    - Post-deployment validation due to model change/drift

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

NOTE: offline training is assumed for the purpose of this project.

NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference.

NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

# 2	Motivation and further consideration of the extension

In the following section we provide our view on the progress related to the above listed study objectives.

## 2.1 Progress related to “CSI feedback enhancement” (RAN1)

### 2.1.1 CSI compression

Agreement

RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

RAN1#118 (August 2024) made the following agreement:

In our view, the background for this agreement is the following :

- Improving trade-off between performance and complexity/overhead

  - Five new use cases were studied in RAN1 by extending the spatial-frequency CSI compression to spatial-temporal-frequency CSI compression. More study is needed to collect system level simulation results and model complexity to make a conclusion on the performance gain and complexity trade-off. It has been agreed in RAN1 #118 to prioritize use cases 2 and 3 in further studies.

- Alleviating/resolving issues related to inter-vendor training collaboration

  - RAN1 has studied many different options and sub-operation for inter-vendor training collaboration. In RAN1 #118, these options were grouped into three directions, A, B, C and multiple issues were identified for each direction. More study is needed for checking the feasibility and performance of each inter-vendor training collaboration options/sub-options and do down-selection among the identified options/sub-options.

- Addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

  - Little progress was made in addressing other aspects due to lack of time. More study is needed to make conclusions on the important aspects like performance monitoring including identifying the cause of performance degradation, data collection, and model pairing. The solutions for these topics are dependent on the selected inter-vendor training collaboration options/sub-options, hence, further study on these topics shall be done in combination with the study on inter-vendor training collaboration options/sub-options.

RAN4 has started the feasibility study of RAN4 option 3 and option 4 for performance testing of CSI compression using two-sided AI/ML model use case. The study is far from concluded on how the performance test can be done. More time is needed for RAN4 to complete the study.

Based on the above, we propose the following

Continue study CSI compression for the remainder of Rel.19

The text in the WID that outlines the objectives for the study can be updated, to better reflect the remaining issues that needs to be addressed and for the efficient progress of the normative work (to avoid the normative work need to handle a down selection of options or to avoid that multiple features for the same functionality is specified).

From RAN1 perspective, these are:

- Further feasibility and performance study and down-selection of inter-vendor collaboration options and/or sub-options.

- How to achieve further complexity reduction of AI/ML models as the studied models are still huge models and the impact of UE energy consumption/battery life has not been assessed.

- Down selection of performance monitoring options

- Error cause detection (e.g., NW side, UE side, data drift) of performance degradation in the field.

- Down-selection, and assessment of specification impact for

  - Data collection,

  - Inference aspects such as model pairing,

  - CQI determination,

  - Higher rank handling,

  - Model structure scalability aspects.

- Understanding the trade-off between system performance gain and complexity/overhead for use case 2 and use case 3.

Discuss whether the WID should be updated to capture the essential aspects (a)-(f) listed above, that needs to be addressed in the study to be able to recommend normative work.

### 2.1.2 CSI prediction

RAN1#118 (August 2024) made the following agreement:

Agreement

From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

A supermajority of companies indicated their support for normative work for this use case in the RAN1 meeting. 12 companies provided the system level simulation results for studying the performance gain of AI based CSI prediction over Rel-18 non-AI based approach, and the following high-level observations were made from RAN1:

- For AI/ML based CSI prediction over non-AI/ML based CSI prediction, 0%~7.8% gain depending on traffic model, in terms of mean UPT, is observed by 7 sources

- For AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3.8%~20.7% gain depending on traffic model, in terms of 5% UE UPT, is observed by 7 sources.

- The ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35 and the median value is 3.3, based on results provided by 15 sources.

- From the perspective of performance impact on channel estimation error and phase discontinuity, compared to non-AI/ML CSI prediction, higher gain is observed by 10 sources for AI/ML based CSI prediction in the presence of channel estimation error or phase discontinuity.

- Generalized performance of the AI/ML model can be achieved if training dataset is constructed with data samples subject to multiple scenarios/configurations including the inference scenario/configuration.

Based on the agreement and observations provided by RAN1, we propose to start normative work for CSI prediction in Rel-19.

Start normative work for CSI prediction in the remainder of Rel.19

Potential specification impact on data collection, performance monitoring for functionality-based LCM, and model inference were identified in RAN1 study for CSI prediction use case.

For data collection, identified potential specification impacts include signalling and procedures for data collection, CSI-RS configuration, assistance information for categorizing the data, if needed.

For performance monitoring, three types of monitoring procedures were proposed, potential specification impact were identified for each type, details including possible down selection of monitoring solutions to be further discussed in the normative phase.

For model inference, it was agreed to reuse legacy CSI-RS configuration and feedback mechanism as specified for legacy “typeII-Doppler-r18” as a starting point and identified potential specification impact includes CSI processing criteria and timeline.

Hence, the normative work on CSI prediction needs to include these objectives:

- Specify necessary signaling/mechanism(s) to facilitate data collection specific to the CSI prediction use case

- Specify necessary signaling/mechanism(s) to facilitate performance monitoring for functionality-based LCM specific to the CSI prediction use case

- Specify necessary signaling/mechanism(s) to facilitate model inference specific to the CSI prediction use case, strive to reuse the configuration and mechanisms specified for legacy codebook type set to “typeII-Doppler-r18”.

Based on the above, we propose the following:

Update the WID text to define the objectives of normative work for CSI prediction use case as in (a) – (c) listed above.

Up to now, CSI prediction has not been discussion much in RAN1. To create requirements for CSI prediction, aspects such as the requirement metrics, baseline/reference model to assume for requirements, amount of requirement/test coverage needed to ensure satisfactory performance, check on suitability of channel model and any need for monitoring related requirements will need to be discussed. With the existing RAN4 TU allocation, RAN4 is pressed for time for discussing the existing issues. For including CSI prediction, we suggest adding a further 0.5 RAN4 TU.

For CSI prediction, add a further 0.5 TU for RAN4.

## 2.2 Progress related to “Necessity and details of model Identification concept and procedure in the context of LCM” and “Model transfer/delivery” (RAN2/RAN1)

RAN1#118 (August 2024) made the following agreement:

Agreement RAN1 is recommending extending the study of the Model identification, and Model transfer/Model delivery based on RAN1 understanding the study is not completed.

The study phase of the Rel-19 WI did not conclude on the need for the model identification, and hence were not considered completed. The only conclusion indicating a possible need for model identification is the following:

Conclusion - From RAN1 perspective, model identification is at least applicable to some of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported)

Note that the study item did not bring any new findings or agreements that justify the need to support model-based LCM for one-sided models, beyond the TR conclusions documented during the Rel-18 SI. In case extending the study of model identification, it should focus on the CSI compression use case where there at least has been some conclusion of a need.

Regarding model transfer, RAN1 has not concluded on the necessity or feasibility of model transfer during the study phase of the rel-19 WI. Model transfer is mainly motivated as one of the options to eliminate the inter-vendor collaboration complexity for the two-sided use case. There are other options being considered that are not dependent on model transfer.

Yet, there is still no concluding agreements in RAN1 on the feasibility of any of the options and which of those options will be standardized if two-sided use case is agreed for normative work.

RAN2 has not further discussed this topic, and no agreements have been made in this regard.

Consider further model identification and model transfer study within the scope of two-sided CSI compression use case only. There is therefore no strong need for a continuation of “other aspects” agenda item in Rel-19, we suggest to RAN1 leadership to remove that agenda point to give more efficient time to other AI/ML agenda points

## 2.3 Progress related to “CN/OAM/OTT collection of UE-sided model training data” (RAN2/RAN1):

Regarding UE-side data collection, the RAN1 related work was already conducted in the Rel-18 study item, and an LS response [4] was sent to RAN2. No further agreements have been made for this aspect.

On the other hand, RAN2 has progressed on this topic and identified different possible options for the collection of UE-sided model training data. The analysis done by RAN2 has taken into account the possible termination entities that may be involved in this procedure, the potential data transfer paths, whether control plane (CP) or user plane (UP) should be used to transfer the data, the protocol layers that may be involved, as well as the possibility for an MNO to have the controllability and visibility over the data collected by the UE, etc. The results of this analysis were captured in an endorsed TP for TR 38.843 [6].  
However, RAN2 did not provide in their study any specific recommendation or identified a preferred solution for a RAN normative work. The identified possible solutions have impacts in SA WGs (see the expected impacted WGs in Table 7.2.1.3.2-1 in [6]), and there are many aspects that are beyond RAN2 area scope on which RAN2 could not reach a conclusion. The open issues captured in [6] include for example the level of possible MNO controllability and visibility for Option 1b, the need for partial and no visibility options, the feasibility of NAS for the transferring of collected data, the feasibility of some UP-based solutions, etc.
Given the above considerations, we suggest involving SA WGs in the discussion.

RAN to liaise SA for the study of solutions in Rel.20 for data collection for the purpose of UE-sided model training, taking into account the endorsed TP for TR 38.843 [6].

## 2.4 Progress related to “Testability and interoperability” [RAN4]:

The basic purpose of RAN4 requirements is to enable interoperability; i.e. that different combinations of NW and UE vendor can work effectively together by means of ensuring predictable and consistent UE behavior and performance. For example, RRM measurements delivered in a known timeframe and with known accuracy.

AI/ML poses some fundamental questions on how to achieve this goal:

- Models may be updated during their lifetime, potentially leading to a divergence in the behavior of different UEs and making management difficult without NW vendor to UE vendor interaction

- It is not clear whether typical levels of RAN4 requirement coverage and RAN5 test coverage can ensure that AI models behave predictably across the range of conditions in which they operate.

- It is not clear how UE model switching etc. may impact behavior, performance etc. as seen from the network

- In some cases, localized training of models may be needed. It is not clear how localized training could be related to meeting conformance requirements.

- Often monitoring is referred to as an effective means of guaranteeing UE performance in the field. Monitoring is indeed promising, but it needs to be established whether requirements are needed on the “accuracy”, timing etc. of monitoring reports in order that the network can receive comparable reports from different UE and model implementations.

- For beam management, testability is a major issue that needs further investigation.

- Although by default so called “synthetic” channel models are a good starting point for RAN4 requirements, it needs to be established on a use-case basis that the test data is realistic enough, so that it ensures predictable performance in the real world and/or does not fail good models.

- For the 2-sided CSI, ensuring a reproduceable, interoperable 3 party solution considering NW vendors, UE vendors and TE is not yet clear. Work is ongoing on the so-called “option 3” and “option 4” solution proposals.

- To create requirements, a basis for developing the requirements is needed, which involves developing and agreeing reference models to assess reasonable performance. This will involve very detailed work for each of the use cases.

With this in mind, although RAN4 has had generic discussions on many of these issues, there remains a huge amount of detailed work in each area. RAN4 progress towards a robust requirements and monitoring framework for each use case and in developing the actual requirements themselves is still in very early phases, and it could even be questioned whether the allocated RAN4 time units are sufficient.

# 3	Regarding ongoing normative work and the 2nd priority cases of positioning accuracy enhancements

The WID have the following objectives for the use case of positioning accuracy enhancements.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

RAN2 Chair guidance for each RAN2 meeting thus far:

#### 8.1.2.3 LCM for Positioning use case Contributions should focus on UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases

In RAN1, so far the discussion followed the verbal guidance from RAN1 vice chair [7]:

The principle is that progress of 1st priority cases is not hindered by 2nd priority cases. RAN1 does not have any artificial blocker to 2nd priority sub-cases.

RAN1 did make some progress on 2nd priority cases, but much diminished compared to the 1st priority cases.

Beyond the fact that Case 2a and 2b are assigned 2nd priority, they are more difficult to make progress. The reasons include:

- Case 2a/2b require tight coordination between UE and LMF, e.g., PRS configuration, measurement type and format, whether/how/what NW-side additional condition(s) the UE should be made aware. Additionally, it’s unclear where the boundaries are to declare consistency vs inconsistency between training and inference.

- For Case 2a/2b, signaling support needs to be explicitly defined for all LCM stages, including training data collection, inference, monitoring, functionality/model management. However, this part is lacking since RAN2 only focus on 1st priority cases for LCM.

- Many issues of Case 2a/2b involve UE capability, e.g., measurement capability, model LCM related capabilities. Different UE vendors have different preferences on UE implementation. On the other hand, there is also strong concern that too much flexibility in UE implementation will lead to market fragmentation.

- The SI phase did not decide on the mapping between the AI ML positioning cases and exisisting positioning methods, or whether new methods should be introduced.  RAN2 will need to discuss the integration of AIML in the positioning methods and inform RAN1 on how much legacy can be re-used.

Five cases together with many variations on UE capability will lead to a large number of possible combinations to support in Rel-19. This discourages companies to invest the effort to bring AI/ML based positioning to real life.

Also, from workload perspective, it is expected to take much effort for RAN1/RAN2/RAN3 to coordinate and finish the work for the three 1st priority cases on time.

Thus, we suggest that Rel-19 only work continues only on the three 1st priority cases listed in the current WID.

For the use case of AI/ML based positioning accuracy enhancements, update the WID to remove the 2nd priority cases.

If the above proposal is agreed, RAN to send an LS to SA and SA2 to inform them about the decision.

# 4	Conclusion

We propose the following:

Proposal 1	Continue study CSI compression for the remainder of Rel.19

Proposal 2	Discuss whether the WID should be updated to capture the essential aspects (a)-(f) listed above, that needs to be addressed in the study to be able to recommend normative work.

Proposal 3	Start normative work for CSI prediction in the remainder of Rel.19

Proposal 4	Update the WID text to define the objectives of normative work for CSI prediction use case as in (a) – (c) listed above.

Proposal 5	For CSI prediction, add a further 0.5 TU for RAN4.

Proposal 6	Consider further model identification and model transfer study within the scope of two-sided CSI compression use case only. There is therefore no strong need for a continuation of “other aspects” agenda item in Rel-19, we suggest to RAN1 leadership to remove that agenda point to give more efficient time to other AI/ML agenda points

Proposal 7	RAN to liaise SA for the study of solutions in Rel.20 for data collection for the purpose of UE-sided model training, taking into account the endorsed TP for TR 38.843 [6].

Proposal 8	For the use case of AI/ML based positioning accuracy enhancements, update the WID to remove the 2nd priority cases.

Proposal 9	If the above proposal is agreed, RAN to send an LS to SA and SA2 to inform them about the decision.

# References

R1-2403834 “LS on 3T6R and 4T6R antenna switching SRS”, RAN4(Huawei)

RP-240087 “WID revision: NR MIMO Phase 5”, Moderator(Samsung)

R1-2401077 “Support for 3 Tx UL transmissions”, Ericsson

R1-2405302 “Discussion on RAN4 LS on 3T6R and 4T6R SRS antenna switching”, Ericsson

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei)

R2-2407807, CR on RAN2 inputs to TR 38.843, Ericsson (Rapporteur)

R1-2401825, “Summary #2 of specification support for positioning accuracy enhancement,” Moderator (Ericsson), RAN1#116, Athens, Greece, February 26th – March 1st, 2024.