3GPP TSG-RAN Meeting #105                                              	RP-242129

Melbourne, Australia, September 9-12, 2024

Source:	CATT

Title:	Discussion on NR mobility enhancements Phase 4

Agenda Item:	9.3.2.1

Document for:	Discussion and Decision

Introduction

As per the WID [1], there are two checkpoints that should be reviewed at RAN#105 regarding the objectives of event triggered L1 measurements and the conditional LTM. Also, there are some controversial issues in RAN1 for the CSI-RS measurement in LTM. This contribution discusses these two checkpoints and the controversial RAN1 issue on CSI-RS measurements.

Discussion

Event triggered L1 measurement reporting

For event triggered L1 measurement reporting, according to the WID [1], whether any modification on this objective is required to avoid/manage any overlap with R19 MIMO WI need to be reviewed at RAN#105.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Up to now, RAN1 and RAN2 have made solid progress on event triggered L1 measurements for the two WIs respectively. We use the following table to make a comparison between MIMO and LTM, based on current RAN1 and RAN2 progress, from the perspectives of purposes, event definition, entity used to evaluate the events and the signaling used to report measurement results:

Table 1: Comparison between LTM and MIMO w.r.t. event triggered L1 measurement reporting

|  | LTM | MIMO |
| --- | --- | --- |
| Purposes | - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. | Beam management for serving cell |
| Event definition | -Event LTM2: Beam of serving cell becomes worse than absolute threshold; -Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; -Event LTM4: Beam of candidate cell becomes better than absolute threshold; -Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2 | - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality |
| Which layer to evaluate the event | MAC | FFS |
| signaling used for measurement results report | MAC CE | UCI |

Based on the above table we can find that the difference between MIMO and LTM for event triggered L1 measurement reporting exists at least in the aspects of purposes, specific event definition and signaling used for measurement result reporting. Though there is similarity between events LTM2/3 and MIMO events 2/1, there is no distinction on the cells where the intended beams belong in MIMO design, which is however required by LTM-defined events. That means how to define the candidate beam or new beam is different for MIMO and LTM.

Observation 1: The purposes, specific event definition and signaling used for measurement result reporting are different for MIMO and LTM.

Based on above comparison in detail, the high level principle and specific designs agreed in MIMO and Mobility WIs so far are quite different, because they are serving different purposes (i.e. cell switch for mobility vs. beam management for MIMO); it is thus not likely to unify the design of event triggered L1 measurements objectives for these two WIs based on the progress till now.

Therefore, we propose to still progress independently on the event triggered L1 measurements objectives of their respective MIMO and Mobility enhancement WIs.

Proposal 1: RAN1 and RAN2 continue progressing independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Remove directly the checkpoint for event triggered L1 measurements objective from the WID.

Conditional LTM

According to the WID [1], the work on conditional LTM has not been started before RAN#105:

- Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint

The benefit of conditional LTM is obvious. Compared with network triggered handover, UE triggered handover could improve the handover robustness, and has advantage of signaling overhead saving to support subsequent conditional LTM.  These are the main technical reasons to support conditional LTM.

The main concern for above checkpoint is the TU availability, which depends on the progress of other objectives already started. As a brief review of the current progress:

- For inter-CU LTM, an essential issue is security update for SA scenario, for which RAN2 waits for SA3 progress. Except for this issue, no other complicated issue is foreseen for the SA case to block the progress. To see this, RAN2 has even started the initial discussion on the scenarios with secondary priority i.e. inter-CU SCG LTM with MCG unchanged and inter-CU MCG LTM.

- As for the event triggered measurement reporting, plenty of progress has been made and the progress is on track.

Hence, the TU now becomes not a problem anymore to start conditional LTM discussion from Q4, 2024, and we think RAN WG work on conditional LTM can be started from Q4, 2024.

Proposal 2: RAN WGs start conditional LTM discussion from Q4, 2024.

Below, our view on what specific aspects should be specified in the normative phase for conditional LTM is given. On top of the existing objective description, we think it should be made clear that the support of conditional RACH-less LTM is within the scope. In Rel-18 LTM, both RACH-based and RACH-less LTM are supported. As for conditional LTM, if only RACH-based LTM were to be supported, the main benefit of LTM on latency reduction would be unable to be achieved, and the benefit on conditional LTM would be no more than the robustness for which support of legacy CHO is already enough. Hence in order to achieve both benefits of LTM (improvement on latency) and UE triggered handover (robustness), conditional RACH-less LTM should be supported.

Considering the key role played by RACH-less LTM to really achieve the benefit, we propose to clarify in the WID that this conditional LTEM objective includes the support of conditional RACH-less LTM, in order to save RAN2 time on debating whether it is in the scope or not.

Proposal 3: Clarify that the support of conditional RACH-less LTM is within the scope in the WID.

On the other hand, to limit this objective to a reasonable scope and to fit the TU of the whole WI, we suggest that the conditional LTM should be discussed only for the essential scenario, i.e. only focusing on intra-CU LTM for SA case.

Proposal 4: The objective on conditional LTM should be limited to intra-CU LTM for SA case.

Necessary physical layer operations on candidate cells before LTM

The following objective is included in WID:

- Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In RAN1#118, a number of enhancements were proposed for the “necessary physical layer operations on candidate cells before LTM”, as summarized in [FL Proposal 5-1-v2] within the FL Summary [2]:

[FL Proposal 5-1-v2] - The following items are further studied in RAN1 for the potential necessary enhancements in Rel-19 LTM - Item 1: CSI acquisition for candidate cell be before cell switch - Item 2: Dynamic update of measurement RS or candidate cells to perform L1-measurement - Item 3: Enhancement on TRS in candidate TCI states to enable faster tracking - Item 4: UL-based measurement - Item 5: Early DL beam management - Item 6: Retention of activated candidate TCI states after cell switch - Item 7: TA acquisition based on CSI-RS - Item 8: Autonomous TCI state activation by event triggered report

The aforementioned enhancements were discussed in RAN1#118 without a consensus. The FL provided the following conclusion in [2]: “FL Proposal 5-1-v2 was not agreed because these items are not clearly defined in the WID and companies has a different understanding whether they are included in the scope. Chair’s guidance is to go RAN plenary to clarify what to be included in the scope.”

Among the items discussed, the first two items are essential features for the WI and should be included within its scope in our view.

- CSI acquisition for candidate cell be before cell switch

The main objective of the WI is to improve the cell-switching performance.  In Rel-18 LTM, the gNB cannot obtain CSI of the target cell, nor can it predict the MCS for PDSCH before cell switch command. This limitation results in performance degradation or delays in scheduling PDSCH with appropriate MCS. Allowing the UE to perform CSI acquisition for candidate cell before cell switch is an effective approach to resolve this problem.

- Dynamic update of measurement RS or candidate cells to perform L1-measurement

Due to the potential rapid changes in the RF environment, the measurement RS or candidate cells configured for a UE may become invalid for ongoing L1-measurement. It is therefore necessary to allow the gNB to update RS configurations for L1-measurements. This capability could potentially reduce unnecessary UE measurement overhead, optimize RS resources, and improve both LTM cell-switching delays and the success rate of LTM cell.

The potential impact for supporting the above two features may not be significant. For Item 1, it needs the signaling support for a UE to report the CSI in additional to other L1 measurements based on L1 measurement on CSI-RS of candidate cells. For Item 2, it requires the support for the gNB to send message to UE to update the measurement configurations.

Based on the above discussion, it is clear that supporting these two features is fundamental to the success of the WI, without significantly impacting the workload. Therefore, we propose explicitly including them in the WID.

Proposal 5: Explicitly include the following two enhancements in WID as part of the “necessary physical layer operations on candidate cells before LTM” objective:

- CSI acquisition based on CSI-RS for candidate cells prior to the cell switch;

- Dynamic update of measurement RS and/or candidate cells for L1-measurement.

Text Proposal to WID

Based on above proposals 1 to 5, we draft a TP to the WID [1] in the Annex. It is proposed that RAN plenary adopts the TP, if above proposals are agreeable.

Proposal 6: Adopt the revised WI objectives in Annex A.

Conclusion

According to the discussion in Section 2, we have the following observations and proposals:

Observation 1: The purposes, specific event definition and signaling used for measurement result reporting are different for MIMO and LTM.

Proposal 1: RAN1 and RAN2 continue progressing independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Remove directly the checkpoint for event triggered L1 measurements objective from the WID.

Proposal 2: RAN WGs start conditional LTM discussion from Q4, 2024.

Proposal 3: Clarify that the support of conditional RACH-less LTM is within the scope in the WID.

Proposal 4: The objective on conditional LTM should be limited to intra-CU LTM for SA case.

Proposal 5: Explicitly include the following two enhancements in WID as part of the “necessary physical layer operations on candidate cells before LTM” objective:

- CSI acquisition based on CSI-RS for candidate cells prior to the cell switch;

- Dynamic update of measurement RS and/or candidate cells for L1-measurement.

Proposal 6: Adopt the revised WI objectives in Annex A.

Reference

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4

- R1-2407445, Final FL summary of Measurements related enhancements for LTM, Moderator (Fujitsu)

- Annex A

----------------------------------------------------------------------------------------------------------------------------------------

### 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs.

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

    - CSI acquisition based on CSI-RS for candidate cells prior to the cell switch

    - Dynamic update of the RS configurations and/or candidate cells for L1 measurements

- Specify support of conditional LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM and RACH-less LTM

  - Focus on  intra-CU LTM when CU is acting as MN and DC is not configured

- Specify RRM requirements related to the above objectives as necessary [RAN4]

Note that Event-1 and Event-7 for MIMO are currently working assumptions.