3GPP TSG RAN Meeting #106				          	RP-242602

Madrid, Spain, December 9-12, 2024

# Source:	Apple Inc, Rohde & Schwarz, Vivo

# Title:	New WID on UE Conformance - Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC)

# Document for:	Endorsement

Agenda Item:	13.4.32

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	UE Conformance - Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC)

## Acronym: NR_FR1_TRP_TRS_enh-UEConTest

## Unique identifier: 	1040111

| This WID includes a Testing part |  | X |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access | X |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-18.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  |  |  |
| No | X | X | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
|  | Stage 2 |
|  | Stage 3 |
| X | Other (e.g. testing) |

* Other = e.g. testing

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NR_FR1_TRP_TRS_enh | RAN4 | 970082 | Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC) |
| NR_FR1_TRP_TRS_enh-Core | RAN4 | 970182 | Core part: Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC) |
| NR_FR1_TRP_TRS_enh-Perf | RAN4 | 970282 | Perf. Part: Enhancement of UE TRP and TRS requirements and test methodologies for FR1 (NR SA and EN-DC) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 960088 | UE Conformance - Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | Release 17 Work Item on NR FR1 TRP TRS Conformance |
| 911110 | Core part: NR_FR1_TRP_TRS | Core Part of Release 17 Work Item on NR FR1 TRP TRS |
| 911210 | Perf. part: NR_FR1_TRP_TRS | Performance Part of Release 17 Work Item on NR FR1 TRP TRS |
| 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | Parent Release 17 Work Item on NR FR1 TRP TRS |

## 3	Justification

RAN5 has completed definition of Rel-17 conformance test cases for NR FR1 TRP TRS. The scope covered FR1 OTA testing on NR FR1 bands n41 and n78 in browsing mode with Hand Phantom. In Rel-18, enhancements for TRP TRS requirements as well as corresponding verification methodologies are introduced:

- Enhancements of TRP TRS test methodology

The RedCap UE was introduced and finalized in Rel-17 with maximum channel bandwidth 20MHz and minimum 1Rx, however the new phantom definitions and UE positioning guidelines are not defined.

In addition, the reverberation-chamber based system was discussed but deprioritized in Rel-17, which is also valuable to perform TRP TRS measurement as an alternative method to save measurement time and re-use large scale of legacy OTA systems.

- TRP TRS requirements for NR 1Tx (Standalone)

In Rel-17, the requirement is only defined for selected bands (n41 and n78) under SA mode with very specific configuration, i.e., wide handset UE (width ≥72mm) operated with 1Tx at PC2 under browsing mode scenario (hand phantom only), while other cases were not defined, e.g., other NR bands, talk mode scenarios (head and hand phantom), and other power classes. 1Tx Requirements for n41 and n78 in talk mode (using head and hand phantoms) and for n1, n28 in browsing and talk mode are defined in Rel-18. The corresponding conformance test cases will be added as part of this work item.

Until RAN#103, the Core part of the RAN4 Rel-18 WI “Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)” is 100% complete and Performance part is 75% completed. This necessitates the introduction of the RAN5 Conformance work item to enable the industry to pursue certification testing associated with new test methods and requirements enhancements for NR FR1 TRP TRS that is the scope of Rel-18.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of this Work Item is to enable UE conformance testing for the corresponding Rel-18 NR FR1 TRP TRS Enhancement Work Items listed under clause 2.2. RAN5 conformance test procedures will be created to verify test requirements as per the scope being covered in the core and performance parts of the Rel-18 Work item namely:

- Enhancements of TRP TRS test methodology

  - Specify reverb-chamber based test methodology and applicability to support SA TRP TRS testing: Align the RAN5 test specifications with any outcome of the harmonization between anechoic chamber and Reverb-chamber based method which can be applied independently for 1Tx TRP TRS testing – for browsing mode, talk mode, and wrist-worn mode.

- Define conformance tests containing test requirements for NR FR1 SISO SA TRP and TRS (OTA) for the following:

-	NR FR1 Bands n41, n78 for PC2 and PC3 UEs in Talk Mode (using Head and Hand Phantoms)

-	NR FR1 Bands n28 and n1 for PC3 UEs in Talk Mode (using Head and Hand phantoms) and Browsing Mode (using Hand Phantoms only).

-	Define test cases and test requirements for the above listed bands and phantom configurations based on the conclusion of core performance requirement definitions.

-	Specifying test requirements of SA with 1 CC is the priority

-	Define a framework for test specifications to incorporate the updated core requirements that will be adopted from RAN#111 for n28 PC3 TRP (Talk Mode and Browsing Mode) and n78 PC2 TRP (Talk Mode).

  - DUT feature focus

    - Rel-18 focus is on smartphone. Test coverage for additional DUT form factors to be determined in future Release(s)

    - The scope covers Wide Grip Hand for UE with Width >72mm and ≤92mm. Additional device widths to be determined in future Release(s)

    - Power Class: Both PC2 and PC3 with 1Tx

- Measurement Uncertainty (MU) and Test tolerances (TT)

  - RAN5 has primary responsibility for MU assessment for FR1 TRP/TRS.

  - A preliminary/placeholder MU table has been added in Annex B of TR 38.870 to cover updates to the MU for browsing mode testing and new values for Talk Mode based FR1 TRP TRS (OTA) testing.

  - This work item scope includes further optimizing and finalizing MU values and then determine TT for the defined test requirements based on recommended Test Tolerances from RAN4.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.561 | Update with Rel-18 conformance test requirements and associated test procedures, test points and MU/TT | TSG RAN#108 (June-25) |  |

## 6	Work item Rapporteur(s)

## Ashwin Mohan (Apple)

ashwin_mohan@apple.com

Jose Fortes (Rohde & Schwarz)

Jose.Fortes@rohde-schwarz.com

Ruixin Wang (Vivo)

ruixin.wang@vivo.com

## 7	Work item leadership

RAN5

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Apple

CAICT

Huawei

HiSilicon

NTT Docomo

Orange

Rohde & Schwarz

SAICT

SGS Wireless

Telecom Italia

Vivo

Vodafone