3GPP TSG RAN Meeting #106								RP-242800

Madrid, Spain, December 9-12, 2024

Agenda Item:	9.3.2.2

Source:	OPPO

Title:	Discussion on WID Scope for NR NTN

Document for:	Discussion & Decision

Introduction

In this paper, we discuss the checkpoint on DL coverage enhancement and we make some suggestions on the update on WID scope.

Discussion

A study has been conducted with the guidance from RAN plenary on the link level and system level evaluations for NR NTN downlink enhancement [1].

- Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

For link level enhancement:

Up to RAN1#119 meeting, RAN1 has conducted all the expected evaluations and based on the evaluation outcomes RAN1 has drawn the following agreements:

Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Agreement For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern.

With the above agreement enhancement, the link level coverage gap can be bridged. Thus, we suggest specifying the enhancement in Rel-19 WI.

Proposal 1: Specify link level enhancement in Rel-19 WI based on RAN1 agreements.

System level enhancement:

RAN1 has discussed the benefits and drawbacks of extending SSB burst periodicity. Based on the simulation assumption of in total 1058 footprints, for satellite Set 1-2, extending SSB burst periodicity up to 320 ms can achieve 96.8% coverage capacity; and with 640 ms SSB burst periodicity can achieve 100% coverage capacity.

Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

Based on the evaluation outcomes, in RAN1#118bis meeting, RAN1 has agreed at least to support SSB burst periodicity extension to 160 ms. However, this can only support up to less than 50% coverage capacity. There were discussions on the implementation solution, e.g., satellite to deploy wider beams than what is assumed in the working assumption. This virtually reduces the total number of the 1058 footprints. So that the achieved coverage capacity rate can be improved. But on the other hand, deploying wider beam may also impact the downlink/uplink link level performance. Moreover, it may also impose some design restrictions on the satellite vendors/operators, which at the end may not be feasible.

Observation 1: With 160 ms SSB burst periodicity, the coverage capacity rate cannot go beyond 50% for satellite Set 1-2.

Observation 2: Wider beam deployment may result in downlink/uplink link level degradation, which on the contrary eats up the benefits from wider beam. However, the feasibility of wider beam deployment may not be confirmed by satellite vendors/operators.

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

Regarding the backward compatibility, RAN1 has drawn an observation to conclude that the Rel-17 and Rel-18 UEs assuming default 20 ms SSB burst periodicity cannot be guaranteed to access the cell deploying extended SSB burst periodicity. But in our understanding, satellite Set 1-2 is targeting to deploy in the field where legacy UE is not expected to access the cell, e.g., green field. In this case, the backward compatibility is not an issue, in particular in some region/country where NR NTN service has never been deployed/commercialized.

Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

Observation 3: Satellite Set 1-2 is targeting to deploy in the field where legacy UE is not expected to access the cell, e.g., green field. In this case, the backward compatibility is not an issue, in particular in some region/country where NR NTN service has never been deployed/commercialized.

Proposal 2: In the normative work, support 320 ms for extended SSB burst periodicity, assumed by UE performing initial access to address the coverage issue for satellite Set 1-2.

As above mentioned analysis, we suggest the capture the following updates in WID for the normative work.

Proposal 3: RAN plenary to update the WID for the DL coverage enhancement as follows:

- Specify solutions for link level enhancements for FR1-NTN and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancement on the following channels are considered: - PDCCH CSS (except for Type-3) time domain repetition - Msg4 PDSCH repetition, targeting coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - SIB1 PDSCH repetition within 20 ms duration, with the number of the repetition fixed to 2. - System level enhancement include the following aspects: - Support the additional default SSB burst periodicity of 320 ms, assumed by UE performing initial access - Necessary enhancement to support beam hopping, where Rel-18 Cell DTX/DRX is considered as baseline.

Conclusion

In this paper, we discussed the checkpoint for DL coverage enhancement, and we have the following observations proposals:

Observation 1: With 160 ms SSB burst periodicity, the coverage capacity rate cannot go beyond 50% for satellite Set 1-2.

Observation 2: Wider beam deployment may result in downlink/uplink link level degradation, which on the contrary eats up the benefits from wider beam. However, the feasibility of wider beam deployment may not be confirmed by satellite vendors/operators.

Observation 3: Satellite Set 1-2 is targeting to deploy in the field where legacy UE is not expected to access the cell, e.g., green field. In this case, the backward compatibility is not an issue, in particular in some region/country where NR NTN service has never been deployed/commercialized.

Proposal 1: Specify link level enhancement in Rel-19 WI based on RAN1 agreements.

Proposal 2: In the normative work, support 320 ms for extended SSB burst periodicity, assumed by UE performing initial access to address the coverage issue for satellite Set 1-2.

Proposal 3: RAN plenary to update the WID for the DL coverage enhancement as follows:

- Specify solutions for link level enhancements for FR1-NTN and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancement on the following channels are considered: - PDCCH CSS (except for Type-3) time domain repetition - Msg4 PDSCH repetition, targeting coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - SIB1 PDSCH repetition within 20 ms duration, with the number of the repetition fixed to 2. - System level enhancement include the following aspects: - Support the additional default SSB burst periodicity of 320 ms, assumed by UE performing initial access - Necessary enhancement to support beam hopping, where Rel-18 Cell DTX/DRX is considered as baseline.

Reference

[1] RP-241667 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3