3GPP TSG RAN meeting #106                                                                                     RP-243126

Madrid, Spain, December 9-12, 2024

Source:	CATT

Title:	5G-A duplex evolution in Rel-20

Agenda Item:	15.2

Document for:	Discussion and decision

Introduction

- SBFD (Sub-band full duplex) operation at gNB side for TDD carrier was studied in Rel-18 and specified in Rel-19. In this contribution, we share our views on 5G-A duplex evolution in Rel-20.

Discussion

In Rel-19, only semi-static SBFD is supported where time and frequency locations of SBFD subbands are semi-statically configured [1]. In order to provide more opportunities for UL transmissions, UL subband is expected to be semi-statically configured in legacy DL symbols. Consequently, DL performance is negatively impacted due to less resource.

Figure 1: Semi-static SBFD

One possible way to mitigate the impact to DL is to support dynamic SBFD, which allows SBFD symbols to fall back to full DL symbols so that the UL/DL resource partitioning can better adapt to the UL/DL traffic requirement. In addition, it allows gNB to fall back to non-SBFD operation in case of strong interference.

Dynamic SBFD was evaluated and studied during Rel-18 SI. Performance evaluation results of dynamic SBFD are captured in TR38.858 section 7.4.1 [2] and the observations are copied below.

For indoor scenario (FR1), for dynamic SBFD compared to semi-static SBFD: - For slot configurations {XXXXU} for dynamic SBFD and semi-static SBFD, - In case of large packet size and dynamic SBFD option 2, based on results from 3 sources, - dynamic SBFD has similar or lower mean and 5% UL Average-UPT for all load levels, - and dynamic SBFD has higher mean and 5% DL Average-UPT for all load levels, except one source reported lower 5% DL Average-UPT for high load level. - In case of large packet size and dynamic SBFD option 3, based on results from 2 sources, - dynamic SBFD has higher mean and 5% UL Average-UPT for low load level, similar or higher mean UL Average-UPT for medium load level, higher or lower 5% UL Average-UPT for medium load level, and lower mean and 5% UL Average-UPT for high load level, - and dynamic SBFD has higher mean DL Average-UPT for low load level, similar or higher 5% DL Average-UPT for low load level, higher mean and 5% DL Average-UPT for medium load level, higher mean DL Average-UPT for high load level, and higher or lower 5% DL Average-UPT for high load level. - In case of small packet size and dynamic SBFD option 3, based on results from 1 source, - dynamic SBFD has lower mean and 5% UL Average-UPT for all load levels, - and dynamic SBFD has similar mean and 5% DL Average-UPT for all load levels, except higher 5% DL Average-UPT for high load level. - For slot configurations {XXXXX} for dynamic SBFD and semi-static SBFD, - In case of large packet size and dynamic SBFD option 3, based on results from 2 sources, - dynamic SBFD has higher mean and 5% UL Average-UPT for all load levels, - and dynamic SBFD has higher mean and 5% DL Average-UPT for all load levels. - In case of small packet size and dynamic SBFD option 3, based on results from 1 source, - dynamic SBFD has lower mean and 5% UL Average-UPT for low load level, similar mean and 5% UL Average-UPT for medium load level, and higher mean and 5% UL Average-UPT for high load level, - and dynamic SBFD has similar mean and 5% DL Average-UPT for low and medium load levels and lower mean and 5% DL Average-UPT for high load level. For Urban Macro (FR1), for dynamic SBFD compared to semi-static SBFD: - For slot configurations {XXXXU} for dynamic SBFD and semi-static SBFD, - In case of large packet size and dynamic SBFD option 2 w/ CLI handling, based on results from 1 source, - dynamic SBFD has similar mean and 5% UL Average-UPT for all load levels, except lower mean UL Average-UPT for low load level, - and dynamic SBFD has higher mean and 5% DL Average-UPT for all load levels, except lower mean DL Average-UPT for high load level. - In case of large packet size and dynamic SBFD option 2 w/o CLI handling, based on results from 1 source, - dynamic SBFD has lower mean and 5% UL Average-UPT for all load levels, - and dynamic SBFD has higher mean and 5% DL Average-UPT for all load levels, except similar 5% DL Average-UPT for high load level. - In case of large packet size and dynamic SBFD option 3 w/o CLI handling, based on results from 2 sources, - dynamic SBFD has higher or lower mean and 5% UL Average-UPT for all load levels, - and dynamic SBFD has similar or higher mean and 5% DL Average-UPT for all load levels, except one source reported lower 5% DL Average-UPT for medium traffic load. - In case of large packet size and dynamic SBFD option 3 w/ CLI handling, based on results from 1 source, - dynamic SBFD has higher mean and 5% UL Average-UPT for all load levels, except similar 5% UL Average-UPT for high load level, - and dynamic SBFD has higher mean DL Average-UPT, similar 5% DL Average-UPT for low load level, similar mean DL Average-UPT for medium load level, lower 5% DL Average-UPT for medium load level, and lower mean and 5% DL Average-UPT for high load level. - For slot configurations {XXXXX} for dynamic SBFD and semi-static SBFD, based on results from 1 source, - In case of large packet size and dynamic SBFD option 3 w/ & w/o CLI handling, - dynamic SBFD has higher mean and 5% UL Average-UPT for all load levels, except a similar 5% UL Average-UPT for low load level without CLI handling, - and dynamic SBFD has similar, higher or lower mean and 5% DL Average-UPT for low and medium load levels, and similar mean and 5% DL Average-UPT for high load level without CLI handling, and lower mean and 5% DL Average-UPT for high load level with CLI handling.

It is proposed to support dynamic SBFD in Rel-20 5G-A. Furthermore, in order to avoid frequent transitions between SBFD and non-SBFD symbols, there should be limitation on the number of transition points within a TDD UL-DL pattern period and the detailed values can be further discussed.

In Rel-19, random access in SBFD symbols is supported for all RRC states. It is desirable to avoid impact on random access procedures in SBFD symbols due to introduction of dynamic SBFD. Whether it is ensured by gNB implementation or any specification efforts are needed can be further discussed.

Proposal 1: Support dynamic SBFD in Rel-20 5G-A to allow SBFD symbols to fall back to non-SBFD symbols.

The following agreement on PDCCH was agreed in Rel-18 SI.

Agreement The following conclusion is to be captured in the TR gNB can configure a CORESET and a search space in a way such that the MOs of the search space occur in either SBFD or non-SBFD symbols, or the MOs of the search space occur in both SBFD and non-SBFD symbols but the associated CORESET does not overlap the boundary of a DL subband in SBFD symbols. If it is agreed to be beneficial that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols, at least the following options can be considered for SBFD-aware UE: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) - Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols Note: These options are applicable to at least USS

Given that there is no enhancement on PDCCH in Rel-19, there is configuration restriction that gNB has to configure a CORESET and a search space in a way such that the MOs of the search space occur in either SBFD or non-SBFD symbols, or the MOs of the search space occur in both SBFD and non-SBFD symbols but the associated CORESET does not overlap the boundary of a DL subband in SBFD symbols.

To achieve better configuration flexibility, it is beneficial that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols. The options identified in the SI can be considered as a starting point. From our perspective, a simple solution with minimal spec impact is preferred. Therefore, Option 3 is our preference.

Figure 2: MOs of a search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols

Proposal 2: For SBFD in Rel-20 5G-A, support a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps with RBs outside DL subband(s) in SBFD symbols.

Summary

In Rel-20 5G-A, we propose to further enhance SBFD to better adapt to the traffic requirements and to improve configuration flexibility for PDCCH.

Proposal 1: Support dynamic SBFD in Rel-20 5G-A to allow SBFD symbols to fall back to non-SBFD symbols.

Proposal 2: For SBFD in Rel-20 5G-A, support a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps with RBs outside DL subband(s) in SBFD symbols.

Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, RAN#104

- TR38.858, Study on Evolution of NR Duplex Operation