3GPP TSG RAN meeting #106										       RP-243146

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.3.4.4

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_FR1_lessthan_5MHz_BW_Ph2 |  |  |  |  |
| Unique ID | 1030081 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241622 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: March/2025 | Performance part: June/2025 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 99% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | R4 |
| --- | --- | --- |
| Rapporteur | Name | Zhang, Meng |
|  | Company | Intel Corporation |
|  | Email | meng.zhang@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	Decisions during RAN4#112bis

- The agreements for UE RF requirements for inter-band NR CA/DC with less than 5MHz CBW are shown below (R4-2417192):

  - SCell SSB position restrictions:

    - SCell with 15 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-2

    - For band n100

      - SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3

      - SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3

    - Capture corresponding restrictions as notes in TS 38.101-1 TS 38.101-1 Table 5.4.3.1-2 and Table 5.4.3.1-3

  - UE capability signaling:

    - UE needs to be able to indicate for each carrier in the CA/DC band combination whether it supports less than 5 MHz channel bandwidth operation for DL/UL

      - To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination UE needs to indicate UE capabilities defined for NR < 5MHz channel bandwidth in Rel-18 and the existing UE capability applicability restriction to single carrier needs to be removed.

      - UE also indicates BCS including support of less than 5MHz channel bandwidth

    - For BCS signalling the following framework is used:

      - Traditional BCSs (i.e., not BCS4 or BCS5): UE is expected to support 3MHz channel bandwidth in the band in the band combination if it indicates a corresponding BCS, which includes 3MHz channel bandwidth in RAN4 specifications. No signalling changes are expected by RAN4.

      - BCS4: if UE indicates support for BCS4 including a band that it also supports 3 MHz channel bandwidth for, then it shall support 3 MHz for CA for that band in the band combination. No signalling changes are expected by RAN4.

      - BCS5: if UE indicates support for BCS5 and minimum channel bandwidth of 3MHz as a part of BCS signalling, then it shall support 3 MHz for CA for that band in the combination.  The existing BCS5 signalling framework needs to be modified to allow indication of 3 MHz as minimum channel bandwidth.

    - It should be possible for UE to indicate support of CA/DC with less than 5MHz channel bandwidth starting from Rel-18 (i.e., allow early implementation from Rel-18)

    - RAN4 has not identified a need for new signaling element. The signalling details are up to RAN2.

- Draft Big CR for less than 5MHz UE RF requirements in Rel-19 was endorsed in R4-2416083.

- An LS was sent to RAN2 on the conclusions reached in this meeting in R4-2417119.

- The agreements for RRM requirements for inter-band NR CA/DC with less than 5MHz CBW are shown below (R4-2416849):

  - Known SCell activation delay requirements

    - For known SCell activation delay requirements RAN4 specifies fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth.

  - Unknown SCell activation delay requirements

    - Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth. FFS the CR details.

  - PSCell addition, (subsequent) conditional PSCell addition delay requirements:

    - Specify new PSCell addition delay requirements for PSCell configured with less than 5MHz channel bandwidth.

    - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

  - NR Conditional Handover including target MCG and target SCG:

    - T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

  - EMR requirements:

    - RAN4 to define EMR requirements for target cell with 12RB PBCH, and the delay requirements for SSB index reading in EMR is extended by [3] more samples.

  - Requirements applicability:

    - SCell activation requirement specified in Rel-19 shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for the target SCell.

##### 2.4.1.2	Decisions during RAN4#113

- Draft Big CR for less than 5MHz UE RF requirements in Rel-19 is endorsed in R4-2418623.

- An LS was sent to RAN2 in R4-2420383 on recent agreement to restrict SSB of less-than-5MHz SCell to sync raster as specified in Tables 5.4.3.1-2 and 5.4.3-3 of TS 38.101-1.

- The agreements for RRM requirements for inter-band NR CA/DC with less than 5MHz CBW are shown below (R4-2416849):

  - Intra-band CA RRM requirements

    - According to the WID, intra-band CA is not in the WI scope.

  - NW indication on PBCH BW in MO configuration

    - No need to introduce network indication on PRB number for PBCH transmission.

  - NW indication on PBCH BW in HO command with direct SCell activation

    - No need to introduce new indication.

  - PSCell change delay requirements

    - No spec impact for this issue given it is already in the spec.

  - Handover with PSCell delay requirements

    - Since the handover with PSCell delay requirements refer to the PSCell addition delay requirements, there is no need to update the specification in Rel-19.

  - Measurement requirements for deactivated SCC

    - Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

Table 9.2.5.1-x: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

  - Conditional handover delay requirements to PCell with 12PRB SSB

    - Not specify CHO requirement for target Pcell with 12 PRB SSB as part of Rel-19 WI.

- Draft Big CR for less than 5MHz RRM requirements in Rel-19 is endorsed in R4-2420241.

#### 2.4.2	Remaining Open issues

None.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

###### RAN4 #112bis

- R4-2415123	On sync raster applicability	CATT

- R4-2415124	Further discussion on UE RF requirements for inter-band NR CA/DC with less than 5MHz CBW	CATT

- R4-2415224	Topic summary for [112bis][120] NR_FR1_5MHz_BW_Ph2	Moderator(Intel)

- R4-2415270	draft LS on UE capability signling for inter-band CA/DC with less than 5MHz channel bandwidth	CATT

- R4-2415484	UE RF requirements for inter-band NR CA/DC with 3MHz CBW	Nokia

- R4-2415540	On RRM core for less than 5MHz Phase 2	Apple

- R4-2415652	Topic summary for [112bis][205] NR_FR1_lessthan_5MHz_BW_Ph2	Moderator (Intel)

- R4-2415818	Discussion on UE RF requirements for inter-band NR CADC with 3MHz CBW	ZTE Corporation, Sanechips

- R4-2415819	LS to RAN2 on introducing UE capability signaling to enable less than 5MHz CBW operation for CA/DC	ZTE Corporation, Sanechips

- R4-2415862	Discussion on RRM requirements for less than 5MHz Ph2	Huawei, HiSilicon

- R4-2416082	Discussion on the remaining issues for Rel-19 less than 5MHz UE RF requirements	Intel Corporation

- R4-2416083	Draft Big CR for less than 5MHz UE RF requirements in Rel-19	Intel Corporation

- R4-2416084	draftCR on SCell SSB transmissions for band n100 with 12 or 20PRB bandwidth	Intel Corporation

- R4-2416085	Discussion on the RRM requirements for Rel-19 less than 5MHz UE	Intel Corporation

- R4-2416086	Draft Big CR for less than 5MHz RRM requirements in Rel-19	Intel Corporation

- R4-2416090	Further Discussion on RRM requirements for NR CA/DC in less than 5 MHz	Ericsson

- R4-2416139	NR Less than 5 MHz and CA/DC	Ericsson

- R4-2416271	LS on applicability of Sync raster for SSB on Scell with 3MHz CBW	Huawei, HiSilicon

- R4-2416288	Discussion on RRM core requirements for Less Than 5Mhz Phase 2	Nokia

- R4-2416383	Discussion on RRM impacts for less than 5MHz BW	ZTE Corporation, Sanechips

- R4-2416389	draft CR to TS 38.133:  NR channel BW less than 5MHz for FR1	ZTE Corporation, Sanechips

- R4-2416455	Discussion on core part for FR1 less than 5MHz	MediaTek inc.

- R4-2416461	Scell bandwidth and sync raster	Qualcomm Inc.

- R4-2416849	WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2	Intel

- R4-2416877	Coffee break discussion minutes for [112bis][205] NR_FR1_lessthan_5MHz_BW_Ph2	Intel

- R4-2417091	Topic summary for [112bis][120] NR_FR1_5MHz_BW_Ph2	Moderator(Intel)

- R4-2417092	WF on NR channel BW less than 5MHz	Intel

- R4-2417118	WF on NR channel BW less than 5MHz	Intel

- R4-2417119	LS on Rel-19 NR channel BW less than 5MHz for FR1	Intel

- R4-2417120	draftCR on SCell SSB transmissions for band n100 with 12 or 20PRB bandwidth	Intel Corporation

- R4-2417192	WF on NR channel BW less than 5MHz	Intel

###### RAN4 #113

- R4-2417541	Draft CR to Big CR for less than 5MHz UE RF requirements in Rel-19	Nokia, Intel

- R4-2417871	On remaining issues on UE RF requirements for inter-band NR CA/DC with 3MHz CBW	CATT

- R4-2417872	draftCR to TS 38.101-1 on BCS definition for CA_n100A-n101A (Option 1)	CATT

- R4-2417873	draftCR to TS 38.101-1 on BCS definition for CA_n100A-n101A (Option 2)	CATT

- R4-2418144	Topic summary for [113][122] NR_FR1_5MHz_BW_Ph2	Moderator (Intel)

- R4-2418272	Topic summary for [113][214] NR_FR1_lessthan_5MHz_BW_Ph2	Moderator (Intel)

- R4-2418360	draftCR to 38.101-1 Less than 5MHz UE RF requirements in Rel-19	ZTE Corporation, Sanechips

- R4-2418502	Discussion on RRM impacts for less than 5MHz BW	ZTE Corporation, Sanechips

- R4-2418566	On RRM core for less than 5MHz Phase 2	Apple

- R4-2418623	Draft Big CR for less than 5MHz UE RF requirements in Rel-19	Intel Corporation

- R4-2418624	Discussion on the RRM requirements for Rel-19 less than 5MHz UE	Intel Corporation

- R4-2418625	Draft Big CR for less than 5MHz RRM requirements in Rel-19	Intel Corporation, ZTE Corporation, Sanechips

- R4-2419117	Discussion on the remaining issues of Rel-19 NR CA/DC in less than 5 MHz	Ericsson

- R4-2419118	Draft CR on SCell and PSCell addition delay requirements for less than 5MHz RRM requirements in Rel-19	Ericsson

- R4-2419142	Discussion on RRM requirements for less than 5MHz Ph2	Huawei, HiSilicon

- R4-2419143	draftCR on RRM requirements for less than 5MHz Ph2	Huawei, HiSilicon

- R4-2419517	Discussion on core part for FR1 less than 5MHz	MediaTek inc.

- R4-2419518	LS on NW indication for MO and HO command for LessThan5MHz	MediaTek inc.

- R4-2419529	LS to RAN2 on sync raster agreements	Qualcomm Incorporated

- R4-2419767	Draft CR for Subsequent Conditional PSCell addition for Less than 5MHz BW	Nokia

- R4-2419768	Discussion on remaining issues for less than 5Mhz ph2	Nokia

- R4-2419770	Draft CR for Subsequent Conditional PSCell addition for Less than 5MHz BW	Nokia

- R4-2420089	WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2	Intel

- R4-2420383	LS to RAN2 on sync raster agreements	Qualcomm Incorporated

- R4-2420384	WF on requirements for less than 5MHz	Intel

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template