3GPP TSG RAN Meeting #106                                                               RP- 243176

# Madrid, Spain, Dec 9 - 12, 2024

Agenda Item:	9.3.1.3

Source:	Apple Inc.

Title:	On Rel-20 5G-Advanced AI/ML for air interface

Document for:	Discussion/Decision

# Introduction

AI/ML for air interface was studied in R18 and extensive evaluation was done by comparing the performance gain of AI-based schemes over non-AI schemes for beam management, positioning, CSI prediction and CSI compression. Use cases using one sided model are being standardized in R19 WI for connected mode UEs. CSI compression is the only two-sided model which has been studied in R18 and R19.

In this paper, we discuss further potential work scope including both one-sided models and two-sided models.

# Discussion

## AI based beam management in RACH procedure for coverage extension

In R18 AI based beam management, both spatial domain and temporal domain beam prediction were studied. In R19 WI, life cycle management for UE side model and NW side model was specified for RRC-connected model UEs, including RS configuration for training, inference and monitoring, and report formats for inference results and performance monitoring report etc. In addition, methods to ensure consistency between training and inference was extensively debated and specified.

It has been observed in the field that FR2 deployment often suffers from a coverage issue, where the UE is having trouble finding the FR2 cells during the RACH procedure. Once connected, with beam refinement and tracking, the link quality is not the bottle neck for FR2 cell usage. Use of wide beams for the RACH procedure is one of the key reasons for limited RACH procedure coverage. Therefore, using AI to predict finer beams for the RACH procedure is one important use case to increase FR2 cell coverage and use.

Both NW side model and UE side model can be used to enable AI based beam management in the RACH procedure.

- For NW side model, the key is how to get the L1-RSRP measurement for model input. This can be done by measuring the L1-RSRP based on multiple transmissions of the RACH sequence. When the gNB measures multiple RACH sequences sent in different ROs associated with different SSBs, the measured L1-RSRP can be used to inference the finer beam for the following message 2 to message 5 transmission and reception assuming beam correspondence at gNB side.

- For UE side model, an association ID needs to be included in SIB 1 to enable the UE to determine whether its UE side model can be used. In addition, the RO configuration for set A is configured in SIB1, so the UE can transmit a RACH sequence corresponding to the finer gNB beam for message 1 to message 5.

Proposal 1: For R20 AI/ML for NR air interface, specify AI based beam management in the RACH procedure to extend the FR2 RACH coverage.

## UE initiated AI based beam management reporting

In R19 MIMO, UE initiated beam reporting is specified, where both the current beam and candidate beams are based on UE measurements. Different events to trigger UE report are specified. UE initiated beam reporting can reduce beam management latency and improve link reliability.

In R19 AI/ML based beam management, with UE side model, a reporting mechanism was discussed using the R18 framework.

To explore the full benefit of AI based beam management using UE side model, UE initiated beam reporting for AI inference results can be used. Compared to R19 MIMO, the key difference is the candidate beams can be based on the AI predicted beam, rather than measured beams.

Given the status in R19 MIMO on UE initiated beam reporting, it might be possible to specify the UE initiated beam reporting for AI based beam management based on R19 MIMO specification.

Proposal 2: For R20 AI/ML for NR air interface, specify UE initiated beam reporting for AI based beam management using UE side model, if not specified already in R19.

## CSI Compression

In [1], the following study scope is captured.

- For CSI compression (two-sided model), further study ways to:

  - Improve trade-off between performance and complexity/overhead

    - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

  - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

R19 CSI compression study focuses on two aspects: alleviate/resolve inter-vendor training complexity and improve performance and complexity tradeoff.

For inter-vendor collaboration, five different options and sub-options are studied to alleviate/resolve inter-vendor training collaboration. Pros/Cons evaluation are further categorized into direction A, B and C.

- Direction A includes option 3, 4 and 5, where either the model structure or the data set format is specified and the model used in deployment can be further updated based on field data using offline engineering.

- Direction B is for model transfer where the model is used in the device directly without offline engineering.

- Direction C is to specify model structure and model parameters in the standard.

Up to RAN1 #119, direction A/B/C have  all been under study. Direction A has been down selected to option 3a-1 and option 4-1. In our view, direction B and direction C have many practical issues under further study in RAN1. Under direction A, option 3a-1 requires the target CSI to enable the UE to handle UE side additional conditions. With target CSI, option 3a-1 works similarly as option 4-1, but requires much larger specification impact including specifying model structure, model delivery, dataset format and delivery. Therefore option 4-1 is preferred.

For further improve performance and complexity tradeoff, time domain compression is introduced, and case 2 (using past CSI at encoder/decoder) and case 3 (joint CSI prediction and compression) are evaluated more extensively compared to other cases. Case 2 has been evaluated by the largest number of companies and shows larger gain. As such, case 2 should be specified.

Proposal 3: For R20 AI/ML based CSI compression using two-sided model, depending on RAN1 study conclusion, specify

- Inter-vendor collaboration option 4-1

- Time-frequency-spatial domain CSI compression using past CSI (case 2)

# Conclusion

In this contribution, we discussed aspects on R20 AI based air interface enhancement WI scope. Based on the discussion, the following proposals have been proposed.

Proposal 1: For R20 AI/ML for NR air interface, specify AI based beam management in RACH procedure to extend the FR2 RACH coverage.

Proposal 2: For R20 AI/ML for NR air interface, specify UE initiated beam reporting for AI based beam management using UE side model, if not specified already in R19.

Proposal 3: For R20 AI/ML based CSI compression using two-sided model, depending on RAN1 study conclusion, specify

- Inter-vendor collaboration option 4-1

- Time-frequency-spatial domain CSI compression using past CSI (case 2)

# Reference

[1] RP-240774, Revised WID on artificial intelligence (AI)/machine learning (ML) for NR air interface, March 18-21, 2024

[2] TR 38.843, Study on AI/ML for NR air interface, Nov 2023