3GPP TSG RAN meeting #106										RP-242465

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.2.4

| WI / SI Name | Study on spatial channel model for demodulation performance requirements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: N/A | Performance part: N/A |  | Testing part: N/A |
| Acronym | FS_NR_demod_SCM |  |  |  |  |
| Unique ID | 1040100 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241610 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2025 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 40 % | Core part: N/A | Performance part: N/A | Testing part: N/A |  |

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Alexander Hamilton |
|  | Company | Nokia |
|  | Email | Alexander.hamilton@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	Agreements from RAN4#112-bis

###### Cases for SU-MIMO

Agreement:

Single-User PDSCH:

8Rx: 8 Layer, MCS 13 on both codewords (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting

(FFS Whether it is already covered in PMI) 4Rx: 4 Layer, MCS 13 (Table 1)  (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting

4 CSI-RS Ports (2,1) for 4 Layer

8 CSI-RS Ports (4,1) for 8 Layer

Single-User PMI

4Rx: 4 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random)

4Rx: 4 Layer (eType II) – Full Throughput Curves (PMI Follow)

4Rx: 2 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random)

4Rx: 2 Layer (eType II) – Full Throughput Curves (PMI Follow)

Interested companies can use initially 8 CSI-RS Ports, more ports can be used to identify and show relevant impacts.

Note: Several companies would prefer to focus on PDSCH until the modelling is stable

###### Cases for MU-MIMO

Agreement:

Interested companies to bring to RAN4#113 views on the following cases:

PDSCH

2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

Interested companies are encouraged to assess 2+2 layers with R-ML / E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

Proponents to highlight the difference between a SCM and TDL for MU-MIMO.

###### UE Speed

Agreement

Interested companies to use 30 km/h and/or 3 km/h for UE speed/velocity.

TDL doppler values to be used for simulation:

- 100 Hz for 30 km/h

- 10 Hz for 3 km/h

Assumption: 3.5 GHz carrier to be used for simulations (aligned with FR1)

###### TDL and CDL Methodologies

Agreement:

Interested companies to use the following candidate SCM options:

- (Option A) CDL (as defined in R4-2415283) with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- (Option B) CDL (as defined in R4-2415283), i.e., with AAV without subarray (fully connected)

  - [(M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports]

- (Option C, for interested companies) CDL (as defined in R4-2415283) without AAV with the following configuration:

  - [(M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports]

  - [(M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports]

- (Option D, for interested companies) CDL from 38.901 with AAV using subarray

  - (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- (Option E, for interested companies) CDL from 38.901 with AAV without subarray (fully connected)

  - [(M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports]

- (Option F, for interested companies) TDL extensions as defined in R4-2412762)

- (Option G for interested companies) TDL extensions as defined in R4-2415382)

Where the BS antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays according to the following description: (M,N,P,Ms,Ns)

M is the number of antenna elements with the same polarization in each column (vertical size)

N is the number of antenna elements with the same polarization in each row (horizontal size)

P is the number of polarizations

Ms is the number of Subarray Elements with the same polarization in each column (vertical size)

Ns is the number of Subarray Elements with the same polarization in each row (horizontal size)

Options on BS antenna configuration for Options A, D:

- Option X: Fixed array size 128 AE (8,8,2,8,Ns)

- Option Y (where most companies views are aligned): Fixed sub-array size 16 AE (8,N,2,8,1)

Companies to clarify their assumptions used for radiation patterns of antenna elements.

Example antenna configuration figures for AAV for Options A and C:

Example for option X with 4 TX ports

Example for option X with 8 TX ports

Example for option Y with 4 TX ports

Example for option Y with 8 TX ports

##### 2.4.1.2	Agreements from RAN4#113

###### Content of Technical Report 38.753

Agreement:

1 Scope 2 References 3 Definitions of terms, symbols and abbreviations 3.1 Terms 3.2 Symbols 3.3 Abbreviations 4 Background 4.1 General 4.2 Study Item Objectives 5 Spatial Channel Modelling Approaches 5.1 CDL Approaches 5.2 TDL Approaches 6 Comparison of Spatial Channel Models 6.1 SU-MIMO (note: 2 companies for the work split) 6.2 MU-MIMO 7 [Implementation/Simulation] Alignment of Spatial Channel Models 8 Conclusions Annex A : Channel Measurements Annex <B>: [Channel Model XYZ] Annex <C>: [Informative: related considerations]

###### Duplex Mode, CBW and SCS

Agreement:

Carrier Frequency:

3.5 GHz

Duplex Mode:

TDD

CBW:

40 MHz

SCS:

30 kHz

###### Receiver Type

Agreement:

IRC Baseline for SU-MIMO;

Interested Companies to use IRC and E-IRC for MU-MIMO and LMMSE for SU-MIMO

###### Cases for MU-MIMO

Agreement:

Interested companies to bring to RAN4#114 views on the following cases:

PDSCH

2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

Interested companies are encouraged to assess 2+2 layers with IRC and E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

###### Antenna Array Virtualisation Notation

Agreement:

- Use 5-argument AAV notation of (M,N,P,Ms,Ns) for all configurations

  - (M,N,P,Ms,Ns) with sub-array

  - (M,N,P,1,1) for direct passthrough

  - (M,N,P,M,N) for ‘fully connected’

###### GCS Antenna Coordinates for gNB

Agreement:

- Height = 25 m

- Azimuth = 0 (placement on the x axis)

- X Coordinate = 0 m

###### LCS Antenna Coordinates for gNB

Agreement:

- α = 0°

- β = 10°

- γ = 0°

###### GCS Antenna Coordinates for UE

Agreement

- Height = 1.5 m;

- Azimuth = 0; (placement on the x axis);

- X Coordinate = 100 m; (Distance from gNB = 100 m)

Note: This set up is for the macro scenario

###### LCS Antenna Coordinates for UE

Agreement:

- α = 180°

- β=0

- γ = 0°

###### BS Antenna Polarisation

Agreement

- BS Polarisation slant (+45, -45)

###### BS Radiation Pattern

Agreement:

- BS antenna radiation pattern as defined in TR 38.901 Table 7.3-1.

###### UE Antenna Polarisation

Agreement

- UE Polarisation (0, 90)

###### UE Radiation Pattern

Agreement:

- UE antenna radiation patterns are omnidirectional.

###### Subcluster Correlation

Agreement:

To avoid inter-cluster correlation, RAN4 companies should make sure they are performing ray-splitting for the sub-clusters as specified by TR 38.827 and TR 38.901 (Table 7.5-5). No further action required.

###### SNR Definition with CDL

Agreement:

Define normalised receive signal power as the following:

The sum of signal power is normalised across all Rx RF chains using a single normalisation factor. The normalisation factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain].

The normalisation factor will be aligned and can be included in TR 38.753.

Interested companies are welcome to provide measured normalization factor for CDL channel with different AAV options in next meeting.

###### TR wording for definition of BS Antenna Panel

Agreement:

- RAN4 to capture the description below in the TR

The BS antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays. One subarray is a unique set of co-polarized antenna elements mapped exclusively to one CSI-RS Port through a virtualizer (set of complex values). The BS antenna panel configuration is defined as (M,N,P,Ms,Ns), according to the description below: M is the number of antenna elements per panel with the same polarisation in each column (vertical size) N is the number of antenna elements per panel with the same polarisation in each row (horizontal size) P is the number of polarisations Ms is the number of antenna elements per subarray with the same polarisation in each column (vertical size) Ns is the number of antenna elements per subarray with the same polarisation in each row (horizontal size)

#### 2.4.2	Remaining Open issues

##### 2.4.2.1	Open Issues following RAN4#112-bis

###### Radiation Pattern

Proposals:

- Option 1: For the purposes of CDL alignment, disregard radiation pattern parameters included in TR 38.827 Section 7.2 and consider Omnidirectional antenna pattern

- Option 2: The antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1

###### Frequency Bands

Proposals:

- Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range

###### Subcluster Correlation

Proposals:

- Option 1: RAN4 should discuss and clarify whether the potential correlation between subclusters is an issue, and whether there should be mechanisms to avoid it.

###### Angles of Departure and Angles of Arrival

Proposals:

- Option 1: Replace the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as  .

###### Time Varying Beam Direction

Proposals:

- Option 1: Further evaluate the method for time variant beam directions in CDL model.

###### Antenna Coordinates, Angles, and other parameters for gNB (for UMa-CDL-C)

Proposals:

- Option 1: For the gNB, use the following parameters for alignment with UMa CDL-C model:

  - GCS Location Coordinates for the gNB:

  - Height = 25m (based on UMa);

  - Azimuth = 0; (placement on the x axis);

  - GCS to LCS Conversion angles  (α,β,γ = bearing, downtilt, slant) for the gNB;

  - α,β,γ=(0,0,0);

- Option 2:

  - Frequency Fc = 3.5 GHz, height BS = 25m, and D2D = 100m.

  - BS orientation: (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees). Intention is that BS points towards horizon along x-axis.

- Other options not precluded

###### Antenna Coordinates, Angles, and other parameters for UE (for UMa-CDL-C)

Proposals:

- Option 1: For the UE, use the following parameters for alignment with UMa CDL-C model:

  - Height = 25m (same as gNB)

  - Azimuth = 0; (placement on the x axis);

  - α,β,γ=(90,0,45);

- Option 2:

  - Frequency Fc = 3.5 GHz, height UE = 1.5m and D2D = 100m.

  - UE orientation: (alpha = -90 degrees, beta = -13.2 degrees, gamma = 45 degrees). Intention is that UE points towards BS and uses "[+ + ... +]" crosspol elements.

- Other options not precluded.

###### BS Antenna Virtualiser choice

Proposals:

- Option 1: Down Tilt Virtualiser : -10 degrees

- Option 2: Broadside

- Option 3: Max Receive Power

- Option 4: DFT Virtualiser according to TR 36.897 with application of panning and tilting angles (as per Table 7.8-1 of 38.901)

###### Assumptions for CDL based modelling:

Proposals:

- Option 1:

  - For companies submitting 38.901 CDL based results, it is encouraged to base themselves on 38.901 CDL-C (i.e., on denormalized 38.901 table 7.7.1-3) and bring final CDLC table (delays, AoDs, ASDs, etc.) in the next meeting for alignment.

  - It is further encouraged to also consider the 38.827 "randomness reduction framework" on top of 901 modelling.

- Other options not precluded.

##### 2.4.2.2	Open Issues following RAN4#113

###### Cases for SU-MIMO

Previous agreements from the WF of RAN4#112-bis:

Agreement: Single-User PDSCH: 8Rx: 8 Layer, MCS 13 on both codewords (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting (FFS Whether it is already covered in PMI) 4Rx: 4 Layer, MCS 13 (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer Single-User PMI 4Rx: 4 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random) 4Rx: 4 Layer (eType II) – Full Throughput Curves (PMI Follow) 4Rx: 2 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random) 4Rx: 2 Layer (eType II) – Full Throughput Curves (PMI Follow) Interested companies can use initially 8 CSI-RS Ports, more ports can be used to identify and show relevant impacts. Note: Several companies would prefer to focus on PDSCH until the modelling is stable

Way Forward (from RAN4#113):

Random PMI as a baseline, FFS Fixed.

Interested companies invited to choose based on most common PMI in PMI histogram.

###### Metrics for comparison

Contributions welcome at RAN4#114.

###### Tap Delay Line Parameters

Contributions welcome at RAN4#114.

###### TDL Model Extensions

Contributions welcome at RAN4#114.

###### Angles of Departure and Angles of Arrival

Way Forward:

Option 1: Replace the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as  .

Set mf,desired based on the LOS direction between the TX and RX

Option 2: Following 38.827 (i.e., mf,desired = mf,model).

###### BS Antenna Virtualiser

Way Forward:

Broadside for BS AAV options except fully connected.

Max receive power for standard fully connected AAV.

Interested companies to study with AAV to use max receive power of single strongest direction.

###### TDL and CDL Methodologies

Way Forward:

Channel Model Configurations:

- Option A1: CDL (TR 38.753) with Table 7.2.1-8 in TR 38.827

- Option A2: CDL (TR 38.753) with Table TBD

- Option B: TDL extensions as defined in (R4-2412762)

- Option C: TDL extensions as defined in (R4-2415382)

- Option D: TDL extensions as defined in (R4-2419895)

AAV Configurations:

- Option 1: with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- Option 2: with AAV without subarray (fully connected)

  - (M,N,P,Ms,Ns) = (8,8,2,8,8) for 8Tx and 4Tx CSI-RS Ports

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

RAN4 to define 38.753 CDL with randomness reduction framework and AAV framework.

###### Definition of New Requirements

Contributions welcome at RAN4#114

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

## 4.	References

From RAN4#112

- RP-241610	SID, Study on spatial channel model for demodulation performance requirements	Nokia, BT plc

- R4-2411557	Scenarios and Requirements for Spatial Channel Modelling	BT plc

- R4-2411044	Work Plan for Spatial Channel Model Study Item	Nokia

- R4-2412320	General view on SCM for demodulation	Ericsson

- R4-2412321	Discussion on SCM methodology	Ericsson

- R4-2412322	Simulation results for SCM	Ericsson

- R4-2412328	Discussion on Spatial Channel Modelling	MediaTek inc.

- R4-2413272	On Rel-19 spatial channel model for demodulation performance requirements	Keysight Technologies UK Ltd

- R4-2413056	Initial Considerations on the study on SCM for demodulation performance requirements	Qualcomm Incorporated

- R4-2412793	Discussion on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2412762	Overviews on spatial channel model for demodulation requirements	Huawei,HiSilicon

- R4-2411119	Discussion on spatial channel modelling methodology	CATT

- R4-2411300	Discussion on Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2411301	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2411390	On Spatial Channel Modeling for Demodulation Requirements	Apple

- R4-2412535	Initial view on spatial channel modelling SI	Samsung

- R4-2413427	Topic summary for [112][327] NR_SCM	Moderator (Nokia)

- R4-2413568	Offline meeting minutes for [112][327] NR_SCM	Nokia

- R4-2413606	Way Forward for [112][327] NR_SCM	Nokia

From RAN4#112-bis

- R4-2414957	Views on spatial channel modeling study	China Telecom

- R4-2415078	Discussion on spatial channel modelling methodology	CATT

- R4-2415282	Discussion on Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2415283	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2415285	TDL and CDL comparison with respect to the CSI resolution feedback 	Orange, BT, Telecom Italia, Vodafone, Bouygues Telecom, Fraunhofer, Deutsche Telekom

- R4-2415296	Discussion on General Items on SCM SI	Nokia

- R4-2415382	Discussion on spatial channel modelling	MediaTek inc.

- R4-2415383	Simulation results of spatial channel modelling	MediaTek inc.

- R4-2415506	On Spatial Channel Modeling for Demodulation Requirements	Apple

- R4-2415636	Discussion on spatial channel modelling methodology	Huawei,HiSilicon

- R4-2415673	Further observations from deployment and MU-MIMO use cases for SCM evaluation	BT plc

- R4-2415682	Discussion on spatial channel model for demodulation performance requirements	Samsung

- R4-2416093	Discussion on general issue of NR SCM for demodulation	Ericsson

- R4-2416094	Discussion on methodology of NR SCM for demodulation	Ericsson

- R4-2416095	Simulation results for NR SCM for demodulation	Ericsson

- R4-2416343	Discussion on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2416344	Simulation results on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2416380	Study of SCM Model Parameters for demodulation performance requirements	Qualcomm Incorporated

- R4-2416451	On Rel-19 spatial channel model for demodulation performance requirements	Keysight Technologies UK Ltd

- R4-2416539	Topic summary for [112bis][318] NR_SCM	Moderator (Nokia)

- R4-2416549	TR 38.753 Skeleton	Nokia

- R4-2416581	Ad-hoc meeting minutes for [112bis][318] NR_SCM	Nokia

- R4-2416591	Way Forward for [112bis][318] NR_SCM	Nokia

From RAN4#113

- R4-2417556	Discussion on General Items on SCM SI	Nokia

- R4-2417801	Discussion on spatial channel modelling	MediaTek inc.

- R4-2417802	Simulation results of spatial channel modelling	MediaTek inc.

- R4-2417827	Further discussion on spatial channel modelling methodology	CATT

- R4-2418028	Further considerations on the study on SCM	Qualcomm Incorporated

- R4-2418042	On Rel-19 Spatial channel model CDL Based Methodologies aspects	Keysight Technologies UK Ltd

- R4-2418043	Discussion on Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2418044	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2418550	On Spatial Channel Modeling for Demodulation Requirements	Apple

- R4-2418620	Discussion on spatial channel model for demodulation performance requirements	Samsung

- R4-2419166	Discussion on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2419251	General issue for Rel-19 SCM for demodulation	Ericsson

- R4-2419252	Discussion on methodology for Rel-19 SCM for demodulation	Ericsson

- R4-2419253	Simulation results for SCM for demodulation	Ericsson

- R4-2419338	Further observations on the properties of MIMO channels in typical deployments	BT plc, Ericsson

- R4-2419347	Initial Simulation Results for Performance Comparison	BT plc

- R4-2419607	Topic summary for [113][320] NR_SCM	Moderator (Nokia)

- R4-2419781	Ad-hoc meeting minutes for [113][320] NR_SCM	Nokia

- R4-2419782	Way Forward for [113][320] NR_SCM	Nokia

- R4-2419895	Discussion on spatial channel model for demodulation requirements	Huawei, HiSilicon