3GPP TSG-RAN WG1 Meeting #103-e	R1-2009516

# e-Meeting, October 26th – November 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0153 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for the prioritization between uplink transmission and sidelink transmission/reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (LG Electronics), Ericsson, Apple, NTT DOCOMO, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current specifications do not specify the UE behavior in handling some cases where a UE is not able to perform simultaneous uplink transmission and sidelink transmission/reception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE behavior is defined for the following cases: - Prioritization between PSCCH/PSSCH TX and UL TX - Prioritization between PSFCH/S-SSB RX and UL TX - Prioritization between sidelink TX/RX and the random access related UL TX |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior in the additional cases remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.4.3, 16.2.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

#### 16.2.4.3	Simultaneous SL and UL transmissions/receptions

If a UE

-	would simultaneously transmit on the UL and on the SL of a serving cell, and

-	the UE is not capable of simultaneous transmissions on the UL and on the SL of the serving cell

the UE transmits only on the link, UL or SL, with the higher priority.

If a UE

-	would simultaneously transmit on the UL and receive on the SL in a carrier,

the UE transmits on UL or receives on SL, with the higher priority.

If a UE

-	is capable of simultaneous transmissions on the UL and on the SL of two respective carriers of a serving cell, or of two respective serving cells,

-	would transmit on the UL and on the SL of the two respective carriers of the serving cell, or of the two respective serving cells,

-	the transmission on the UL would overlap with the transmission on the SL over a time period, and

-	the total UE transmission power over the time period would exceed PCMAX

the UE

-	reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

-	reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

##### 16.2.4.3.1	Prioritizations for sidelink and uplink transmissions/receptions

A UE performs prioritization between SL transmissions/receptions and UL transmissions after performing the procedures described in Clause 9.2.5 and in Clause 6.1 of [6, TS 38.214].

PSFCH transmissions in a slot have a same priority value as the smallest priority value among PSSCH receptions with corresponding HARQ-ACK information provided by the PSFCH transmissions in the slot.

PSFCH receptions in a slot have a same priority value as the smallest priority value among PSSCH transmissions with corresponding HARQ-ACK information provided by the PSFCH receptions in the slot.

A priority of S-SS/PSBCH block transmission or reception is provided by sl-SSB-PriorityNR.

For prioritization between SL transmission or PSFCH/S-SS/PSBCH block reception and UL transmission other than a PRACH, or a PUSCH scheduled by an UL grant in a RAR and its retransmission, or a PUSCH corresponding to Type-2 random access procedure and its retransmission, or a PUCCH with sidelink HARQ-ACK information report

-	if the UL transmission is for a PUSCH or for a PUCCH with priority index 1,

-	if sl-PriorityThresholdULURLLC is provided

-	the SL transmission or reception has higher priority than the UL transmission if a smallest priority value of the SL transmission(s) is smaller than sl-PriorityThresholdULURLLC; otherwise, the UL transmission has higher priority than the SL transmission or reception

-	else

-	the UL transmission has higher priority than the SL transmission or reception

-	else

-	the SL transmission or reception has higher priority than the UL transmission if the smallest priority value of the SL transmission(s) or reception is smaller than sl-PriorityThreshold; otherwise, the UL transmission has higher priority than the SL transmission or reception

A PRACH transmission, or a PUSCH scheduled by an UL grant in a RAR and its retransmission, or a PUSCH for Type-2 random access procedure and its retransmission has higher priority than a SL transmission or reception.

## *** Unchanged text is omitted ***