3GPP TSG-RAN WG5 Meeting # 92-e	R5-215574

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1376 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE CA_41E channel spacing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria RFFE GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LTE CA_41E channel spacing in TS36.508 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected channel spacing in Table 4.3.1.2.9A-9: Test frequencies for CA_41E. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE CA_41E channel spacing in TS36.508 remains incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.9A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

##### 4.3.1.2.9A	TDD reference test frequencies for CA in operating band 41

Table 4.3.1.2.9A-1: Test frequencies for CA_41C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 25+100 | 25 | 39683 | 2499.3 | 100 | 39800 | 2511 |
|  |  | 100 | 39750 | 2506 | 25 | 39867 | 2517.7 |
|  | 50+75 | 50 | 39703 | 2501.3 | 75 | 39823 | 2513.3 |
|  |  | 75 | 39725 | 2503.5 | 50 | 39845 | 2515.5 |
|  | 50+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 |
|  | 75+75 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 |
|  | 75+100 | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 |
|  | 100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 |
| Mid | 25+100 | 25 | 40528 | 2583.8 | 100 | 40645 | 2595.5 |
|  |  | 100 | 40595 | 2590.5 | 25 | 40712 | 2602.2 |
|  | 50+75 | 50 | 40549 | 2585.9 | 75 | 40669 | 2597.9 |
|  |  | 75 | 40571 | 2588.1 | 50 | 40691 | 2600.1 |
|  | 50+100 | 50 | 40526 | 2583.6 | 100 | 40670 | 2598.0 |
|  |  | 100 | 40571 | 2588.1 | 50 | 40715 | 2602.5 |
|  | 75+75 | 75 | 40545 | 2585.5 | 75 | 40695 | 2600.5 |
|  | 75+100 | 75 | 40523 | 2583.3 | 100 | 40694 | 2600.4 |
|  |  | 100 | 40546 | 2585.6 | 75 | 40717 | 2602.7 |
|  | 100+100 | 100 | 40521 | 2583.1 | 100 | 40719 | 2602.9 |
|  | 100+1002 | 100 | 40529 | 2583.9 | 100 | 40712 | 2602.2 |
| High | 25+100 | 25 | 41373 | 2668.3 | 100 | 41490 | 2680 |
|  |  | 100 | 41440 | 2675 | 25 | 41557 | 2686.7 |
|  | 50+75 | 50 | 41395 | 2670.5 | 75 | 41515 | 2682.5 |
|  |  | 75 | 41417 | 2672.7 | 50 | 41537 | 2684.7 |
|  | 50+100 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 75+75 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  | 75+100 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  | 100+100 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. Note 2: This test frequency is applicable only for intra-band contiguous CA which requires channel spacing to be less than norminal channel spacing. |  |  |  |  |  |  |  |

<< Unchanged sections skipped>>

Table 4.3.1.2.9A-9: Test frequencies for CA_41E

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |  |  |  | CC3 Note1 |  |  |  |  |  |  |  |  |  | CC4 Note1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] |  | NUL/DL |  |  |  | fUL/DL [MHz] |  |  | BW [RB] |  |  | NUL/DL |  | fUL/DL [MHz] |  |  |
| Low | 75+75+75+100 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 |  | 75 |  | 40025 |  |  |  | 2533.5 |  |  | 100 |  |  | 40196 |  | 2550.6 |  |  |
|  | 75+75+100+100 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 |  | 100 |  | 40046 |  |  |  | 2535.6 |  |  | 100 |  |  | 40244 |  | 2555.4 |  |  |
|  |  | 75 | 39725 | 2503.5 | 100 | 39896 | 2520.6 |  | 75 |  | 40067 |  |  |  | 2537.7 |  |  | 100 |  |  | 40238 |  | 2554.8 |  |  |
|  |  | 100 | 39750 | 2506.0 | 75 | 39921 | 2523.1 |  | 75 |  | 40071 |  |  |  | 2538.1 |  |  | 100 |  |  | 40242 |  | 2555.2 |  |  |
|  | 75+100+100+100 | 75 | 39725 | 2503.5 | 100 | 39896 | 2520.6 |  | 100 |  | 40094 |  |  |  | 2540.4 |  |  | 100 |  |  | 40292 |  | 2560.2 |  |  |
|  |  | 100 | 39750 | 2506.0 | 75 | 39921 | 2523.1 |  | 100 |  | 40092 |  |  |  | 2540.2 |  |  | 100 |  |  | 40290 |  | 2560.0 |  |  |
|  |  | 100 | 39750 | 2506.0 | 100 | 39948 | 2525.8 |  | 75 |  | 40119 |  |  |  | 2542.9 |  |  | 100 |  |  | 40290 |  | 2560.0 |  |  |
|  | 100+100+100+100 | 100 | 39750 | 2506.0 | 100 | 39948 | 2525.8 |  | 100 |  | 40146 |  |  |  | 2545.6 |  |  | 100 |  |  | 40344 |  | 2565.4 |  |  |
| Mid | 75+75+75+100 | 75 | 40370 | 2568.0 | 75 | 40520 | 2583.0 |  | 75 |  | 40670 |  |  |  | 2598.0 |  |  | 100 |  |  | 40841 |  | 2615.1 |  |  |
|  | 75+75+100+100 | 75 | 40345 | 2565.5 | 75 | 40495 | 2580.5 |  | 100 |  | 40666 |  |  |  | 2597.6 |  |  | 100 |  |  | 40864 |  | 2617.4 |  |  |
|  |  | 75 | 40345 | 2565.5 | 100 | 40516 | 2582.6 |  | 75 |  | 40687 |  |  |  | 2599.7 |  |  | 100 |  |  | 40858 |  | 2616.8 |  |  |
|  |  | 100 | 40370 | 2568.0 | 75 | 40541 | 2585.1 |  | 75 |  | 40691 |  |  |  | 2600.1 |  |  | 100 |  |  | 40862 |  | 2617.2 |  |  |
|  | 75+100+100+100 | 75 | 40320 | 2563.0 | 100 | 40491 | 2580.1 |  | 100 |  | 40689 |  |  |  | 2599.9 |  |  | 100 |  |  | 40887 |  | 2619.7 |  |  |
|  |  | 100 | 40345 | 2565.5 | 75 | 40516 | 2582.6 |  | 100 |  | 40687 |  |  |  | 2599.7 |  |  | 100 |  |  | 40885 |  | 2619.5 |  |  |
|  |  | 100 | 40345 | 2565.5 | 100 | 40543 | 2585.3 |  | 75 |  | 40714 |  |  |  | 2602.4 |  |  | 100 |  |  | 40885 |  | 2619.5 |  |  |
|  | 100+100+100+100 | 100 | 40320 | 2563.0 | 100 | 40518 | 2582.8 |  | 100 |  | 40716 |  |  |  | 2602.6 |  |  | 100 |  |  | 40914 |  | 2622.4 |  |  |
| High | 75+75+75+100 | 75 | 41015 | 2632.5 | 75 | 41165 | 2647.5 |  | 75 |  | 41315 |  |  |  | 2662.5 |  |  | 100 |  |  | 41486 |  | 2679.6 |  |  |
|  | 75+75+100+100 | 75 | 40965 | 2627.5 | 75 | 41115 | 2642.5 |  | 100 |  | 41286 |  |  |  | 2659.6 |  |  | 100 |  |  | 41484 |  | 2679.4 |  |  |
|  |  | 75 | 40965 | 2627.5 | 100 | 41136 | 2644.6 |  | 75 |  | 41307 |  |  |  | 2661.7 |  |  | 100 |  |  | 41478 |  | 2678.8 |  |  |
|  |  | 100 | 40990 | 2630.0 | 75 | 41161 | 2647.1 |  | 75 |  | 41311 |  |  |  | 2662.1 |  |  | 100 |  |  | 41482 |  | 2679.2 |  |  |
|  | 75+100+100+100 | 75 | 40915 | 2622.5 | 100 | 41086 | 2639.6 |  | 100 |  | 41284 |  |  |  | 2659.4 |  |  | 100 |  |  | 41482 |  | 2679.2 |  |  |
|  |  | 100 | 40940 | 2625.0 | 75 | 41111 | 2642.1 |  | 100 |  | 41282 |  |  |  | 2659.2 |  |  | 100 |  |  | 41480 |  | 2679.0 |  |  |
|  |  | 100 | 40940 | 2625.0 | 100 | 41138 | 2644.8 |  | 75 |  | 41309 |  |  |  | 2661.9 |  |  | 100 |  |  | 41480 |  | 2679.0 |  |  |
|  | 100+100+100+100 | 100 | 40890 | 2620.0 | 100 | 41088 | 2639.8 |  | 100 |  | 41286 |  |  |  | 2659.6 |  |  | 100 |  |  | 41484 |  | 2679.4 |  |  |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>