3GPP TSG-RAN WG Meeting #93 Electronic	RP-212169

Elbonia, 13 – 17 September 2021

Agenda item:	9.9

Source:	Nokia, Nokia Shanghai Bell

Title:	Resolving the band n77 US extension

WID/SID:	NR_RF_FR1-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

During RAN#92e, the topic of band n77 extension in the US for the frequency range 3.45-3.55 GHz (often referred to as the "DoD band", despite this moniker being slightly inaccurate as the US DoD frequency ranges cover more than just 3.45-3.55 GHz) was discussed, with the following proposals agreed (as per outcome of RP-211587):

- RAN4 focuses on the necessary updates to RAN4 requirements and leave signaling work, if any, to RAN2.

- RAN2 focuses on signaling aspects, with an aim to ensure the network can properly deal with legacy n77 UEs that do not support 3.45-3.55 GHz operation in US

- RAN tasks RAN4/2 to complete the required work in Aug. and report back to RAN#93-e

- RAN4 chair is kindly asked to use an appropriate agenda to facilitate the work in Aug. meeting, i.e., R16 maintenance, R16 TEI, etc.

RAN2#115e and RAN4#100e both discussed the topic, with the following conclusions:

-	RAN2#115e: Agreed that signalling is needed to distinguish UEs supporting and not supporting the extended frequency range in the US, and technically endorsed the CRs in R2-2109182, R2-2109183, R2-2109184 and R2-2109185. An LS was also sent to RAN4 and RAN in R2-2109223 to inform of this decision and to request RAN4 to define an NS-value to prevent IDLE mode UEs not supporting the extended band n77 in the US from camping on the cells using that frequency range.

-	RAN4#100e: Agreed to the WF R4-2115080, which meant that the CRs R4-2112050 and R4-2115112 were agreed to TS38.101-1 based on the modified n77 definition for the US. However, RAN4 was not able to conclude on CRs for adding an NS-value (although proposd b the email discussion outcome R4-2115005).

Hence, it seems it would be RAN#93e needs to resolve this matter and we present the proposed way forward in this document.

# 2	Way forward with band n77 in the US

First, we would note that both the RAN2 and RAN4 agreements are compatible: Both enalbe the use of the frequency range 3.45-3.55 GHz in specifications. Apart from finally agreeing to the CRs, it seems only one aspect is missing: Adding a new NS-value to RAN4 specifications. We think this is a simple matter as such, and not even a very technically challenging one: RAN#93e can approve the 1) CRs endorsed in RAN2 (or their revised versions in RP-2102208, RP-2102209, RP-2102210, RP-2102211), 2) CRs agreed in RAN4, and 3) company CRs adding the new NS-value to RAN4 specifications (i.e. RP-212212, RP-212213).

Observation 1: The RAN2#115e and RAN4#100e agreements are compatible with each other and only lack the addition of an NS-value to band n77.

Proposal 1: RAN to resolve the n77 extension in the US by approving the with company contributed CRs that add the capability signalling to LTE and NR along with NS-value to TS38.101-1.

Such CRs can be found in RP-2102208, RP-2102209, RP-2102210, RP-2102211, RP-2102212 and RP-2102213.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: The RAN2#115e and RAN4#100e agreements are compatible with each other and only lack the addition of an NS-value to band n77.

Proposal 1: RAN to resolve the n77 extension in the US by approving the with company contributed CRs that add the capability signalling to LTE and NR along with NS-value to TS38.101-1.