3GPP TSG-RAN5 Meeting #94-e	R5-220075

# Electronic Meeting, 21st February– 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1485 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause title styles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are some incorrect clause title styles in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some clause title styles were corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some clause title styles will remain incorrect in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2.4, 6.5.4, 6.5E.2.2.1, 6.5F.2.2, 6.5F.3.1, 7.6A.2.2, 7.6A.2.3, 7.6C.2, 7.6C.3, 7.6D.2, 7.6D.3, 7.6D.4, 7.7, 7.7A.1, 7.7A.2, 7.7A.3, 7.7D, 7.8.2, 7.8A.2.1.5, 7.8A.2.2.5, 7.8A.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_07, NS_40, NS_09 not complete.

- PC1 requirements for NR operating bands other than Band n14 are not fully defined in RAN4 Rel-15 and Rel-16 specifications due to the lack of MPR requirements.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, and NS_35 to all types of NR UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply in test case 6.5.2.4 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and 3 UE release 15 and forward.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 |
| NS_06 | 6.5.2.3.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5, 10 |  |  |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n83 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.3.17-1 | Table 6.2.3.3.17-2 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.26-1 | Table 6.2.3.3.26-1 |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.27-1 | Table 6.2.3.3.27-1 |
| NS_50 | 6.5.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz,10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

6.2.3.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double-counting MPR.

Allowed maximum power reduction is defined as A-MPR=max(MPR, A-MPR'),

Note that A-MPR'=0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A2 relative to 23 dBm for power class 3, PC2 A4 relative to 26 dBm for power class 2, and PC1.5_A6 relative to 29 dBm for power class 1.5,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A1 relative to 23 dBm for power class 3, PC2 A3 relative to 26 dBm for power class 2, and PC1.5_A5 relative to 29 dBm for power class 1.5,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Parameters for region edges and frequency offsets

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | CP-OFDM |  | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) |  | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.3.2-2: A-MPR' values Access

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16-QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64-QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256-QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256-QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.2.

6.2.3.3.3	A-MPR for NS_10

Table 6.2.3.3.3-1: A-MPR for NS_10

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB (RBs) | 1 –20 |
|  | A (dB) | ≤ 36 |
| 20 | RBstart | 0 – 15 |
|  | LCRB (RBs) | 1 – 20 |
|  | A (dB) | ≤ 66 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. NOTE 5: The A-MPR for DFT-s-OFDM is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3.3.1-1 and Table 6.2.4-1 in TS 36.521-1 [21] and A value specified in Table 6.2.3.3.3-1. NOTE 6: The A-MPR for CP-OFDM is the total backoff and is obtained by adding the A value in Table 6.2.3.3.3-1 to the corresponding MPR specified in Table 6.2.2.3-1. |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.3.

6.2.3.3.4	A-MPR for NS_05 and NS_05U

Table 6.2.3.3.4-1: A-MPR regions for NS_05 and NS_05U

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | <1.62MHz/12/SCS | >2.52MHz/12/SCS | A3 |  |  |  |  |  |  |
| 10 | 1925 ≤ FC < 1935 | <1.62MHz/12/SCS | >0 | A1 | >1.62MHz/12/SCS ≤3.60MHz/12/SCS | >5.4MHz/12/SCS | A7 | ≥7.2MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 10 | 1935 ≤ FC < 1945 |  | >4.5MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | <3.24MHz/12/SCS | >0 | A1 | >3.24MHz/12/SCS ≤5.40MHz/12/SCS | >8.1MHz/12/SCS | A7 | ≥10.08MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15 | 1932.5 ≤ FC < 1942.5 | <1.62MHz/12/SCS | >0 | A1 |  |  |  | ≥12.24 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | >7.2MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | <4.86MHz/12/SCS | >0 | A1 | >4.86MHz/12/SCS ≤7.20MHz/12/SCS | >9.0MHz/12/SCS | A7 | ≥13.68 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 20 | 1950 ≤ FC <1960 |  | >9.0MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.4-2 and 6.2.3.3.4-3. NOTE 2: Void. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-2: A-MPR for NS_05 and NS_05U

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 10 | ≤ 5 | ≤ 4 |  |
|  | QPSK | ≤ 10 | ≤ 5 | ≤ 4.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 6 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 6 |  |
|  | 256 QAM | ≤ 13 | ≤ 5 | ≤ 7 |  |
| CP-OFDM | QPSK | ≤ 10 | ≤ 5 | ≤ 7.5 | ≤ 2 |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 8 |  |
|  | 256 QAM | ≤ 13 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.3.4-3: A-MPR for NS_05

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1 |  | ≤ 1 | N/A | ≤ 6 |
|  | QPSK |  |  | ≤ 1.5 |  | ≤ 1.5 |  | ≤ 6 |
|  | 16 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 64 QAM |  |  |  | ≤ 4 |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-4: A-MPR for modulation and waveform type for NS_05U

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 2 |  | ≤ 2 | N/A | ≤ 6 |
|  | QPSK | ≤ 2 |  | ≤ 2 |  | ≤ 2 |  | ≤ 6 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 6 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |  | ≤ 3 |  | ≤ 6 |
|  | 256 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 16 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 64 QAM | ≤ 4 |  | ≤ 4 | ≤ 4 | ≤ 4 |  | ≤ 6 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.4.

6.2.3.3.5	A-MPR for NS_40

Table 6.2.3.3.5-1: A-MPR for NS_40

| Modulation/Waveform |  | A (dB) |  |
| --- | --- | --- | --- |
|  |  | Channel bandwidth: 5 MHz |  |
|  |  | Outer | Inner |
| DFT-s-OFDM | QPSK | ≤ 15.5 | ≤ 12 |
|  | 16 QAM | ≤ 14.5 | ≤ 11 |
|  | 64 QAM | ≤ 14.5 | ≤ 10 |
|  | 256 QAM | ≤ 12.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 14.5 | ≤ 10 |
|  | 16 QAM | ≤ 14.5 | ≤ 10 |
|  | 64 QAM | ≤ 14 | ≤ 8 |
|  | 256 QAM | ≤ 11 | ≤ 5.5 |
| NOTE 1: The A-MPR for NS_40 is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3.3.1-1 and Table 6.2.4-30a in TS 36.101 and MPR+A specified in Table 6.2.2.3-1 and Table 6.2.3.3.5-1. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.5.

6.2.3.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.3.6-1: A-MPR regions for NS_43

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 15 | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 40 | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 45 | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | < 1.8 MHz /12/SCS > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.6-2. NOTE 2: 15 kHz SCS unless otherwise stated NOTE 3: Void |  |  |  |  |  |  |  |

Table 6.2.3.3.6-2: A-MPR for NS_43

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  |  |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer |  | Inner |  | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  | N/A |  | N/A |  |  |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  |  | ≤ 2.5 |  |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  |  | ≤ 4 |  |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  |  | ≤ 4 |  |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 6.2.3.3.6-3: Void

When NS_43U is signalled for 5 and 10 MHz channel bandwidths A-MPR is defined in Table 6.2.3.3.1-2 except for DFT-s-OFDM QPSK when LCRB > 5.4 MHz/12/SCS the A-MPR is 2.5 dB. For 15 MHz channel bandwidth Table 6.2.3.3.6-4 applies.

Table 6.2.3.3.6-4: A-MPR for NS_43U

| Modulation/Waveform |  | 15 MHz |
| --- | --- | --- |
|  |  | Outer / Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9 |
|  | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |
| CP-OFDM | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.6.

6.2.3.3.7	A-MPR for NS_03 and NS_03U

Table 6.2.3.3.7-1: A-MPR for NS_03

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
|  | 256 QAM | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.5 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |

In case UE operates in a band where NS_03U applies and it receives additionalSpectrumEmission value of 3 then A-MPR values specified in Table 6.2.3.3.7-1 apply with an exception that DFT-s-OFDM Pi/2 BPSK A-MPR is 2 dB.

6.2.3.3.8	A-MPR for NS_37

Table 6.2.3.3.8-1: A-MPR regions for B11/B21 protection (NS_37) (1447.9 - 1462.9 MHz)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |  | Region C (Outer/Inner) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 10 | 1452.9 < FC ≤1457.9 | ≥0 | > 7.2 MHz/12/SCS | ≤ A1 | N/A | N/A | N/A | N/A | N/A | N/A |
| 15 | FC =1455.4 | ≥0 | > 9.9 MHz/12/SCS | ≤ A1 | < 0.54 MHz/12/SCS | < 1.08 MHz/12/SCS | ≤ A2 | > 13.86 MHz/12/SCS | < 1.08 MHz/12/SCS | ≤ A2 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.8-2. NOTE 2: Void NOTE 3: Void NOTE 4: No A-MPR for SCS = 60 kHz for region B and C only. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.8-2: A-MPR for NS_37

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 3 |
|  | QPSK | ≤ 1.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 3 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3 |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.8.

6.2.3.3.9	A-MPR for NS_38

Table 6.2.3.3.9-1: A-MPR for EESS (NS_38) Protection (1430 – 1470 MHz)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A Outer/Inner |  |  | Region B Outer/Inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR (dB) | RBstart+LCRB | A-MPR (dB) |
| 10 | 1435 ≤ FC < 1442 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 12 | ≤ 2.16 MHz/12/SCS | ≤ 9 |
| 15 | 1437.5 ≤ FC < 1447.5 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 13 | ≤ 3.6 MHz/12/SCS | ≤ 10 |
| 20 | 1440 ≤ FC < 1450 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 13 | ≤ 5.4 MHz/12/SCS | ≤ 10 |
| NOTE 1 - 4: Void |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.9.

6.2.3.3.10	A-MPR for NS_39

Table 6.2.3.3.10-1: A-MPR for own RX (NS_39) Protection (1440 – 1470 MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A (Outer/Inner) |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBstart+LCRB | A-MPR (dB) |  |
| 10 | 1462 < FC ≤1465 | > 7.9 MHz/12/SCS | ≤ 6 |
| 15 | 1456.3 < FC ≤ 1462.5 | > 11.2 MHz/12/SCS | ≤ 6 |
| 20 | 1450.8 < FC ≤ 1460 | > 14.4 MHz/12/SCS | ≤ 6 |
| NOTE 1 - 4: Void |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.10.

6.2.3.3.11	A-MPR for NS_41

Table 6.2.3.3.11-1: A-MPR for NS_41

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR (dB) | RBstart+LCRB | A-MPR (dB) |  |
| 5 | - | - | - | - | - | - |
| 10 | 1437 ≤ FC < 1442 | ≤ -4.5 MHz/12/SCS + LCRB | > 4.5 MHz/12/SCS | ≤ 9 | < 1.8MHz/12/SCS | ≤ 9 |
| 15 | 1439.5 ≤ FC < 1447.5 | ≤ -5.4 MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 11 | < 3.42 MHz/12/SCS | ≤ 9 |
| 20 | 1442 ≤ FC < 1450 | ≤ -5.4 MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 12 | < 5.04 MHz/12/SCS | ≤ 9 |
| 40 | 1452 ≤ FC < 1497 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 11.7 MHz/12/SCS | ≤ 13.5 |
| 50 | 1457 ≤ FC < 1492 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 15.12 MHz/12/SCS | ≤ 13.5 |
| 60 | 1462 ≤ FC < 1487 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 18.72 MHz/12/SCS | ≤ 13.5 |
| NOTE 1 - 4: Void |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.11.

6.2.3.3.12	A-MPR for NS_42

Table 6.2.3.3.12-1: A-MPR for NS_42

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart+LCRB | A-MPR Outer/Inner (dB) | RBstart | RBstart+LCRB | A-MPR Inner (dB) | A-MPR Outer (dB) |
| 5 | 1512 ≤ FC ≤ 1514.5 | >3.1 MHz / 12 / SCS | ≤ 7 | <0.9 MHz / 12 / SCS | ≤ 3.1 MHz / 12 / SCS | ≤ 1.5 | ≤ 4 |
| 10 | 1497 ≤ FC ≤ 1512 | >6.2 MHz / 12 / SCS | ≤ 8 | <0.9 MHz / 12 / SCS | ≤ 6.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 15 | 1502 FC ≤ 1509.5 | >9.3 MHz / 12 / SCS | ≤ 8 | <3.06 MHz / 12 / SCS | ≤ 9.3 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 20 | 1497 ≤ FC ≤ 1507 | >12.4 MHz / 12 / SCS | ≤ 8 | <4.5 MHz / 12 / SCS | ≤ 12.4 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 40 | 1477 ≤ FC ≤ 1497 | >24.8 MHz / 12 / SCS | ≤ 8 | <5.4 MHz / 12 / SCS | ≤ 24.8 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 50 | 1467 ≤ FC ≤ 1492 | >31 MHz / 12 / SCS | ≤ 8 | <7.2 MHz / 12 / SCS | ≤ 31 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 60 | 1462 ≤ FC ≤ 1487 | >37.2 MHz / 12 / SCS | ≤ 8 | <7.2 MHz / 12 / SCS | ≤ 37.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| NOTE 1 - 5: Void |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.12.

6.2.3.3.13	A-MPR for NS_18

Table 6.2.3.3.13-0: Band n28 and n83 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |

Table 6.2.3.3.13-1: A-MPR for NS_18

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.13.

6.2.3.3.14	A-MPR for NS_21

Table 6.2.3.3.14-1: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 1.44MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS | Region A1b RBstart ≤ 1.44MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A2 LCRB > 5.4MHz/12/SCS | Region A3b RBend ≥ 7.74MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A3a RBend ≥ 7.74MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 10 | DFT-s-OFDM | PI/2 BPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | QPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | 16 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 64 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 256 QAM | 6 | 3 | 4 | 3 | 6 |
|  | CP-OFDM | QPSK | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 16 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 64 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 256 QAM | 6 | 4 | 5.5 | 4 | 6 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.14.

6.2.3.3.15	A-MPR for NS_24

Table 6.2.3.3.15-1: A-MPR for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rbend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | Rbend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | Rbend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | Fc=1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | Fc=1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | Fc=2002.5 |  | >2.16 | A1 | >3.78 | ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
| 10MHz | Fc=1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | Fc=1995 |  | >4.5 | A1 | >7.56 | ≤4.32 | A2 | ≤7.38 | ≤4.32 | A3 |
| 10MHz | Fc=2000 | >6.84 |  | A5 | <2.88 |  | A5 | ≥3.06 ≤6.66 | >1.44 | A6 |
| 15MHz | Fc=1987.5 |  | >7.02 | A1 | >11.52 | ≤6.84 | A2 | ≤11.34 | ≤6.84 | A3 |
| 15MHz | Fc=1997.5 | >9.36 |  | A5 | <3.6 |  | A5 | ≥3.78 ≤9.18 | >1.44 | A6 |
| 20MHz | Fc=1990 | >13.5 |  | A5 | <4.5 |  | A5 | ≥4.68 ≤13.32 | >2.16 | A6 |
| 20MHz | Fc=1995 | >12.6 |  | A5 | <5.4 |  | A5 | ≥5.58 ≤12.42 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.15-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 11 | ≤ 5 |  | ≤ 8.5 | ≤ 20 | ≤ 10 |  |
| CP-OFDM QPSK | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13 | ≤ 6.5 |  | ≤ 8.5 | ≤ 20 | ≤ 12 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2.3-1 NOTE 2: Outer and inner allocations are defined in clause 6.1 |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.15.

6.2.3.3.16	A-MPR for NS_27

Table 6.2.3.3.16-1: A-MPR for NS_27

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  | Region B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR | LCRB*12*SCS | A-MPR |
| 15 MHz | 3557.5 ≤ FC < 3562.5 | <1.8 MHz |  |  | A3 | ≥10.8 MHz | A3 |
|  | 3687.5 < FC ≤ 3692.5 | >11.52 MHz |  |  |  |  |  |
| 15 MHz | 3562.5 ≤ FC < 3567.5 | ≤1.08 MHz |  | <1.44 MHz | A4 | ≥11.52 MHz | 2 |
|  | 3682.5 < FC ≤ 3687.5 |  | ≥13.22 MHz |  |  |  |  |
| 20 MHz | 3560 ≤ FC < 3570 | <3.6 MHz |  |  | A5 | ≥10.8 MHz | A5 |
|  | 3680 < FC ≤ 3690 | >12.96 MHz |  |  |  |  |  |
| 20 MHz | 3570 ≤ FC < 3580 | ≤2.16 MHz |  | <1.44 MHz | A6 | ≥14.4 MHz | 2 |
|  | 3670 < FC ≤ 3680 |  | ≥16.92 |  |  |  |  |
| 40 MHz | 3570 ≤ FC < 3600 | <11.34 MHz |  |  | A7 |  |  |
|  |  | ≥11.34 MHz, ≤31.0 MHz |  | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  | >31.0 MHz |  | <1.8 MHz | A7 |  |  |
|  | 3650 < FC ≤ 3680 |  | >24.48 MHz |  | A7 |  |  |
|  |  |  | ≤24.48 MHz, ≥6.48 MHz | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  |  | <6.48 MHz | <1.8 MHz | A7 |  |  |
| 40 MHz | 3600 ≤ FC ≤ 3650 | ≤6.12 MHz |  | <1.44 MHz | A8 | >20 MHz | 4.5 |
|  |  |  | ≥ 32.76 |  |  |  |  |

Table 6.2.3.3.16-2: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | QPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | 16 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 64 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 256 QAM |  | 6 |  |  |  |  | 11 |  |
| CP-OFDM | QPSK | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 16 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 64 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 256 QAM |  | 7 |  |  |  |  | 11.5 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.16.

6.2.3.3.17	A-MPR for NS_46

Table 6.2.3.3.17-1: A-MPR regions for NS_46

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2534.5 ≤ FC < 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: > 9.72 MHz for DFT-s-OFDM, > 16.02 MHz for CP-OFDM. |  |  |  |  |

Table 6.2.3.3.17-2: A-MPR for NS_46

| Modulation/Waveform |  | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM | 6 |  |  |  | 7 | 11 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.17.

6.2.3.3.18	A-MPR for NS_47

Table 6.2.3.3.18-1: A-MPR regions for NS_47

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.3.18-2. |  |  |  |  |

Table 6.2.3.3.18-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1(dB) |  | A2(dB) |  | A3(dB) |  | A4(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 |  | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 |  | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  |  |  | ≤ 7 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.18.

6.2.3.3.19	A-MPR for NS_50

Table 6.2.3.3.19-1: A-MPR regions for NS_50 (Power Class 3)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.48 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 8.64 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-2. |  |  |  |

Table 6.2.3.3.19-2: A-MPR for NS_50 (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 2 |  |
|  | QPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2 |  |
|  | 16 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 |  | ≤ 5 |  |  |
|  | 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5 |  | ≤ 5 |  |  |
| CP-OFDM | QPSK | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 16 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 64 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 |  | ≤ 6.5 |  |  |
|  | 256 QAM | ≤ 12 | ≤ 8 |  |  |  | ≤ 6.5 |  |  |

Table 6.2.3.3.19-3: A-MPR regions for NS_50 (Power Class 2)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.3 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.12 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 5.04 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A1 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A1 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-4. |  |  |  |

Table 6.2.3.3.19-4: A-MPR for NS_50 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤4.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 |
|  | QPSK | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 |
|  | 16 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 |  | ≤ 4 |  |  |
|  | 256 QAM | ≤ 11.5 | ≤ 5.5 |  |  |  |  |  |
| CP-OFDM | QPSK | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 | ≤ 2 | ≤ 5 |
|  | 16 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 |
|  | 64 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 |
|  | 256 QAM | ≤ 12.5 | ≤ 7 |  |  |  |  |  |

6.2.3.3.20	A-MPR for NS_44

Table 6.2.3.3.20-1: A-MPR regions for NS_44

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2582.5≤ FC ≤ 2602.5 | <18.0 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥18.0 | <7.2 | A3 |
|  |  | ≥18.0 | ≥7.2 | A6 |
| 30 MHz | 2585 ≤ FC ≤ 2600 | <21.6 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥21.6 | <12.6 | A3 |
|  |  | ≥21.6 | ≥12.6 | A6 |
| 40 MHz | 2590 ≤ FC ≤ 2595 | ≥0, <2.88 | >0 | A1 |
|  |  | ≥2.88, <14.4 | >max (0, 12*SCS*RBend - 3.6) | A2 |
|  |  | ≥14.4, <23.4 | >10.8 | A3 |
|  |  | ≥23.4, <32.4 | >16.2 | A4 |
|  |  | ≥32.4 | >0 | A5 |

Table 6.2.3.3.20-2: A-MPR for NS_44

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | QPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 16 QAM | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 64 QAM | 5 |  | 3 | 7 | 12 | 4 |
|  | 256 QAM | 5 |  |  | 7 | 12 |  |
| CP-OFDM | QPSK | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 16 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 64 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 256 QAM |  |  |  | 8 | 12 |  |

6.2.3.3.21	A-MPR for NS_12

Table 6.2.3.3.21-1: A-MPR regions for NS_12

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 5MHz | ≤1.8 | >0 | A1 |
| 10MHz | ≤3.6 | >0 | A1 |

Table 6.2.3.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 |
| CP-OFDM QPSK | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 9.5 |

6.2.3.3.22	A-MPR for NS_13

Table 6.2.3.3.22-1: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |

Table 6.2.3.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |

6.2.3.3.23	A-MPR for NS_14

Table 6.2.3.3.23-1: A-MPR regions for NS_14

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | ≥0 | ≥9 | A2 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | ≥0 | ≥9 | A2 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | ≥0 | ≥9 | A2 |

Table 6.2.3.3.23-2: A-MPR for NS_14

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3 |  | ≤ 3 |
| DFT-s-OFDM 256 QAM |  |  | ≤ 8 |
| CP-OFDM QPSK | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 6 |  | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 |  | ≤ 8 |

6.2.3.3.24	A-MPR for NS_15

Table 6.2.3.3.24-1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |

Table 6.2.3.3.24-2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 |

6.2.3.3.25	A-MPR for NS_45

Table 6.2.3.3.25-1: A-MPR for NS_45

| Modulation/Waveform |  | Outer |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.5 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |

6.2.3.3.26	A-MPR for NS_48

Table 6.2.3.3.26-1: A-MPR regions for NS_48

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.<1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥13.5 | A3 |
|  |  | ≥21.6 | <1.08<1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A2 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 3.6) | A3 |
|  |  | ≥17.1, <27.36 | ≥13.5 | A4 |
|  |  | ≥27.36, <34.56 | ≥13.5 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <6.12 | >0 | A2 |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥20.7, <41.04 | ≥17.1 | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 6.2.3.3.26-2: A-MPR for NS_48

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | QPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 16 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 64 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 256 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
| CP-OFDM | QPSK | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 16 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 64 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 256 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |

6.2.3.3.27	A-MPR for NS_49

Table 6.2.3.3.27-1: A-MPR regions for NS_49

| Channel Bandwidth (MHz) | Carrier Center Frequency, Fc (MHz) | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤3.96 | <1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <6.48 | ≥0 | A5 |
|  |  | ≥6.48, <14.4 | ≥max (0,12*SCS* RBend - 3.6) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.8 | A4 |
|  |  | ≥21.6 | ≥10.8 | A2 |
|  |  | ≥21.6 | <1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <7.2 | ≥0 | A1 |
|  |  | ≥7.2, <10.44 | <1.08 | A5 |
|  |  | ≥7.2, <18 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥18, <34.56 | ≥14.4, <28.8 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥28.8 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥7.74, <14.4 | < min (1.08, max(0,12*SCS* RBend-7.74)) | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥18, <max (0, 12*SCS*RBend – 7.74) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 7.74) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6.2.3.3.27-2: A-MPR for NS_49

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | QPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 16 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 64 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 256 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
| CP-OFDM | QPSK | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 16 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 64 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 256 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |

6.2.3.3.28	A-MPR for NS_51

Table 6.2.3.3.28: void

6.2.3.3.29	A-MPR for NS_07

FFS

6.2.3.3.30	A-MPR for NS_56

For 5 MHz channel centred on frequencies (FC) = 1630.0, 1630.3 MHz, A-MPR is defined as

if RBstart <= ceil{3/SCS/15 kHz)}and LCRB <= ceil{17/SCS/15 kHz)},

then

the A-MPR = 14 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart <= ceil{3/(SCS/15 kHz)} and LCRB > ceil{17/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 4 dB.

For 5 MHz channel centred on frequencies (Fc) = 1635.0, 1649.0, 1654.0 MHz, no A-MPR is needed.

For Channel 10 MHz with centre frequency of 1632.5 MHz, A-MPR is defined as

if RBstart < ceil{3/(SCS/15 kHz)} and LCRB <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 12 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart < ceil{9/(SCS/15 kHz)}, and LCRB > ceil{8/(SCS/15 kHz)},

then

the A-MPR = 8 dB,

else,

if RBstart <= ceil{18/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)}], and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 5 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 3 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 4 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 2 dB.

For 10 MHz channel centered on frequency of 1651.5 MHz, no A-MPR is needed.

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

<< Unchanged sections omitted >>

6.2.4	Configured transmitted power

6.2.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c is within the specified bounds.

6.2.4.2	Test applicability

This test case applies to all types of NR Power Class 2 and 3 UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

6.2.4.3	Minimum conformance requirements

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1.3-1 without taking into account the tolerance specified in the Table 6.2.1.3-1;

When the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 capable UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 capable UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass = 3 dB for a power class 2 capable UE or 6 dB for a power class 1.5 UE, when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); 3 dB for a power class 1.5 capable UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); otherwise ΔPPowerClass = 0 dB;

∆TIB,c is the additional tolerance for serving cell c as specified in 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC ; = 0 dB otherwise; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.0.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4] for the applicable operating bands.

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise ∆TC,c = 0dB;

MPRc and A-MPRc for serving cell c are specified in subclause 6.2.2.3 and subclause 6.2.3.3, respectively; ∆MPRc for serving cell c is specified in clause 6.2.2.

∆TRxSRS is applied when UE transmits SRS to other than first SRS port when the SRS-TxSwitch capability is indicated as '1T2R', '1T4R' or, '1T4R/2T4R' with UE configured with 4 SRS resources in the SRS resource set, and when UE transmits SRS to other than first or second SRS port when the SRS-TxSwitch capability is indicated as '2T4R' or '1T4R/2T4R' with the UE configured with 2 SRS resources in the SRS resource set. The value of ∆TRxSRS is 4.5dB for n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self defence requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4.3-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4.3-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c – MAX{TL,c, T(PCMAX_L,f,c)} ≤ PUMAX,f,c ≤ PCMAX_H,f,c + T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4.3-2. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1.3-1.

Table 6.2.4.3-2: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.4.

6.2.4.4	Test description

6.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.4.4.3.

6.2.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE reaches the Pumax level of the test point.

3.	Measure the mean power of the UE in the channel bandwidth for each test point in table 6.2.4.5-1 according to the test configuration from table 6.2.4.4.1-1. The period of measurement shall be at least the continuous duration of one active slot and in the uplink symbols. For TDD slots with transient periods are not under test.

6.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.2.4.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.4.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2.4.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2.4.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

Table 6.2.4.4.3-4: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2.4.4.3-5: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | Test ID 1, 2 |
|  | 1 |  | Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2.4.5-1.

Table 6.2.4.5-1: PCMAX configured UE output power

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) |
| Measured UE output power test point 4 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: Power class 3 is default power class unless otherwise stated. Note 3: The maximum output power shall be within the range in Table 6.2.4.5-1a. Note 4: The maximum output power shall be within the range in Table 6.2.4.5-1b. |  |  |

Table 6.2.4.5-1a: Measured UE output power test point 4 for Test ID 1,2

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 20 dBm ±(2.5+TT) |
| n2 | 20 dBm ±(2.51+TT) |
| n3 | 20 dBm ±(2.51+TT) |
| n5 | 20 dBm ±2.5±TT |
| n7 | 20 dBm ±(2.51+TT) |
| n8 | 20 dBm ±(2.51+TT) |
| n12 | 20 dBm ±(2.51+TT) |
| n14 | 20 dBm ±(2.5+TT) |
| n20 | 20 dBm ±(2.51+TT) |
| n24 | 20 dBm + 2.5+TT/-31-TT |
| n25 | 20 dBm ±(2.5+TT) |
| n26 | 20 dBm ±(2.51+TT) |
| n28 | 20 dBm ±(2.5+TT) |
| n30 | 20 dBm ±(2.5+TT) |
| n34 | 20 dBm ±(2.5+TT) |
| n38 | 20 dBm ±(2.5+TT) |
| n39 | 20 dBm ±(2.5+TT) |
| n40 | 20 dBm ±(2.5+TT) |
| n41 | 20 dBm ±(2.51+TT) |
| n50 | 20 dBm ±(2.5+TT) |
| n51 | 20 dBm ±(2.5+TT) |
| n65 | 20 dBm ±(2.5+TT) |
| n66 | 20 dBm ±(2.5+TT) |
| n70 | 20 dBm ±(2.5+TT) |
| n71 | 20 dBm ±(2.5+TT) |
| n74 | 20 dBm ±(2.5+TT) |
| n77 | 20 dBm + 2.5+TT/-3-TT |
| n78 | 20 dBm + 2.5+TT/-3-TT |
| n79 | 20 dBm + 2.5+TT/-3-TT |
| NOTE 1: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.0 dB. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-1b: Measured UE output power test point 4 for Test ID 3

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 23 dBm ±(2+TT) |
| n2 | 23 dBm ±(21+TT) |
| n3 | 23 dBm ±(21+TT) |
| n5 | 23 dBm ±(2+TT) |
| n7 | 23 dBm ±(21+TT) |
| n8 | 23 dBm ±(21+TT) |
| n12 | 23 dBm ±(21+TT) |
| n20 | 23 dBm ±(21+TT) |
| n24 | 23 dBm + 2.0+TT/-31-TT |
| n25 | 23 dBm ±(2+TT) |
| n26 | 23 dBm ±(21+TT) |
| n28 | 23 dBm +2+TT/-2.5-TT |
| n34 | 23 dBm ±(2+TT) |
| n38 | 23 dBm ±(2+TT) |
| n39 | 23 dBm ±(2+TT) |
| n40 | 23 dBm ±(2+TT) |
| n41 | 23 dBm ±(21+TT) |
| n50 | 23 dBm ±(2+TT) |
| n51 | 23 dBm ±(2+TT) |
| n65 | 23 dBm ±(2+TT) |
| n66 | 23 dBm ±(2+TT) |
| n70 | 23 dBm ±(2+TT) |
| n71 | 23 dBm +2+TT/-2.5-TT |
| n74 | 23 dBm ±(2+TT) |
| n77 | 23 dBm +2+TT/-3-TT |
| n78 | 23 dBm +2+TT/-3-TT |
| n79 | 23 dBm +2+TT/-3-TT |
| NOTE 1: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-2: Test Tolerance (Configured transmitted power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

<< Unchanged sections omitted >>

6.5.4	Transmit intermodulation

6.5.4.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.4.3	Minimum conformance requirements

UE transmit intermodulation is defined by the ratio of the mean power of the wanted signal to the mean power of the intermodulation product when an interfering CW signal is added at a level below the wanted signal at each transmitter antenna port with the other antenna port(s) if any terminated. Both the wanted signal power and the intermodulation product power are measured through NR rectangular filter with measurement bandwidth shown in Table 6.5.4.3-1.

The requirement of transmit intermodulation is specified in Table 6.5.4.3-1.

Table 6.5.4.3-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel centre | BWChannel | 2*BWChannel |
| Interference CW signal level | -40dBc |  |
| Intermodulation product | < -29dBc | < -35dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCSs for the channel BW as defined in Table 6.5.2.4.1.3-1 |  |
| Measurement offset from channel centre | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.4.

6.5.4.4	Test description

6.5.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 6.5.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for transmit intermodulation test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.3.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5.4.4.3.

6.5.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 6.5.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE until the UE transmits at its PUMAX level.

3.	Measure the rectangular filtered mean power of the UE. For TDD, only slots consisting of only UL symbols are under test for the wanted signal and for the intermodulation product.

4.	Set the interference signal frequency below the UL carrier frequency using the first offset in table 6.5.4.5-1.

5.	Set the interference CW signal level according to table 6.5.4.5-1.

6.	Search the intermodulation product signals below and above the UL carrier frequency, then measure the rectangular filtered mean power of transmitting intermodulation for both signals, and calculate the ratios with the power measured in step 3.

7.	Set the interference signal frequency above the UL carrier frequency using the first offset in table 6.5.4.5-1.

8.	Search the intermodulation product signals below and above the UL carrier frequency, then measure the rectangular filtered mean power of transmitting intermodulation for both signals, and calculate the ratios with the power measured in step 3.

9.	Repeat the measurement using the second offset in table 6.5.4.5-1.

6.5.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exception:

Table 6.5.4.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.5.4.5	Test requirement

The ratio derived in step 6 and 8, shall not exceed the described value in table 6.5.4.5-1.

Table 6.5.4.5-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel centre | BWChannel | 2*BWChannel |
| Interference CW signal level | -40dBc |  |
| Intermodulation product | < -29dBc | < -35dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCSs for the channel BW as defined in Table 6.5.2.4.1.5-1 |  |
| Measurement offset from channel centre | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |
| Note 1: The test requirements do not apply when the interfering signal overlaps with the channel bandwidth of the downlink signal. |  |  |

<< Unchanged sections omitted >>

6.5E.2.2	Spectrum emission mask for V2X

6.5E.2.2.1	Spectrum emission mask for V2X / non-concurrent operation

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Connection diagram is TBD

- Preconfiguration is TBD in 38.508-1

- Test state and generic procedure are TBD in 38.508-1

6.5E.2.2.1.1	Test purpose

<< Unchanged sections omitted >>

6.5F.2.2	Spectrum emission mask for operation with shared spectrum channel access

Editor’s Note:

- Test config table is FFS

- Message content is TBD

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in subclause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5F.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is operating with shared spectrum channel access.

6.5F.2.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR standalone shared spectrum channel access.

6.5F.2.2.3	Minimum conformance requirements

Instead of the general spectrum emission mask requirement in clause 6.5.2.2, when operating with shared spectrum channel access the relative power of any UE emission shall not exceed the levels specified in Table 6.5F.2.2.3-1 for the specified channel bandwidth or -30 dBm/MHz whichever is the greatest. The spectrum emission mask for operation with shared spectrum channel access is defined relative to the maximum power density in a 1 MHz measurement bandwidth within the channel bandwidth.

The spectrum emission mask for operation with shared spectrum channel access applies to frequencies (ΔfOOB) starting from the  edge of the assigned channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

Table 6.5F.2.2.3-1: Spectrum emission mask for operation with shared spectrum channel access

| Spectrum emission limit (dBr) / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz |  | 20 MHz |  | 40 MHz |  | 60 MHz | 80 MHz | Measurement bandwidth (MBW) |
| ± 0-1 | -20 a??fOOB |  |  |  |  |  |  |  | [100kHz]3 |
| ± 1-5 | NOTE 1 | NOTE 1 |  | NOTE 1 |  | NOTE 1 |  | NOTE 1 | 1 MHz |
| ± 5-10 | NOTE 2 |  |  |  |  |  |  |  |  |
| ± 10-20 | -40 | NOTE 2 |  |  |  |  |  |  |  |
| ± 20-30 |  | -40 |  | NOTE 2 |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  | NOTE 2 |  |  |  |
| ± 40-50 |  |  |  | -40 |  |  |  | NOTE 2 |  |
| ± 50-60 |  |  |  |  |  |  |  |  |  |
| ± 60-70 |  |  |  |  |  | -40 |  |  |  |
| ± 70-80 |  |  |  |  |  |  |  |  |  |
| ± 80-100 |  |  |  |  |  |  |  | -40 |  |
| NOTE 1: Given as: -20-8A a??fOOB-1 where A= Channel Bandwidth2-1 NOTE 2: Given as: -16-12B a??fOOB where B= Channel Bandwidth2 NOTE 3: The measured value shall be scaled by a factor equal to the ratio of the reference bandwidth (1 MHz) to the measurement bandwidth before the emission limit (dBr) is applied. NOTE 4: The carrier leakage exceptions from Table 6.4F.2.3-1 apply and carrier leakage contribution shall be removed prior to setting the 0dBr level of the mask, the reported carrier frequency location in txDirectCurrentLocation field of the UplinkTxDirectCurrentBWP can be used to cancel the carrier leakage contribution. If txDirectCurrentLocation is not available or is reported with value 3300 or 3301, a carrier frequency location at the centre of the channel shall be assumed. |  |  |  |  |  |  |  |  |  |

For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2.

6.5F.2.2.3.1	Spectrum emission mask for non-transmitted channels

In the case of non-transmitted 20 MHz channel(s) on the edges of an assigned channel bandwidth the spectrum emission mask for operation with shared spectrum channel access, specified in Table 6.5F.2.2.3-1, is applied by using the total bandwidth of the remaining transmitted channels. The spectrum emission mask for non-transmitted channels is floored at -28dBr.

The relative power of any UE emission shall not exceed the most stringent levels given by the spectrum emission mask for operation with shared spectrum channel access with full channel bandwidth and the spectrum emission mask for non-transmitted channels with the channel bandwidth of the transmitted channels in the case of non-transmitted channels at the edge of an assigned channel bandwidth.

An exception to the spectrum emission mask for non-transmitted channels allows a single [2] MHz bandwidth to extend to [-28] dBc relative to total transmit power, or [-20] dBm, whichever is the greatest.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5F.2.2.

6.5F.2.2.4	Test description

6.5F.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands with shared spectrum channel access specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5F.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5F.2.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | FFS |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | FFS |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | FFS |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration |  | Uplink Configuration |  |
|  |  | Default | Default | N/A for Spectrum Emission Mask test case |  | Modulation | RB allocation |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5F.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5F.2.2.4.3

6.5F.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5F.2.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2F.1.5-1. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5F.2.2.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.5F.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5F.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

FFS

6.5F.2.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2F.1.5-1, and the power of any UE emission shall fulfil requirements in Table 6.5F.2.2.5-1.

Table 6.5F.2.2.5-1: General NR spectrum emission mask

FFS

Table 6.5F.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | 6.0GHz < f ≤ 7.125GHz |
| --- | --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB | TBD |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<< Unchanged sections omitted >>

6.5F.3.1	General spurious emissions

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined

- Test configuration table is FFS

- TP analysis is TBD

- Test state and generic procedure are TBD in 38.508-1

6.5F.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5F.3.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR standalone shared spectrum channel access.

6.5F.3.1.3	Minimum conformance requirements

The requirements for general spurious emission requirements in clause 6.5.3.1 apply.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5F.3.1.

6.5F.3.1.4	Test description

6.5F.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5F.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5F.3.1.4.1-1: Test Configuration Table

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5F.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5F.3.1.4.3.

6.5F.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 6.5F.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5F.3.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.5F.3.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5F.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.5F.3.1.5	Test requirement

This clause specifies the requirements for the specified NR standalone shared spectrum channel access for Transmitter Spurious emissions requirement with frequency range as indicated in table 6.5F.3.1.5-1.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5F.3.1.5-1 apply for all transmitter band configurations (NRB) and channel bandwidths for shared spectrum channel access.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5F.3.1.5-1.

Table 6.5F.3.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz |  |  |  |

<< Unchanged sections omitted >>

7.6A.2.2	In-band Blocking for CA (3DL CA)

7.6A.2.2.1	Test purpose

Same test purpose as in clause 7.6A.2.1.

7.6A.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support 3DL CA.

7.6A.2.2.3	Minimum conformance requirements

Minimum requirements are defined in clause 7.6A.2.0.

7.6A.2.2.4	Test description

7.6A.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.6A.2.2.4.1-1, 7.6A.2.2.4.1-2 or 7.6A.2.2.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.2.2.4.1-1: Test Configuration Table for 3DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Intra-band contiguous: Mid range for all CCs Inter-band: NOTE 1, NOTE 5 Intra-band contiguous + Inter-band: NOTE 1, NOTE 5 Intra-band non-contiguous + Inter-band: NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.2.4.1-1 (NOTE 5) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest NRB_agg, NOTE 1 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |
| Network signalling value |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous CA) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA，CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.2.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for in-band blocking testing. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8 NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |  |

Table 7.6A.2.2.4.1-2: Void

Table 7.6A.2.2.4.1-3: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.7 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.6A.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.6A.2.2.4.3.

7.6A.2.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.6A.2.2.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC according to Tables 7.6A.2.2.4.1-1, 7.6A.2.2.4.1-2 or 7.6A.2.2.4.1-3 on both SCC and PCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Tables 7.6A.2.2.4.1-1, 7.6A.2.2.4.1-2 or 7.6A.2.2.4.1-3 on PCC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the parameters of the signal generator for an interfering signal below the aggregated component carriers in Case 1 according to Table 7.6A.2.2.4.2-1.

7	Set the downlink signal level according to the Table 7.6A.2.2.4.2-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in table 7.6A.2.2.4.2-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

8	Measure the average throughput for the carrier(s) indicated in Table 7.6A.2.2.4.2-1 for duration sufficient to achieve statistical significance according to Annex H.2A.

9	Repeat steps from 6 to 8, using an interfering signal above the measured carrier(s) according to Table 7.6A.2.2.4.2-1 in Case 1 at step 6.

10	Repeat steps from 6 to 9, using interfering signals in Case 2 at step 6 and 9. The ranges of case 2 are covered in steps equal to the interferer bandwidth.

11.	Repeat steps 1 to 10 for all component carriers listed in Table 7.6A.2.2.4.2-1.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

Table 7.6A.2.2.4.2-1: Test repetition and measurement configuration

| CA configuration | CA configuration ID in REFSENS | Throughput measured on | Table with test parameters to select |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.6A.2.2.5-3 7.6A.2.2.5-3a 7.6A.2.2.5-4 7.6A.2.2.5-4a |
| Inter-band | 11 | SCC1, SCC2 | 7.6A.2.2.5-1 7.6A.2.2.5-1a 7.6A.2.2.5-1b 7.6A.2.2.5-2 7.6A.2.2.5-2a |
|  | 21 |  |  |
|  | 31 |  |  |
| Intra-band contiguous + Inter-band | 12 | SCC2 | 7.6A.2.2.5-1 7.6A.2.2.5-1a 7.6A.2.2.5-1b 7.6A.2.2.5-2 7.6A.2.2.5-2a |
|  | 22 | SCC1, SCC2 | 7.6A.2.2.5-3 7.6A.2.2.5-3a 7.6A.2.2.5-4 7.6A.2.2.5-4a |
| Intra-band non-contiguous + Inter-band | 23 | SCC2 | 7.6A.2.2.5-1 7.6A.2.2.5-1a 7.6A.2.2.5-1b 7.6A.2.2.5-2 7.6A.2.2.5-2a |
|  | 33 | SCC1, SCC2 | 7.6A.2.2.5-1 7.6A.2.2.5-1a 7.6A.2.2.5-2 7.6A.2.2.5-2a |
| NOTE 1: CA configuration ID as defined in “Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band)” in table 7.3A.2.4.1-1. NOTE 2: CA configuration ID as defined in “Default Test Settings for a CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band)” in table 7.3A.2.4.1-1. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band)” in table 7.3A.2.4.1-3. |  |  |  |

7.6A.2.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.6A.2.2.5	Test requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables below, according to the type of CA. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.6A.2.2.5-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. |  |  |  |  |  |  |

Table 7.6A.2.2.5-1a: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band, intra-band non-contiguous)

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |  | -CBW/2-11 |
| n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n25, n26, n28,n34, n38,n39, n40, n41, n483, n50, n51, n53, n65, n66, n70, n74, n75, n76 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. |  |  |  |  |  |  |

7.6A.2.2.5-1b: In-band blocking for additional NR operating bands for carrier aggregation with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
|  | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
| n29 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal NOTE 4: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.2.5-2: In-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| FIoffset, case 1 | MHz | 15 | 22.5 | 30 | 37.5 | 45 |
| FIoffset, case 2 | MHz | 25 | 37.5 | 50 | 62.5 | 75 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| FIoffset, case 1 | MHz | 60 | 75 | 90 | 105 | 120 |
| FIoffset, case 2 | MHz | 100 | 125 | 150 | 175 | 200 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| FIoffset, case 1 | MHz | 135 | 150 |  |  |  |
| FIoffset, case 2 | MHz | 225 | 250 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |

Table 7.6A.2.2.5-2a: In-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (inter-band, intra-band non-contiguous)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and BW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3CBW to FDL_high + 3CBW |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.2.5-3: In-band blocking parameters with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (intra-band contiguous CA)

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dB | REFSENS + CA bandwidth class specific value below |  |  |  |
|  |  | 10.0 | 6 | 13.8 |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FIoffset, case 1 | MHz | 30 | BWchannel CA+ BWchannel CA/2 | 75 |  |
| FIoffset, case 2 | MHz | 50 | BWInterferer + FIoffset, case 1 | 125 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattrn OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |

Table 7.6A.2.2.5-3a: In-band blocking parameters with FDL_low < 2700 MHz and FUL_low < 2700 MHz (intra-band contiguous CA)

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below |  |
|  |  | 16.0 | 19.0 |
| BWInterferer | MHz | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |

Table7.6A.2.2.5-4: In-band blocking with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (intra-band contiguous CA)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 3BWchannel CA to FDL_high + 3BWchannel CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.2.5-4a: In-band blocking with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz (intra-band contiguous CA)

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n41, n66, n484,n40 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.3-1, Pinterferer power defined in Table 7.6A.2.2.5-1a and Table 7.6A.2.2.5-2a is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.3-1.

7.6A.2.3	In-band Blocking for CA (4DL CA)

7.6A.2.3.1	Test purpose

Same test purpose as in clause 7.6A.2.1.

7.6A.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support 4DL CA.

7.6A.2.3.3	Minimum conformance requirements

Minimum requirements are defined in clause 7.6A.2.0.

7.6A.2.3.4	Test description

7.6A.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.6A.2.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.2.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest NRB_agg, NOTE 1 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (Intra-band contiguous CA) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA-ZA and CA_nXB-nYA-ZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA-ZA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nY(2A) Configurations (Intra-band non-contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for in-band blocking testing. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8 NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.7 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.6A.2.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.6A.2.2.4.3.

7.6A.2.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.6A.2.3.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC according to Table 7.6A.2.3.4.1-1 on both SCC and PCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 7.6A.2.3.4.1-1 on PCC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the parameters of the signal generator for an interfering signal below the aggregated component carriers in Case 1 according to Table 7.6A.2.3.4.2-1.

7	Set the downlink signal level according to the Table 7.6A.2.3.4.2-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in table 7.6A.2.2.4.2-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

8	Measure the average throughput for the carrier(s) indicated in Table 7.6A.2.3.4.2-1 for duration sufficient to achieve statistical significance according to Annex H.2A.

9	Repeat steps from 6 to 8, using an interfering signal above the measured carrier(s) according to Table 7.6A.2.3.4.2-1 in Case 1 at step 6.

10	Repeat steps from 6 to 9, using interfering signals in Case 2 at step 6 and 9. The ranges of case 2 are covered in steps equal to the interferer bandwidth.

11.	Repeat steps 1 to 10 for all component carriers listed in Table 7.6A.2.3.4.2-1.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

Table 7.6A.2.3.4.2-1: Test repetition and measurement configuration

| CA configuration | CA configuration ID in REFSENS | Throughput measured on | Table with test parameters to select |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2, SCC3 | 7.6A.2.3.5-3 7.6A.2.3.5-3a 7.6A.2.3.5-4 7.6A.2.3.5-4a |
| Inter-band | 11 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-1b 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 21 |  |  |
|  | 31 |  |  |
| Intra-band contiguous + Inter-band | 12 | SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-1b 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 22 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-3 7.6A.2.3.5-3a 7.6A.2.3.5-4 7.6A.2.3.5-4a |
|  | 32 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-3 7.6A.2.3.5-3a 7.6A.2.3.5-4 7.6A.2.3.5-4a |
| Intra-band non-contiguous + Inter-band | 13 | SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-1b 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 23 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-1b 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 33 | SCC1, SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-2 7.6A.2.3.5-2a |
| Intra-band non-contiguous + Intra-band non-contiguous | 14 | SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-2 7.6A.2.3.5-2a |
|  | 24 | SCC2, SCC3 | 7.6A.2.3.5-1 7.6A.2.3.5-1a 7.6A.2.3.5-2 7.6A.2.3.5-2a |
| NOTE 1: CA configuration ID as defined in “Default Test Settings for a CA_nXA-nYA-nZA-VA Configuration (Inter-band)” in table 7.3A.3.4.1-1. NOTE 2: CA configuration ID as defined in “Default Test Settings for a CA_XC-YA-ZA and CA_XB-YA-ZA Configurations (Intra-band contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 4: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intra-band non-contiguous)” in table 7.3A.2.4.1-1. |  |  |  |

7.6A.2.3.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.6A.2.3.5	Test requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables below, according to the type of CA.

Table 7.6A.2.3.5-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. |  |  |  |  |  |  |

Table 7.6A.2.3.5-1a: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band, intra-band non-contiguous)

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |  | -CBW/2-11 |
| n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n25, n26, n28,n34, n38,n39, n40, n41, n483, n50, n51, n53, n65, n66, n70, n74, n75, n76 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. |  |  |  |  |  |  |

7.6A.2.3.5-1b: In-band blocking for additional NR operating bands for carrier aggregation with FDL_high < 2700 MHz and FUL_high < 2700 MHz (inter-band)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
|  | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
| n29 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal NOTE 4: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.3.5-2: In-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (inter-band, intra-band non-contiguous)

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| FIoffset, case 1 | MHz | 15 | 22.5 | 30 | 37.5 | 45 |
| FIoffset, case 2 | MHz | 25 | 37.5 | 50 | 62.5 | 75 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| FIoffset, case 1 | MHz | 60 | 75 | 90 | 105 | 120 |
| FIoffset, case 2 | MHz | 100 | 125 | 150 | 175 | 200 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| FIoffset, case 1 | MHz | 135 | 150 |  |  |  |
| FIoffset, case 2 | MHz | 225 | 250 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |

Table 7.6A.2.3.5-2a: In-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (inter-band, intra-band non-contiguous)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and BW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3CBW to FDL_high + 3CBW |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.3.5-3: In-band blocking parameters with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (intra-band contiguous CA)

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dB | REFSENS + CA bandwidth class specific value below |  |  |  |
|  |  | 10.0 | 6 | 13.8 |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FIoffset, case 1 | MHz | 30 | BWchannel CA+ BWchannel CA/2 | 75 |  |
| FIoffset, case 2 | MHz | 50 | BWInterferer + FIoffset, case 1 | 125 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattrn OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |

Table 7.6A.2.3.5-3a: In-band blocking parameters with FDL_low < 2700 MHz and FUL_low < 2700 MHz (intra-band contiguous CA)

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below |  |
|  |  | 16.0 | 19.0 |
| BWInterferer | MHz | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |

Table7.6A.2.3.5-4: In-band blocking with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (intra-band contiguous CA)

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 3BWchannel CA to FDL_high + 3BWchannel CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.3.5-4a: In-band blocking with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz (intra-band contiguous CA)

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n41, n66, n484,n40 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.3-1, Pinterferer power defined in Table 7.6A.2.3.5-1a and Table 7.6A.2.3.5-2a is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.3-1.

<< Unchanged sections omitted >>

7.6C.2	In-band blocking for SUL

7.6C.2.1	Test purpose

Same test purpose as in clause 7.6.2.1.

7.6C.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

7.6C.2.3	Minimum conformance requirements

For SUL operation, the in-band blocking requirement for downlink bands specified in clause 7.6.2.3 shall be met.

The normative reference for this requirement is TS 38.101-1 [2] clauses 7.6C.2.

7.6C.2.4	Test description

Same test description as specified in clause 7.6.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1

Instead of table 7.6.2.4.1-1  use Table 7.6C.2.4-1.

Table 7.6C.2.4-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Mid, Highest for Non-SUL carrier For SUL band: n80: 30 MHz n81: 20 MHz n82: 20 MHz n83: 20 MHz n84: 20 MHz n86: 40 MHz n95: 15 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and lowest SCS for Non-SUL carrier |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | DL Configuration |  | UL Configuration |  | SUL Configuration |  |
|  | Mod'n | RB allocation | N/A |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) |  |  | DFT-s-OFDM QPSK | REFSENS (NOTE 1) |
| NOTE 1: The specific configuration of SUL and DL are defined in Table 7.3C.2.4.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.8 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.1-28 with condition SUL AND (RF OR RRM), 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, additionally the following exception shown in Table 7.6C.2.4-2 is considered.

Table 7.6C.2.4-2: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 7.6C.2.4-3: Void

7.6C.2.5	Test requirement

Same test requirement specified in clause 7.6.2.5 for downlink bands shall be met for in-band blocking testing for SUL.

<< Unchanged sections omitted >>

7.6C.3	Out-of-band blocking for SUL

7.6C.3.1	Test Purpose

Same test purpose as in clause 7.6.3.1.

7.6C.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

7.6C.3.3	Minimum conformance requirements

For SUL operation, the out-of-band blocking requirement for downlink bands specified in clause 7.6.3 shall be met. For operation band combination listed in Table 7.6C.3.3-1, exceptions to the requirement specified in Table 7.6C.3.3-2 are allowed when the second order intermodulation product of the SUL carrier and the CW interfering signal fully or partially overlaps with the DL carrier.

Table 7.6C.3.3-1: SUL operating band combination with exceptions allowed

NR Band combination for SUL

SUL_n78-n81

SUL_n78-n82

SUL_n78-n83

SUL_n79-n81

SUL_n79-n83

Table 7.6C.3.3-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when, where and are the channel bandwidths configured for SUL and DL (victim) bands in MHz, respectively. |  |  |

For all interferer frequency ranges specified in clause 7.6.3 a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

The normative reference for this requirement is TS 38.101-1 [2] clauses 7.6C.3.

7.6C.3.4	Test description

Same test description as specified in clause 7.6.3.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1

Instead of table 7.6.3.4.1-1  use Table 7.6C.3.4-1.

Table 7.6C.3.4-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for SUL carrier One frequency chosen arbitrarily from low or high range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Mid, Highest for Non-SUL carrier For SUL band: n80: 30 MHz n81: 20 MHz n82: 20 MHz n83: 20 MHz n84: 20 MHz n86: 40 MHz n95: 15 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and lowest for Non-SUL carrier |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | DL Configuration |  | UL Configuration |  | SUL Configuration |  |
|  | Mod'n | RB allocation | N/A |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) |  |  | DFT-s-OFDM QPSK | REFSENS (NOTE 1) |
| NOTE 1: The specific configuration of SUL and DL are defined in Table 7.3C.2.4.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.4.9 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.1-28 with condition SUL AND (RF OR RRM), 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, additionally the following exception shown in Table 7.6C.3.4-2 is considered.

Table 7.6C.3.4-2: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 7.6C.3.4-3: Void

7.6C.3.5	Test Requirement

For SUL operation, the out-of-band blocking requirement for downlink bands specified in clause 7.6.3.5 shall be met. For operation band combination listed in Table 7.6C.3.5-1, exceptions to the requirement specified in Table 7.6C.3.5-2 are allowed when the second order intermodulation product of the SUL carrier and the CW interfering signal fully or partially overlaps with the DL carrier.

Table 7.6C.3.5-1: SUL operating band combination with exceptions allowed

NR Band combination for SUL

SUL_n78-n81

SUL_n78-n82

SUL_n78-n83

SUL_n79-n81

SUL_n79-n83

Table 7.6C.3.5-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when, where and are the channel bandwidths configured for SUL and DL (victim) bands in MHz, respectively. |  |  |

For all interferer frequency ranges, a maximum of

exceptions are allowed for the spurious response frequencies recorded in the final step of test procedure in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

<< Unchanged sections omitted >>

7.6D.2	In-band blocking for UL MIMO

7.6D.2.1	Test purpose

In-band blocking for UL MIMO is defined for an unwanted interfering signal falling into the range from 15MHz below to 15MHz above the receive band of an UE that support UL MIMO, with FDL_high < 2700 MHz and FUL_high < 2700 MHz, or into the range from 3CBW below to 3CBW above the receive band of an UE that support UL MIMO, with FDL_high < 3300 MHz and FUL_high < 3300 MHz, at which the relative throughput shall meet or exceed the requirement for the specified measurement channels.

The lack of in-band blocking ability will decrease the coverage area when other g-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6D.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support UL MIMO on FDD bands.

7.6D.2.3	Minimum conformance requirements

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in subclause 7.6 shall be met with the UL MIMO configurations described in sub-clause 6.2D.1. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.6D.

7.6D.2.4	Test description

7.6D.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 7.6D.2.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6D.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | CP-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3D.2.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS and interfering source to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.4 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	The DL and UL Reference Measurement channels are set according to Table 7.6D.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On , Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message content are defined in clause 7.6D.2.4.3.

7.6D.2.4.2	Test procedure

Same test procedure as specified in 7.6.2.4.2.

7.6D.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

7.6D.2.5	Test requirement

Same test requirement as specified in 7.6.2.5.

Table 7.6D.2.5-1: Void

7.6D.3	Out-of-band blocking for UL MIMO

7.6D.3.1	Test purpose

Out-of-band blocking for UL MIMO is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the receive band of an UE that support UL MIMO, with FDL_high < 2700 MHz and FUL_high < 2700 MHz, or falling more than 3CBW below or above the receive band of an UE that support UL MIMO, with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

The lack of out-of-band blocking ability will decrease the coverage area when other g-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6D.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support UL MIMO on FDD bands.

7.6D.3.3	Minimum conformance requirements

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in subclause 7.6 shall be met with the UL MIMO configurations described in sub-clause 6.2D.1. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.6D.

7.6D.3.4	Test description

7.6D.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 7.6D.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6D.3.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | One frequency chosen arbitrarily from low or high range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | CP-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3D.2.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS and interfering source to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	The DL and UL Reference Measurement channels are set according to Table 7.6D.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On , Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message content are defined in clause 7.6D.3.4.3.

7.6D.3.4.2	Test procedure

Same test procedure as specified in 7.6.3.4.2.

7.6D.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

7.6D.3.5	Test requirement

Same test requirement as specified in 7.6.3.5.

Table 7.6D.3.5-1: Void

7.6D.4	Narrow band blocking for UL MIMO

7.6D.4.1	Test purpose

Narrow band blocking for UL MIMO is defined for a receiver's ability of an UE that supports UL MIMO to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

The lack of narrow-band blocking ability will decrease the coverage area when other g-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6D.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support UL MIMO on FDD bands.

7.6D.4.3	Minimum conformance requirements

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in subclause 7.6 shall be met with the UL MIMO configurations described in sub-clause 6.2D.1. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.6D.

7.6D.4.4	Test description

7.6D.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 7.6D.4.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6D.4.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | CP-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3D.2.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

1.	Connect the SS and interfering source to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	The DL and UL Reference Measurement channels are set according to Table 7.6D.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On , Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message content are defined in clause 7.6D.4.4.3.

7.6D.4.4.2	Test procedure

Same test procedure as specified in 7.6.4.4.2.

7.6D.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

7.6D.4.5	Test requirement

Same test requirement as specified in 7.6.4.5.

Table 7.6D.4.5-1: Void

7.7	Spurious response

7.7.1	Test Purpose

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in subclause 7.6.3 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

7.7.3	Minimum Conformance Requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3(with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters for the wanted signal as specified in Table 7.7.3-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and in Table 7.7.3-1a for NR bands with FDL_high ≥ 3300 MHz and FUL_high ≥ 3300 MHz and for the interferer as specified in Table 7.7.3-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.7.3-1: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.7.3-1a: Spurious response parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.7.3-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.7.

7.7.4	Test Description

7.7.4.1	Initial Conditions

The initial conditions shall be the same as in clause 7.6.3.4.1 in order to test spurious responses obtained in clause 7.6.3 under the same conditions.

7.7.4.2	Test Procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6.3.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6.3.4.1-1. Since the UE has no payload and no loopback data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7.5-2. The spurious frequencies are taken from records in the final step of test procedures in clause 7.6.3.4.2.

4.	Set the downlink signal level according to the table 7.7.5-1 or 7.7.5-1a. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.7.5-1 or 7.7.5-1a for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

5.	For the spurious frequency, measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

7.7.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

7.7.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 with parameters for the wanted signal as specified in Table 7.7.5-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and in Table 7.7.5-1a for NR bands with FDL_high ≥ 3300 MHz and FUL_high ≥ 3300 MHz and for the interferer as specified in Table 7.7.5-2.

Table 7.7.5-1: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.7.5-1a: Spurious response parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.7.5-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

Table 7.7.5-3: Void

<< Unchanged sections omitted >>

7.7A.1	Spurious response for CA (2DL CA)

7.7A.1.1	Test Purpose

Spurious response for 2DL CA verifies the receiver's ability to receive a wanted 2DL carrier aggregated signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in sub-clause 7.6A.3 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7A.1.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 2DL CA.

7.7A.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7A.0.

7.7A.1.4	Test Description

7.7A.1.4.1	Initial Conditions

The initial conditions shall be the same as in clause 7.6A.3.1.4.1 in order to test spurious responses obtained in clause 7.6A.3.1 under the same conditions.

7.7A.1.4.2	Test Procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.7A.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Test Configuration Table 7.6A.3.1.4.1-1, 7.6A.3.1.4.1-2 or 7.6A.3.1.4.1-3 in Clause 7.6A.3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Test Configuration Table 7.6A.3.1.4.1-1, 7.6A.3.1.4.1-2 or 7.6A.3.1.4.1-3 in Clause 7.6A.3 on both PCC and SCC. Since the UE has no payload data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7A.0.1-2. The spurious frequencies are taken from records in the final step of test procedures in clause 7.6A.3 Out-of-band blocking for CA.

7.	Set the downlink signal level according to Table 7.7A.0.1-1 for both carriers. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.7A.0.1-1 + (10log(P_LCRB/NRB_alloc) for PCC, 10log(S_LCRB/NRB_alloc) for SCC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

8.	For each spurious frequency, measure the average throughput for each component carrier for duration sufficient to achieve statistical significance according to Annex H.2A.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

Table 7.7A.1.4.2-1: Void

7.7A.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

7.7A.1.5	Test Requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.7A.0.1-1 and 7.7A.0.1-2. For the UE which supports inter-band 2DL CA configuration in Table7.3A.0.3.2.1-1, PInterferer power defined in Table 7.7A.0.1-2 is increased by the amount given by ΔRIB,c in Table  7.3A.0.3.2.1-1.

7.7A.2	Spurious response for 3DL CA

7.7A.2.1	Test Purpose

Spurious response for 3DL CA verifies the receiver's ability to receive a wanted 3DL carrier aggregated signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in sub-clause 7.6A.3 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7A.2.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 3DL CA.

7.7A.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7A.0.

7.7A.2.4	Test Description

7.7A.2.4.1	Initial Conditions

The initial conditions shall be the same as in clause 7.6A.3.2.4.1 in order to test spurious responses obtained in clause 7.6A.3.2 under the same conditions.

7.7A.2.4.2	Test Procedure

Same test procedure as sub-clause 7.7A.1.4.2 with the following exceptions:

Step 1, 2 and 4 of Test Procedure as in clause 7.7A.1.4.2 is replaced by:

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Test Configuration Table 7.6A.3.1.4.1-1, 7.6A.3.1.4.1-2 or 7.6A.3.1.4.1-3 in Clause 7.6A.3 on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Test Configuration Table 7.6A.3.1.4.1-1, 7.6A.3.1.4.1-2 or 7.6A.3.1.4.1-3  in Clause 7.6A.3 on both PCC and SCCs. Since the UE has no payload data to send, the UE sends uplink MAC padding bits on the UL RMC.

4.	Set the downlink signal level according to Table 7.7A.0.1-1 for both carriers. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.7A.0.1-1 + (10log(P_LCRB/NRB_alloc) for PCC, 10log(S_LCRB/NRB_alloc) for SCC) for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

7.7A.2.4.3	Message Contents

Same message contents as sub-clause 7.7A.1.4.3.

7.7A.2.5	Test Requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.7A.0.1-1 and 7.7A.0.1-2. For the UE which supports inter-band 3DL CA configuration in Table 7.3A.0.3.2.3-1, PInterferer power defined in Table 7.7A.0.1-2 is increased by the amount given by ΔRIB,c in Table 7.3A.0.3.2.3-1.

7.7A.3	Spurious response for 4DL CA

7.7A.3.1	Test Purpose

Spurious response for 4DL CA verifies the receiver's ability to receive a wanted 4DL carrier aggregated signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in sub-clause 7.6A.3 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7A.3.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 4DL CA.

7.7A.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.7A.0.

7.7A.3.4	Test Description

7.7A.3.4.1	Initial Conditions

The initial conditions shall be the same as in clause 7.6A.3.3.4.1 in order to test spurious responses obtained in clause 7.6A.3.3 under the same conditions.

7.7A.3.4.2	Test Procedure

Same test procedure as sub-clause 7.7A.2.4.2.

7.7A.3.4.3	Message Contents

Same message contents as sub-clause 7.7A.1.4.3.

7.7A.3.5	Test Requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.7A.0.1-1 and 7.7A.0.1-2.For the UE which supports inter-band 4DL CA configuration in Table 7.3.2_1.3-1 and Table 7.3A.0.3.2.4-1, PInterferer power defined in Table 7.7A.0.1-2 is increased by the amount given by ΔRIB,c in Table 7.3.2_1.3-1 and Table 7.3A.0.3.2.4-1.

7.7D	Spurious response for UL MIMO

7.7D.1	Test Purpose

Spurious response verifies the ability of the UE that support UL MIMO to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking for UL MIMO limit as specified in sub-clause 7.6D.3 is not met.

The lack of the spurious response ability decreases the coverage area when other unwanted interfering signal exists at any other frequency.

7.7D.2	Test Applicability

This test applies to all types of NR UE release 15 and forward that support UL MIMO on FDD bands.

7.7D.3	Minimum Conformance Requirements

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements in Clause 7.7D.3 shall be met with the UL MIMO configurations specified in Table 6.2D.1.4.1-1 in Clause 6.2 D.1 UE maximum output power for UL MIMO. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmitter antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.7D.

7.7D.4	Test Description

7.7D.4.1	Initial Conditions

The initial conditions shall be the same as in clause 7.6D.3.4.1 in order to test spurious responses obtained in clause 7.6D.3 under the same conditions.

7.7D.4.2	Test Procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Test Configuration Table 7.6D.3.4.1-1 in Clause 7.6D.3. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Test Configuration Table 7.6D.3.4.1-1 in Clause 7.6D.3. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

3.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7D.5-2. The spurious frequencies are taken from records in the final step of test procedures in clause 7.6D.3.4.2.

4.	Set the downlink signal level according to the Table 7.7D.5-1. Send uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in table 7.7D.5-1 for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the throughput measurement.

5.	For the spurious frequency, measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

7.7D.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

7.7D.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 with parameters specified in Tables 7.7D.5-1 and 7.7D.5-2.

Table 7.7D.5-1: Spurious response parameters

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 30MHz | 40 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2.4. Note 2: The reference measurement channel is specified in Annex A.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.7D.5-2: Spurious Response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

<< Unchanged sections omitted >>

7.8.2	Wide band Intermodulation

7.8.2.1	Test purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

7.8.2.3	Minimum conformance requirements

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8.2.3-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and Table 7.8.2.3-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.8.2.3-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 6 | 6 | 7 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 15 | 16 |
| PInterferer 1 (CW) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| BWInterferer 2 |  | MHz | 5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) |  | MHz | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) |  | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.8.2.3-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 |  |  |  |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |
| BWInterferer 2 | MHz | BW |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz | -2BW / +2BW |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.8.2.

7.8.2.4	Test description

7.8.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 7.8.2.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.8.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range (NOTE 4) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest (NOTE 3) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3.2.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: For n70, highest test channel bandwidth shall be Highest UL / Highest DL according to asymmetric channel bandwidths specified in clause 5.3.6. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 , C.3.1 , and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	The DL and UL Reference Measurement channels are set according to Table 7.5.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message content are defined in clause 7.5.4.3.

7.8.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table  7.8.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.8.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.8.2.5-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.8.2.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

4.	Set the Interfering signal levels to the values as defined in Table 7.8.2.5-1 and frequency below the wanted signal.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

6.	Repeat steps from 3 to 5, using an interfering signal above the wanted signal at step 4.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

7.8.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with DFT-s-OFDM condition in Table 4.6.3-118 PUSCH-Config.

7.8.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Table 7.8.2.5-1 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8.2.5-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 6 | 6 | 7 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 15 | 16 |
| PInterferer 1 (CW) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| BWInterferer 2 |  | MHz | 5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) |  | MHz | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) |  | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.8.2.5-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 |  |  |  |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |
| BWInterferer 2 | MHz | BW |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz | -2BW / +2BW |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |

Table 7.8.2.5-3: Void

<< Unchanged sections omitted >>

7.8A.2.1.5	Test requirement

7.8A.2.1.5.1	Wide band intermodulation for Intra-band contiguous CA

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Table 7.8A.2.1.5.1-1 or 7.8A.2.1.5.1-2 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8A.2.1.5.1-1: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 10 | REFSENS + 6 |
| PInterferer 1 (CW) | dBm | -46 |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |
| BWInterferer 2 | MHz | 20 | BWChannel_CA |
| FInterferer 1 (Offset) | MHz | -Foffset-30 / Foffset+30 | -2BWChannel_CA / +2BWChannel_CA |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the closest carrier. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |

Table 7.8A.2.1.5.1-2: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 16 | REFSENS + 19 |
| PInterferer 1 (CW) | dBm | -46 | -46 |
| PInterferer 2 (Modulated) | dBm | -46 | -46 |
| BWInterferer 2 | MHz | 5 | 5 |
| FInterferer 1 (Offset) | MHz | -Foffset-7.5 / Foffset+7.5 | -Foffset-7.5 / Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |

7.8A.2.1.5.2	Wide band intermodulation for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two downlink carriers, the wide band intermodulation requirements are defined with the uplink configuration in accordance with Table 7.3A.0.2.3-1. For this uplink configuration, the UE shall meet the requirements for each carrier as specified in subclause 7.8.2 for each component carrier respectively. The requirements apply for out-of-gap interferers while all downlink carriers are active.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters defined in Table 7.8A.2.1.5.3-1 or 7.8A.2.1.5.3-2.

7.8A.2.1.5.3	Wide band intermodulation for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters defined in Table 7.8A.2.1.5.3-1 or 7.8A.2.1.5.3-2.

Table 7.8A.2.1.5.3-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 6 | 6 | 7 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 15 | 16 |
| PInterferer 1 (CW) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| BWInterferer 2 |  | MHz | 5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) |  | MHz | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) |  | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band CA configuration in Table 7.3A.0.3.2.1-1, Pinterferer power defined in Table 7.8A.2.1.5.3-1 and 7.8A.2.1.5.3-2 is increased by the amount given by ΔRIB,c in Table 7.3A.0.3.2.1-1.

Table 7.8A.2.1.5.3-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 |  |  |  |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |
| BWInterferer 2 | MHz | BW |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz | -2BW / +2BW |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.8A.2.2.5	Test requirement

7.8A.2.2.5.1	Wide band intermodulation for Intra-band contiguous CA

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Table 7.8A.2.2.5.1-1 or 7.8A.2.2.5.1-2 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8A.2.2.5.1-1: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | B | C | D |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 10 | REFSENS + 6 | REFSENS + 13.8 |
| PInterferer 1 (CW) | dBm | -46 |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |
| BWInterferer 2 | MHz | 20 | BWChannel_CA | 50 |
| FInterferer 1 (Offset) | MHz | -Foffset-30 / Foffset+30 | -2BWChannel_CA / +2BWChannel_CA | -Foffset-75 / Foffset+75 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the closest carrier. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |

Table 7.8A.2.2.5.1-2: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 16 | REFSENS + 22 |
| PInterferer 1 (CW) | dBm | -46 | -46 |
| PInterferer 2 (Modulated) | dBm | -46 | -46 |
| BWInterferer 2 | MHz | 5 | 5 |
| FInterferer 1 (Offset) | MHz | -Foffset-7.5 / Foffset+7.5 | -Foffset-7.5 / Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |

7.8A.2.2.5.2	Wide band intermodulation for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters defined in Table 7.8A.2.2.5.2-1 or 7.8A.2.2.5.2-2.

Table 7.8A.2.2.5.2-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 6 | 6 | 7 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 15 | 16 |
| PInterferer 1 (CW) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| BWInterferer 2 |  | MHz | 5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) |  | MHz | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) |  | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band CA configuration in Table 7.3A.0.3.2.1-1, Pinterferer power defined in Table 7.8A.2.2.5.2-1 and 7.8A.2.2.5.2-2 is increased by the amount given by ΔRIB,c in Table 7.3A.0.3.2.1-1.

Table 7.8A.2.2.5.2-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 |  |  |  |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |
| BWInterferer 2 | MHz | BW |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz | -2BW / +2BW |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.8A.2.3.5	Test requirement

7.8A.2.3.5.1	Wide band intermodulation for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters defined in Table 7.8A.2.3.5.1-1 or 7.8A.2.3.5.1-2.

Table 7.8A.2.3.5.1-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 6 | 6 | 7 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 15 | 16 |
| PInterferer 1 (CW) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) |  | dBm | -46 |  |  |  |  |  |  |  |  |  |  |  |
| BWInterferer 2 |  | MHz | 5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) |  | MHz | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) |  | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band CA configuration in Table 7.3A.0.3.2.1-1, Pinterferer power defined in Table 7.8A.2.3.5.1-1 and 7.8A.2.3.5.1-2 is increased by the amount given by ΔRIB,c in Table 7.3A.0.3.2.1-1.

Table 7.8A.2.3.5.1-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 |  |  |  |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |
| BWInterferer 2 | MHz | BW |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz | -2BW / +2BW |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |

<< End of changes >>