3GPP TSG RAN Meeting #99	RP-230182

Rotterdam, the Netherlands, March 20-23, 2023

Agenda Item:	9.2.3

Source:	NEC

Title:	Discussion on Deployment Scenarios and Design Targets for Ambient IoT in RAN

Document for:	Discussion and Decision

### 1	Introduction

A new study item on ambient IoT [1] has been approved in RAN#97e meeting. The following objectives in SID will be studied in RAN. In RAN#98e meeting, some proposals are endorsed after discussion [2][3]. In this contribution, we give our views on deployment scenarios and design targets according to objectives.

- Identify the suitable deployment scenarios and their characteristics, at least for the use cases/services agreed in SA1’s “Study on Ambient power-enabled internet of Things”, comprising among at least the following aspects - Indoor/outdoor environment - Basestation characteristics, e.g. macro/micro/pico cells-based deployments - Connectivity topologies, including which node(s) , e.g. basestation, UE, relay, repeater, etc. can communicate with target devices - TDD/FDD, and frequency bands in licensed or unlicensed spectrum - Coexistence with UEs and infrastructure in frequency bands for existing 3GPP technologies - Device originated and/or device terminated traffic assumption NOTE: There can be more than one deployment scenario identified for a use case, and a deployment scenario may be common to more than one use case. NOTE: Where more than one deployment scenario is identified for a use case, the trade-offs between them should also be studied. NOTE: The study shall not prioritize deployment aspects that should be coordinated with SA, e.g. public or private network, with or without CN connection. NOTE: A representative use case can be studied for a group of use cases that have similar requirements. - Formulate a set of RAN design targets based on the identified deployment scenarios and their characteristics for the relevant use cases, at least including - Power consumption - Complexity - Coverage - Data rate - Positioning accuracy NOTE: The requirements from SA1 on the relevant use cases shall be taken into consideration. NOTE: The study shall aim to provide better coverage compared to existing non-3GPP technologies for the relevant use cases. NOTE: Other RAN design targets in relation to connection density, mobility, security, latency, reliability etc. may be discussed, if necessary for the relevant use cases. NOTE: Detailed definitions of the RAN design targets should be discussed during the study. - Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported.

### 2	Discussion

#### 2.1	Topology

There are some FFS parts for topology discussion after RAN#98e meeting. For topology (3) BS <-> assisting node <-> Ambient IoT device <-> BS, it’s FFS that whether the two BS can be different. If the two BS are different, signaling exchange for coordination could be expected between the two BS. Since the coverage of assisting node is expected to be better than ambient IoT device, it is seldom that an ambient IoT device is under coverage of one BS but the assisting node is out of coverage of the BS. Thus, the same BS could fulfill most of scenarios of topology (3) and meanwhile could refrain from inter-node coordination.

As for topology (5) UE <-> Ambient IoT device <-> {BS or UE}, it is similar to topology (3). From Ambient IoT device’s perspective, it has single link between ambient IoT device and node that connecting with ambient IoT device for topology (1)(2)(4) where the link is for both transmission and reception. On the other hand, topology (3) and (5) have dual links between IoT device and nodes that connecting with ambient IoT device, one link for transmission and the other link for reception. Supporting topology (5) may not have large impact and burden on standardization from ambient IoT device’s perspective since topology (3) have been supported. Above all, we think topology (5) could be supported.

Proposal 1: For topology (3) BS <-> assisting node <-> Ambient IoT device <-> BS, the two BS cannot be different.

Proposal 2: Support topology (5) UE <-> Ambient IoT device <-> {BS or UE}.

#### 2.2	Device categorization

It has agreed to consider three sets of ambient IoT device and three energy storage capacities. It’s obvious that storage type 1 (no storage at all) corresponds to device A (No energy storage, no harvesting ambient sources, no independent signal generation). Both storage 2 (Up to E1 joules) and storage 3 (Up to E2 joules) could be replaced by a single description such as “limited energy storage”, but having different storage categories has benefit for distinguishing device B (Has energy storage from harvesting ambient sources, no independent signal generation) and device C (Has energy storage from harvesting ambient sources, has independent signal generation). Independent signal generation requires more energy than no independent signal generation, so storage 2 could be used for device B and storage 3 could be used for device C. We could discuss the require maximum energy for device B and device C separately and give the guidance on implementation.

One issue needs to discuss is whether the backscattering transmission for device A and C is different from active RF component for transmission for device C. The unified transmission between backscattering transmission and active transmission could reduce standardization work load that only one radio interface needed to be design and implementation complexity that only one receiver is needed to detect the signal. On the other hand, the unified transmission could limit the transmission efficiency of device C since higher spectrum efficient method could be used for device C.

Proposal 3: Confirm the working assumption for this framework to categorize energy storage capacities and support three storage categories, i.e. assuming E1 < E2.

Proposal 4: Study whether the backscattering transmission for device A and C is different from active RF component for transmission for device C.

### 3	Conclusion

In this contribution, we give our views on deployment scenarios and design targets for ambient IoT in RAN, and propose that:

Proposal 1: For topology (3) BS <-> assisting node <-> Ambient IoT device <-> BS, the two BS cannot be different.

Proposal 2: Support topology (5) UE <-> Ambient IoT device <-> {BS or UE}.

Proposal 3: Confirm the working assumption for this framework to categorize energy storage capacities and support three storage categories, i.e. assuming E1 < E2.

Proposal 4: Study whether the backscattering transmission for device A and C is different from active RF component for transmission for device C.

### References

[1]	RP-222685, New SID: Study on Ambient Power Enabled IoT, Huawei, HiSilicon, TSG RAN Meeting #97e, Electronic Meeting, September 12 – 16, 2022.

[2]	RP-230052, revised_draft_report_RAN_98e.

[3]	RP-223451, [98e-11-Ambient-IoT] Extended round summary and GTW proposal, Moderator (Huawei), 3GPP TSG RAN Meeting #98e, Electronic Meeting, December 12-16, 2022.

### Annex A: Endorsed proposal

Proposal 4-3b-v2: Agree that for basestation deployments (when present), “Coexistence with existing 3GPP technologies” can be:

- Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type

- Deployed on new sites without an assumption of an existing 3GPP deployment

Proposal 5-3b-v2: the following set of Ambient Iot devices are considered in the SI:

- Device A: No energy storage, no harvesting ambient sources, no independent signal generation, i.e. backscattering transmission

- Device B: Has energy storage from harvesting ambient sources, no independent signal generation, i.e. backscattering transmission. Use of stored energy can include amplification for reflected signals

- Device C: Has energy storage from harvesting ambient sources, has independent signal generation, i.e. active RF component for transmission

- FFS: Whether to include device function

- FFS: Whether to include a target maximum power consumption for each device

- FFS: Whether/how to describe what stored energy is used for (in addition to the statement for Device B)

- FFS: if combination of these devices will be considered.

Proposal 5-2a-extended: Make a working assumption for this framework to categorize energy storage capacities:

- Storage 1: no storage at all

- Storage 2: Up to E1 joules

- Storage 3: Up to E2 joules

- FFS: In RAN#99 value(s) of E1, E2 and it is possible that E1=E2, in which case we have only two storage categories. Note in this case that storage 2 and 3 could be replaced by a single description such as “limited energy storage”, instead.

Proposal 4-4v2: Define the following topologies as starting points for the next meeting:

- Topology (1): BS <-> Ambient IoT device

- NOTE 1: Includes the possibility of BS Rx and BS Tx in different BSs

- Topology (2): BS <-> Intermediate node <-> Ambient IoT device

- NOTE 1: Intermediate node can be relay, IAB, UE, repeater, etc. which is capable of ambient IoT

- Topology (3): BS <-> assisting node <-> Ambient IoT device <-> BS

- NOTE 1: Assisting node can be relay, IAB, UE, repeater, etc. which is capable of ambient IoT

- FFS: If the two BS can be different

- Topology (4): UE <-> Ambient IoT device

- FFS: Topology (5) UE <-> Ambient IoT device <-> {BS or UE}

- NOTE: For potential topology (5), discuss its relation with other topologies, its necessity, etc. in RAN#99.

- NOTE: For all topologies: The Ambient IoT device may be provided with carrier wave from another node(s) either inside or outside the topology

- NOTE: For all topologies: The links in each topology may be bidirectional or unidirectional

- FFS: Whether to consider combination of different topologies in the study.

- FFS: BS, UE, or assisting node could be multiple BSs, UEs or assisting nodes, respectively.

Proposal 2 for agreement:

- Define the groups of Grouping A as follows, as a start point:

  - Indoor

  - Outdoor

  - Indoor/outdoor

- Define the groups of Grouping B as follows, as a start point:

  - Inventory

  - Sensors

  - Positioning

  - Command

- Whether to incorporate Grouping A and Grouping B according to Approach 1 (include both separately) or Approach 2 (Group first by A, and second by B) will be decided in RAN#99.

- Mapping of SA1 use cases to the groups of each grouping will be discussed in the next meeting, including whether RAN needs to attempt that mapping, or only has to define the groups.

Conclusion 3-2: Discussion of CN-related aspects will be under the third objective of the SID.

Proposal 3-1: Agree to capture deployment scenarios as follows:

Table 3: Deployment Scenario <X>

Applicable representative use

cases

Characteristic Description

Environment (of device)

Basestation characteristic (if any)

Connectivity topology

rUC1, rUC2, ...., ... Spectrum

Coexistence with existing 3GPP technologies

Traffic assumption

FFS: whether/which rows can be indicated with more than one value, e.g. Environment = Indoor/Outdoor

FFS: Possible values for each characteristic row (see following questions)

FFS: Whether device characteristic is added to the table.

Proposal 4-1v2: Agree that: ‘Environment of device’ can be ‘indoor’, ‘outdoor’, ‘indoor or outdoor’.

- FFS: Whether to further describe other aspects, e.g. the size of the served area.

Proposal 4-2: Agree that ‘basestation characteristic’ can be: macro-cell based deployment, micro-cell based

deployment, pico-cell base deployment, or none.

- FFS: If/how it is necessary to reflect mobile IAB basestation.

Proposal 4-3a: Agree that the study considers Ambient IoT deployment in-band to NR, in guardband of NR, and standalone band from NR, and FFS: relationship to deployment scenarios.

- Note: Prioritization among them can be discussed in later meetings.

Proposal 4-5: Agree that spectrum in a deployment scenario is: licensed FDD, licensed TDD, unlicensed.

- Note: Further discuss if the study should apply any limitations to the cases for which unlicensed spectrum is studied.

Conclusion 4-6: The moderator records the following for this meeting:

- FFS: whether the TR will describe different types of device-terminated traffic, e.g. Device-Terminated command and Device-Terminated reporting trigger, and whether to describe relationships between device-originated and device-terminated traffic, etc.