3GPP TSG-RAN5 Meeting #98	R5-231626

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2172 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Spurious emissions for UE co-existence for CA_n1A-n8A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-2-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R16 CA configuration CA_n1A-n8A is currently missing in TC 6.5A.3.2.0 and TC 6.5A.3.2.1, the test configuration need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the minimum requirements for CA_n1A-n8A into Table 6.5A.3.2.0.3-2 and Table 6.5A.3.2.0.3-3 Update the test configuration for CA_n1A-n8A into Table 6.5A.3.2.1.4.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CA_n1A-n8A will be missing in TC 6.5A.3.2.0 and TC 6.5A.3.2.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.2.0 6.5A.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR38.905, CR0748 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The following WIC is also covered by this CR：NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an outcome of R5-231204 |  |  |  |  |  |  |  |  |

<<START OF CHANGES>>

6.5A.3.2.0	Minimum conformance requirements

6.5A.3.2.0.1	Spurious emissions for UE co-existence for intra-band contiguous CA

This clause specifies the requirements for the specified intra-band contiguous carrier aggregation configurations for coexistence with protected bands, the requirements in Table 6.5A.3.2.0.1-1 apply.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.2.0.1-1: Requirements for uplink intra-band contiguous carrier aggregation

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2, 3 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2, 3 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 1, 2 |
| CA_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
| CA_n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 2: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 4: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 6: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |

6.5A.3.2.0.2	Void

6.5A.3.2.0.3	Spurious emissions for UE co-existence for Inter-band CA

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 apply on each component carrier with both component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1; Table 6.5A.3.2.0.3-2 and Table 6.5A.3.2.0.3-3 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.5A.3.2.0.3-1: Requirements for uplink inter-band carrier aggregation (two bands) Rel-15

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n78-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.3.2.0.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Table 6.5A.3.2.0.3-2: Requirements for uplink inter-band carrier aggregation (two bands) Rel-16

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n8 | E-UTRA Band 20, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n2-n5 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, 48, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n48 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n2A-n77A | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n3-n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | Same requirements as in Table 6.5A.3.2.0.3-1 |  |  |  |  |  |  |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | Same requirements as in Table 6.5A.3.2.0.3-1 |  |  |  |  |  |  |
| CA_n28-n40 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 27, 28, 31, 34, 38, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 8 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n66-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

Table 6.5A.3.2.0.3-3: Requirements for uplink inter-band carrier aggregation (two bands) Rel-17

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n8 | E-UTRA Band 20, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | Same requirements as in Table 6.5A.3.2.0.3-2 |  |  |  |  |  |  |
| CA_n2-n5 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 28, 29, 30, 42, 48, 50, 51 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n48 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n2A-n77A | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n3-n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 1,8, 11, 20, 21, 28, 34, 39, 40, 65, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n24-n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n24-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n24-n77 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n26-n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 43, 47, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 53 NR band 77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n70 | E-UTRA Band 2, 5, 10, 12, 13, 14, 17, 24, 25, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n39-n41 | Same requirements as in Table 6.5A.3.2.0.3-2 |  |  |  |  |  |  |
| CA_n41-n79 | Same requirements as in Table 6.5A.3.2.0.3-2 |  |  |  |  |  |  |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n48-n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 | 16 |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2, 16 |
| CA_n48-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n66-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: The same configuration applies to corresponding NR-DC configuration in Table 4.3.1.1.7.1-1 in [5]. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in this test. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.2.

6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  | UL Allocation |  |  |  |  |  |
|  | CA Configuration |  |  |  |  |  |  |  | PCC NRB @SCS |  |  | SCC NRB@SCS |  | CC MOD |  |  | PCC & SCC RB allocation |  |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
|  | PCC |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |  |  |
|  | Band |  | Range |  | Band |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Low |  | n8 |  | Low |  |  | 25@15kHz |  |  | 25@15kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Low |  | n78 |  | Low |  | 106@15kHz |  |  | 273@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  | 1@272 |
| 2 | n1 | Low |  | n78 |  | 3421,14 MHz (NOTE 2) |  | 106@15kHz |  |  | 52@15kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@0 |
| 3 | n1 | Low |  | n78 |  | High |  | 106@15kHz |  |  | 273@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n1A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | LOW |  | n79 |  | MID |  | 106@15kHz |  |  | 273@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@181 |
| 2 | n1 | LOW |  | n79 |  | 4618,41 MHz (NOTE 3) |  | 79@15kHz |  |  | 216@15kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@0 |
| 3 | n1 | LOW |  | n79 |  | MID |  | 106@15kHz |  |  | 273@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  | 1@245 |
| 4 | n1 | LOW |  | n79 |  | 4785,54 MHz (NOTE 4) |  | 106@15kHz |  |  | 216@15kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n2A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | Low |  | n5 |  | low |  | 20/106 |  |  | 20/106 |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@0 |
| 2 | n2 | High |  | n5 |  | High |  | 20/106 |  |  | 20/106 |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  | 1@105 |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | Low |  | n77 |  | Mid |  | 106@15kHz |  |  | 273@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@272 |
| 2 | n2 | High |  | n77 |  | 3850,23 MHz (NOTE 5) |  | 106@15kHz |  |  | 52@15kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@0 |
| 3 | n2 | High |  | n77 |  | 3965,25 MHz (NOTE 6) |  | 106@15kHz |  |  | 52@15kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | HIGH |  | n41 |  | HIGH |  | 160@15kHz |  |  | 273@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@151 |
| 2 | n3 | LOW |  | n41 |  | HIGH |  | 160@15kHz |  |  | 273@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@149 |  | 1@18 |
| 3 | n3 | LOW |  | n41 |  | HIGH |  | 133@15kHz |  |  | 217@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@132 |  | 1@155 |
| 4 | n3 | HIGH |  | n41 |  | LOW |  | 160@15kHz |  |  | 217@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  | 1@151 |
| 5 (NOTE 7) | n3 | HIGH |  | n41 |  | HIGH |  | 160@15kHz |  |  | 217@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@159 |  | 1@105 |
| 6 (NOTE 7) | n3 | LOW |  | n41 |  | HIGH |  | 79@15kHz |  |  | 273@30kHz |  | CP-OFDM QPSK |  |  | NA |  |  |  | CP-OFDM QPSK |  |  |  | 1@78 |  | 1@153 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | LOW |  | n78 |  | HIGH |  | 160@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@159 |  |  | 1@272 |  |
| 2 | n3 | HIGH |  | n78 |  | LOW |  | 160@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@159 |  |  | 1@0 |  |
| 3 | n3 | LOW |  | n78 |  | MID |  | 133@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@262 |  |
| 4 | n3 | LOW |  | n78 |  | MID |  | 160@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@41 |  |
| Test Settings for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | HIGH |  | n77 |  | MID |  | 79@15kHz |  | 162@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@161 |  |
| 2 | n5 | HIGH |  | n77 |  | MID |  | 106@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@17 |  |
| 3 | n5 | LOW |  | n77 |  | 3458,79 MHz (NOTE 8) |  | 106@15kHz |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| 4 | n5 | HIGH |  | n77 |  | 3593,79 MHz (NOTE 9) |  | 106@15kHz |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| 5 | n5 | LOW |  | n77 |  | 4021,89 MHz (NOTE 10) |  | 52@15kHz |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@0 |  |
| 6 | n5 | HIGH |  | n77 |  | HIGH |  | 106@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@272 |  |
| 7 | n5 | LOW |  | n77 |  | 3538,29 MHz (NOTE 11) |  | 106@15kHz |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | HIGH |  | n78 |  | LOW |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| 2 | n8 | LOW |  | n78 |  | MID |  | 79@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@192 |  |
| 3 | n8 | HIGH |  | n78 |  | HIGH |  | 106@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@35 |  |
| 4 | n8 | LOW |  | n78 |  | LOW |  | 106@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@0 |  |
| 5 | n8 | LOW |  | n78 |  | HIGH |  | 106@15kHz |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@139 |  |
| Test Settings for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | High |  | n41 |  | Low |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | Low |  | n41 |  | Low |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@52 |  |  | 1@108 |  |
| 3 | n24 | High |  | n41 |  | Low |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |
| 4 | n24 | Low |  | n41 |  | Low |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@94 |  |
| 5 | n24 | High |  | n41 |  | High |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@192 |  |
| Test Settings for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | High |  | n48 |  | Mid |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | High |  | n48 |  | High |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | Low |  | n77 |  | Mid |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | Low |  | n77 |  | 3930 MHz |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@273 |  |
| 3 | n24 | Low |  | n77 |  | 3930 MHz |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@262 |  |
| 4 | n24 | Low |  | n77 |  | 3930 MHz |  | 52@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@52 |  |  | 1@196 |  |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | High |  | n66 |  | High |  | 106@15kHz |  | 216@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@0 |  |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | Low |  | n70 |  | Low |  | 106@15kHz |  | 79@15kHz |  |  | QPSK/CP-OFDM QPSK |  | NA |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | LOW |  | n41 |  | HIGH |  | 79@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@78 |  |  | 1@157 |  |
| 2 | n28 | HIGH |  | n41 |  | HIGH |  | 79@15kHz |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@78 |  |  | 1@115 |  |
| 3 | n28 | HIGH |  | n41 |  | LOW |  | 79@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@194 |  |
| Test Settings for CA_n39A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n39 | LOW |  | n41 |  | HIGH |  | 216@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@137 |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | LOW |  | n79 |  | LOW |  | 273@30kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@272 |  |  | 1@0 |  |
| 2 | n41 | HIGH |  | n79 |  | LOW |  | 273@30kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@272 |  |  | 1@259 |  |
| 3 | n41 | LOW |  | n79 |  | MID |  | 273@30kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@272 |  |  | 1@150 |  |
| 4 | n41 | HIGH |  | n79 |  | LOW |  | 273@30kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| 5 | n41 | HIGH |  | n79 |  | LOW |  | 273@30kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@129 |  |  | 1@0 |  |
| 6 | n41 | HIGH |  | n79 |  | LOW |  | 106@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| 7 | n41 | HIGH |  | n79 |  | HIGH |  | 273@30kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@272 |  |  | 1@272 |  |
| 8 | n41 | HIGH |  | n79 |  | MID |  | 217@30kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@216 |  |  | 1@133 |  |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | High |  | n66 |  | Low |  | 216@15kHz |  | 216@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | High |  | n70 |  | High |  | 216@15kHz |  | 79@15kHz |  |  | QPSK |  | NA |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@215 |  |  | 1@78 |  |
| 2 | n48 | High |  | n70 |  | Low |  | 216@15kHz |  | 79@15kHz |  |  | QPSK |  | NA |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@215 |  |  | 1@0 |  |
| Test Settings for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | HIGH |  | n71 |  | LOW |  | 106@15kHz |  | 106@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| 2 | n48 | LOW |  | n71 |  | HIGH |  | 216@15kHz |  | 106@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@105 |  |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | Low |  | n71 |  | Low |  | 216@15kHz |  | 216@15kHz |  |  | QPSK/CP-OFDM QPSK |  | NA |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | LOW |  | n77 |  | 3825,24 MHz (NOTE 12) |  | 106@15kHz |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| 2 | n66 | LOW |  | n77 |  | HIGH |  | 216@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@151 |  |
| 3 | n66 | LOW |  | n77 |  | 4065,18 MHz (NOTE 13) |  | 79@15kHz |  | 52@15kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| 4 | n66 | LOW |  | n77 |  | HIGH |  | 216@15kHz |  | 217@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@48 |  |
| 5 | n66 | LOW |  | n77 |  | HIGH |  | 106@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@217 |  |
| 6 (NOTE 14) | n66 | LOW |  | n77 |  | HIGH |  | 79@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@214 |  |
| 7 | n66 | LOW |  | n77 |  | HIGH |  | 216@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@211 |  |
| 8 | n66 | LOW |  | n77 |  | MID |  | 106@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@13 |  |
| 9 | n66 | HIGH |  | n77 |  | MID |  | 106@15kHz |  | 273@30kHz |  |  | CP-OFDM QPSK |  | NA |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@24 |  |
| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | Low |  | n71 |  | Low |  | 79@15kHz |  | 106@15kHz |  |  | QPSK/CP-OFDM QPSK |  | NA |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |
| NOTE 1: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 2: Test Point ID 2 for CA_n1A-n78A and Band n78 is defined as a carrier with CBW=10MHz and centre frequency at 3421.14MHz (NR ARFCN=628067). NOTE 3: Test Point ID 2 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4618.41MHz (NR ARFCN=707867). NOTE 4: Test Point ID 4 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4785.54MHz (NR ARFCN=719000). NOTE 5: Test Point ID 2 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3850.23MHz (NR ARFCN=656667). NOTE 6: Test Point ID 3 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3965.25MHz (NR ARFCN=664333). NOTE 7: Test point ID 5 and 6 for CA_n3A-n41A test protected NR band 100 introduced in Rel-17. NOTE 8: Test Point ID 3 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3458.79MHz (NR ARFCN=630533). NOTE 9: Test Point ID 4 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3593.79MHz (NR ARFCN=639533). NOTE 10: Test Point ID 5 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4021.89MHz (NR ARFCN=668067). NOTE 11: Test Point ID 7 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3538.29MHz (NR ARFCN=635867). NOTE 12: Test Point ID 1 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3825.24MHz (NR ARFCN=655000). NOTE 13: Test Point ID 3 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4065.18MHz (NR ARFCN=671000). NOTE 14: Test point ID 6 for CA_n66A-n77A tests protected E-UTRA band 103 introduced in Rel-17. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

<<END OF CHANGES>>