3GPP TSG-RAN5 Meeting #98	R5-231949

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0324 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 6.4 - MBMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The names of generic procedures in 36.579-1 clause 5.4 have been changed to be applicable for MCVideo and MCData too. - Inconsistent wording of test steps and MMIs - Further editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Refernces to procedures of 36.579-1 clause 5.4 corrected. - Hormonised wording of test steps and MMIs - Further editorial corrections, further editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0291 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-230144. |  |  |  |  |  |  |  |  |

## 6.4	MBMS

### 6.4.1	On-network / MBMS / MBMS Bearer Announcement / MBMS Bearer Listening Status / Transition to MBMS from Unicast / MBMS Floor Control / Transition to Unicast from MBMS

6.4.1.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the UE (MCPTT Client) receives MBMS bearer announcements via SIP MESSAGE messages }

then { UE (MCPTT Client) responds by sending a SIP 200 (OK) to the SIP MESSAGE message and the UE (MCPTT Client) sends an MBMS listening status message via SIP MESSAGE }

}

(2)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the UE (MCPTT Client) receives MBMS subchannel control messages }

then { UE (MCPTT Client) responds by sending an MBMS listening status message via SIP MESSAGE }

}

(3)

with { UE (MCPTT Client) listening to the MBMS subchannel during an ongoing MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the MCPTT User (MCPTT Client) engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server on the MBMS subchannel (Floor Taken and Floor Idle) and continues to respect floor control imposed by the MCPTT Server via unicast (Floor Ack and Floor Release)}

}

(4)

with { UE (MCPTT Client) no longer listening to the MBMS subchannel during an ongoing MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the MCPTT User (MCPTT Client) engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects floor control imposed by the MCPTT Server via unicast (Floor Idle)}

}

(5)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the UE (MCPTT Client) receives an explicit MuSiK download message }

then { UE (MCPTT Client) sends a 200 OK response to accept the key and associates the key with the group}

}

6.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.379, clause 14.3.3.2, TS 24.380, clauses 10.3.2, 10.3.3, 10.3.4, 6.2.4.3.5, 6.2.4.5.3, 6.2.4.4.2, 6.2.4.3.2. Unless otherwise stated, these are Rel-13 requirements.

[TS 24.379, clause 14.3.3.2]

When the MCPTT client wants to report the MBMS bearer listening status, the MCPTT client:

NOTE 1:	The application/vnd.3gpp.mcptt-mbms-usage-info+xml can contain both the listening status "listening" and "not listening" at the same time.

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]; and

2)	the SIP MESSAGE request:

a)	shall include in the Request-URI the MBMS public service identity of the participating MCPTT function received in the P-Asserted-Identity header field of the announcement message;

b)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media-feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

c)	should include a public user identity in the P-Preferred-Identity header field as specified in 3GPP TS 24.229 [4];

d)	shall include a P-Preferred-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.mcptt";

e)	shall include an application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body with the <version> element set to "1";

f)	if the MCPTT client is listening to the MBMS bearer, the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body:

i)	shall include an <mbms-listening-status> element set to "listening";

ii)	if the intention is to report that the MCPTT client is listening to the MBMS subchannel for an ongoing conversation in a session (e.g. as the response to the Map Group To Bearer message), shall include the MCPTT session identity of the ongoing conversation in <session-identity> element;

iii)	shall include one or more <TGMI> elements for which the listening status applies; and

iv)	if the intention is to report that the MCPTT client is listening to the general purpose MBMS subchannel, shall include the <general-purpose> element set to "true";

g)	if the MCPTT client is not listening, the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body:

i)	shall include an <mbms-listening-status> element set to "not-listening";

iii)	shall include one or more <TGMI> elements for which the listening status applies;

iii)	if the intention is to report that the MCPTT client is no longer listening to the MBMS subchannel in an ongoing session (e.g. as the response to Unmap Group to Bearer message), shall include the MCPTT session identity in <session-identity> elements; and

iv)	if the intention is to report that the MCPTT client is no longer listening to general purpose MBMS subchannel, shall include the <general-purpose> element set to "false"; and

NOTE 2:	If the MCPTT client reports that the MCPTT client is no longer listening to the general purpose MBMS subchannel, it is implicitly understood that the MCPTT client no longer listens to any MBMS subchannel in ongoing conversations that the MCPTT client previously reported status "listening".

h)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-request-uri> set to the MCPTT ID; and

3)	shall send the SIP MESSAGE request according to 3GPP TS 24.229 [4].

[TS 24.380, clause 10.3.2]

When receiving a Map Group To Bearer message over the general purpose MBMS subchannel, the MBMS interface in the MCPTT client:

1.	shall associate the TMGI in the TMGI field, the MBMS subchannel for audio and for floor control with the MCPTT group identity in the MCPTT Group ID field.

[TS 24.380, clause 10.3.3]

If the MBMS interface receives RTP media packets or floor control messages over the MBMS subchannel, the MBMS interface in the MCPTT client:

1.	if there is an association between the TMGI, the MBMS subchannel for audio and for floor control to an ongoing conversation in a group session:

a.	shall forward the received floor control messages to the floor participant in the conversation; and

b.	if the received RTP medias contains a different SSRC value than the SSRC value used by the MCPTT client, shall forward the received RTP packets to the media mixer in the conversation; and

2.	if there is no such association:

a.	shall ignore the received floor control message or received RTP media packet.

[TS 24.380, clause 10.3.4]

When receiving the Unmap Group To Bearer message over a MBMS subchannel, the MBMS interface in the MCPTT client:

1.	shall remove the association between the TMGI, the MBMS subchannel for audio and for floor control from the conversation in the group session identified by the MCPTT Group ID field, if such an association exists.

[TS 24.380, clause 6.2.4.3.5]

Upon receiving an indication from the user to request permission to send media, the floor participant:

1.	shall send the Floor Request message toward the floor control server; The Floor Request message:

a.	if a different priority than the normal priority is required, shall include the Floor Priority field with the priority not higher than negotiated with the floor control server as specified in subclause 14.3.3; and

b.	if the floor request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Floor Indicator field indicating the relevant call types;

2.	shall start timer T101 (Floor Request) and initialise counter C101 (Floor Request) to 1; and

3.	shall enter the 'U: pending Request' state.

[TS 24.380, clause 6.2.4.5.3]

Upon receiving an indication from the user to release the permission to send RTP media, the floor participant:

1.	shall send a Floor Release message towards the floor control server The Floor Release message:

a.	may include the first bit in the subtype of the Floor Release message set to '1' (acknowledgement is required) as specified in subclause 8.2.2;

NOTE:	It is an implementation option to handle the receipt of the Floor Ack message and what action to take if the Floor Ack message is not received.

b.	if the session is a broadcast call and if the session was established as a normal call, shall include the Floor Indicator with the A-bit set to '1' (Normal call); and

c.	if the Floor Granted message included the G-bit set to '1' (Dual floor), shall include the Floor Indicator with the G-bit set to '1' (Dual floor);

2.	shall remove the indication that the participant is overriding without revoke if this indication is stored;

3.	shall remove the indication that the participant is overridden without revoke if this indication is stored;

4.	shall start timer T100 (Floor Release) and initialize counter C100 (Floor Release) to 1; and

5.	shall enter the 'U: pending Release' state.

[TS 24.380, clause 6.2.4.4.2]

Upon receiving a Floor Granted message from the floor control server or a floor granted indication in an SIP 200 (OK) response in the application and signalling layer, the floor participant:

1.	if the first bit in the subtype of the Floor Granted message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '1' (Floor Granted); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	shall provide floor granted notification to the user, if not already done;

NOTE:	Providing the floor granted notification to the user prior to receiving the Floor Granted message is an implementation option.

3.	if the Floor Indicator field is included and the B-bit is set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	if the G-bit in the Floor Indicator is set to '1' (Dual floor) shall store an indication that the participant is overriding without revoke;

5.	shall stop the optional timer T103 (End of RTP media), if running;

6.	shall stop timer T101 (Floor Request); and

7.	shall enter the 'U: has permission' state.

[TS 24.380, clause 6.2.4.3.2]

Upon receiving a Floor Idle message, the participant:

1.	if the first bit in the subtype of the Floor Idle message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '5' (Floor Idle); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	may provide floor idle notification to the user, if it has not already done so;

3.	shall stop the optional timer T103 (End of RTP media), if it is running; and

4.	shall remain in the 'U: has no permission' state.

6.4.1.3	Test description

6.4.1.3.1	Pre-test conditions

System Simulator:

-	Void

IUT:

-	Void

Preamble:

-	A pre-activated MBMS bearer exists

6.4.1.3.2	Test procedure sequence

Table 6.4.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a group call with automatic commencement mode and without implicit floor request. (NOTE 1) | - | - | - | - |  |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCPTT call establishment are described in TS 36.579-1 [2], clause 5.4.3 'MCX CO communication in E-UTRA'. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |  |
| 2 | The UE (MCPTT client) sends an initial SIP INVITE request requesting the establishment of a group call with automatic commencement mode and floor control, | --> | SIP INVITE | - | - |  |
| 3 | The SS sends a SIP 100 Trying | <-- | SIP 100 Trying | - | - |  |
| 4 | The SS sends a SIP 180 Ringing | <-- | SIP 180 Ringing | - | - |  |
| 5 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |  |
| 6 | The UE (MCPTT client) sends a SIP ACK. | --> | SIP ACK | - | - |  |
| 7 | The UE (MCPTT client) notifies the user that the call has been established. (NOTE 1) | - | - | - | - |  |
| 8 | Make the UE (MCPTT client) request the floor. (NOTE 1) | - | - | - | - |  |
| 9 | The procedure 'MCPTT Floor Release – Floor Idle' as described in TS 36.579-1 [2] Table 5.3A.15.3-1 is performed. | - | - | - | - |  |
| 10-11 | Void | - | - | - | - |  |
| 12 | The SS pre-activates an MBMS bearer and sends a SIP MESSAGE containing an MBMS bearer announcement message to announce the availability of an MBMS bearer. NOTE: The related E-UTRA/EPC actions are described in TS 36.579-1 [2], clause 5.4.12 'MCX communication over MBMS'. The test sequence below shows only the MCPTT relevant messages exchanged. | <-- | SIP MESSAGE | - | - |  |
| 13 | Check: Does the UE (MCPTT client) respond with a SIP 200 (OK)? | --> | SIP 200 (OK) | 1 | P |  |
| 14 | Check: Does the UE (MCPTT client) send a SIP MESSAGE containing an MBMS Bearer listening status report? | --> | SIP MESSAGE | 1 | P |  |
| 15 | The SS responds with a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |  |
| 16 | Make the UE (MCPTT client) request the floor. (NOTE 1) | - | - | - | - |  |
| 17 | The UE (MCPTT client) sends a Floor Request message. | --> | Floor Request | - | - |  |
| 18 | The SS decides that a MBMS subchannel shall be used for the conversation and sends a Map Group To Bearer message over the general purpose MBMS subchannel. NOTE: The related E-UTRA/EPC actions are described in TS 36.579-1 [2], clause 5.4.12 'MCX communication over MBMS'. The test sequence below shows only the MCPTT relevant messages exchanged. | <-- | Map Group To Bearer | - | - |  |
| 19 | Check: Does the UE (MCPTT client) respond to the Map Group To Bearer message by sending a SIP MESSAGE containing an MBMS Bearer listening status report? | --> | SIP MESSAGE | 2 | P |  |
| 20 | The SS responds with a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |  |
| 21 | The SS sends a Floor Granted message with request for acknowledgement using unicast messaging. | <-- | Floor Granted | - | - |  |
| 22 | Check: Does the UE (MCPTT client) respond with a Floor Ack message? | --> | Floor Ack | 3 | P |  |
| 22a1 | The SS sends a SIP MESSAGE containing a MuSiK download message to set the explicit MuSiK key to be used for the floor control encryption over the MBMS subchannel. | <-- | SIP MESSAGE | - | - |  |
| 22a2 | Check: Does the UE (MCPTT client) respond with a SIP 200 (OK)? | --> | SIP 200 (OK) | 5 | P |  |
| 23 | The SS sends a Floor Taken (by client A) message over the MBMS subchannel | <-- | Floor Taken | - | - |  |
| 24 | Make the UE (MCPTT client) release the floor. (NOTE 1) | - | - | - | - |  |
| 25 | Check: Does the UE (MCPTT client) send a Floor Release message? | --> | Floor Release | 3 | P |  |
| - | EXCEPTION: Step 26a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE requests an acknowledgement to the Floor Release message. | - | - | - | - |  |
| 26a1 | The SS responds with a Floor Ack message using unicast messaging | <-- | Floor Ack | - | - |  |
| 27 | The SS sends a Floor Idle message with request for acknowledgement over the MBMS subchannel | <-- | Floor Idle | - | - |  |
| 28 | Check: Does the UE (MCPTT client) respond with a Floor Ack message sent over the MBMS subchannel? | --> | Floor Ack | 3 | P |  |
| 29 | The SS sends a Floor Taken message over the MBMS subchannel | <-- | Floor Taken | - | - |  |
| - | EXCEPTION: Step 30a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE (MCPTT client) notifies the user of the status of the floor; i.e. if the UE (MCPTT client) notifies the user when the floor is idle or who currently has the floor. | - | - | - | - |  |
| 30a1 | Check: Does the UE (MCPTT client) notify the user that the floor is taken? (NOTE 1) | - | - | 3 | P |  |
| 31 | The SS sends a Floor Idle message with request for acknowledgement over the MBMS subchannel | <-- | Floor Idle | - | - |  |
| 32 | Check: Does the UE (MCPTT client) respond with a Floor Ack message sent over the MBMS subchannel? | --> | Floor Ack | 3 | P |  |
| 33 | The SS ends the conversation on the MBMS subchannel and sends an Unmap Group to Bearer message on the MBMS subchannel. NOTE: The related E-UTRA/EPC actions are described in TS 36.579-1 [2], clause 5.4.12 'MCX communication over MBMS'. The test sequence below shows only the MCPTT relevant messages exchanged. | <-- | Unmap Group to Bearer | - | - |  |
| 34 | Check: Does the UE (MCPTT client) respond to the Unmap Group To Bearer message with a SIP MESSAGE containing an MBMS bearer listening status report? | --> | SIP MESSAGE | 2 | P |  |
| 35 | The SS responds with a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |  |
| 36 | The SS cancels the MBMS bearer announcement and sends a SIP MESSAGE. | <-- | SIP MESSAGE | - | - |  |
| 37 | Check: Does the UE (MCPTT client) respond with a SIP 200 (OK)? | --> | SIP 200 (OK) | 1 | P |  |
| 38 | Check: Does the UE (MCPTT client) send a SIP MESSAGE containing an MBMS Bearer listening status report? | --> | SIP MESSAGE | 1 | P |  |
| 38a | The SS responds with a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |  |
| 39 | The SS sends a Floor Idle message with request for acknowledgement over the unicast channel | <-- | Floor Idle | - | - |  |
| 40 | Check: Does the UE respond with a Floor Ack message? | --> | Floor Ack | 4 | P |  |
| 41 | Make the UE (MCPTT client) release the call. (NOTE 1) | - | - | - | - |  |
| 42 |  | The UE (MCPTT client) performs procedure 'MCX CO call release' as described in TS 36.579-1 [2] Table 5.3.10.3-1. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.4.1.3.3	Specific message contents

Table 6.4.1.3.3-1: SIP INVITE (step 2, Table 6.4.1.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition GROUP-CALL

Table 6.4.1.3.3-2: SIP MESSAGE (step 12, Table 6.4.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| addr-spec |  |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/sdp" |  |  |  |
| Content-Disposition | "render" |  |  |  |
| MIME-part-body | SDP message as described in Table 6.4.1.3.3-3 |  |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.1.3.3-4 |  |  |  |

Table 6.4.1.3.3-3: SDP in SIP MESSAGE (Table 6.4.1.3.3-2)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media descriptions |  |  |  |  |
| media description |  | m= line media = audio |  |  |
| media | "audio" |  |  |  |
| port | "9" |  |  |  |
| proto | "RTP/AVP" |  |  |  |
| fmt | "99" |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" or “IP6” depending on IP address |  |  |  |
| connection-address | "0.0.0.0" (IPv4) or domain name within “.invalid” DNS top level domain (IPv6) |  |  |  |
| media description |  | m= line media = application |  |  |
| media | "application" |  |  |  |
| port | "9" |  |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" or “IP6” depending on IP address |  |  |  |
| connection-address | "0.0.0.0" (IPv4) or domain name within “.invalid” DNS top level domain (IPv6) |  |  |  |
| media description |  | m= line media = application |  |  |
| media | "application" |  |  |  |
| port | port number assigned by the SS |  |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" |  |  |  |
| connection-address | multicast IP assigned by the SS |  |  |  |
| media attribute |  | a= line attribute = key-mgmt |  |  |
| key-mgmt |  |  | TS 24.379 [9] clause 14.2.2.2 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in TS 36.579-1 [2], Table 5.5.9.1-x. | MSCCK distribution as specified in TS 33.180 [33] Annex H | RFC 4567 [44] |  |

Table 6.4.1.3.3-4: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.1.3.3-2)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status | not present |  |  |  |
| announcement |  |  |  |  |
| TMGI |  |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [X] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |  |
| QCI | "65" | Mission Critical user plane Push To Talk voice |  |  |
| mbms-service-areas | "0" | The selected value is randomly chosen. The value 0 has a special meaning; it shall denote the whole PLMN as the MBMS Service Area |  |  |
| GPMS | "2" | The number of the "m=application" media line in the application/sdp MIME |  |  |
| version | "1" |  |  |  |

Table 6.4.1.3.3-5: SIP MESSAGE (step 14, Table 6.4.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | The value received in the P-Asserted-Identity header field of the announcement message (px_MBMS_Service_ID) |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCPTT_User_A_ID |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.1.3.3-6 |  |  |  |

Table 6.4.1.3.3-6: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.1.3.3-5)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status |  |  |  |  |
| mbms-listening-status | "listening" |  |  |  |
| session-id | not present |  |  |  |
| general-purpose | "true" |  |  |  |
| TMGI | same value as the announcement in the SIP MESSAGE |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.1.3.3-7: SIP MESSAGE (step 19, Table 6.4.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | The value received in the P-Asserted-Identity header field of the announcement message (px_MBMS_Service_ID) |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCPTT_User_A_ID |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.1.3.3-8 |  |  |  |

Table 6.4.1.3.3-8: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.1.3.3-7)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status |  |  |  |  |
| mbms-listening-status | "listening" |  |  |  |
| session-id | px_sesson_A_ID |  |  |  |
| general-purpose | not present |  |  |  |
| TMGI | same value as the announcement in the SIP MESSAGE |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.1.3.3-9: SIP MESSAGE (step 34, Table 6.4.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | The value received in the P-Asserted-Identity header field of the announcement message (px_MBMS_Service_ID) |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCPTT_User_A_ID |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.1.3.3-10 |  |  |  |

Table 6.4.1.3.3-10: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.1.3.3-10)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status |  |  |  |  |
| mbms-listening-status | "not-listening" |  |  |  |
| session-id | px_sesson_A_ID |  |  |  |
| general-purpose | not present |  |  |  |
| TMGI | same value as the announcement in the SIP MESSAGE |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.1.3.3-11: SIP MESSAGE (step 36, Table 6.4.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | px_MBMS_Service_ID | The MBMS public service identity of the participating MCPTT function |  |  |
| port | not present |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.1.3.3-12 |  |  |  |

Table 6.4.1.3.3-12: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.1.3.3-11)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status | not present |  |  |  |
| announcement |  |  |  |  |
| TMGI |  |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [X] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |  |
| QCI | "65" | Mission Critical user plane Push To Talk voice |  |  |
| mbms-service-areas | not present |  |  |  |
| GPMS | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.1.3.3-13: SIP MESSAGE (step 38, Table 6.4.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | The value received in the P-Asserted-Identity header field of the announcement message (px_MBMS_Service_ID) |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCPTT_User_A_ID |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.1.3.3-14 |  |  |  |

Table 6.4.1.3.3-13a: SIP MESSAGE (step 22a1, Table 6.4.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| addr-spec |  |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.1.3.3-13B |  |  |  |
| MIME body part |  | MIKEY message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | As described in TS 36.579-1 [2], Table 5.5.9.1-x. | MIKEY message containing the MuSiK |  |  |

Table 6.4.1.3.3-13B: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.1.3.3-13a)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status | not present |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |
| anyExt |  |  |  |  |
| mbms-explicitMuSiK-download |  |  |  |  |
| group | px_MCPTT_Group_A_ID |  |  |  |

Table 6.4.1.3.3-14: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.1.3.3-13)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status |  |  |  |  |
| mbms-listening-status | "not-listening" |  |  |  |
| session-id | not present |  |  |  |
| general-purpose | "false" |  |  |  |
| TMGI | same value as the announcement in the SIP MESSAGE |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.1.3.3-19: Floor Idle (steps 27, 31, 39, Table 6.4.1.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.6.6-1, condition ACK

Table 6.4.1.3.3-21: Floor Granted (step 21, Table 6.4.1.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.6.3-1, condition EMERGENCY-CALL. ACK

Table 6.4.1.3.3-21..24: Void

### 6.4.2	On-network / MBMS / Multi Talker

6.4.2.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { the MCPTT User requests the establishment of an MCPTT On-demand Pre-arranged Group Call  and implicit floor control }

then { UE (MCPTT Client) requests On-demand Automatic Commencement Mode Pre-arranged Group Call establishment with implicit floor control by sending a SIP INVITE message, and, after indication from the MCPTT Server that the call was established and receiving a Floor Granted message, notifies the MCPTT User and respects basic floor control (Floor Release, Floor Idle, Floor Ack, Floor Request)}

}

(2)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the UE (MCPTT Client) receives MBMS bearer announcements via SIP MESSAGE messages }

then { UE (MCPTT Client) responds by sending a SIP 200 (OK) to the SIP MESSAGE message and the UE (MCPTT Client) sends an MBMS listening status message via SIP MESSAGE }

}

(3)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the UE (MCPTT Client) receives MBMS subchannel control messages }

then { UE (MCPTT Client) responds by sending an MBMS listening status message via SIP MESSAGE }

}

(4)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the UE (MCPTT Client) receives an explicit MuSiK download message }

then { UE (MCPTT Client) sends a 200 OK response to accept the key and associates the key with the group}

}

(5)

with { UE (MCPTT Client) listening to the MBMS subchannel during an ongoing MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the (MCPTT Client) receives multi-talker floor control messages from the SS (MCPTT Server) over the MBMS subchannel }

then { UE (MCPTT Client) notifies the MCPTT User }

}

(6)

with { UE (MCPTT Client) listening to the MBMS subchannel during an ongoing MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the MCPTT User engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server on the MBMS subchannel (Floor Taken and Floor Idle) and continues to respect floor control imposed by the MCPTT Server via unicast (Floor Ack and Floor Release)}

}

(7)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call with Automatic  Commencement Mode }

ensure that {

when { the MCPTT User requests terminate the ongoing MCPTT Group Call }

then { UE (MCPTT Client) sends a SIP BYE request and leaves the MCPTT session }

}

6.4.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.379, clauses 10.1.1.2.1.1, 6.2.4.1, 14.3.3.2, TS 24.380, clauses 10.3.2, 10.3.3, 10.3.4, 6.2.4.2.2, 6.2.4.4.2, 6.2.4.5.3, 6.2.4.6.4, 6.2.4.3.5, 6.2.4.5.8, 6.2.4.5.9, 6.2.4.3.2. Unless otherwise stated, these are Rel-15 requirements.

[TS 24.379, clause 10.1.1.2.1.1]

Upon receiving a request from an MCPTT user to establish an MCPTT prearranged group session the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	if the MCPTT user has requested the origination of an MCPTT emergency group call or is originating an MCPTT prearranged group call and the MCPTT emergency state is already set, the MCPTT client shall comply with the procedures in clause 6.2.8.1.1;

2)	if the MCPTT user has requested the origination of an MCPTT imminent peril group call, the MCPTT client shall comply with the procedures in clause 6.2.8.1.9;

3)	if the MCPTT user has requested the origination of a broadcast group call, the MCPTT client shall comply with the procedures in clause 6.2.8.2;

4)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

5)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

7)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

8)	should include the "timer" option tag in the Supported header field;

9)	should include the Session-Expires header field according to IETF RFC 4028 [7]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

10)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCPTT function serving the MCPTT user;

NOTE 1:	The MCPTT client is configured with public service identity identifying the participating MCPTT function serving the MCPTT user.

11)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

12)	if the MCPTT client emergency group state for this group is set to "MEG 2: in-progress" or "MEG 4: confirm-pending", the MCPTT client shall include the Resource-Priority header field and comply with the procedures in clause 6.2.8.1.2;

13)	if the MCPTT client imminent peril group state for this group is set to "MIG 2: in-progress" or "MIG 4: confirm-pending" shall include the Resource-Priority header field and comply with the procedures in clause 6.2.8.1.12;

14)	shall contain in an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <session-type> element set to a value of "prearranged";

b)	the <mcptt-request-uri> element set to the group identity;

c)	the <mcptt-client-id> element set to the MCPTT client ID of the originating MCPTT client;

NOTE 2:	The MCPTT client does not include the MCPTT ID of the originating MCPTT user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCPTT function.

d)	if the group identity can be determined to be a TGI and if the MCPTT client can associate the TGI with a MCPTT group ID, the <associated-group-id> element set to the MCPTT group ID; and

e)	if the MCPTT client is aware of active functional-aliases, and an active functional alias is to be included in the initial SIP INVITE request, the <functional-alias-URI> set to the URI of the used functional alias;

NOTE 3:	The text "can associate the TGI with a MCPTT group ID" means that the MCPTT client is able to determine that there is a constituent group of the temporary group that it is a member of.

NOTE 4:	The MCPTT client is informed about temporary groups and regrouping of MCPTT groups that the user is a member of as specified in 3GPP TS 24.481 [31].

NOTE 5:	If the MCPTT user selected a TGI where there are several MCPTT groups where the MCPTT user is a member, the MCPTT client selects one of those MCPTT groups.

NOTE 6:	The MCPTT client learns the functional aliases that are activated for an MCPTT ID from procedures specified in clause 9A.2.1.3.

15)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarifications given in clause 6.2.1;

16)	if an implicit floor request is required, shall indicate this as specified in clause 6.4; and

17)	shall send the SIP INVITE request towards the MCPTT server according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP INVITE request, the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5] ;

2)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted" or the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted", the MCPTT client shall perform the actions specified in clause 6.2.8.1.4;

2A)	may notify the answer state to the user (i.e. "Unconfirmed" or "Confirmed") if received in the P-Answer-State header field; and

3)	may subscribe to the conference event package as specified in clause 10.1.3.1.

[TS 24.379, clause 6.2.4.1]

Upon receiving a request from an MCPTT user to leave an MCPTT session established using on-demand session signalling, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	shall generate a SIP BYE request according to 3GPP TS 24.229 [4];

3)	shall set the Request-URI to the MCPTT session identity to leave; and

4)	shall send a SIP BYE request towards MCPTT server according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCPTT client shall interact with the media plane as specified in 3GPP TS 24.380 [5].

[TS 24.379, clause 14.3.3.2]

When the MCPTT client wants to report the MBMS bearer listening status, the MCPTT client:

NOTE 1:	The application/vnd.3gpp.mcptt-mbms-usage-info+xml can contain both the listening status "listening" and "not listening" at the same time.

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]; and

2)	the SIP MESSAGE request:

a)	shall include in the Request-URI the MBMS public service identity of the participating MCPTT function received in the P-Asserted-Identity header field of the announcement message;

b)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media-feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

c)	should include a public user identity in the P-Preferred-Identity header field as specified in 3GPP TS 24.229 [4];

d)	shall include a P-Preferred-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.mcptt";

e)	shall include an application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body with the <version> element set to "1";

f)	if the MCPTT client is listening to the MBMS bearer, the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body:

i)	shall include an <mbms-listening-status> element set to "listening";

ii)	if the intention is to report that the MCPTT client is listening to the MBMS subchannel for an ongoing conversation in a session (e.g. as the response to the Map Group To Bearer message), shall include the MCPTT session identity of the ongoing conversation in <session-identity> element;

iii)	shall include one or more <TGMI> elements for which the listening status applies; and

iv)	if the intention is to report that the MCPTT client is listening to the general purpose MBMS subchannel, shall include the <general-purpose> element set to "true";

g)	if the MCPTT client is not listening, the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body:

i)	shall include an <mbms-listening-status> element set to "not-listening";

iii)	shall include one or more <TGMI> elements for which the listening status applies;

iii)	if the intention is to report that the MCPTT client is no longer listening to the MBMS subchannel in an ongoing session (e.g. as the response to Unmap Group to Bearer message), shall include the MCPTT session identity in <session-identity> elements; and

iv)	if the intention is to report that the MCPTT client is no longer listening to general purpose MBMS subchannel, shall include the <general-purpose> element set to "false"; and

NOTE 2:	If the MCPTT client reports that the MCPTT client is no longer listening to the general purpose MBMS subchannel, it is implicitly understood that the MCPTT client no longer listens to any MBMS subchannel in ongoing conversations that the MCPTT client previously reported status "listening".

h)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-request-uri> set to the MCPTT ID; and

3)	shall send the SIP MESSAGE request according to 3GPP TS 24.229 [4].

[TS 24.380, clause 10.3.2]

When receiving a Map Group To Bearer message over the general purpose MBMS subchannel, the MBMS interface in the MCPTT client:

1.	shall associate the TMGI in the TMGI field, the MBMS subchannel for audio and for floor control with the MCPTT group identity in the MCPTT Group ID field.

[TS 24.380, clause 10.3.3]

If the MBMS interface receives RTP media packets or floor control messages over the MBMS subchannel, the MBMS interface in the MCPTT client:

1.	if there is an association between the TMGI, the MBMS subchannel for audio and for floor control to an ongoing conversation in a group session:

a.	shall forward the received floor control messages to the floor participant in the conversation; and

b.	if the received RTP medias contains a different SSRC value than the SSRC value used by the MCPTT client, shall forward the received RTP packets to the media mixer in the conversation; and

2.	if there is no such association:

a.	shall ignore the received floor control message or received RTP media packet.

[TS 24.380, clause 10.3.4]

When receiving the Unmap Group To Bearer message over a MBMS subchannel, the MBMS interface in the MCPTT client:

1.	shall remove the association between the TMGI, the MBMS subchannel for audio and for floor control from the conversation in the group session identified by the MCPTT Group ID field, if such an association exists.

[TS 24.380, clause 6.2.4.2.2]

When a call is initiated as described in 3GPP TS 24.379 [2], the floor participant:

1.	shall create an instance of the 'Floor participant state transition diagram for basic operation';

2.	if the originating floor participant receives a floor control message before it receives the SIP 200 (OK) response, shall store the floor control message;

NOTE:	The originating floor participant might receive a floor control message before the SIP 200 (OK) response when initiating, joining or rejoining a call because of processing delays of the SIP 200 (OK) response in the SIP core.

3.	if the established MCPTT call is a chat group call and the SIP INVITE request is not an implicit floor request, shall enter the 'U: has no permission' state; and

4.	if for the established MCPTT call the SIP INVITE request is an implicit floor request:

a.	shall start timer T101 (Floor Request) and initialise counter C101 (Floor Request) to 1;

b.	shall enter the 'U: pending Request' state; and

c.	if the floor participant has received and stored a floor control message before the reception of the SIP 200 (OK) response, shall act as if the floor control message was received in the 'U: pending Request' state after entering the 'U: pending Request' state.

When the floor participant is rejoining an ongoing MCPTT call as described in 3GPP TS 24.379 [2] the floor participant shall enter the 'U: has no permission' state.

[TS 24.380, clause 6.2.4.4.2]

Upon receiving a Floor Granted message from the floor control server or a floor granted indication in a SIP 200 (OK) response in the application and signalling layer, the floor participant:

1.	if the first bit in the subtype of the Floor Granted message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '1' (Floor Granted); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	if the call is not an ambient listening call, shall provide floor granted notification to the user, if not already done;

NOTE:	Providing the floor granted notification to the user prior to receiving the Floor Granted message is an implementation option.

3.	if the Floor Indicator field is included and the B-bit is set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	if the G-bit in the Floor Indicator is set to '1' (Dual floor) shall store an indication that the participant is overriding without revoke;

5.	shall stop the optional timer T103 (End of RTP media), if running;

6.	shall stop timer T101 (Floor Request); and

7.	shall enter the 'U: has permission' state.

[TS 24.380, clause 6.2.4.5.3]

Upon receiving an indication from the user to release the permission to send RTP media, the floor participant:

1.	shall send a Floor Release message towards the floor control server. The Floor Release message:

a.	may include the first bit in the subtype of the Floor Release message set to '1' (acknowledgement is required) as specified in subclause 8.2.2;

NOTE:	It is an implementation option to handle the receipt of the Floor Ack message and what action to take if the Floor Ack message is not received.

b.	if the session is a broadcast call and if the session was established as a normal call, shall include the Floor Indicator with the A-bit set to '1' (Normal call); and

c.	if the Floor Granted message included the G-bit set to '1' (Dual floor), shall include the Floor Indicator with the G-bit set to '1' (Dual floor);

2.	shall remove the indication that the participant is overriding without revoke if this indication is stored;

3.	shall remove the indication that the participant is overridden without revoke if this indication is stored;

4.	shall start timer T100 (Floor Release) and initialize counter C100 (Floor Release) to 1; and

5.	shall enter the 'U: pending Release' state.

[TS 24.380, clause 6.2.4.6.4]

Upon receiving a Floor Idle message, the floor participant:

1.	if the first bit in the subtype of the Floor Idle message to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '5' (Floor Idle); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	may provide a floor idle notification to the MCPTT user;

3.	if the Floor Indicator field is included and the B-bit set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	shall stop timer T100 (Floor Release);

5.	if the session is not a broadcast group call or if the A-bit in the Floor Indicator field is set to '1' (Normal call), shall enter the 'U: has no permission' state; and

6.	if the session was initiated as a broadcast group call:

a.	shall indicate to the MCPTT client the media transmission is completed; and

b	shall enter the 'Releasing' state.

[TS 24.380, clause 6.2.4.3.5]

Upon receiving an indication from the user to request permission to send media, the floor participant:

1.	shall send the Floor Request message toward the floor control server; The Floor Request message:

a.	if a different priority than the normal priority is required, shall include the Floor Priority field with the priority not higher than negotiated with the floor control server as specified in subclause 14.3.3;

b.	if the floor request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Floor Indicator field indicating the relevant call types; and

c.	shall include the Location field:

i.	if the current location of the talker is not available or is not to be reported according to the MCPTT user profile, then the location type is set to '0' (Not provided); or

ii.	if the current location of the talker is available and may be reported according to the MCPTT user profile, then the location type and location value are set as specified in table 8.2.3.21-3;

2.	shall start timer T101 (Floor Request) and initialise counter C101 (Floor Request) to 1; and

3.	shall enter the 'U: pending Request' state.

[TS 24.380, clause 6.2.4.5.8]

Upon receiving a Floor Taken message:

1.	if the G-bit in the Floor Indicator set to '1' (Dual Floor) the floor participant:

a.	shall inform the user that the call is overridden without revoke;

b.	shall store an indication that the participant is overridden without revoke; and

c.	shall remain in the 'U: has permission' state; and

2.	if the I-bit in the Floor Indicator set to '1' (Multi-talker) the floor participant:

a.	may provide a floor taken notification to the user;

b.	should start the optional timer T103 (End of RTP media) for the participant for which Floor Taken message was received;

c.	shall provide a notification to the user indicating the type of call and may provide a list of current talkers; and

d.	shall remain in the 'U: has permission' state.

[TS 24.380, clause 6.2.4.5.9]

Upon receiving the Floor Release Multi Talker message, the floor participant:

1.	shall provide a notification to the user indicating that the participant has released the floor in a multi-talker group; and

2.	shall remain in the 'U: has permission' state.

[TS 24.380, clause 6.2.4.3.2]

Upon receiving a Floor Idle message, the participant:

1.	if the first bit in the subtype of the Floor Idle message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '5' (Floor Idle); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	may provide floor idle notification to the user, if it has not already done so;

3.	shall stop the optional timer T103 (End of RTP media), if it is running; and

4.	shall remain in the 'U: has no permission' state.

6.4.2.3	Test description

6.4.2.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT Client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

Preamble:

-	A pre-activated MBMS bearer exists

-	The <multi-talker-control> element of the MCPTT Group Configuration Defaults as specified in TS 36.579-1 [2], Table 5.5.7.1-1 is set to "true".

-	The <max-number-simultaneous-talkers> element of the MCPTT Group Configuration Defaults as specified in TS 36.579-1 [2], Table 5.5.7.1-1 is set to "2".

-	The UE has performed procedure 'MCPTT UE registration' as specified in TS 36.579-1 [2] clause 5.4.2.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.4.2.3.2	Test procedure sequence

Table 6.4.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a group call with automatic commencement mode and implicit floor request. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO session establishment/modification without provisional responses other than 100 Trying' as described in TS 36.579-1 [2] Table 5.3A.1.3-1 to establish a group call with automatic commencement mode and implicit floor control according to option b.ii of NOTE 1 in TS 36.579.1 [2] Table 5.3A.1.3-1? | - | - | 1 | - |
| 3 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 1 | - |
| 4 | Make the UE (MCPTT client) release the floor. (NOTE 1) | - | - | - | - |
| 5 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT Floor Release – Floor Idle' as described in TS 36.579-1 [2] Table 5.3A.15.3-1? | - | - | 1 | P |
| 6 | The SS pre-activates an MBMS bearer and sends a SIP MESSAGE containing an MBMS bearer announcement message to announce the availability of an MBMS bearer. NOTE: The related E-UTRA/EPC actions are described in TS 36.579-1 [2], clause 5.4.12 'MCX communication over MBMS'. The test sequence below shows only the MCPTT relevant messages exchanged. | <-- | SIP MESSAGE | - | - |
| 7 | Check: Does the UE (MCPTT client) respond with a SIP 200 (OK)? | --> | SIP 200 (OK) | 2 | P |
| 8 | Check: Does the UE (MCPTT client) send a SIP MESSAGE containing an MBMS Bearer listening status report? | --> | SIP MESSAGE | 2 | P |
| 9 | The SS responds with a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 10 | Make the UE (MCPTT client) request the floor. (NOTE 1) | - | - | - | - |
| 11 | Check: Does the UE (MCPTT client) send a Floor Request message? | --> | Floor Request | 1 | P |
| 12 | The SS decides that a MBMS subchannel shall be used for the conversation and sends a Map Group To Bearer message over the general purpose MBMS subchannel. NOTE: The related E-UTRA/EPC actions are described in TS 36.579-1 [2], clause 5.4.12 'MCX communication over MBMS'. The test sequence below shows only the MCPTT relevant messages exchanged. | <-- | Map Group To Bearer | - | - |
| 13 | Check: Does the UE (MCPTT client) respond to the Map Group To Bearer message by sending a SIP MESSAGE containing an MBMS Bearer listening status report? | --> | SIP MESSAGE | 3 | P |
| 14 | The SS responds with a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 15 | The SS sends a Floor Granted message with request for acknowledgement using unicast messaging. | <-- | Floor Granted | - | - |
| 16 | Check: Does the UE (MCPTT client) respond with a Floor Ack message? | --> | Floor Ack | 1 | P |
| 17 | The SS sends a SIP MESSAGE containing a MuSiK download message to set the explicit MuSiK key to be used for the floor control encryption over the MBMS subchannel. | <-- | SIP MESSAGE | - | - |
| 18 | Check: Does the UE (MCPTT client) respond with a SIP 200 (OK)? | --> | SIP 200 (OK) | 4 | P |
| 19 | The SS sends a Floor Taken (by client A) message over the MBMS subchannel. | <-- | Floor Taken | - | - |
| 20 | The SS (MCPTT server) sends a Floor Taken message with I-bit in the Floor Indicator set to '1' (Multi-talker) over the MBMS subchannel. | <-- | Floor Taken | - | - |
| 21 | Check: Does the UE (MCPTT client) provide a notification to the user indicating the type of call? (NOTE 1) | - | - | 5 | P |
| 22 | The SS (MCPTT server) sends a Floor Release Multi Talker message over the MBMS subchannel. | <-- | Floor Release Multi Talker | - | - |
| 23 | Check: Does the UE (MCPTT client) provide a notification to the user indicating that the participant has released the floor in a multi-talker group? (NOTE 1) | - | - | 5 | P |
| 24 | Make the UE (MCPTT client) release the floor. (NOTE 1) | - | - | - | - |
| 25 | Check: Does the UE (MCPTT client) send a Floor Release message? | --> | Floor Release | 6 | P |
| - | EXCEPTION: Step 26a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE requests an acknowledgement to the Floor Release message. | - | - | - | - |
| 26a1 | The SS responds with a Floor Ack message using unicast messaging | <-- | Floor Ack | - | - |
| 27 | The SS sends a Floor Idle message with request for acknowledgement over the MBMS subchannel. | <-- | Floor Idle | - | - |
| 28 | Check: Does the UE (MCPTT client) respond with a Floor Ack message over the MBMS subchannel? | --> | Floor Ack | 6 | P |
| 29 | The SS ends the conversation on the MBMS subchannel and sends an Unmap Group to Bearer message on the MBMS subchannel. NOTE: The related E-UTRA/EPC actions are described in TS 36.579-1 [2], clause 5.4.12 'MCX communication over MBMS'. The test sequence below shows only the MCPTT relevant messages exchanged. | <-- | Unmap Group to Bearer | - | - |
| 30 | Check: Does the UE (MCPTT client) respond to the Unmap Group To Bearer message with a SIP MESSAGE containing an MBMS bearer listening status report? | --> | SIP MESSAGE | 3 | P |
| 31 | The SS responds with a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 32 | The SS cancels the MBMS bearer announcement and sends a SIP MESSAGE message. | <-- | SIP MESSAGE | - | - |
| 33 | Check: Does the UE (MCPTT client) respond with a SIP 200 (OK)? | --> | SIP 200 (OK) | 2 | P |
| 34 | Check: Does the UE (MCPTT client) send a SIP MESSAGE containing an MBMS Bearer listening status report? | --> | SIP MESSAGE | 2 | P |
| 35 | The SS responds with a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 36 | The SS sends a Floor Idle message with request for acknowledgement over the unicast channel | <-- | Floor Idle | - | - |
| 37 | Check: Does the UE respond with a Floor Ack message? | --> | Floor Ack | 1 | P |
| 38 | Make the UE (MCPTT client) release the call. (NOTE 1) | - | - | - | - |
| 39 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release' as described in TS 36.579-1 [2] Table 5.3.10.3-1? | - | - | 7 | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.4.2.3.3	Specific message contents

Table 6.4.2.3.3-1: SIP INVITE (step 2, Table 6.4.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.3A.1.4-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP Message as described in Table 6.4.2.3.3-2 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.4.2.3.3-3 |  |  |  |

Table 6.4.2.3.3-2: SDP in SIP INVITE (Table 6.4.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1, condition FIRST_SDP_FROM_UE, INITIAL_SDP_OFFER, IMPLICIT_GRANT_REQUESTED

Table 6.4.2.3.3-3: MCPTT-Info in SIP INVITE (Table 6.4.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition GROUP-CALL, INVITE_REFER

Table 6.4.2.3.3-4: SIP 200 (OK) (step 2, Table 6.4.2.3.2-1; step 4, TS 36.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.3A.1.4-2, condition INVITE_RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.4.2.3.3-5 |  |  |  |

Table 6.4.2.3.3-5: SDP in SIP 200 (OK) (Table 6.4.2.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1, condition SDP_ANSWER, IMPLICIT_GRANT_REQUESTED

Table 6.4.2.3.3-7: SIP MESSAGE (step 6, Table 6.4.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| addr-spec |  |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/sdp" |  |  |  |
| Content-Disposition | "render" |  |  |  |
| MIME-part-body | SDP message as described in Table 6.4.2.3.3-8 |  |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.2.3.3-9 |  |  |  |

Table 6.4.2.3.3-8: SDP in SIP MESSAGE (Table 6.4.2.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media descriptions |  |  |  |  |
| media description |  | m= line media = audio |  |  |
| media | "audio" |  |  |  |
| port | "9" |  |  |  |
| proto | "RTP/AVP" |  |  |  |
| fmt | "99" |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" or “IP6” depending on IP address |  |  |  |
| connection-address | "0.0.0.0" (IPv4) or domain name within “.invalid” DNS top level domain (IPv6) |  |  |  |
| media description |  | m= line media = application |  |  |
| media | "application" |  |  |  |
| port | "9" |  |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" or “IP6” depending on IP address |  |  |  |
| connection-address | "0.0.0.0" (IPv4) or domain name within “.invalid” DNS top level domain (IPv6) |  |  |  |
| media description |  | m= line media = application |  |  |
| media | "application" |  |  |  |
| port | port number assigned by the SS |  |  |  |
| proto | "udp" |  |  |  |
| fmt | "MCPTT" |  |  |  |
| Connection Data |  | c= line |  |  |
| nettype | "IN" |  |  |  |
| addrtype | "IP4" |  |  |  |
| connection-address | multicast IP assigned by the SS |  |  |  |
| media attribute |  | a= line attribute = key-mgmt |  |  |
| key-mgmt |  |  | TS 24.379 [9] clause 14.2.2.2 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in TS 36.579-1 [2], Table 5.5.9.1-4. | MSCCK distribution as specified in TS 33.180 [33] Annex H | RFC 4567 [44] |  |

Table 6.4.2.3.3-9: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.2.3.3-7)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status | not present |  |  |  |
| announcement |  |  |  |  |
| TMGI |  |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [32] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-1 | Mobile Country Code |  |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-1 | Mobile Network Code |  |  |
| QCI | "65" | Mission Critical user plane Push To Talk voice |  |  |
| mbms-service-areas | "0" | The selected value is randomly chosen. The value 0 has a special meaning; it shall denote the whole PLMN as the MBMS Service Area |  |  |
| GPMS | "2" | The number of the "m=application" media line in the application/sdp MIME |  |  |
| version | "1" |  |  |  |

Table 6.4.2.3.3-10: SIP MESSAGE (step 8, Table 6.4.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | The value received in the P-Asserted-Identity header field of the announcement message (px_MBMS_Service_ID) |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCPTT_User_A_ID |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.2.3.3-11 |  |  |  |

Table 6.4.2.3.3-11: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.2.3.3-10)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status |  |  |  |  |
| mbms-listening-status | "listening" |  |  |  |
| session-id | not present |  |  |  |
| general-purpose | "true" |  |  |  |
| TMGI | same value as the announcement in the SIP MESSAGE |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.2.3.3-13: SIP MESSAGE (step 13, Table 6.4.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | The value received in the P-Asserted-Identity header field of the announcement message (px_MBMS_Service_ID) |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCPTT_User_A_ID |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.2.3.3-14 |  |  |  |

Table 6.4.2.3.3-14: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.2.3.3-13)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status |  |  |  |  |
| mbms-listening-status | "listening" |  |  |  |
| session-id | px_sesson_A_ID |  |  |  |
| general-purpose | not present |  |  |  |
| TMGI | same value as the announcement in the SIP MESSAGE |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.2.3.3-16: Floor Ack (steps 16, 28, 37, Table 6.4.2.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.6.11-1, condition UPLINK

Table 6.4.2.3.3-17: SIP MESSAGE (step 17, Table 6.4.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| addr-spec |  |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.2.3.3-18 |  |  |  |
| MIME body part |  | MIKEY message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | As described in TS 36.579-1 [2], Table 5.5.9.1-5. | MIKEY message containing the MuSiK |  |  |

Table 6.4.2.3.3-18: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.2.3.3-17)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status | not present |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |
| anyExt |  |  |  |  |
| mbms-explicitMuSiK-download |  |  |  |  |
| group | px_MCPTT_Group_A_ID |  |  |  |

Table 6.4.2.3.3-20: Floor Taken (step 20, Table 6.4.2.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.6.7-1, condition Multi-Talker

Table 6.4.2.3.3-22: Floor Ack (step 26a1, Table 6.4.2.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.6.11-1, condition DOWNLINK

Table 6.4.2.3.3-23: SIP MESSAGE (step 30, Table 6.4.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | The value received in the P-Asserted-Identity header field of the announcement message (px_MBMS_Service_ID) |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCPTT_User_A_ID |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.2.3.3-24 |  |  |  |

Table 6.4.2.3.3-24: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.2.3.3-23)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status |  |  |  |  |
| mbms-listening-status | "not-listening" |  |  |  |
| session-id | px_sesson_A_ID |  |  |  |
| general-purpose | not present |  |  |  |
| TMGI | same value as the announcement in the SIP MESSAGE |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.2.3.3-25: SIP MESSAGE (step 32, Table 6.4.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | px_MBMS_Service_ID | The MBMS public service identity of the participating MCPTT function |  |  |
| port | not present |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.2.3.3-26 |  |  |  |

Table 6.4.2.3.3-26: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.2.3.3-25)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status | not present |  |  |  |
| announcement |  |  |  |  |
| TMGI |  |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [X] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |  |
| QCI | "65" | Mission Critical user plane Push To Talk voice |  |  |
| mbms-service-areas | not present |  |  |  |
| GPMS | not present |  |  |  |
| version | "1" |  |  |  |

Table 6.4.2.3.3-27: SIP MESSAGE (step 34, Table 6.4.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | The value received in the P-Asserted-Identity header field of the announcement message (px_MBMS_Service_ID) |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Identity |  |  |  |  |
| PPreferredID-value | px_MCPTT_User_A_ID |  |  |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| Message-body | MIME body not including MCPTT-Affiliation-Command | not including any MIME body part with Content-Type "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |
| MIME body part |  | MCPTT MBMS usage info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-mbms-usage-info+xml" |  |  |  |
| MIME-part-body | MCPTT-MBMS-Usage-Info as described in Table 6.4.2.3.3-28 |  |  |  |

Table 6.4.2.3.3-28: MCPTT-MBMS-Usage-Info in SIP MESSAGE (Table 6.4.2.3.3-27)

| Derivation Path: TS 24.379 [9], Clause F.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-mbms-usage-info |  |  |  |  |
| mbms-listening-status |  |  |  |  |
| mbms-listening-status | "not-listening" |  |  |  |
| session-id | not present |  |  |  |
| general-purpose | "false" |  |  |  |
| TMGI | same value as the announcement in the SIP MESSAGE |  |  |  |
| announcement | not present |  |  |  |
| version | "1" |  |  |  |