3GPP TSG RAN Meeting #99																		RP-230621

Rotterdam, Netherlands, March 20-23, 2023

Source: 	ZTE

Title: 	Discussion on NG-RAN support of eNPN Phase 2

Agenda item:	9.1.3

Document for:	Discussion/Decision

Introduction

Non-Public Network was introduced in Rel-16 with basic functions, and further enhanced in Rel-17. This contribution provides our view on NG-RAN support for enhanced NPN phase2 in Rel-18.

Discussion

SA2 has completed the study on enhanced support of Non-Public Networks phase 2, and is still working on eNPN_Ph2 WID in SP-220805 approved at SA#97(revised in SP-221340 at SA#98).

This WID was originally planned to be completed in March, but now postponed to June. The overall status of the SID and WID of eNPN phase2 at SA#99 can be seen below :

According to the eNPN_Ph2 WID in Rel-18, SA2 focuses on the following enhancements:

1.Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection.

2.Support for non-3GPP access for SNPN.

3.Support for providing access to Localized Services.

For enhancement 1, NG-RAN needs to support signaling Equivalent SNPN list of the UE over NGAP and XnAP e.g., by including it in the Mobility Restriction List; NG-RAN supports signaling the selected SNPN ID to the target NG-RAN node during Xn and NG based handovers. In addition, it is not clear whether there is an impact on NR-DC operation, e.g, across SNPNs. Anyway, inter-SNPN mobility has impact on both RAN2 and RAN3 specs, UE/NG-RAN should take equivalent SNPN list into consideration, for supporting relevant functions, e.g. idle/connected mode mobility.

Observation 1: Support for inter-SNPN mobility has impact on both RAN2 and RAN3 specs.

In order to support enhancement 2, it seems only few impact on RAN3 specs, e.g, N3IWF being able to include the "selected NID" in the [NGAP] INITIAL UE MESSAGE.

Observation 2: Support for non-3GPP access for SNPN has few impact on RAN3 specs.

SA2 has approved some CRs for TS23.501 related to enhancement3 (packed in SP-230063 at SA#99) . No RAN impact can be identified regarding these CRs. Since the SA2 WID is ongoing, RAN WGs need to follow SA2 progress to check if any RAN impact.

Observation 3: Until now, no RAN impact can be identified for Localized Services supporting.

Based on the above observations, Since some enhancement involves RAN2 and RAN3, we give the following proposal.

Proposal 1: RAN is kindly asked to allocate new WID for RAN work for Enhanced Non-Public Networks Phase 2 supporting.

Conclusion

The following observations and proposal are provided:

Observation 1: Support for inter-SNPN mobility has impact on both RAN2 and RAN3 specs.

Observation 2: Support for non-3GPP access for SNPN has few impact on RAN3 specs.

Observation 3: Until now, no RAN impact can be identified for Localized Services supporting.

Proposal 1: RAN is kindly asked to allocate new WID for RAN work for Enhanced Non-Public Networks Phase 2 supporting.