3GPP TSG RAN meeting #99										RP-230636

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:			9.3.4.7

| WI / SI Name | Further enhancements on NR and MR-DC measurement gaps and measurements without gaps |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MG_enh2 |  |  |  |  |
| Unique ID | 950182 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222337 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 45% RAN4: 45% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ato (Tsang-Wei) YU, Rui HUANG |
|  | Company | MediaTek Inc., Intel Corporation |
|  | Email | ato.yu@mediatek.com, rui.huang@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #106 (Feb, 2023, Athens, Greece)

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

    - The following agreements were captured in the WF [64]

    - Scope and general issues

      - RAN4 has discussed this issue for several meetings and the current definitions of Type-1 MG and Type-2 MG are sufficient. No need to specify another definition.

    - Case 1 (Pre-configured MG and concurrent MG)

      - It is up to UE capability to support the simultaneous activation/deactivation of two Pre-MGs in the same FR

      - Gap combination configuration of case 1 can use gap combination of concurrent gaps defined in TS38.133 table 9.1.8-1 as formatting baseline with the clarification that each configured gap can be Pre-MG or Type-2 MG.

      - Definitions for simultaneous and non-simultaneous multiple Pre-MGs activation/deactivation were agreed.

      - No requirements are defined for equal priority in Pre-MG with concurrent gaps

      - Dynamic collisions are gap collisions involving at least one [activated] pre-configured MG, where gap instances of other MGs (which has lower priority) are dropped.

        - [activated] is based on the assumption that only activated Pre-MG can cause collisions.

      - RAN4 shall not define implicit association of intra-frequency layers with Pre-MG (i.e. the priority of a Pre-MG which concurrent with other gaps should be up to network assignment)

    - Case 2 (NCSG and concurrent MG)

      - Gap combination configuration of case 2 can use gap combination of concurrent gaps defined in TS38.133 table 9.1.8-1 as formatting baseline with the clarification that each configured gap can be NCSG or Type-2 MG.

      - No requirements are defined for equal priority in NCSG with concurrent gaps

      - Update the existing gap interruption requirements for Case 2 as follows.

        - A slot is considered as interrupted if it is interrupted by an occasion of any of the configured concurrent measurement gaps following the measurement gap interruption requirements in clause 9.1.2, or by VIL occasion of any of the configured NCSG following the NCSG interruption requirements in clause 9.1.9.1, except for a dropped measurement gap or NCSG occasion.

- RRM core requirements for measurements without gaps

    - The following agreements were captured in the WF [67] with an LS [69] sent to RAN2

    - Measurement without gaps for UEs reporting NeedForGapsInfoNR

      - Inform RAN2 on RAN4 #105 agreements on signaling for UE to indicate UE supporting ”no-gap” with interruption

      - At least interruption length will be defined in the requirements of interruption

      - Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2

      - Condition for intra-frequency requirements without gaps

        - When the target SSB is completely contained in active BWP of UE or the active downlink BWP is initial BWP, the intra-frequency measurement should be without gap regardless of the NeedForGaps’ status reporting.

        - When the target SSB is outside active BWP (not initial BWP), the intra-frequency measurement will be

          - without gap without interruption, if UE reports ‘[no gap no interruption:TBD]’ for the intra-band;

          - without gap with interruption, if UE reports ‘[no gap with interruption:TBD]’ for the intra-band

    - Inter-RAT measurement without gap

      - Add the request on the additional signaling from UE to indicate the inter-RAT NR measurements without gap but interruption needed in the inter-RAT NR measurement without gap (case a-1) to the LS to RAN2

#### 2.4.2	Remaining Open issues

RAN4 #106 (Feb, 2023, Athens, Greece)

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

    - The following open issues were captured in the WF [64]

    - Scope and general issues

      - Whether to consider Type-1 MG

    - Case 1 (Pre-configured MG and concurrent MG)

      - Detail in gap combination configuration

      - Delay requirements for simultaneous multiple Pre-MGs activation/deactivation

      - UE behavior when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion

      - Whether to define new capability/requirements for dynamic collision

      - Measurement delay requirements

    - Case 2 (NCSG and concurrent MG)

      - Whether to consider a new capability for NCSG + NCSG in an FR

      - Whether the same RF chain is assumed for the two NCSG patterns

      - Detail in gap combination configuration

      - Whether to consider parallel measurements upon gap collision

      - Whether to support parallel measurements for two NCSG

      - Potential changes for NCSG upon SCell activation

- RRM core requirements for measurements without gaps

    - The following open issues were captured in the WF [67]

    - Measurement without gaps for UEs reporting NeedForGapsInfoNR

      - FFS detail interruption ratio, possible measurement delay requirements extension and the need to define the interruption location

      - Detail of interruption length

      - Requirement for intra/inter-freq measurement without gap when interruption allowed (case 2)

      - Requirement for inter-freq measurement without gap (Inter-f case 1)

      - Whether to define a mapping between NeedForGap and NCSG capabilities when UE supports both of them

      - FFS other impacts on the legacy UE behavior

      - General principles to define scheduling restriction requirements

      - FFS on top of which existing requirements to define scheduling restriction requirements

      - FFS whether and how to define a default SMTC pattern for scheduling restriction

    - Inter-RAT measurement without gap

      - On top of which UE capability to support the inter-RAT LTE measurement requirements when UE has vacant RF chain available (Case b-1)

      - On top of which UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP (Case b-2)

      - Whether to introduce an additional capability to support inter-RAT measurement without gap with mixed numerology

      - Frameworks used to define inter-RAT measurement without gap

      - Scheduling restriction for inter-RAT NR/LTE measurements

      - Whether any searcher limitation

      - CSSF

      - Effective measurement window

      - Release independent

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

- R4-2301135	Discussion on RRM core requirement for pre-MG, multiple concurrent MGs and NCSG	LG Electronics Finland

- R4-2302333	On joint requirements for Rel-17 measurement gap enhancements	Qualcomm Incorporated

- R4-2300220	On R18 gap enhancement scope and general issues	Apple

- R4-2300471	Discussion on using scenarios for pre-configured MGs, multiple concurrent MGs and NCSG	Intel Corporation

- R4-2300873	Discussion on general issues for pre-configured MGs, multiple concurrent MGs and NCSG	CMCC

- R4-2301279	Consideration on scope and general issues for Pre-MG, concurrent MGs and NCSG	vivo

- R4-2301405	Discussion on general and scope of enhancements on measurement gaps	ZTE Corporation

- R4-2300221	On R18 gap enhancement case 1 requirements	Apple

- R4-2300472	Discussion on RRM requirements for combinations of pre-configured MGs and multiple concurrent MGs	Intel Corporation

- R4-2300584	Discussion on case 1 requirements for combination of pre-MG and concurrent MGs	CATT

- R4-2300865	Discussion on Pre-configured MG and concurrent MG	CMCC

- R4-2300888	RRM requirements for the combination of Pre-MG and concurrent MG	Xiaomi

- R4-2301011	Discussion on Case 1 requirements for Pre-MG and concurrent MG	China Telecom

- R4-2301280	Consideration on case 1 requirements for Pre-MG and concurrent MGs	vivo

- R4-2301406	Discussion on Case 1 RRM requirements	ZTE Corporation

- R4-2301639	Discussion on case 1 requirements for Pre-MG and concurrent MG	OPPO

- R4-2301850	Discussion on Case 1 requirements	Nokia, Nokia Shanghai Bell

- R4-2301973	Discussion on joint working of pre-MG and con-MG	Huawei, HiSilicon

- R4-2302118	Discussion on PreMG and ConMGs	Ericsson

- R4-2302309	Discussion on case 1 requirements (Pre-configured MG and concurrent MG)	MediaTek inc.

- R4-2300222	On R18 gap enhancement case 2 requirements	Apple

- R4-2300473	Discussion on RRM requirements for combinations of NCSG and multiple concurrent MGs	Intel Corporation

- R4-2300585	Discussion on case 2 requirements for combination of NCSG and concurrent MGs	CATT

- R4-2300864	Discussion on NCSG and concurrent MG	CMCC

- R4-2300889	RRM requirements for the combination of NCSG and concurrent MG	Xiaomi

- R4-2301012	Discussion on Case 2 requirements for NCSG and concurrent MG	China Telecom

- R4-2301281	Consideration on case 2 requirements for concurrent MGs and NCSG	vivo

- R4-2301407	Discussion on Case 2 RRM requirements	ZTE Corporation

- R4-2301640	Discussion on case 2 requirements NCSG and conMG	OPPO

- R4-2301851	Discussion on Case 2 requirements	Nokia, Nokia Shanghai Bell

- R4-2301974	Discussion on joint working of NCSG and con-MG	Huawei, HiSilicon

- R4-2302119	Discussion on NCSG and ConMGs	Ericsson

- R4-2302310	Discussion on case 2 requirements (NCSG and concurrent MG)	MediaTek inc.

- R4-2300223	On measurement without gaps for UEs reporting NeedForGapsInfoNR	Apple

- R4-2300224	LS On measurement without gaps for UEs reporting NeedForGapsInfoNR	Apple

- R4-2300440	Requirements for measurement without gap	Qualcomm Incorporated

- R4-2300474	Discussion on measurements without gaps when UE reporting NFG	Intel Corporation

- R4-2300528	Discussion on measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2300586	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR	CATT

- R4-2300872	Discussion on measurements without gaps for UEs reporting NeedForGapsInfoNR	CMCC

- R4-2300900	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	Xiaomi

- R4-2301282	Considerations on Measurement without gaps for UEs reporting NeedForGapsInfoNR	vivo

- R4-2301408	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	ZTE Corporation

- R4-2301641	On measurement without gaps for UEs reporting NeedForGapsInfoNR	OPPO

- R4-2301975	Discussion on requirements for NeedForGaps	Huawei, HiSilicon

- R4-2302120	Discussion on NeedForGaps measurement	Ericsson

- R4-2302311	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	MediaTek inc.

- R4-2300225	On R18 inter-RAT measurement without gap	Apple

- R4-2300441	Requirements for Inter-RAT measurement without gap	Qualcomm Incorporated

- R4-2300475	Discussion on inter-RAT measurement without gaps	Intel Corporation

- R4-2300587	Discussion on RRM requirements for Inter-RAT measurement without gap	CATT

- R4-2300868	Discussion on inter-RAT measurements without gaps	CMCC

- R4-2300901	Discussion on inter-RAT measurement without gap	Xiaomi

- R4-2301283	"Considerations on 	Inter-RAT measurement without gap"	vivo

- R4-2301642	On RRM requirements for Inter-RAT measurement without gap	OPPO

- R4-2301976	Discussion on inter-RAT MG-less measurement	Huawei, HiSilicon

- R4-2302121	Discussion on Inter-RAT measurement without gap	Ericsson

- R4-2302312	Discussion on inter-RAT measurements	MediaTek inc.

- R4-2302771	Topic summary for [106][213] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2302772	Topic summary for [106][214] NR_MG_enh2_part2	Moderator (Intel)

- R4-2303172	Ad-hoc minutes for NR and MR-DC measurement gaps and measurements without gaps WI 	MediaTek inc.

- R4-2303301	Ad-hoc minutes for NR and MR-DC measurement gaps and measurements without gaps WI (part 2) 	MediaTek inc.

- R4-2302771	Topic summary for [106][213] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2303197	WF on Rel-18 NR MG enhancements	MediaTek

- R4-2302772	Topic summary for [106][214] NR_MG_enh2_part2	Moderator (Intel)

- R4-2303198	WF on Rel-18 NR measurements without gaps	Intel

- R4-2303305	WF on Rel-18 NR measurements without gaps	Intel

- R4-2303199	LS on measurements without gap	Intel, CATT

- R4-2303306	LS on measurements without gap	Intel, CATT

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template