Title:	Spatial Channel Model Text Description

File:	input.doc

Source:	Spatial Channel Model AHG (Combined ad-hoc from 3GPP & 3GPP2)

Date:	December 19, 2002

Notice

©2002 Third Generation Partnership Project Two (3GPP2).  All rights reserved.  Permission is granted for copying, reproducing, or duplicating this document only for the legitimate purposes of 3GPP2 and its organizational partners.  No other copying, reproduction, or distribution is permitted.

1	Introduction	4

1.1	Terminology (usage in 3GPP & 3GPP2)	4

1.2	Scope	4

1.2.1	System level evaluation.	4

1.2.2	Link level evaluation.	5

2	Link Level Channel Models for Calibration Purposes	5

2.1	Purpose	5

2.2	Link Level Channel Model Parameter Summary	5

2.3	Spatial Parameters per Path	7

2.4	BS and MS Array Topologies	7

2.5	Spatial Parameters for the BS	7

2.5.1	BS Antenna Pattern	7

2.5.2	Per Path BS Angle Spread (AS)	10

2.5.3	Per Path BS Angle of Arrival	10

2.5.4	Per Path BS Power Azimuth Spectrum	10

2.6	Spatial Parameters for the MS	11

2.6.1	MS Antenna Pattern	11

2.6.2	Per Path MS Angle Spread (AS)	11

2.6.3	Per Path MS  Angle of Arrival	11

2.6.4	Per Path MS Power Azimuth Spectrum	12

2.6.5	MS Direction of Travel	12

2.6.6	Per Path Doppler Spectrum	12

2.7	Generation of Channel Model	12

2.8	Calibration and Reference Values	13

3	System Level Spatial Channel Model	13

3.1	System Wide Definitions and Parameters	14

3.2	Scenarios	15

3.3	Channel Generation Steps	16

3.4	BS and MS Array Topologies	18

3.4.1	Polarized arrays	18

3.4.2	Reference Antenna Configurations	19

3.4.3	General Modeling Assumptions	20

3.4.4	Micro-cell Assumptions	20

3.5	Options for Urban Macro and Micro Environments	20

3.5.1	Far scatterer clusters	20

3.5.2	Line of sight	21

3.5.3	Urban canyon	22

3.6	Correlation Between Channel Parameters	23

3.6.1	Correlation between Delay Spread, Angle Spread, and Log Normal Shadowing	23

3.7	System Level Calibration	25

4	Appendix A: MMSE receiver description	29

5	Reference	29

# Introduction

This document details the current discussion of the combined 3GPP-3GPP2 Spatial Channel Ad-hoc group.  A similar document, developed independently in the 3GPP2 Spatial Channel Modeling Ad-hoc group, was used for reference.

## Terminology (usage in 3GPP & 3GPP2)

In this document the following are terms that are commonly used interchangeably and are equivalent.  To promote consistency, the term on the left will be preferred in this document unless otherwise stated.

MS = Mobile Station = UE = User Equipment = Terminal = Subscriber Unit

BS = Base Station = Node-B = BTS

AS = Angle Spread = Azimuth Spread = PAS

Path = Ray

Path Component = Sub-ray

PAS = Power Azimuth Spectrum

DoT = Direction of Travel

AoA = Angle of Arrival

PDP = Power Delay Profile

## Scope

The scope of the 3GPP-3GPP2 SCM AHG is to develop and specify parameters and methods associated with the spatial channel modeling that are common to the needs of the 3GPP and 3GPP2 organizations (harmonization).  The scope includes development of specifications for:

### System level evaluation.

Within this category, a list of four focus areas are identified, however the emphasis of the SCM AHG work is on items a & b.

- Physical parameters (e.g. power delay profiles, angle spreads, dependencies between parameters)

- System evaluation methodology.

- Antenna arrangements, reference cases and definition of minimum requirements.

- Some framework (air interface) dependent parameters.

### Link level evaluation.

The link level models are defined only for calibration purposes. It is a common view within the group that the link level simulation assumptions will not be used for evaluation and comparison of proposals.

# Link Level Channel Models for Calibration Purposes

(Editor’s Note: A harmonized link level model is being proposed, and further discussion is expected prior to agreement.  This includes all aspects of the link level model described in Section 2.)

## Purpose

Link level simulations alone will not be used for algorithm comparison because they reflect only one snapshot of the channel behavior. Furthermore, they do not account for system attributes such as scheduling and HARQ. For these reasons, link level simulations do not allow any conclusions about the typical behavior of the system. Only system level simulations can achieve that. Therefore we require system level simulations for the final algorithm comparison.

Link level simulations will not be used to compare performance of different algorithms. Rather, they will be used only for calibration, which is the comparison of performance results from different implementations of a given algorithm.

## Link Level Channel Model Parameter Summary

(Editor’s Note: The following table is under discussion as a harmonized set of link level parameters and values.)

Table 2-1. Summary of Suggested SCM Link Level Parameters for Calibration Purposes

| Model |  |  | Case I |  | Case II |  | Case III |  | Case IV |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding 3GPP Designator* |  |  | Case B |  | Case C |  | Case D |  | Case A |  |
| Corresponding 3GPP2 Designator* |  |  | Model A, D, E |  | Model C |  | Model B |  | Model F |  |
| PDP |  |  | Modified Pedestrian A |  | Vehicular A |  | Pedestrian B |  | Single Path |  |
| # of Paths |  |  | - 4+1 (LOS on, K = 6dB) - 4 (LOS off) |  | 6 |  | 6 |  | 1 |  |
| Relative Path Power (dB) |  | Delay (ns) | - 0.0 - -Inf | 0 | 0,0 | 0 | 0.0 | 0 | 0 | 0 |
|  |  |  | - -6.51 - 0.0 | 0 | -1.0 | 310 | -0.9 | 200 |  |  |
|  |  |  | - -16.21 - -9.7 | 110 | -9.0 | 710 | -4.9 | 800 |  |  |
|  |  |  | - -25.71 - –19.2 | 190 | -10.0 | 1090 | -8.0 | 1200 |  |  |
|  |  |  | - -29.31 - -22.8 | 410 | -15.0 | 1730 | -7.8 | 2300 |  |  |
|  |  |  |  |  | -20.0 | 2510 | -23.9 | 3700 |  |  |
| Speed (km/h) |  |  | - 3 - 30, 120 |  | 3, 30, 120 |  | 3, 30, 120 |  | 3 |  |
| UE/Mobile Station | Topology |  | Reference 0.5λ |  | Reference 0.5λ |  | Reference 0.5λ |  | N/A |  |
|  | PAS |  | - LOS on: Fixed AoA for LOS component, remaining power has 360 degree uniform PAS. - LOS off: PAS with a Lapacian distribution, RMS angle spread of 35 degrees per path |  | RMS angle spread of 35 degrees per path with a Lapacian distribution Or 360 degree uniform PAS. |  | RMS angle spread of 35 degrees per path with a Lapacian distribution |  | N/A |  |
|  | DoT (degrees) |  | 0 |  | 22.5 |  | -22.5 |  | N/A |  |
|  | AoA (degrees) |  | 22.5 (LOS component) 67.5 (all other paths) |  | 67.5 (all paths) |  | 22.5 (odd numbered paths), -67.5 (even numbered paths) |  | N/A |  |
| Node B/ Base Station | Topology |  | Reference: ULA with 0.5λ-spacing or 4λ-spacing or 10λ-spacing |  |  |  |  |  | N/A |  |
|  | PAS |  | Lapacian distribution with RMS angle spread of 2 degrees or 5 degrees, per path depending on AoA/AoD |  |  |  |  |  | N/A |  |
|  | AoD/AoA (degrees) |  | 50 for 2 RMS angle spread per path 20 for 5 RMS angle spread per path |  |  |  |  |  | N/A |  |

*Designators correspond to channel models previously proposed in 3GPP and 3GPP2 ad-hoc groups.

## Spatial Parameters per Path

Each resolvable path is characterized by its own spatial channel parameters (angle spread, angle of arrival, power azimuth spectrum). All paths are assumed independent. These assumptions apply to both the BS and the MS specific spatial parameters.  The above assumptions are in effect only for the Link Level channel model.

## BS and MS Array Topologies

The spatial channel model should allow any type of antenna configuration to be selected, although details of a given configuration must be shared to allow others to reproduce the model and verify the results.

Calibrating simulators at the link level requires a common set of assumptions including a specific set of antenna topologies to define a baseline case.  At the MS, the reference element spacing is 0.5. At the BS, three values for reference element spacing are defined: 0.5, 4, and 10.

## Spatial Parameters for the BS

### BS Antenna Pattern

The 3-sector antenna pattern used for each sector, Reverse Link and Forward Link, is plotted in Figure 2-1 and is specified by

(1)

θis defined as the angle between the direction of interest and the boresight of the antenna, θ3dB is the 3dB beamwidth in degrees, and  Am is the maximum attenuation.  For a 3 sector scenario θ3dB is 70o, Am=20dB,and the antenna boresight pointing direction is given by Figure 2-2.  For a 6 sector scenario θ3dB is 35o, Am=23dB, which results in the pattern shown in Figure 2-3, and the boresight pointing direction defined by Figure 2-4.  The boresight is defined to be the direction to which the antenna shows the maximum gain.  The gain specified by previous 3GPP documents[] for the 3-sector 70o antenna is 14dBi.  By reducing the beamwidth by half to 35o, the corresponding gain will be 3dB higher resulting in 17dBi. The antenna pattern shown is targeted for diversity oriented implementations (i.e. large inter-element spacings). For beamforming applications that require small spacings, alternative antenna designs may have to be considered leading to a different antenna pattern.

Figure 2-1  Antenna Pattern for 3-Sector Cells

Figure 2-2 Boresight pointing direction for 3-sector cells

Figure 2-3  Antenna Pattern for 6-Sector Cells

Figure 2-4 Boresight Pointing direction for 6-Sector Cells

### Per Path BS Angle Spread (AS)

The base station per path angle spread is defined as the root mean square (RMS) of angles with which an arriving path’s power is received by the base station array. The individual path powers are defined in the temporal channel model described in Table 2-1. Two values of BS angle spread (each associated with a corresponding mean angle of arrival, AoA) are considered:

-	AS: 2 degrees at AoA 50 degrees

- AS: 5 degrees at AoA 20 degrees

It should be noted that attention should be paid when comparing the link level performance between the two angle spread values since the BS antenna gain for the two corresponding AoAs will be different.  The BS antenna gain is applied to the path powers specified in Table 2-1.

### Per Path BS Angle of Arrival

The Angle of Arrival (AoA) or Angle of Departure (AoD) is defined to be the mean angle with which an arriving or departing path’s power is received or transmitted by the BS array with respect to the boresite. The two values considered are:

- AoA: 50 degrees (associated with the RMS Angle Spread of 2 degrees)

- AoA: 20 degrees (associated with the RMS Angle Spread of 5 degrees)

### Per Path BS Power Azimuth Spectrum

The Power Azimuth Spectrum (PAS) of a path arriving at the base station is assumed to have a Laplacian distribution. For an incoming AOA θ and RMS angle-spread , the BS per path PAS value at an angle  is given by expression  below:

P(θ,σ,θ)=Noexp−2θ−θσG(θ)

where both angles θand  are given with respect to the boresight of the antenna elements. It is assumed that all antenna elements’ orientations are aligned. Also, P is the average received power and G is the numeric base station antenna gain described in Section 2.5.1 by

G(θ)=100.1A(θ)

Finally, No is the normalization constant:

No−1=exp−2(θ−θ)σ⋅G(θ)dθ

In the above equation,  represents path components (sub-rays) of the path power arriving at an incoming AoA θ.  The distribution of these path components is TBD.

## Spatial Parameters for the MS

### MS Antenna Pattern

For each and every antenna element at the MS, the antenna pattern will be assumed omni directional with an antenna gain of -1 dBi.

### Per Path MS Angle Spread (AS)

The MS per path AS is defined as the root mean square (RMS) of angles of an incident path’s power at the MS array.  Two values of the path’s angle spread are considered:

-	AS: 104 degrees (results from a uniform over 360 degree PAS),

- AS: 35 degrees for a Laplacian PAS with a certain path specific Angle of Arrival (AoA).

### Per Path MS  Angle of Arrival

The per path Angle of Arrival (AOA) is defined as the mean of angles of an incident path’s power at the UE/Mobile Station array with respect to the broadside as shown Figure 2-5.

Figure 2-5. Angle of Arrival orientation at the MS.

Three different per path AoA values at the MS are suggested for the cases of a non-uniform PAS, see Table 2-1 for details:

- AoA: -67.5 degrees (associated with an RMS Angle Spread of 35 degrees)

- AoA: +67.5 degrees (associated with an RMS Angle Spread of 35 degrees)

- AoA: +22.5 degrees (associated with an RMS Angle Spread of 35 degrees or with an LOS component)

### Per Path MS Power Azimuth Spectrum

The Laplacian distribution and the Uniform distribution are used to model the per path Power Azimuth Spectrum (PAS) at the MS.

The Power Azimuth Spectrum (PAS) of a path arriving at the MS is modeled as either a Laplacian distribution or a uniform over 360 degree distribution. Since an omni directional  MS antenna gain is assumed, the received per path PAS will remain either Laplacian or uniform. For an incoming AOA θ and RMS angle-spread , the MS per path Laplacian PAS value at an angle  is given by expression  below:

P(θ,σ,θ)=Noexp−2θ−θσ,

where both angles θand  are given with respect to the boresight of the antenna elements. It is assumed that all antenna elements’ orientations are aligned. Also, P is the average received power and No is the normalization constant:

No−1=P(θ,σ,θ)dθ.

In the above equation,  represents path components (sub-rays) of the path power arriving at an incoming AoA θ.  The distribution of these path components is TBD.

### MS Direction of Travel

The mobile station direction of travel is defined with respect to the broadside of the mobile antenna array as shown in Figure 2-5.

### Per Path Doppler Spectrum

The per path Doppler Spectrum is defined as a function of the direction of travel and the per path PAS and AoA at the MS.  This should correspond to the per path fading behavior for either the correlation-based or ray-based method.

## Generation of Channel Model

The proponent can determine the model implementation. Examples of implementations include correlation or ray-based techniques.

Further details on examples of the model implementation are TBD.

Outline of methodology, including doppler spectrum filter required for correlation method.

## Calibration and Reference Values

For the purpose of link level simulations, reference values of the average correlation are given below in Table 2-2. The reference values are provided for the calibration of the simulation software and to assist in the resolution of possible errors in the simulation methods implemented. Specifically, the average complex correlation and magnitude of the complex correlation is reported between BS antennas and between MS antennas.  The spatial parameter values used are those defined already throughout Section 2. Nokia to provide formulas for computing correlation matrices

Table 2-2. Reference Correlation Values.

|  | Antenna Spacing | AS (degrees) | AOA (degrees) | Correlation (magnitude) | Complex Correlation |
| --- | --- | --- | --- | --- | --- |
| BS | 0.5λ | 5 | 20 | 0.9688 | 0.4743+0.8448i |
|  | 0.5λ | 2 | 50 | 0.9975 | -0.7367+0.6725i |
|  | 4λ | 5 | 20 | 0.3224 | -0.2144+0.2408i |
|  | 4λ | 2 | 50 | 0.8624 | 0.8025+0.3158i |
|  | 10λ | 5 | 20 | 0.0704 | -0.0617+i0.034 |
|  | 10λ | 2 | 50 | 0.5018 | -0.2762-i0.4190 |
| MS | λ/2 | 104 | 0 | 0.3042 | -0.3042 |
|  | λ/2 | 35 | -67.5 | 0.7744 | -0.6948-i0.342 |
|  | λ/2 | 35 | 22.5 | 0.4399 | 0.0861+0.431i |
|  | λ/2 | 35 | 67.5 | 0.7744 | -0.6948+i0.342 |

# System Level Spatial Channel Model

A Spatial Channel Model for use at the system level is described below. Parameter values and their ranges are specified to enable the generation of a Spatio-Temporal channel model for use in evaluating the system-level performance of multiple antenna schemes. The goal of this section is to define the methodology for generating the spatial channel coefficients between a given base and mobile for use in system level simulations. For a given base and mobile pair, this procedure consists of three basic steps

- Specify an environment, either suburban macro, urban macro, or urban micro.

- Obtain the parameters associated with that environment.

- Generate the channel coefficients based on the parameters.

The remainder of this section gives the details for these steps. Figure X below provides a roadmap for generating the channel coefficients. (This diagram should be greatly expanded and should show which section numbers each of the items is discussed.)

## System Wide Definitions and Parameters

The received signal at the MS consists of N time-delayed multipath replicas of the transmitted signal. These N paths are defined by powers and delays and are chosen randomly according to the channel generation procedure. Each path consists of M subpaths which will be described later.

Figure 3-6 shows the angular parameters used in the proposed model. The following definitions are used:

ΩBS	Base Station orientation. It is defined as the difference between the broadside of the BS array and a North (N) reference direction.

θ0	  Angle of the LOS direction between the BS and MS, referenced to the broadside of the BS array.

δi	mean angle of arrival difference for the nth (n = 1 … N) path relative to the LOS AOA θ0.

θl,i	Absolute AOA for the mth (m = 1 … M) subpath of the nth path at the BS with respect to the BS broadside.

θ0,MS	Angle between the BS-MS LOS and the MS broadside.

ΩMS	MS orientation. It is defined as the difference between the broadside of the MS array and the North reference direction.

θv,MS	The angle reference of the MS velocity vector.

The angles shown in Figure 1 that are measured in a clockwise direction are assumed to be negative in value.

Figure 3-6  BS and MS angle parameters

For system level simulation purposes, the fast fading per-path will be evolved in time, although bulk parameters including Angle Spread, Delay Spread, Log Normal Shadowing, and MS location will remain fixed during the evaluation of the given MS.

## Scenarios

To limit the number of cases for consideration, the following represent a definition of the unique environments for simulation.

- Suburban Macro (approximately 3Km distance BS to BS)

- Urban Macro (approximately 3Km distance BS to BS)

- Urban Micro (less than 1Km distance BS to BS)

The macro cell definition applies when the base station antennas are above rooftop height. The micro cell definition applies otherwise. Table 3-3 Scenario parameters describes the parameters used in each of the scenarios.

Table 3-3 Scenario parameters

| Channel Scenario | Suburban Macro | Urban Macro | Urban Micro |
| --- | --- | --- | --- |
| Mean composite AS at BS | E(AS)=50 | E(AS)=80, 150 | N/A |
| rds (delays/DS) Input | 1.4 TBD | 1.7 TBD |  |
| rds Output | 1.29 TBD | 1.54 TBD |  |
| ras (AoDs/PAS) Output | 1.22 TBD | 1.37 TBD |  |
| Composite AS at BS as a lognormal RV when simulating with 6 paths AS=10Ax+A, x~N(0,1) | A= 0.69 A= 0.17 | A= 0.87 A= 0.36 | N/A |
| Per path AS at BS (Fixed) | 2 deg | 2 deg | 5 deg |
| Per path AoD Distribution st dev | rAOD,CS=1.07 TBD | rAOD,CS=1.3 TBD | U(-30deg, 30deg) |
| Mean of RMS composite AS at UE | E(AS, comp,UE)=720 | E(AS, comp, UE)=720 | E(AS, comp, UE)=720 |
| Per path AS at UE (fixed) | 350 | 350 | 350 |
| Per path AoA Distribution | N(0,AoA2(Pr)) Note 1 | N(0,AoA2(Pr)) Note 1 | N(0,AoA2(Pr)) Note 1 |
| Mean total RMS Delay Spread | E(DS)=0.17 s | E(DS)=0.65 s | U(0,0.8) s |
| Narrowband composite delay spread as a lognormal RV when simulating with 6 paths DS=10Dx+D, x~N(0,1) | D = - 0.80 D = 0.288 | D = -0.175 D = 0.17 |  |
| Lognormal shadowing standard deviation | 8dB | 8dB | 9-10dB TBD |

Note 1, Per path AoA is chosen from Normal distribution whose sigma is a function of the relative of path power,  N(0,AoA2(Pr)), where 0 = LOS, and AoA = 104.12(1-exp(-0.3125*|Pr|), with Pr equal to the fraction of power in the given path in dBr.

## Channel Generation Steps

From these parameters, realizations of the user parameters such as the path delays, powers, and subpath angles of departure and arrival can be derived using the following 12-step procedure.

- Step 1: Choose a single channel scenario common to all drops (i.e. to be applied to the entire simulation).

- Step 2: Generate Drops. Assign geometry (LOS direction and distance of UE from NodeB), UE antenna structure orientation, UE speed vector direction and magnitude.

- Step 3: Select lognormal random draws for DS, AS (at NodeB: σASk), LN as described in Section 3.4.2 below.

- Step 4: Assign N = 6 paths (for macro channels). Assign 6 random delays. Delays are ordered and the minimum delay is subtracted from all so that the first delay is always zero. k is the drop and UE index.

Ratio values rds  are rds =1.17 suburban macro, rds =1.41 urban macro. Realization of random delays are made according to the model below:

τn=−rDSσDSklogzn        zn:U(0,1)

Note that while there is some evidence that delay spread may depend on distance between the transmitter and receiver, the effect is considered to be minor (compared to other dependencies: DS-AS, DS-LN.)    Various inputs based on multiple data sets indicate that the trend of DS can be either slightly positive or negative, and may sometimes be relatively flat with distance.   For these reasons and also for simplicity, a distance dependence on DS is not modeled.

In the development of the Spatial Channel Model, care was taken to include the statistical relationships between Angles and Powers, as well as Delays and Powers.  This was done using the proportionality factors rds = delays/DS and ras = AoDs/PAS that were based on measurements.  (the r value is calculated by a ratio of statistically biased estimators).

- Step 5: Assign a power to each path n:

Pn=e(1−rDS)⋅τnrDS⋅σDSk⋅10−ξn    where    ξn:N(0,σ2RND)

where σRND is a shadowing randomization effect on the per-path powers. The value for RND = 3 dB.  Powers are normalized so that total power (for all six paths is equal to one).

The equations presented here for the power of the nth path are based on an power-delay envelope which is the average behavior of the power-delay profile.  Defining the powers to reproduce the average behavior limits the dynamic range of the result and does not reproduce the expected randomness from trial to trial.  The randomizing noise n is used to vary the powers with respect to the average envelope to reproduce the variations experienced in the actual channel.  This parameter is also necessary to produce a dynamic range comparable to measurements.

- Step 6: Generation of AODs per path and ordering at NodeB. Random draws of AODs from . The rAOD,CS=1.07 (0.3 in dB) (Suburban macro), rAOD,CS=1.3  (1.0 in dB) (urban macro).

- Step 7: The 6 AODs generated in Step 6 are ordered in increasing absolute value and each of the six delays (Step 4) is assigned to each AOD. Increasing delays are matched to increasing relative AOD angle δn is defined in Figure 3-6 based on their absolute value and deterministically.

- Step 8: 20 sub-rays are used to generate a 2o Laplacian spread for each path (ray) at the NodeB.  All 20 sub-rays have identical powers (1/20 of the path power) but random phase. Sub-components have a predefined fixed angle distribution as shown in Table 3-4.

- Step 9: At the UE, assign per-path (ray) AOA variance as a function of the path (ray) relative power. Draw the relative AOA (with respect to LOS) from a distribution: N(0,σ2AOA).  Where 0 = LOS, andAoA = 104.12(1-exp(-0.3125*|Pr|), with Pr equal to the fraction of power in the given path in dBr.

- Step 10: 20 sub-rays are used to generate a 35o Laplacian spread for each path (ray) at the UE.  All 20 sub-rays have identical powers (1/20 of the path power) but random phase.  Sub-components have a predefined fixed angle distribution as shown in Table 3-4.

- Step 11: Pairing of each NodeB sub-ray with a corresponding UE sub-ray is made to create the actual channel gain. Random pairing is used.

- Step 12: Assign antenna gains to NodeB paths (rays). Assign the antenna gain to each UE sub-ray.

Table 3-4. Fixed Sub-path components to produce per-path spreads

| Sub-path # | (+/-) degrees for a 2 deg Laplacian (Node-B Macro-cell) | (+/-) degrees for a 5 deg Laplacian (Node-B Micro-cell) | (+/-) degrees for a 35 deg Laplacian (UE) |
| --- | --- | --- | --- |
| 1, 2 | 0.08716 | 0.2179 | 1.525 |
| 3, 4 | 0.2754 | 0.6884 | 4.819 |
| 5, 6 | 0.4858 | 1.215 | 8.502 |
| 7, 8 | 0.7243 | 1.811 | 12.68 |
| 9, 10 | 0.9997 | 2.499 | 17.49 |
| 11, 12 | 1.325 | 3.313 | 23.19 |
| 13, 14 | 1.724 | 4.310 | 30.17 |
| 15, 16 | 2.238 | 5.595 | 39.16 |
| 17, 18 | 2.962 | 7.405 | 51.84 |
| 19, 20 | 4.201 | 10.50 | 73.51 |

## BS and MS Array Topologies

The spatial channel model should allow any type of antenna configuration to be selected, although details of a given configuration must be shared to allow others to reproduce the model and verify the results.  It is intended that the spatial channel model be capable of operating on any given antenna array configuration.

### Polarized arrays

Practical antennas on handheld devices require spacings much less than /2.  Polarized antennas are likely to be the primary way to implement multiple antennas.  A cross-polarized model is therefore included here.

A method of describing polarized antennas is presented, which is compatible with the 12 step procedure given in section 3.3.  The following steps extend the original 12 to add the additional polarized components necessary.

- Step 13:  For each of the 6 paths of step 4, generate 20 sub-rays at the MS and 20 sub-rays at the BS, to represent the portion of each signal that leaks into the quadrature antenna orientation due to scattering.  Randomly pair each of the new sub-rays with a corresponding sub-ray that was generated in step 8 for the BS and step 10 for the MS.

- Step 14:. Set the AoD and AoA of each sub-ray in step 13 equal to that of the corresponding sub-ray of the inline antenna orientation.  (Orthogonal sub-rays arrive/depart at common angles.)

- Step 15: Set the phase angle of each sub-ray in step 13 to a random angle drawn from U(0,2pi).

- Step 16: The power P2 of each ray in the quadrature orientation is set relative to the power P1 of each ray in the inline orientation according to an XPD ratio, defined as XPD= P1/P2.  For urban macrocells:  P2 = P1 - A - B*N(0,1), where A=0.34*(mean relative path power)+7.2 dB, and B=5.5dB is the standard deviation of the XPD variation.

For urban microcells:  P2 = P1 - A - B*N(0,1), where A=8 dB, and B=8dB is the standard deviation of the XPD variation.

- Step 17: Decompose each of the inline and quadrature sub-rays into vertical and horizontal components based on the in-line and quadrature orientations.

- Step 18: At the receive antennas, decompose each of the vertical and horizontal components into components that are in-line and quadrature with the receive antennas and sum the in-line components.

The fading behavior between the cross pol elements will be a function of the per-ray spreads and the Doppler.  The fading between orthogonal polarizations has been observed to be independent and therefore the sub-rays phases are chosen randomly.

Figure 3-7  Dual Polarization Example

Additional description of the polarization model will be included in this section with equations describing the various components.

Expressions describing the per antenna resulting complex waveforms can be described as follows (variables defined as in Figure 3-6).

hs,u(t)=PiLGBS(θl,i)⋅ej⋅k⋅ds⋅sin(θl,i)⋅GMS(θl,iMS)⋅ej⋅k⋅duMS⋅sin(θl,iMS)ej⋅φl,iej⋅k⋅v⋅cos(θl,iMS−θvMS)⋅ t      	(1)

The polarization model can be illustrated by a matrix describing the propagation of and mixing between horizontal and vertical amplitude of each sub-path.  The resulting channel realization is:

hs,u(t)=PiL⋅GsV(θl,i)GsH(θl,i)T⋅ejβl,i1rx,iejβl,i3rx,iejβl,i2ejβl,i4GMSuV(θl,iMS)GMSuH(θl,iMS)ej⋅k⋅ds⋅sin(θl,i)⋅ej⋅k⋅duMS⋅sin(θl,iMS)ej⋅k⋅v⋅cos(θl,iMS−θvMS)⋅ t			(2)

where:

s	is the antenna index at the BS (NodeB) where s=1...S

u	is the antenna index at the MS (UE) where u=1...U

T	is the number of paths as defined in the temporal channel model

Pi	is the relative power of the i-th path

L	s the number of sub-paths used to model  each path (L=20).

GsV(θl,i)GsH(θl,i)	is the antenna complex response in the vertical and horizontal polarizations at the BS (NodeB) as functions of the AOA.

GMSuV(θl,iMS)GMSuH(θl,iMS)	is the antenna gain at the MS (UE) as a function of the AOA for the two polarizations.

θl,i	is the angle, with respect to the BS broadside, of the l-th sub-path in the i-th path. Note that θl,i=θo+δi+θl,isub−path, where θl,isub−pathis the sub-path’s relative angle within the path as defined in Table 4 in the SCM-Text.

θl,iMS	is the angle, with respect to the UE broadside, of the l-th sub-path in the i-th path.

φl,i	is the uniformly distributed random phase of the l-th sub-path in the i-th path.

is the wave number.

ds	is the distance in wavelengths of the antenna s from the reference (s=0 ) antenna. If s is the reference antenna then the distance is zero.

v	is the MS (UE) speed vector in meters/sec. The norm of the vector is the magnitude of the speed.

θvMS	is the angle of the speed vector with respect to the MS array’s broadsise

The 2x2 matrix represents the scattering phases and amplitudes of a plane wave leaving the UE with a given angle and polarization and arriving Node B with another direction and polarization. rx,i is the average power ratio of waves leaving the UE in the vertical direction and arriving at Node B in the horizontal direction (v-h) to those arriving at Node B in the vertical direction (v-v). By symmetry the power ratio of the opposite process (h-v over v-v) is chosen to be the same.  Note that: rx,i=1/XPD; for the macrocell model, the XPD is dependent on the path index; for the microcell model, the XPD is independent of path index.

βl,ix where x=1,2,3,4	are the uniform and i.i.d random phases of the l-th sub-path in the i-th path for each of the four mixing directions.

Expression (2) assumes a random pairing of the of the sub-paths from the MS and BS. The random orientation of the MS (UE) array affects the value of the angle θl,iMS of each sub-path.

If for example, vertically polarized antennas are used only at both NodeB and UE then the antenna responses become 10 and expression (2) becomes identical to (1). For an ideal dipole antenna at the NodeB tilted with respect to the z-axis at αdegrees the above vector becomes cos(α)sin(α)⋅cos(θl,i).

The elevation spectrum is not modeled.

### Reference Antenna Configurations

In order to compare algorithms, reference antenna configurations based on uniform linear array configurations with 0.5, 4, and 10 wavelength inter-element spacing will be used.

### General Modeling Assumptions

- Mapping of paths to resolvable paths. TBD

- Fractional Unrecovered Power (FURP) is TBD.

- The macrocell pathloss from 3GPP2 evaluation methodology will be used for the SCM model as shown below.

| Propagation Model (BTS Ant Ht=32m, MS=1.5m) | 28.6+ 35log10(d) dB, d in meters | Modified Hata Urban Prop. Model @1.9GHz (COST 231). Minimum of 35 meters separation between MS and BS. |
| --- | --- | --- |

- Uplink-Downlink Reciprocity: The AOD/AOA values are identical between uplink and downlink.

- Random path phases between UL, DL are uncorrelated.

- mobile-to-mobile shadowing is uncorrelated.

### Micro-cell Assumptions

  - The BS antenna is always located at rooftop.

  - The pathloss model is TBD

  - Antenna patterns at BS/NodeB will be those already defined in the SCM-Text.  Question about backlobes remain since micro-cells typically have poor backlobes.  TBD.

  - Site-to-site correlation. Shadowing between different paths (delay) are iid. Site-to-site correlation for the same delay follows the macro model (50% correlation).  Some questions remain on this method.

  - Log Normal St.Dev of 4-5dB per path should be used prior to normalization.  The log normal per-path sets the distribution and the dynamic range for the path powers, and the distribution for the composite base angle spread.  A separate bulk log normal shadowing is needed for the path losse03

  - .  The value of 10dB was suggested.  The final value is TBD.

  - The hexagonal cell repeats will be the assumed layout.

## Options for Urban Macro and Micro Environments

### Far scatterer clusters

The Far scatterer cluster model is switch selectable.  It represents the bad-urban case where additional clusters are seen in the environment.  This model is limited to use with the urban macro-cell where the first cluster will be the primary cluster and the second will be the far scattering cluster (FSC). When the model is active, it will have the following characteristics:

- There is a reduction in the number of paths in the primary cluster from N = 6 to N = 4, with the far scattering cluster then having N = 2.  Thus the total number of paths will stay the same, now N = 4 + 2.  This is a modification to the SCM channel generation procedure in section 3.3.

- FSCs will lie only outside a 1000m radius from the BS/NodeB. (the 500-600m radius is under consideration also but not finalized).

- The model statistics of the two clusters are identical (cluster DS, AS, PDP) but independently drawn. The FSC also has independent shadowing.

- The FCS is attenuated by 1dB/microsec delay with respect to the 1st cluster with a 10dB maximum.  The excess delay will be defined as the difference in propagation time between the BS-MS LOS distance, and the BS-FSC-MS distance.  The delay of the FSCs of the other cells (Ioc components) remains an open issue.

- There will be one FSC modeled within each cell and dropped following a uniform distribution. FSCs will be modeled for all the cells in the simulation in order to capture the Ioc spatial characteristics.

An exact method of setting the powers is TBD.  The following approach is a possible method:  Draw the N=4 path powers from the channel generation procedure in section 3.3, then draw a separate set of N=2 path powers from the same procedure.  The two groups are kept separate and un-normalized.  Now the delay based attenuation is applied to the group of N=2 paths, and the N=6 total paths are normalized to unity power.

### Line of sight

The Line-of-sight model an option that is switch selectable.  It can be selected for the urban macro and micro cases.  Suburban is TBD.  It uses the following description when this function is selected.

For the NLOS case, the Rice factor is set to 0, thus the fading is determined by the combination of sub-rays as described in section 3.3 of the model.

For the LOS case, the Rice factor is based on a simplified version of [Foster 1994], and is applied based on a probability factor:

K = 13-0.03*d (dB)

where d is the distance between MS and BS in meters.

The probability for LOS or NLOS depends on various environmental factors, including clutter, street canyons, and distance.  For simplicity, the probability of LOS is defined to be unity at zero distance, and decreases linearly until a cutoff point at d=300m, where the LOS probability is zero.

P(LOS) =	(300 – d)/300,  for 0< d < 300m.

0, for d > 300m.

The K-factor will be formed by adding a direct component (sine wave) at the average AoD and AoA of the path such that the ratio of the power assigned to the direct component to the power assigned to the 6 paths is equal to the K-factor measured in dB.  After the power of the direct component is added, the total power in the channel is normalized to unity power.  The K-factor is defined as the ratio of power in the LOS component to the total power in the diffused-NLOS component.  The LOS path will coincide in time with the first (earliest) diffused path. When pairing sub-rays between transmitter and receiver, the direct components are paired representing the LOS path.

### Urban canyon

The urban canyon model is switch selectable.  When switched on, the model modifies the AoAs of the paths arriving at the subscriber unit.  It is for use in both the urban macro and urban micro scenarios.

Urban-canyons exist in dense urban areas served by macro-cells, and below-rooftop micro-cells.  When this model is used, the spatial channel for all subscribers in the simulated universe will be defined by the statistical model given below.  Thus for the SCM channel generation steps given in Section 3.3, Step 9 is replaced with steps 9a-d given below, which describe the AoAs of the paths arriving at the subscriber in the urban canyon scenario.

The following procedure is used to determine the subscriber mean AoAs of the six paths.  This model does not use a building grid, but assigns angles based on statistical data presented in the figures below.  The procedures is defined in terms of the subscriber terminal:

Step 9a.  Pick a random direction of travel from a uniform random variable U(-180,180). This is going to be also the assumed street orientation for this subscriber, and their direction of travel.

Step 9b.  Pick the AoA corresponding to the strongest ray in the channel impulse response according to the CDF depicted in Figure 3-8.  Note that +/- angles are equally likely.

Step 9c.  For the remaining 5 weaker paths, pick the AoAs according to the CDF shown in Figure 3-9, where zero degrees is defined to be AoA of the strongest path from step 9b.

Step 9d.  For interferers, steps 9b and 9c are repeated, assuming the street orientation and direction of travel did not change from step 9a.

The fixed 35 degree per-path Azimuth Spread is applied after the six ray AoA are defined.

Figure 3-8,  AoA of Strongest Path (+ & - angles have equal probability)

The CDF for the function given in Figure 3-8, is defined by:

Fx(x)=x500o≤x<15ox541.66+0.272315o≤x<80ox125−0.2280o≤x<100ox541.66+0.3954100o≤x<165ox50−2.6165o≤x<180o

Figure 3-9, Angle difference between Strongest Path and remaining paths

The CDF of the function given in Figure 3-9, is defined by:

Fx(x)=x1200+0.15−180o≤x<0o0.7x=0x1200+0.850o<x<180o

## Correlation Between Channel Parameters

### Correlation between Delay Spread, Angle Spread, and Log Normal Shadowing

In [], Greenstein presents a model for correlating delay spread (DS) with log normal (LN) shadow fading.  Since both are shown to be log-normal distributed, the correlation between the DS and LN are correlated by the coefficient .  The best value for suburban and urban data was shown to be  = -0.75, presented in [Error: Reference source not found] from data measured by [].

The result of the correlation between log normal shadowing and delay spread is significant because it indicates that for a strong signal (positive LN), the DS is reduced, and for a weak signal condition (negative LN), the DS is increased.

Cost 259[] presents the azimuth spread (AS) as also being log-normal distributed, and likewise being correlated to the DS and LN.  Since the correlation of these parameters is quite high, a spatial channel model needs to be specified that can reproduce this correlation behavior along with the expected probability and range of each parameter.  For a macro-cell environment, the following values are given in [Error: Reference source not found]:

 = Correlation between DS & AS = +0.5

 = Correlation between LN & AS = -0.75

 = Correlation between LN & DS = -0.75

The random variables for correlating DS, AS, and LN shadowing are generated using

αnβnγn=c11c12c13c21c22c23c31c32c33wn1wn2wn3,			(1)

where n refers to the nth base station (BS).  and are the correlated zero-mean unit variance Gaussian distributed random variables associated with the DS, AS, and LN, respectively, and wn1, wn2, and wn3 are unit-variance, independent Gaussian noise samples.  Equation 1 is performed once for each BS per drop.

c11c12c13c21c22c23c31c32c33=ρααραβραγρβαρββρβγργαργβργγ12,			(2)

Equation 2 incorporates the matrix square root function of correlation coefficients.

The LN shadow fading component is typically correlated between Base Stations (BS).  To produce correlated shadow fading components () between BS1 and BS2, a simplified approach is shown where the independent unit-variance Gaussian noise samples w13 and w23 are replaced with correlated, unit-variance Gaussian noise samples that are generated using:

w13=ξcccc33+1−ccc332ξ1				(3)

and

w23=ξcccc33+1−ccc332ξ2,			(4)

where c, 1, 2 are independent, unit-variance Gaussian noise samples and cc is the desired site-to-site shadow fading correlation between the MS and nodes BS1 and BS2.  A value for the common component cc=0.5 or 50% site-to-site correlation will be assumed.  The cumulative distribution functions of DS, LN,  and AS are described in terms of the correlated Gaussian random variables that characterize their distributions [Error: Reference source not found][Error: Reference source not found][].

Using nomenclature from[Error: Reference source not found] the distribution of DS is given by:

DS = 10^(Dn +D)					(5)

where n is the correlated zero-mean unit variance Gaussian distributed random variable associated with the DS for the nth BS, D = E{log10(DS)} is the logarithmic mean of the distribution of DS,  and D = Std{log10(DS)} is the logarithmic standard deviation of the distribution of DS.

Similarly the distribution of AS is given by:

AS = 10^(An +A)					(6)

where	n is the correlated zero-mean unit variance Gaussian distributed random variable associated with the AS, A = E{log10(AS)} is the logarithmic mean of the distribution of AS,  and A = Std{log10(AS)} is the logarithmic standard deviation of the distribution of AS.

Finally, the distribution for the LN is given by:

LN = 10^(SFn/10),					(7)

where	n is the correlated zero-mean unit variance Gaussian distributed random variable associated with the LN, SF (shadow fading) is obtained from analysis of the standard deviation from the regression line of the path loss versus distance, and the value is given in dB.  Since the LN is normally used as a dB value, there is no need to convert it to linear, but rather simply use the quantity SFn.

Correlated channel parameters may be drawn within a system simulation based on the equations described above.

## System Level Calibration

The following examples are given for calibration purposes. A resolvable path at the receiver is assumed to be the energy from one (or more) paths falling within one chip interval. The Chip rate in UMTS is 3.84Mcps. The PDF of the number of resulting resolvable paths is recorded.

Table 3-5, SCM Parameter Summary with Simulated Outputs

| Parameter | Urban 8 RND = 3dB |  | Urban 15 RND = 3dB |  | Suburban 5 RND = 3dB |  |
| --- | --- | --- | --- | --- | --- | --- |
| rDS | Input | Output | Input | Output | Input | Output |
|  | 1.7 | 1.54 | 1.7 | 1.54 | 1.4 | 1.29 |
| DS | Input | Ideal | Input | Ideal | Input | Ideal |
|  | -6.195 | -6.26 | -6.195 | -6.26 | -6.80 | -6.92 |
| DS | Input | Ideal | Input | Ideal | Input | Ideal |
|  | 0.18 | 0.25 | 0.18 | 0.25 | 0.288 | 0.363 |
| rAS | Input | Output | Input | Output | Input | Output |
|  | 1.3 | 1.37 | 1.3 | 1.37 | 1.2 | 1.22 |
| AS | Input | Ideal | Input | Ideal | Input | Ideal |
|  | 0.810 | 0.75* | 1.18 | 1.0938 | 0.66 | 0.66 |
| AS | Input | Ideal | Input | Ideal | Input | Ideal |
|  | 0.3295 | 0.37 | 0.210 | 0.2669 | 0.12 | 0.18 |
| E[DS] | Ideal | Simulated | Ideal | Simulated | Ideal | Simulated |
|  | 0.65s | 0.63s | 0.65s | 0.63s | 0.17s | 0.172s |
| E[ AS Node B] | Ideal | Simulated | Ideal | Simulated | Ideal | Simulated |
|  | 8 | 7.97 | 15 | 14.94 | 5 | 5.01 |
| E[ AS UE] | Ideal | Simulated | Ideal | Simulated | Ideal | Simulated |
|  | 72 | 72.05 | 72 | 72.69 | 72 | 74.07 |

*This value was 0.77, however this will produce a value of E[ AS Node B]=8.5 instead of 8.

The following figures: Figure 3-10, Figure 3-11, Figure 3-12, Figure 3-13, Figure 3-14, represent calibration cases for the current SCM model.  These curves correspond to the parameters presented in Table 3-5, and include the 3dB randomizing factor for the generation of path powers.

Figure 3-10, Probability of Urban and Suburban Time Resolvable Paths

Figure 3-11, RMS Delay Spread, Simulated versus Ideal

Figure 3-12, Node-B Composite Angle Spread, Simulated versus Ideal

Figure 3-13, Dynamic Range (dB) for each channel model

Figure 3-14, CDF of all Path Powers

Channel Scenario: Urban Microcellular

Figure 4 Statistics for Urban Microcellular Environment with chip-rate frequency = 3.84MHz (clockwise from top left): (a) CDF of total rms DS, (b) Probabilities for number of resolvable paths present (in occurrences per 10,000 drops). (c) CDF of Composite AS

# Appendix A: MMSE receiver description

(The following text is a preliminary description of the MMSE receiver).

This procedure generates SINR values at the output of a linear MMSE receiver for a single instant in time.

Step 1:  Given the space-time propagation model and transmitter state, form a channel (expressed here as one or more convolution matrices) relating all transmitting sources and receive antennas from every sector in the system.

At the UE, the received samples are represented as a column vector,

r=[r1T,r2T,⋯,rMT]T   =[r1(1),r1(2),⋯,r1(N),r2(1),r2(2),⋯,r2(N),⋯,rM(1),rM(2),⋯,rM(N)]T,

where M is the number of receive antennas at the UE, and N is the number of received symbols per antenna. This received time-space vector is related to the transmitted symbols as follows:

r=G(1)x(1)+j=2JG(j)x(j)+n=G1(1)G2(1)⋮GM(1)⋅x(1)+j=2JG1(j)G2(j)⋮GM(j)⋅x(j)+n1n2⋮nM

where Gi(j), 1 ≤ i, j ≤ M are Toeplitz convolution matrices defining the channel between the i-th receive antenna and the j-th transmitted data stream, x(j)is the j-th transmitted data stream, J is the total number of data streams in the system, and n is the vector of noise samples.  The j = 1 data stream is the primary data stream intended for the user.  The j-th data stream can be a transmission from an interfering base station, another sector of the desired base station, or another data stream intended for the desired user (which is considered interference to the primary data stream). If the composite channel response is limited to K samples, then each of the convolution matrices has N rows by (N+K-1) columns,

Gi(j)=gi(j)(K)gi(j)(K−1)⋯gi(j)(1)00⋯00gi(j)(K)gi(j)(K−1)⋯gi(j)(1)0⋯0⋮⋮⋱⋱⋯⋱⋯000⋯0gi(j)(K)gi(j)(K−1)⋯gi(j)(1),

and gi(j) is the vector of discrete channel samples of length K.

Note that in the above formulation, the vector x has M(N+K-1) rows, and thus, it is longer than the received vector, r.  Also, the vector x will be interleaved with zero values if a fractionally-spaced approach with more than one received sample per symbol is used.

Step 2:  Using the above channel, produce an estimate of the channel.

gi(j)=gi(j)+Δgi(j),

where Δgi(j) is a vector representing the channel estimation error for the i-th receive antenna and the j-th transmitted data stream.  The estimation error is due to noise and interference in the pilot channel and can also be due to the channel estimator’s inability to track a fast fading channel.

Step 3:  Using the estimated channel, compute the SINR per data stream at the output of the MMSE filters.

SINRj=fjHΩj−1fj2fjHΩj−1ΩjΩj−1fj,

where

m≠jJ+EnnH,

Ωj is an estimate of Ωj, d=max(N−K)2+K,K, fjis the d-th column of G(j), x(j)(d) is the d-th element (desired symbol) of the x(j) data stream vector, and SINRj represents the SINR for the j-th transmitted data stream in the system.  In this example, the primary data stream sent to a user will be j = 1.  In a MIMO system where multiple data streams are sent to a single user, the second stream could be j = 2, etc.

# Reference

[?] WG5 Evaluation Methodology – Addendum (V6), WG5 Evaluation AHG, July 25, 2001

[?] Nokia, Ericsson, Motorola.  Common HSDPA system simulation assumptions. TSG-R1 document, TSGR#15(00)1094, 22-25th, August, 2000, Berlin, Germany, 12 pp.

[?] L. Greenstein, V. Erceg, Y. S. Yeh, M. V. Clark, “A New Path-Gain/Delay-Spread Propagation Model for Digital Cellular Channels,” IEEE Transactions on Vehicular Technology, VOL. 46, NO.2, May 1997, pp.477-485.

[?] E. Sousa, V. Jovanovic, C. Daigneault, “Delay Spread Measurements for the Digital Cellular Channel in Toronto,” IEEE Transactions on Vehicular Technology, VOL. 43, NO.4, Nov 1994, pp.837-847.

[?] L. M. Correia,  Wireless Flexible Personalized Communications, COST 259: European Co-operation in Mobile Radio Research, Chichester:  John Wiley & Sons, 2001.

[?]  A. Algans, K. I. Pedersen, P. Mogensen, “Experimental Analysis of the Joint Statistical Properties of Azimuth Spread, Delay Spread, and Shadow Fading,” IEEE Journal on Selected Areas in Communications, Vol. 20, No. 3, April 2002, pp. 523-531.

Actually, this is the number of received samples per antenna, if more than one sample per symbol is collected.