3GPP TSG RAN WG1 Meeting #100b-e	R1-2003xxx

e-Meeting, April 20th – 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | xxxx | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Additional enhancements for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections needed on PUR transmission, multi-TB scheduling, and coexistence features of Rel-16 NB-IoT. - The relationship between PO_NPUSCH,c3 and the higher layer parameter might be unclear; in and , the character “c” overlaps the character “H”. - Clarify that If an UE has received DCI scheduling multiple SC-MTCH TBs, it is not required to monitor Type2A-CSS during the scheduling gap or the processing gap. - Editorial correction to delete the Extra ' ', and NPDCCH UE-specific search space text, and replacing “n+4 subframe” by “subframe n+4”. - As in previous releases, the subframes containing SIB1-NB transmission should not be considered as NB-IoT downlink subframes. - With the sentence “associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space”, the SPS NPUSCH is not considered in resource reservation as it’s not using UE-specific NPDCCH search space. Also, there is no clear definition for fully reserved subframe. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - [100b-e-LTE-NB_IoTenh3-PUR-02] on PUR power control, include the endorsed text proposal for Sec.16.2.1.1.1. link PO_NPUSCH,c3 to the corresponding higher layer parameter; replacing “” by “PO_NPUSCH,cj”, and replacing “” by “MNPUSCH,c(i)” - [100b-e-LTE-NB_IoTenh3-Multi-TB-01] Type2A CSS during the scheduling gap and the processing gap, include the endorsed text proposal for Sec. 16.6. Specify that a UE is not required to monitor Type2A CSS during the scheduling gap and the processing gap - [100b-e-LTE-NB_IoTenh3-Multi-TB-03] editioral correction of deleting the extra '', and NPDCCH UE-specific search space text include the endorsed text proposal for Sec.16.5.1. - [100b-e-LTE-NB_IoTenh3-Coex-NR-01] on Resource reservation for TDD NB-IoT, include the endorsed text proposal for Sec.16.4,16.5. To determine whether a subframe is a NB-IoT downlink subframe, UE shall first determine the subframes containing NPSS/NSSS/NPBCH/SIB1-NB transmission are not assumed as NB-IoT DL subframes. - [100b-e-LTE-NB_IoTenh3-Coex-NR-03] on clarification for SPS and fully reserved subframe, include the endorsed text proposal for Sec.16.4, 16.5. The NPUSCH transmission without a corresponding NPDCCH is also considered in resource reservation. The definition for fully reserved subframe is added. - Editorial correction replacing “n+4 subframe” by “subframe n+4” in Sec 16.6 - Issue#8 in R1-2002714 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect functionality of Rel-16 NB-IoT features. - the relationship between PO_NPUSCH,c3 and the higher layer parameter might be unclear; and and are not friendly for reading. - Some UE may not be able to monitor Type2-CSS during the scheduling gap or the processing gap but the eNB may have different assumptions - Specification may lead to erroneous implementation. - The subframes containing SIB1-NB may be considered as NB-IoT DL subframes incorrectly. - The SPS NPUSCH may incorrectly not be considered for resource reservation. There may be ambiguity on the fully reserved subframe. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1.1.1, 16.4, 16.5, 16.5.1, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.211 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 16.2.1.1.1	UE behaviour

The setting of the UE Transmit power for a Narrowband Physical Uplink Shared Channel (NPUSCH) transmission is defined as follows. For FDD, if the UE is capable of enhanced random access power control [12], and it is configured by higher layers, and for TDD,  enhanced random access power control shall be applied for a UE which started the random access procedure in the first or second configured NPRACH repetition level.

The UE transmit power  for NPUSCH transmission in NB-IoT UL slot i for the serving cell is given by:

For NPUSCH (re)transmissions corresponding to the random access response grant if enhanced random access power control is not applied, and for all other NPUSCH transmissions except for NPUSCH transmission using preconfigured uplink resource, when the number of repetitions of the allocated NPUSCH RUs is greater than 2:

[dBm]

otherwise

[dBm]

where,

-	is the configured UE transmit power defined in [6] in NB-IoT UL slot i for serving cell .

-	MNPUSCH,c(i) is {1/4} for 3.75 kHz subcarrier spacing and {1, 3, 6, 12} for 15kHz subcarrier spacing

-	PO_NPUSCH,cj is a parameter composed of the sum of a component  provided from higher layers and a component  provided by higher layers for j=1, 3 and for serving cell where . For NPUSCH (re)transmissions corresponding to a dynamic scheduled grant or a semi-persistent grant then j=1, for NPUSCH (re)transmissions corresponding to the random access response grant then j=2 and for NPUSCH transmission using preconfigured uplink resource then j=3. PO_UE_NPUSCH,c2=0. If enhanced random access power control is not applied, PO_NOMINAL_NPUSCH,c(2)=PO_PRE+∆PREAMBLE_Msg3, where the parameter preambleInitialReceivedTargetPower [8] () and ∆PREAMBLE_Msg3 are signalled from higher layers for serving cell . If enhanced random access power control is applied,

PO_NOMINAL_NPUSCH,c2=MSG3_RECEIVED_TARGET_POWER +ΔPREAMBLE_Msg3

-	For j=1, for NPUSCH format 2, =1; for NPUSCH format 1, is provided by higher layers for serving cell . For j=2,  For j=3,  is the parameter pur-NPUSCH-power-control-alpha provided by higher layers for serving cell .

-	 is the downlink path loss estimate calculated in the UE for serving cell  in dB and  = nrs-Power + nrs-PowerOffsetNonAnchor – NRSRP, where nrs-Power is provided by higher layers and Subclause 16.2.2, and nrs-PowerOffsetNonAnchor is set to zero if it is not provided by higher layers and NRSRP is defined in [5] for serving cell .

< Unchanged parts are omitted>

## 16.4	Narrowband physical downlink shared channel related procedures

A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows

-	If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe.

-	Else if higher layer parameter valid-subframe-config-DL or slot-reserved-resource-config-DL is configured

-	for NPDSCH transmission associated with C-RNTI using UE-specific NPDCCH search space

-	if the Resource reservation field in the DCI is set to 0, then the subframe is assumed as a NB-IoT DL subframe

-	else if the Resource reservation field in the DCI is set to 1, then the subframe is assumed as a NB-IoT DL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols are reserved in the subframe).

-	for NPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space

-	the subframe is assumed as a NB-IoT DL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols are reserved in the subframe).

-	In all other cases, a NB-IoT UE shall assume a subframe as a NB-IoT DL subframe if

-	for a NB-IoT carrier that a UE receives higher layer parameter operationModeInfo, the subframe is configured as NB-IoT DL subframe or the subframe is a TDD special subframe configured for NB-IoT DL transmission after the UE has obtained SystemInformationBlockType1-NB.

-	the subframe is configured as NB-IoT DL subframe by the higher layer parameter downlinkBitmapNonAnchor.

-	except when the UE is configured with higher layer parameter additionalTxSIB1-Config set to TRUE, subframe #3 not containing additional SystemInformationBlockType1-NB transmission is assumed as a NB-IoT DL subframe if the UE monitors a NPDCCH UE-specific search space or decodes NPDSCH transmission scheduled by NPDCCH in the UE-specific search space.

For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter multi-TB-Unicast-config, there shall be a maximum of 2 downlink HARQ processes.

< Unchanged parts are omitted>

## 16.5	Narrowband physical uplink shared channel related procedures

For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter multi-TB-Unicast-config, there shall be a maximum of 2 uplink HARQ processes.

For a NB-IoT UE and NPUSCH transmission using preconfigured uplink resource, there shall be 1 uplink HARQ process.

A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows

-	If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured

-	for NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space including NPUSCH format 1 transmission without a corresponding NPDCCH

-	if the Resource reservation field in the DCI is set to 0, then the subframe is assumed as a NB-IoT UL subframe

-	else if the Resource reservation field in the DCI is set to 1, then the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols are reserved in the subframe).

-	for NPUSCH format 2 transmission

-	the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols are reserved in the subframe).

-	In all other cases,

-	for TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers

-	for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe.

### 16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14] , while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a NPDCCH with DCI format N0.

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-	n+k0 DL subframe for FDD,

-	k0 NB-IoT UL subframes following the end of n+8 subframe for TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and

-	, where the value of is determined by the repetition number field in the corresponding DCI (see Subclause 16.5.1.1), the value of is determined by the resource assignment field in the corresponding DCI (see Subclause 16.5.1.1), the value of  is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the  allocated number of subcarriers (as determined in Subclause 16.5.1.1) in the corresponding DCI, and the value of is determined by the number of scheduled TB field, if present, in the corresponding DCI,  otherwise

-	n0 is the first NB-IoT UL slot starting after the end of subframe n+k0 for FDD

-	n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8 subframe for TDD

-	value of k0 is determined by the scheduling delay field () in the corresponding DCI according to Table 16.5.1-1 for FDD and Table 16.5.1-1A for TDD

-	For ,

-	if the UE is configured with higher layer parameter multi-TB-UL-Unicast-Interleaving-config, and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and  where  for ,  otherwise.

-	NB-IoT UL slots  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT UL slots  with  are associated with TBr+1 ,

Table 16.5.1-1: for DCI format N0 for FDD.

| 0 | 8 |
| --- | --- |
| 1 | 16 |
| 2 | 32 |
| 3 | 64 |

Table 16.5.1-1A: for DCI format N0 for TDD.

| 0 | 0 |
| --- | --- |
| 1 | 8 |
| 2 | 16 |
| 3 | 32 |

If a NPUSCH transmission without a corresponding NPDCCH collides partially or fully with a NPDSCH transmission, the NPUSCH transmission is dropped.

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the C-RNTI, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-2 and transmit a corresponding NPUSCH. The scrambling initialization of this NPUSCH corresponding to these NPDCCHs and the NPUSCH retransmission for the same transport block is by C-RNTI.

Table 16.5.1-2: NPDCCH and NPUSCH configured by C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by C-RNTI |

If a UE is configured to receive random access procedures initiated by "PDCCH orders", the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-3.

Table 16.5.1-3: NPDCCH configured as "PDCCH order" to initiate random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format N1 | UE specific by C-RNTI |

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the Temporary C-RNTI regardless of whether UE is configured or not configured to decode NPDCCH with the CRC scrambled by the C-RNTI during random access procedure, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-4 and transmit the corresponding NPUSCH. The scrambling initialization of NPUSCH corresponding to these NPDCCHs is by Temporary C-RNTI.

If a Temporary C-RNTI is set by higher layers, the scrambling initialization of NPUSCH corresponding to the Narrowband Random Access Response Grant in Subclause 16.3.3 and any NPUSCH retransmission(s) for the same transport block is by Temporary C-RNTI. Otherwise, the scrambling initialization of NPUSCH corresponding to the Narrowband Random Access Response Grant in Subclause 16.3.3 and any NPUSCH retransmission(s) for the same transport block is by C-RNTI.

If a UE is also configured by higher layers to decode NPDCCH with CRC scrambled by the C-RNTI during random access procedure, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-4 and transmit the corresponding NPUSCH. The scrambling initialization of NPUSCH corresponding to these NPDCCH is by C-RNTI.

Table 16.5.1-4: NPDCCH and NPUSCH configured by Temporary C-RNTI and/or C-RNTI during random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | Type-2 Common |

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the SPS C-RNTI, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-5 and transmit a corresponding NPUSCH if a transport block corresponding to the HARQ process of the NPUSCH transmission is generated as described in [8]. 
The scrambling initialization of this NPUSCH corresponding to these NPDCCHs and NPUSCH retransmission for the same transport block is by SPS C-RNTI. The scrambling initialization of initial transmission of this NPUSCH without a corresponding NPDCCH and the NPUSCH retransmission for the same transport block is by SPS C-RNTI.

Table 16.5.1-5: NPDCCH and NPUSCH configured by SPS C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by C-RNTI |

A UE may transmit NPUSCH on preconfigured uplink resources as configured by higher layers. The scrambling initialization of NPUSCH transmission using preconfigured uplink resource is by PUR C-RNTI.

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the PUR C-RNTI, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-6 and in case the indication in the DCI corresponds to the retransmission of a transport block transmitted using preconfigured uplink resource, transmit a corresponding NPUSCH. The scrambling initialization of this NPUSCH corresponding to these NPDCCHs and the NPUSCH retransmission for the same transport block is by PUR C-RNTI.

Table 16.5.1-6: NPDCCH and NPUSCH configured by PUR C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by PUR C-RNTI |

< Unchanged parts are omitted>

## 16.6	Narrowband physical downlink control channel related procedures

A UE shall monitor a set of NPDCCH candidates (described in Subclause 10.2.5.1 of [3]) as configured by higher layer signalling for control information, where monitoring implies attempting to decode each of the NPDCCHs in the set according to all the monitored DCI formats.

The set of NPDCCH candidates to monitor are defined in terms of NPDCCH search spaces.

The UE shall monitor one or more of the following search spaces

-	a Type1-NPDCCH common search space,

-	a Type1A-NPDCCH common search space,

-	a Type2-NPDCCH common search space,

-	a Type2A-NPDCCH common search space, and

-	a NPDCCH UE-specific search space.

A UE is not required to simultaneously monitor a NPDCCH UE-specific search space and a Type-1-NPDCCH common search space.

A UE is not required to simultaneously monitor a NPDCCH UE-specific search space and a Type2-NPDCCH common search space.

A UE is not required to simultaneously monitor a Type-1-NPDCCH common search space and a Type2-NPDCCH common search space.

A UE is not required to monitor Type1A-NPDCCH common search space or Type2A-NPDCCH common search space in subframes in which the UE monitors a Type1-NPDCCH common search space or in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by P-RNTI

A UE is not required to monitor Type1A-NPDCCH common search space or Type2A-NPDCCH common search space in subframes in which the UE monitors a Type2-NPDCCH common search space or in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by C-RNTI or Temporary C-RNTI.

A UE is not required to monitor Type2A-NPDCCH common search space in the same subframe in which it monitors Type1A-NPDCCH common search space.

UE is not required to monitor Type1A-NPDCCH common search space in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by SC-RNTI.

UE is not required to monitor Type2A-NPDCCH common search space in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by G-RNTI or SC-RNTI.

Until UE receives higher layer configuration of NPDCCH UE-specific search space, the UE monitors NPDCCH according to the same configuration of NPDCCH search space as that for NPDCCH scheduling Msg4.

A UE is not required to monitor Type1-NPDCCH common search space if the set of subframes comprising the NPDCCH candidates include any subframes in which the UE has initiated an NPUSCH transmission using preconfigured uplink resource on a given serving cell.

An NPDCCH search space  at aggregation level  ( for TDD special subframe,  otherwise), and repetition level  is defined by a set of NPDCCH candidates where each candidate is repeated in a set of consecutive NB-IoT downlink subframes excluding subframes used for transmission of SI messages starting with subframe .

For NPDCCH UE-specific search space, the aggregation and repetition levels defining the search spaces and the corresponding NPDCCH candidates are listed in Table 16.6-1 by substituting the value of with the higher layer configured parameter npdcch-NumRepetitions or npdcch-NumRepetitions-PUR after the UE has initiated a NPUSCH transmission using preconfigured uplink resource.

For Type1-NPDCCH common search space and Type1A-NPDCCH common search space, the aggregation and repetition levels defining the search spaces are listed in Table 16.6-2 by substituting the value of

-	with the higher layer configured parameter npdcch-NumRepetitionPaging for Type1-NPDCCH common search space;

-	with the higher layer configured parameter npdcch-NumRepetitions-SC-MCCH for Type1A-NPDCCH common search space.

For Type2-NPDCCH common search space and Type2A-NPDCCH common search space, the aggregation and repetition levels defining the search spaces and the corresponding monitored NPDCCH candidates are listed in Table 16.6-3 by substituting the value of

-	with the higher layer configured parameter npdcch-NumRepetitions-RA for Type2-NPDCCH common search space;

-	with the higher layer configured parameter npdcch-NumRepetitions-SC-MTCH for Type2A-NPDCCH common search space.

The locations of starting subframe  are given by where is the th consecutive NB-IoT DL subframe from subframe , excluding subframes used for transmission of SI messages, and , and , and where

-	subframe  is a subframe satisfying the condition , where , T≥4.

-	for NPDCCH UE-specific search space,

-	is given by the higher layer parameter npdcch-StartSF-USS or npdcch-StartSF-USS-PUR after the UE has initiated a NPUSCH transmission using preconfigured uplink resource,

-	is given by the higher layer parameter npdcch-Offset-USS or npdcch-Offset-USS-PUR after the UE has initiated a NPUSCH transmission using preconfigured uplink resource,

-	for NPDCCH Type2-NPDCCH common search space,

-	is given by the higher layer parameter npdcch-StartSF-CSS-RA,

-	is given by the higher layer parameter npdcch-Offset-RA,

-	for NPDCCH Type2A-NPDCCH common search space,

-	is given by the higher layer parameter npdcch-startSF-SC-MTCH,

-	is given by the higher layer parameter npdcch-Offset-SC-MTCH,

For Type1-NPDCCH common search space,and is determined from locations of NB-IoT paging opportunity subframes.

For Type1A-NPDCCH common search space, and subframe  is a subframe satisfying the condition , where , T≥4 and

-	is given by the higher layer parameter npdcch-StartSF-SC-MCCH,

-	is given by the higher layer parameter npdcch-Offset-SC-MCCH.

If the UE is configured by high layers with a NB-IoT carrier for monitoring of NPDCCH UE-specific search space,

-	the UE shall monitor the NPDCCH UE-specific search space on the higher layer configured NB-IoT carrier,

-	the UE is not expected to receive NPSS, NSSS, NPBCH on the higher layer configured NB-IoT carrier.

otherwise,

-	the UE shall monitor the NPDCCH UE-specific search space on the same NB-IoT carrier on which NPSS/NSSS/NPBCH are detected.

If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4 with duration given by higher layer parameter pur-SS-window-duration. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR C-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration.

Table 16.6-1: NPDCCH UE- specific search space candidates

|  |  | DCI subframe repetition number | NCCE indices of monitored NPDCCH candidates |  |
| --- | --- | --- | --- | --- |
|  |  |  | L'=1 | L'=2 |
| 1 | 1 | 00 | {0},{1} | {0,1} |
| 2 | 1 | 00 | {0},{1} | {0,1} |
|  | 2 | 01 | - | {0,1} |
| 4 | 1 | 00 | - | {0,1} |
|  | 2 | 01 | - | {0,1} |
|  | 4 | 10 | - | {0,1} |
| >=8 |  | 00 | - | {0,1} |
|  |  | 01 | - | {0,1} |
|  |  | 10 | - | {0,1} |
|  |  | 11 | - | {0,1} |
| Note 1: {x}, {y} denotes NPDCCH Format 0 candidate with NCCE index 'x', and NPDCCH Format 0 candidate with NCCE index 'y' are monitored Note 2: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs 'x' and 'y' is monitored. |  |  |  |  |

Table 16.6-2: Type 1/Type 1A - NPDCCH common search space candidates

|  |  |  |  |  |  |  |  |  | NCCE indices of monitored NPDCCH candidates |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | L'=1 | L'=2 |
| 1 | 1 | - | - | - | - | - | - | - | - | {0,1} |
| 2 | 1 | 2 | - | - | - | - | - | - | - | {0,1} |
| 4 | 1 | 2 | 4 | - | - | - | - | - | - | {0,1} |
| 8 | 1 | 2 | 4 | 8 | - | - | - | - | - | {0,1} |
| 16 | 1 | 2 | 4 | 8 | 16 | - | - | - | -- | {0,1} |
| 32 | 1 | 2 | 4 | 8 | 16 | 32 | - | - | - | {0,1} |
| 64 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | - | - | {0,1} |
| 128 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | - | {0,1} |
| 256 | 1 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | - | {0,1} |
| 512 | 1 | 4 | 16 | 32 | 64 | 128 | 256 | 512 | - | {0,1} |
| 1024 | 1 | 8 | 32 | 64 | 128 | 256 | 512 | 1024 | - | {0,1} |
| 2048 | 1 | 8 | 64 | 128 | 256 | 512 | 1024 | 2048 | - | {0,1} |
| DCI subframe repetition number | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |  |  |
| Note 1: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs 'x' and 'y' is monitored. |  |  |  |  |  |  |  |  |  |  |

Table 16.6-3: Type 2/Type 2A - NPDCCH common search space candidates

|  |  | DCI subframe repetition number | NCCE indices of monitored NPDCCH candidates |  |
| --- | --- | --- | --- | --- |
|  |  |  | L'=1 | L'=2 |
| 1 | 1 | 00 | - | {0,1} |
| 2 | 1 | 00 | - | {0,1} |
|  | 2 | 01 | - | {0,1} |
| 4 | 1 | 00 | - | {0,1} |
|  | 2 | 01 | - | {0,1} |
|  | 4 | 10 | - | {0,1} |
| >=8 |  | 00 | - | {0,1} |
|  |  | 01 | - | {0,1} |
|  |  | 10 | - | {0,1} |
|  |  | 11 | - | {0,1} |
| Note 1: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs 'x' and 'y' is monitored. |  |  |  |  |

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or multi-TB-Unicast-config and if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k,

-	if the corresponding NPDCCH with DCI format N0 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the number of scheduled TB field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1, otherwise the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1; and

- the UE does not expect to receive a DCI Format N0 before subframe n+k-2 for which the corresponding NPUSCH format 1 transmission ends later than subframe n+k+255 if the corresponding NPDCCH with DCI format N0 schedules one transport block.

-	for TDD, and if the corresponding NPUSCH format1 transmission ends in subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

otherwise

-	if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission ends in n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k.

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or multi-TB-Unicast-config

-	and if the NB-IoT UE detects NPDCCH with DCI Format N1 or N2 ending in subframe n, and if a NPDSCH transmission starts from n+k,

-	if the corresponding NPDCCH with DCI format N1 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the number of scheduled TB field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1;

-	otherwise, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1;

otherwise

-	if the NB-IoT UE detects NPDCCH with DCI Format N1 or N2 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

If a NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, and

-	for FDD, if the corresponding NPUSCH format 2 transmission starts from subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

-	for TDD, if the corresponding NPUSCH format 2 transmission ends in subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and

-	for FDD, if the corresponding NPRACH transmission starts from subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n’+1 to subframe n+3.

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5

otherwise

-	the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap.

An NB-IoT UE is not required to monitor NPDCCH candidates of a Type2A-NPDCCH common search space during the scheduling gap or the processing gap.

For an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, if the transmission of a narrowband random access preamble for SR ends on subframe n,

-	in case of frame structure type 1 with NPRACH format 0 and 1 when the number of NPRACH repetitions is greater than or equal to 64, or NPRACH format 2 when the number of NPRACH repetitions is greater than or equal to 16, the UE is not required to monitor NPDCCH UE-specific search space from subframe n to subframe n+40,

-	otherwise, the UE is not required to monitor NPDCCH UE-specific search space from subframe n to subframe n+3.

< Unchanged parts are omitted>