3GPP TSG RAN WG1 #100bis			                                   					       												        R1-2001865

e-Meeting, April 20th – 30th, 2020

Agenda Item:	6.2.5.4

Source:	Moderator (AT&T)

Title:	Summary on UE features for LTE-based 5G terrestrial broadcast

Document for:	Discussion/Decision

# Introduction

This document was drafted by the moderator of the agenda item under the direction of the RAN1 Chairman following the below guidance whose purpose it serves:

- April 13-17: preparation phase - April 13th – 14th: FLs to prepare summary - April 15th – 17th: FLs to lead the discussion identifying the set of email threads - Note: PLEASE KEEP THE EMAIL DISCUSSION SCOPE PER EMAIL THREAD REASONABLE! - Too much scope will force Chairman/Vice Chairman to step in to do the necessary cut down using the best judgement à if so, no complain please.

All Sections except Section 3 were exclusively prepared by the moderator of the agenda item. Specifically, Section 2 is the moderator’s summary of contributions submitted to RAN1 #100bis-e in this agenda item according to the Chairman’s guidance. During the preparation phase, companies were given the opportunity to revise their views in the moderator’s summary in Section 2 using revision marks as shown below, if any. Section 3 was jointly drafted by the moderator and contributing companies during the preparation phase of RAN1 #100bis-e whereby companies present their views on the moderator’s proposals according to their Chairman’s guidance above in the respective tables. After conclusion of the preparation phase, the moderator submitted the final document as input to RAN1 #100bis-e with recommendations captured in Section 4.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary on UE features for LTE-based 5G terrestrial broadcast

The following table represents the version of the LTE UE feature list for LTE-based 5G terrestrial broadcast agreed by RAN1 as baseline for RAN1 #100bis-e [1][2]. The moderator notes that no papers were submitted to RAN1 #100bis-3 in agenda item 6.2.5.4.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. LTE_TERR_BCAST | 4-1 | 2.5kHz PMCH | 1. Support of 2.5kHz SCS for PMCH | Support of fembmsMixedCell or fembmsDedicatedCell | Yes | N/A | UE cannot receive services transmitted with 2.5kHz numerology | Per band | No | N/A | Need to add capability for support, plus scaling factor, like mbms-ScalingFactor1dot25. The values for the scaling factor are {2, 4, 6, 8} | Optional with capability signalling |
|  | 4-2 | 0.370kHz PMCH with T_d = 2 | 1. Support of 0.370kHz SCS for PMCH with stagger length of 2 slots | Support of fembmsMixedCell or fembmsDedicatedCell | Yes | N/A | UE cannot receive services transmitted with 0.370kHz numerology and reference signal with stagger length of 2 slots | Per band | No | N/A | Need to add capability for support, plus scaling factor, like mbms-ScalingFactor1dot25. The values for the scaling factor are {12, 24, FFS, FFS} (the scaling factor is common for T_d=2 and T_d=4) | Optional with capability signalling |
|  | 4-3 | 0.370kHz PMCH with T_d = 4 | 1. Support of 0.370kHz SCS for PMCH with stagger length of 4 slots | Support of fembmsMixedCell or fembmsDedicatedCell | Yes | N/A | UE cannot receive services transmitted with 0.370kHz numerology and reference signal with stagger length of 4 slots | Per band | No | N/A | Need to add capability for support, plus scaling factor, like mbms-ScalingFactor1dot25. The values for the scaling factor are {12, 24, FFS, FFS} (the scaling factor is common for T_d=2 and T_d=4) | Optional with capability signalling |
|  | 4-4 | PDCCH AL16 | - Support of PDCCH AL16 for CAS in MBMS-dedicated cell. | Support of fembmsDedicatedCell | No | N/A | Reduced coverage for PDCCH | Per UE | No | N/A |  | Optional without capability signalling |
|  | 4-5 | CFI indication in MIB-MBMS | - Support of semi-static CFI indication in MIB | Support of fembmsDedicatedCell | No | N/A | Reduced coverage for PDCCH due to PCFICH | Per UE | No | N/A |  | Optional without capability signalling |
|  | 4-6 | PBCH repetition | - Support of PBCH repetition in CAS | Support of fembmsDedicatedCell | No | N/A | Reduced coverage for PBCH | Per UE | No | N/A |  | Optional without capability signalling |

# Issues for discussion during the preparation phase

Despite the lack of contributions in this agenda item, the moderator notes that there are still four FFS points remaining in the LTE UE feature list for LTE-based 5G terrestrial broadcast. The moderator would thus like to propose to resolve these during RAN1 #100bis-e and companies are invited to provide their views on the moderator’s proposal and specific values for the FFS points in the table below.

| 4-2 | 0.370kHz PMCH with T_d = 2 | 1. Support of 0.370kHz SCS for PMCH with stagger length of 2 slots | Support of fembmsMixedCell or fembmsDedicatedCell | Yes | N/A | UE cannot receive services transmitted with 0.370kHz numerology and reference signal with stagger length of 2 slots | Per band | No | N/A | Need to add capability for support, plus scaling factor, like mbms-ScalingFactor1dot25. The values for the scaling factor are {12, 24, FFS, FFS} (the scaling factor is common for T_d=2 and T_d=4) | Optional with capability signalling | 4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-3 | 0.370kHz PMCH with T_d = 4 | 1. Support of 0.370kHz SCS for PMCH with stagger length of 4 slots | Support of fembmsMixedCell or fembmsDedicatedCell | Yes | N/A | UE cannot receive services transmitted with 0.370kHz numerology and reference signal with stagger length of 4 slots | Per band | No | N/A | Need to add capability for support, plus scaling factor, like mbms-ScalingFactor1dot25. The values for the scaling factor are {12, 24, FFS, FFS} (the scaling factor is common for T_d=2 and T_d=4) | Optional with capability signalling | 4-3 |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Despite the lack of contributions in this agenda item, the moderator would still like to give companies an opportunity to comment on any other issue that could potentially be discussed during RAN1 #100bis-e as it relates to LTE UE features for LTE-based 5G terrestrial broadcast.

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Conclusion

[To be completed towards the end of the preparation phase]

# References

- R1-2001485, RAN1 UE features list for Rel-16 LTE after RAN1#100-E, AT&T & NTT DOCOMO

- R1-2001486, LS on Rel-16 RAN1 UE features lists for LTE, RAN1 (NTT DOCOMO)