3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.3

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-IAB-01]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-IAB-01] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-IAB-01] Email discussion/approval of proposal 1 in R1-2001866 till 4/24 – Ralf (ATT)

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-IAB-01] “Email discussion/approval of Proposal 1 in R1-2001866” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-IAB-01]

The following is the proposal in Error: Reference source not found for approval in this email discussion:

FL Proposal 1 (high priority): Inform RAN3 of the following IAB-DU features and add references(s) in 38.306 in order to indicate any links between IAB-MT and IAB-DU features (e.g. if they should be supported jointly)

- Inter-IAB-node discovery and measurements: SSB transmission Configuration: Support up to 4 STCs configured for an IAB node DU per cell per frequency location, including IAB-specific SSB transmission periodicities

- Extension of RACH occasions and periodicities for backhaul RACH resources: Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors

- IAB node non-TDM multiplexing capability: TDM not required between IAB-MT and IAB-DU functions

- UL-Flexible-DL slot formats: Support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU resources

- Dynamic indication of soft resource availability: Support DCI Format 2_5 based indication of soft resource availability to an IAB node

- FFS: Case 1 OTA timing alignment: Support IAB-DU DL timing alignment with parent node based on

Companies are asked to provide their views and comments in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei | - We would like to understand the motivation of sending the DU feature list to RAN3. Particularly, what do RAN1 expect RAN3 to do with the list of DU features? Given that the typical way to handle DU capabilities is via OAM, it is not clear what RAN3 will do it differently for IAB since it is still a network node. We suggest at least adding a note: how to handle IAB-DU features should be up to RAN3 decision. - We are not sure about adding reference(s) in 38.306 since we have not yet agreed to capture the IAB-MT feature in 38.306. RAN2 is discussing how to capture the IAB-MT features into the specification. The formality issue does not seem to be urgent at the moment. We can focus on the RAN1 related feature lists first. |

…

# Conclusions

…

# References

- R1-2001866, Summary on UE features for IAB, Moderator (AT&T)